USER'S  MANUAL

RHINO 9

Pentium PCI Local Bus Motherboard

Copyright Notice

Copyright 1996 Ocean Office Automation Ltd.

All rights reserved.

Ocean Office Automation Ltd.

5th Floor, Kader Industrial Building,

22 Kai Cheung Road, Kowloon Bay,

Kowloon, Hong Kong.

All of the information contained in this manual is copyrighted and all rights reserved.  No part of this document, in whole or in part, may be reproduced or copied in any form without prior consent in writing from Ocean Office Automation Ltd..

___________________________________________________________________

Limitations of  Liability

While the information in this manual has been carefully reviewed and is believed to be accurate, Ocean Office Automation Ltd. (Ocean) assumes no responsibility in the event that any inaccuracies are found.  In no event shall Ocean be held liable for any loss or expenses whether directly or indirectly caused by the support materials provided with this product.  It is further acknowledged that Ocean is under no obligation to update the manual or to notify purchasers of any forthcoming updates.

___________________________________________________________________

Trademarks

Intel i486, Pentium and Intel are registered trademarks of Intel Corporation.

Octek is a registered trademark of  Ocean Office Automation Ltd.

XT, AT, PS/2, OS/2, & IBM are registered trademarks of International Business Machines Corporation.

All other brand and product names mentioned in this manual are trademarks or copyrights of their respective holders.

___________________________________________________________________

REVISION 1.1  July 1996

Table of Contents

Chapter 1:  System Overview

1.1      General Specification Overview	1

Chapter 2: Installation and Upgrade

2.1      CPU Installation	5

2.2      Fast Page mode / EDO DRAM Installation	6

2.3      Control of System Speed	8

2.4      Fan voltage	8

2.5      Fan Connector	8

2.6      Reset CMOS	8

Appendix-A: Connectors Pinout

A.1      Turbo LED Connector (P1)	9

A.2      Speaker Connector (P2)	9

A.3      HD LED Connector (P3)	9

A.4      Reset Connector (P4)	9

A.5      Keylock Connector (P5)	10

A.6      Fan Connector (P6)	10

A.7      External Battery Connector (P9)	10

A.8      Sleep Switch (P10)	10

A.9      USB connector (P24)	10

A.10    Power Connector (P15,P18)	11

A.11    PS/2 Mouse Connector (P21) 	11

A.12    Keyboard Connector (KB1) 	11

Appendix-B: Hardware Settings

B.1      System Component Map	12

B.2      Layout of RHINO 9 Main Board	13

B.3      Jumper Settings	13

B.3.1      CPU related settings	14

B.3.2      External cache (L2 cache) setting	16

B.3.3      Peripheral setup	17

B.3.4      Miscellaneous	17

# CHAPTER 1

# SYSTEM OVERVIEW

OCTEK RHINO 9 is a powerful PC machine offering unparalleled performance. The advanced external cache system implemented meets the demand of the most memory-intensive applications today. The support of Synchronous Cache RAM (pipelined burst SRAM) and EDO DRAM results in better performance when compared with traditional asynchronous SRAM and Fast Page Mode DRAM. With the Pentium processor and a high bandwidth 32-bit PCI expansion bus, the I/O  bottleneck that plagues most PC systems is now removed.

All of the I/O is integrated inside the mainboard to further facilitate system installation. The built-in IDE can support up to 4 fast Enhanced IDE devices whereas it can  also support up to two  floppy drives. The  mainboard also includes two serial ports and one parallel port as the basic configuration for end user.  All that is needed is just a VGA card plugged into a PCI or ISA slot to complete the whole system.

## 1.1	General Specifications Overview

Processor:

w Processor Type			Intel  Pentium CPU including

P54C,  P54CQS, P54CS, P55C, 					AMD K5 and Cyrix 6x86 CPU.

w External CPU clock		50/60/66 Mhz

Chipset:

w Motherboard  chipset		Intel 82430HX  PCI/ISA chipset

w Super I/O chipset		665/669 super I/O chipset

Cache Architecture:

w Internal Cache 	 		8KB data cache

8KB code cache

w External Cache			Total 256/512KB Synchronous Pipelined 				Burst SRAM

Memory Subsystem:

w DRAM SIMM sockets		6 x 72 pin  4MB / 8MB / 16MB / 32MB / 				64MB DRAM modules

w Max. Memory Size		384MB

w DRAM Type			Fast Page Mode or EDO DRAM with 				or without parity bit.

w Enhancement			Mix of Fast Page Mode or EDO DRAM 				supported

Input/Output Subsystem

w PCI bus slots		         	3 x 32-bit PCI Bus slots (3 masters)

w ISA bus slots			3 x 16-bit ISA slots

w shared bus slots		1 x 32 bit PCI bus slot (master) OR 1 				x 16-bit ISA slot

w I/O bus speed			Up to 33MHz (PCI bus)

Integrated IDE, Super I/O Subsystem

w IDE support			Chipset built-in PCI IDE support up to 				4 IDE Drives

w On board I/O 			One Floppy Port supporting 2 floppy 				drives of 360K / 720K / 1.44M/ 2.88M 				capacity.

Two serial ports (16550 Fast UART 				compatibles)

One parallel Port (Standard, ECP, 					EPP support)

PS/2 Mouse

w PS/2 Mouse			Supports PS/2 Mouse through a 1x4 				header

Power Management

w Green functions			Support various Power Management 				schemes

Sleep Switch for power saving

BIOS Subsystem

w BIOS Shadowing		Shadow RAM for System and Vide	BIOS

w BIOS Features			Built-in setup, Power-on self test, Drive				table optimization, User-definable drive 				types, Password protection, Shadowing 				options

Plug & Play / BIOS Update

w Plug & Play BIOS		Support Plug & Play for easy  installation

w Flash EEPROM			Use Flash EEPROM to allow easy BIOS 				update

USB Devices

w USB Devices			Interfaced with both host and hub

control functions

2 programmable USB ports

System Support Functions

w System functions		7 DMA channels, 16 level interrupts, 				Programmable timers

w Support functions		Fast A20 gate and Fast Reset

w Clock				Enhanced real time clock/calendar 					with battery back-up

Other Features

w Power good			On board power good signal generation

w 3.3V supply			On board 3.3V supply to eliminate the 				need for special power supply for 3.3Volt 				-3.6Volt component e.g. CPU, SRAM.

Maximum rating : 30 W.

w 2.8V supply			On board 2.8 V supply to directly support 				Intel Pentium P55C CPU,

Maximum rating : 30 W.

w Switches			Reset, Keylock  switches, Sleep Switch.

w Size				8.5" (W) x 13" (L)

# CHAPTER 2

# INSTALLATION AND UPGRADE

## 2.1	CPU Installation

The CPU is composed of pins that can easily be bent during installation, causing permanent damage to the processor.  It is therefore very important that you make sure the pins are straight before installing the CPU onto the SPGA socket located on RHINO 9 (refer to layout for exact location).  To properly align the CPU  with the socket, align pin 1 of the CPU ( with a notch at the corner) with pin 1 of the CPU socket as demonstrated below.

### Figure  1.   Socket 7 for Pentium CPU

## 2.2	Fast Page mode / EDO DRAM Installation

There are six SIMM sockets located on the RHINO 9 motherboard, marked SM1, SM2, SM3, SM4, SM5 and SM6. The SM1, SM2, SM3, SM4, SM5 and SM6 are counted starting from left to right consecutively. Start to install the SIMM modules ( IN PAIRS) from either Bank 1 (SM5/SM6), Bank 2 (SM3/SM4) or Bank 3 (SM1/SM2) depending on how your memory is configured, you may not need to use all the memory banks. Either X32 or X36 of  72 pins SIMM can be installed.

Bank 3  Bank 2  Bank 1

1  1  1  1  1 1 11  11 11 1

Figure 2     SIMM  Sockets Location

Below is some of the possible configuration table for RHINO 9.

| BANK 3 |  | BANK 2 |  | BANK 1 |  | TOTAL |
| --- | --- | --- | --- | --- | --- | --- |
| SM1 | SM2 | SM3 | SM4 | SM5 | SM6 |  |
| -------- | -------- | -------- | -------- | 512Kx32 | 512Kx32 | 4MB |
| -------- | -------- | -------- | -------- | 1Mx32 | 1Mx32 | 8MB |
| -------- | -------- | -------- | -------- | 2Mx32 | 2Mx32 | 16MB |
| -------- | -------- | -------- | -------- | 4Mx32 | 4Mx32 | 32MB |
| -------- | -------- | -------- | -------- | 8Mx32 | 8Mx32 | 64MB |
| -------- | -------- | -------- | -------- | 16Mx32 | 16Mx32 | 128MB |
| -------- | -------- | 512Kx32 | 512Kx32 | 512Kx32 | 512Kx32 | 8MB |
| -------- | -------- | 1Mx32 | 1Mx32 | 512Kx32 | 512Kx32 | 12MB |
| -------- | -------- | 2Mx32 | 2Mx32 | 512Kx32 | 512Kx32 | 20MB |
| -------- | -------- | 4Mx32 | 4Mx32 | 512Kx32 | 512Kx32 | 36MB |
| -------- | -------- | 8Mx32 | 8Mx32 | 512Kx32 | 512Kx32 | 68MB |
| -------- | -------- | 16Mx32 | 16Mx32 | 512Kx32 | 512Kx32 | 132MB |
| -------- | -------- | 512Kx32 | 512Kx32 | 1Mx32 | 1Mx32 | 12MB |
| -------- | -------- | 1Mx32 | 1Mx32 | 1Mx32 | 1Mx32 | 16MB |
| -------- | -------- | 2Mx32 | 2Mx32 | 1Mx32 | 1Mx32 | 24MB |
| -------- | -------- | 4Mx32 | 4Mx32 | 1Mx32 | 1Mx32 | 40MB |
| -------- | -------- | 8Mx32 | 8Mx32 | 1Mx32 | 1Mx32 | 72MB |
| -------- | -------- | 16Mx32 | 16Mx32 | 1Mx32 | 1Mx32 | 136MB |

..			..			..

| -------- | -------- | 512Kx32 | 512Kx32 | 16Mx32 | 16Mx32 | 132MB |
| --- | --- | --- | --- | --- | --- | --- |
| -------- | -------- | 1Mx32 | 1Mx32 | 16Mx32 | 16Mx32 | 136MB |
| -------- | -------- | 2Mx32 | 2Mx32 | 16Mx32 | 16Mx32 | 144MB |
| -------- | -------- | 4Mx32 | 4Mx32 | 16Mx32 | 16Mx32 | 160MB |
| -------- | -------- | 8Mx32 | 8Mx32 | 16Mx32 | 16Mx32 | 192MB |
| -------- | -------- | 16Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 256MB |

..			..			..

| 512Kx32 | 512Kx32 | 512Kx32 | 512Kx32 | 512Kx32 | 512Kx32 | 12MB |
| --- | --- | --- | --- | --- | --- | --- |
| 1Mx32 | 1Mx32 | 512Kx32 | 512Kx32 | 512Kx32 | 512Kx32 | 16MB |
| 2Mx32 | 2Mx32 | 512Kx32 | 512Kx32 | 512Kx32 | 512Kx32 | 24MB |
| 4Mx32 | 4Mx32 | 512Kx32 | 512Kx32 | 512Kx32 | 512Kx32 | 40MB |
| 8Mx32 | 8Mx32 | 512Kx32 | 512Kx32 | 512Kx32 | 512Kx32 | 72MB |
| 16Mx32 | 16Mx32 | 512Kx32 | 512KX32 | 512Kx32 | 512Kx32 | 136MB |

..			..			..

| 512Kx32 | 512Kx32 | 16Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 260MB |
| --- | --- | --- | --- | --- | --- | --- |
| 1Mx32 | 1Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 264MB |
| 2Mx32 | 2Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 272MB |
| 4Mx32 | 4Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 288MB |
| 8Mx32 | 8Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 320MB |
| 16Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 384MB |

Table 1      Memory Configuration Table

## 2.3	Control of System Speed

System speed can be controlled by keyboard. To change the speed by keyboard, use the minus sign (-) and the plus sign (+).  Press <control> + <alt> + <“-”> for slow speed and  <control> + <alt> + <“+”> for fast speed.

## 2.4	Fan voltage

There are two different voltages available for CPU fans; +5 volts and + 12 volts. Select a proper setting for jumper JP1 before you turn on the computer.

## 2.5	Fan Connector

This Fan connector (P6) allows the CPU fan to draw current directly from the motherboard, without having to rely on a separate power connector.

## 2.6	Reset CMOS

If the setting of the system setup is done improperly, it may make the system malfunction. If this happens, turn off the power and set jumper JP21 to 2-3 to clear the internal CMOS status register. Wait at least 5 seconds to ensure that the CMOS content has been completely cleared.

Next, set the jumper JP21 back to 1-2 and turn on the power.  The BIOS will find the CMOS status register is  reset and will regard the setup information invalid, so it will prompt you to correct the information.

# APPENDIX-A

# CONNECTORS  PINOUT

### A.1	Turbo LED Connector (P1)

| Pin | Signal Name |
| --- | --- |
| 1 | Pull_Up_150 |
| 2 | LED_Turbo- |

### A.2	Speaker Connector (P2)

| Pin | Signal Name |
| --- | --- |
| 1 | Speaker Data_Out |
| 2 | N.C. |
| 3 | Ground |
| 4 | +5Vdc |

### A.3	HD LED Connector (P3)

| Pin | Signal Name |
| --- | --- |
| 1 | Pull_Up_330 |
| 2 | HD_LED- |
| 3 | HD_LED- |
| 4 | Pull_Up_330 |

### A.4	Reset Connector (P4)

| Pin | Signal Name |
| --- | --- |
| 1 | Reset |
| 2 | Ground |

## A.5	Keylock Connector (P5)

| Pin | Signal Name |
| --- | --- |
| 1 | +5Vdc |
| 2 | Mechanical Key |
| 3 | Ground |
| 4 | Keyboard Inhibit |
| 5 | Ground |

### A.6	Fan Connector (P6)

| Pin | Signal Name |
| --- | --- |
| 1 | +5/+12dc Power |
| 2 | Ground |

### A.7	External Battery Connector (P9)

| Pin | Signal Name |
| --- | --- |
| 1 | +3.6Vdc |
| 2 | N.C. |
| 3 | Ground |
| 4 | Ground |

### A.8	Sleep  Switch (P10)

| Pin | Signal Name |
| --- | --- |
| 1 | EPMI |
| 2 | Ground |

### A.9	USB Connector (P24)

| Signal Name | Pin | Pin | Signal Name |
| --- | --- | --- | --- |
| VCC | 6 | 1 | VCC |
| Port 1- | 7 | 2 | Port 0- |
| Port 1+ | 8 | 3 | Port 0+ |
| Ground | 9 | 4 | Ground |
| NC | 10 | 5 | NC |

## A.10	Power Connector (P15,P18)

| Pin | Signal Name |
| --- | --- |
| 1 | Power Good |
| 2 | +5Vdc |
| 3 | +12Vdc |
| 4 | -12V dc |
| 5 | Ground |
| 6 | Ground |
| 7 | Ground |
| 8 | Ground |
| 9 | -5Vdc |
| 10 | +5Vdc |
| 11 | +5Vdc |
| 12 | +5Vdc |

### A.11	PS/2 Mouse Connector (P21)

| Pin | Signal Name |
| --- | --- |
| 1 | +5V dc |
| 2 | GND |
| 3 | MDATA |
| 4 | MCLK |

### A.12	Keyboard Connector (KB1)

| Pin | Signal Name |
| --- | --- |
| 1 | Keyboard clock |
| 2 | Keyboard data |
| 3 | Not used |
| 4 | Ground |
| 5 | VCC |

# APPENDIX-B

# HARDWARE  SETTINGS

#### B.1	System Component Map

| Jumper Connectors | Function |
| --- | --- |
| P1 | Turbo LED |
| P2 | Speaker |
| P3 | IDE LED Connector |
| P4 | Reset |
| P5 | Keylock |
| P6 | Fan Connector |
| P7 | Secondary IDE Connector |
| P8 | Primary IDE Connector |
| P9 | External Battery Connector |
| P10 | Sleep Connector |
| P12 | Printer Port Connector |
| P14 | Floppy Connector |
| P15 | Power Connector |
| P17 | Serial Port 1 |
| P18 | Power Connector |
| P20 | Serial Port 2 |
| P21 | PS/2 Mouse Connector |
| P24 | USB Connector |
| KB1 | Keyboard Connector |

#### B.2	Layout of RHINO 9 Main Board

## B.3	Jumper Settings

### All factory settings are marked by * in the following sections.

B.3.1	CPU related settings

CPU Voltage Core Selection

RHINO 9 has on board 2.8V and 3.3V regulators to directly support  Intel Pentium P55C CPU. The P55C will specially need a 2.8V CPU Core voltage in order to function properly. Others Pentium CPU such as P54C, P54CQS, P54CS, Cyrix 6x86 CPU and AMD K5-PR75 - PR100 will need a Core voltage of 3.3V. The voltage selection for Core voltage is done by JP9, JP10, JP11, JP15, JP17 and JP18 as follows :

| CPU Core voltage | JP9 | JP10 | JP11 | JP15 | JP17 | JP18 | CPU TYPE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.8V | 2-3 | 2-3 | 2-3 | 1-2 | 2-3 | 2-3 | Intel P55C CPU |
| 3.3V (3.3Volt -3.6Volt) | 1-2 | 1-2 | 1-2 | 2-3 | 1-2 | 1-2 | Intel P54C, P54CQS or Intel P54CS,P54CTB or Cyrix 6x86 CPU or AMD K5-PR75 - PR100 |

NOTE :  Be careful to select the appropriate Core voltage for  different CPU.  Improper Core voltage supplied to CPU may result in “PERMANENT DAMAGE” to CPU !

NOTE :  Be careful to select the appropriate Core voltage for  different CPU.  Improper Core voltage supplied to CPU may result in “PERMANENT DAMAGE” to CPU !

CPU CLOCK

| JP2 | JP3 | JP8 | JP25 | JP28 | JP29 | CPU Clock | CPU TYPE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-2 2-3 1-2 | 1-2 1-2 1-2 | 1-2 1-2 1-2 | 1-2 1-2 1-2 | 2-3 2-3 2-3 | 2-3 2-3 2-3 | 50MHz | Intel P54C-75 Cyrix 6x86-P120+ (100Mhz) AMD K5-PR75 (75Mhz) |
| 1-2 2-3 2-3 1-2 2-3 1-2 | 1-2 1-2 2-3 2-3 1-2 1-2 | 1-2 1-2 1-2 1-2 1-2 1-2 | 2-3 2-3 2-3 2-3 2-3 2-3 | 1-2 1-2 1-2 1-2 1-2 1-2 | 2-3 2-3 2-3 2-3 2-3 2-3 | 60MHz | Intel P54C-90 Intel P54C-120 Intel P54C-150 Intel P54C-180 Cyrix 6x86-P150+ (120Mhz) AMD K5-PR90 (90Mhz) |
| 1-2 2-3 2-3 1-2 2-3 1-2 | 1-2 1-2 2-3 2-3 1-2 1-2 | 2-3 2-3 2-3 2-3 2-3 2-3 | 2-3 2-3 2-3 2-3 2-3 2-3 | 2-3 2-3 2-3 2-3 2-3 2-3 | 1-2 1-2 1-2 1-2 1-2 1-2 | 66MHz | Intel P54C-100 Intel P54C-133 Intel P54C-166 Intel P54C-200 Cyrix 6x86-P166+ (133Mhz) AMD K5-PR100 (100Mhz) |

CPU Fan voltage select

|  | JP1 |
| --- | --- |
| +5V | 1-2 * |
| +12V | 2-3 |

B.3.2	External cache (L2 cache) setting  Synchronous Cache size

| Total Cache Size | JP4 | JP5 | JP6 | JP7 | JP12 |
| --- | --- | --- | --- | --- | --- |
| 0KB | 1-2 | 1-2 | 1-2 | 1-2 | 1-2 |
| 256KB | 2-3 | 1-2 | 1-2 | 1-2 | 1-2 |
| 512KB | 1-2 | 2-3 | 2-3 | 2-3 | 2-3 |

Synchronous SRAM device requirement

256K Configuration :

1. 256KB cache using on board 256K SRAM

| SRAM | location | type | speed | voltage |
| --- | --- | --- | --- | --- |
| tag RAM | U6,U7 (on board) | 8Kx8 / 32Kx8 | 15ns | 5V I/O |
| data RAM | U1,U4 (on board) | 2 pcs 32Kx32 pipelined burst SRAM | 7ns (Clock to o/p valid) | 3.3V I/O |

2. 256KB cache using 256K Cache Module w/o Tag RAM

| SRAM | location | type | speed | voltage |
| --- | --- | --- | --- | --- |
| tag RAM | U6,U7 (on board) | 8Kx8 /32Kx8 | 15ns | 5V I/O |
| data RAM | DM1 | 256K Cache Module w/o Tag | 7ns (Clock to o/p valid) | 3.3V I/O |

512K Configuration:

512KB cache using 256K Cache Module w/o Tag RAM  and 256K SRAM on board

| SRAM | location | type | speed | voltage |
| --- | --- | --- | --- | --- |
| tag RAM | U6,U7 | 32Kx8 | 15ns | 5V I/O |
| data RAM (bank 0) | U1,U4 (on board) | 2 ps 32Kx32 pipelined burst SRAM | 7ns (Clock to o/p valid) | 3.3V I/O |
| data RAM (bank 1) | DM1 | 256K Cache Module w/o Tag RAM | 7ns (Clock to o/p valid) | 3.3V I/O |

B.3.3	 Peripheral setup

PS/2 MOUSE support

|  | JP49 | JP43 | JP44 |
| --- | --- | --- | --- |
| Enabled | 1-2 * | 2-3 * | 2-3 * |
| Disabled | 2-3 | 1-2 | 1-2 |

B.3.4	Miscellaneous

Power Good Signal select

|  | JP42 |
| --- | --- |
| External Power Good | 1-2 * |
| On-board Power Good | 2-3 |

CMOS discharge

|  | JP21 |
| --- | --- |
| Preserve CMOS | 1-2 * |
| Clear CMOS | 2-3 |

Battery select

|  | JP23 |
| --- | --- |
| On-board Battery | 1-2 * |
| External Battery | 2-3 |

Reserved Jumpers

| JP16 | 1-2 |
| --- | --- |
| JP20 | 2-3 |
| JP35 | 1-2 |