USER'S  MANUAL

RHINO 9

Pentium PCI Local Bus Motherboard

Copyright Notice

Copyright 1996 Ocean Office Automation Ltd.

All rights reserved.

Ocean Office Automation Ltd.

5th Floor, Kader Industrial Building,

22 Kai Cheung Road, Kowloon Bay,

Kowloon, Hong Kong.

All of the information contained in this manual is copyrighted and all rights reserved.  No part of this document, in whole or in part, may be reproduced or copied in any form without prior consent in writing from Ocean Office Automation Ltd..

___________________________________________________________________

Limitations of  Liability

While the information in this manual has been carefully reviewed and is believed to be accurate, Ocean Office Automation Ltd. (Ocean) assumes no responsibility in the event that any inaccuracies are found.  In no event shall Ocean be held liable for any loss or expenses whether directly or indirectly caused by the support materials provided with this product.  It is further acknowledged that Ocean is under no obligation to update the manual or to notify purchasers of any forthcoming updates.

___________________________________________________________________

Trademarks

Intel i486, Pentium and Intel are registered trademarks of Intel Corporation.

Octek is a registered trademark of  Ocean Office Automation Ltd.

XT, AT, PS/2, OS/2, & IBM are registered trademarks of International Business Machines Corporation.

All other brand and product names mentioned in this manual are trademarks or copyrights of their respective holders.

___________________________________________________________________

REVISION 1.2  July 1996

Table of Contents

Chapter 1:  General Specification Overview

1.       General Specification Overview	1

Chapter 2: Installation and Upgrade

2.1      System Component Map	3

2.2      CPU Installation	3

2.3      Fast Page mode / EDO DRAM Installation	4

2.4      Control of System Speed	6

2.5      Fan voltage	6

2.6      Fan Connector	6

2.7      Reset CMOS	6

Appendix-A: Connectors Pinout

A.1      Turbo LED Connector (P1)	7

A.2      Speaker Connector (P2)	7

A.3      HD LED Connector (P3)	7

A.4      Reset Connector (P4)	7

A.5      Keylock Connector (P5)	7

A.6      Fan Connector (P6)	7

A.7      External Battery Connector (P9)	7

A.8      Sleep Switch (P10)	8

A.9      USB connector (P24)	8

A.10    Power Connector (P15,P18)	8

A.11    PS/2 Mouse Connector (P21) 	8

A.12    Keyboard Connector (KB1) 	8

Appendix-B: Hardware Settings

B.1      Layout of RHINO 9 Main Board	9

B.2      Jumper Settings	10

B.2.1      CPU related settings	10

B.2.2      External cache (L2 cache) setting	11

B.2.3      Peripheral setup	12

B.3.4      Miscellaneous	12

## CHAPTER 1          General Specifications Overview

Processor:

w Processor Type			Intel  Pentium CPU including

P54C,  P54CQS, P54CS, P55C, 					AMD K5 and Cyrix 6x86 CPU.

w External CPU clock		50/60/66 Mhz

Chipset:

w Motherboard  chipset		Intel 82430HX  PCI/ISA chipset

w Super I/O chipset			Advanced super I/O chipset

Cache Architecture:

w Internal Cache 	 		8KB / 16KB data cache

8KB / 16KB code cache

w External Cache			Total 256/512KB Synchronous Pipelined 				Burst SRAM

Memory Subsystem:

w DRAM SIMM sockets		6 x 72 pin  4MB / 8MB / 16MB / 32MB / 	64MB 				DRAM modules

w Max. Memory Size		384MB

w DRAM Type			Fast Page Mode or EDO DRAM with or					 without parity bit.

w Enhancement			Mix of Fast Page Mode or EDO DRAM 					supported

Input/Output Subsystem

w PCI bus slots		         	3 x 32-bit PCI Bus slots (3 masters)

w ISA bus slots			3 x 16-bit ISA slots

w shared bus slots			1 x 32 bit PCI bus slot (master) OR					1x 16-bit ISA slot

w I/O bus speed			Up to 33MHz (PCI bus)

Integrated IDE, Super I/O Subsystem

w IDE support			Chipset built-in PCI IDE support up to 					4 IDE Drives

w On board I/O 			One Floppy Port supporting 2 floppy drives				of 360K / 720K / 1.2M / 1.44M/ 2.88M 					capacity.

Two serial ports (16550 Fast UART 					compatibles)

One parallel Port (Standard, ECP, EPP 					support)

PS/2 Mouse

w PS/2 Mouse			Supports PS/2 Mouse through a 1x4 header

Power Management

w Green functions			Support various Power Management schemes

Sleep Switch for power saving

BIOS Subsystem

w BIOS Shadowing			Shadow RAM for System and Video BIOS

w BIOS Features			Built-in setup, Power-on self test, Drive	table				optimization, User-definable drive types, 				Password protection, Shadowing options

Plug & Play / BIOS Update

w Plug & Play BIOS			Support Plug & Play for easy  installation

w Flash EEPROM			Use Flash EEPROM to allow easy BIOS 				update

USB Devices

w USB Devices			Interfaced with both host and hub control 				functions

2 programmable USB ports

System Support Functions

w System functions			7 DMA channels, 16 level interrupts, 					Programmable timers

w Support functions			Fast A20 gate and Fast Reset

w Clock				Enhanced real time clock/calendar with 					battery back-up

Other Features

w Power good			On board power good signal generation

w 3.5V (VRE) supply		On board 3.5V supply to eliminate the 					need for special power supply for  					component e.g. CPU, SRAM.

Maximum rating : 30 W.

w Dual Voltage supply		On board Dual Voltage supply to directly 				support Intel Pentium CPU, Cyrix 6x86 					CPU and AMD K5 CPU.

Maximum rating : 30 W.

w Switches			Reset, Keylock  switches, Sleep Switch.

w Size				8.5" (W) x 13" (L)

## CHAPTER 2          INSTALLATION AND UPGRADE

## 2.1	System Component Map

| Jumper Connectors | Function |
| --- | --- |
| P1 | Turbo LED |
| P2 | Speaker |
| P3 | IDE LED Connector |
| P4 | Reset |
| P5 | Keylock |
| P6 | Fan Connector |
| P7 | Secondary IDE Connector |
| P8 | Primary IDE Connector |
| P9 | External Battery Connector |
| P10 | Sleep Connector |
| P12 | Printer Port Connector |
| P14 | Floppy Connector |
| P15,P18 | Power Connector |
| P17 | Serial Port 1 |
| P20 | Serial Port 2 |
| P21 | PS/2 Mouse Connector |
| P24 | USB Connector |
| KB1 | Keyboard Connector |

## 2.2	CPU Installation

The CPU is composed of pins that can easily be bent during installation, causing permanent damage to the processor.  It is therefore very important that you make sure the pins are straight before installing the CPU onto the SPGA socket located on RHINO 9 (refer to layout for exact location).  To properly align the CPU  with the socket, align pin 1 of the CPU ( with a notch at the corner) with pin 1 of the CPU socket as demonstrated below.

### Figure  1.   Socket 7 for Pentium CPU

## 2.3	Fast Page mode / EDO DRAM Installation

There are six SIMM sockets located on the RHINO 9 motherboard, marked Bank 1 ,Bank 2 and Bank 3. The Bank 1, Bank 2 and Bank 3 are counted starting from right to left consecutively. Start to install the SIMM modules ( IN PAIRS) from either Bank 1, Bank 2  or Bank 3 depending on how your memory is configured, you may not need to use all the memory banks. Either X32 or X36 of  72 pins SIMM can be installed.

Bank 3      Bank 2       Bank 1

1  1  1  1  1 1 11  11 11 1

Figure 2     SIMM  Sockets Location

Below is some of the possible configuration table for RHINO 9.

| BANK 3 |  | BANK 2 |  | BANK 1 |  | TOTAL |
| --- | --- | --- | --- | --- | --- | --- |
| -------- | -------- | -------- | -------- | 512Kx32 | 512Kx32 | 4MB |
| -------- | -------- | -------- | -------- | 1Mx32 | 1Mx32 | 8MB |
| -------- | -------- | -------- | -------- | 2Mx32 | 2Mx32 | 16MB |
| -------- | -------- | -------- | -------- | 4Mx32 | 4Mx32 | 32MB |
| -------- | -------- | -------- | -------- | 8Mx32 | 8Mx32 | 64MB |
| -------- | -------- | -------- | -------- | 16Mx32 | 16Mx32 | 128MB |
| -------- | -------- | 512Kx32 | 512Kx32 | 512Kx32 | 512Kx32 | 8MB |
| -------- | -------- | 1Mx32 | 1Mx32 | 512Kx32 | 512Kx32 | 12MB |
| -------- | -------- | 2Mx32 | 2Mx32 | 512Kx32 | 512Kx32 | 20MB |
| -------- | -------- | 4Mx32 | 4Mx32 | 512Kx32 | 512Kx32 | 36MB |
| -------- | -------- | 8Mx32 | 8Mx32 | 512Kx32 | 512Kx32 | 68MB |
| -------- | -------- | 16Mx32 | 16Mx32 | 512Kx32 | 512Kx32 | 132MB |
| -------- | -------- | 512Kx32 | 512Kx32 | 1Mx32 | 1Mx32 | 12MB |
| -------- | -------- | 1Mx32 | 1Mx32 | 1Mx32 | 1Mx32 | 16MB |
| -------- | -------- | 2Mx32 | 2Mx32 | 1Mx32 | 1Mx32 | 24MB |
| -------- | -------- | 4Mx32 | 4Mx32 | 1Mx32 | 1Mx32 | 40MB |
| -------- | -------- | 8Mx32 | 8Mx32 | 1Mx32 | 1Mx32 | 72MB |
| -------- | -------- | 16Mx32 | 16Mx32 | 1Mx32 | 1Mx32 | 136MB |

..			..			..

| -------- | -------- | 512Kx32 | 512Kx32 | 16Mx32 | 16Mx32 | 132MB |
| --- | --- | --- | --- | --- | --- | --- |
| -------- | -------- | 1Mx32 | 1Mx32 | 16Mx32 | 16Mx32 | 136MB |
| -------- | -------- | 2Mx32 | 2Mx32 | 16Mx32 | 16Mx32 | 144MB |
| -------- | -------- | 4Mx32 | 4Mx32 | 16Mx32 | 16Mx32 | 160MB |
| -------- | -------- | 8Mx32 | 8Mx32 | 16Mx32 | 16Mx32 | 192MB |
| -------- | -------- | 16Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 256MB |

..			..			..

| 512Kx32 | 512Kx32 | 512Kx32 | 512Kx32 | 512Kx32 | 512Kx32 | 12MB |
| --- | --- | --- | --- | --- | --- | --- |
| 1Mx32 | 1Mx32 | 512Kx32 | 512Kx32 | 512Kx32 | 512Kx32 | 16MB |
| 2Mx32 | 2Mx32 | 512Kx32 | 512Kx32 | 512Kx32 | 512Kx32 | 24MB |
| 4Mx32 | 4Mx32 | 512Kx32 | 512Kx32 | 512Kx32 | 512Kx32 | 40MB |
| 8Mx32 | 8Mx32 | 512Kx32 | 512Kx32 | 512Kx32 | 512Kx32 | 72MB |
| 16Mx32 | 16Mx32 | 512Kx32 | 512KX32 | 512Kx32 | 512Kx32 | 136MB |

..			..			..

| 512Kx32 | 512Kx32 | 16Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 260MB |
| --- | --- | --- | --- | --- | --- | --- |
| 1Mx32 | 1Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 264MB |
| 2Mx32 | 2Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 272MB |
| 4Mx32 | 4Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 288MB |
| 8Mx32 | 8Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 320MB |
| 16Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 16Mx32 | 384MB |

Table 1      Memory Configuration Table

## 2.4	Control of System Speed

System speed can be controlled by keyboard. To change the speed by keyboard, use the minus sign (-) and the plus sign (+).  Press <control> + <alt> + <“-”> for slow speed and  <control> + <alt> + <“+”> for fast speed.

## 2.5	Fan voltage

There are two different voltages available for CPU fans; +5 volts and + 12 volts. Select a proper setting for jumper JP1 before you turn on the computer.

## 2.6	Fan Connector

This Fan connector (P6) allows the CPU fan to draw current directly from the motherboard, without having to rely on a separate power connector.

## 2.7	Reset CMOS

If the setting of the system setup is done improperly, it may make the system malfunction. If this happens, turn off the power and set jumper JP21 to 2-3 to clear the internal CMOS status register. Wait at least 5 seconds to ensure that the CMOS content has been completely cleared.

Next, set the jumper JP21 back to 1-2 and turn on the power.  The BIOS will find the CMOS status register is  reset and will regard the setup information invalid, so it will prompt you to correct the information.

## APPENDIX-A          CONNECTORS  PINOUT

## A.1	Turbo LED Connector (P1)

| Pin | Signal Name |
| --- | --- |
| 1 | Pull_Up_150 |
| 2 | LED_Turbo- |

A.2	Speaker Connector (P2)

| Pin | Signal Name |
| --- | --- |
| 1 | Speaker Data_Out |
| 2 | N.C. |
| 3 | Ground |
| 4 | +5Vdc |

## A.3	HD LED Connector (P3)

| Pin | Signal Name |
| --- | --- |
| 1 | Pull_Up_330 |
| 2 | HD_LED- |
| 3 | HD_LED- |
| 4 | Pull_Up_330 |

## A.4	Reset Connector (P4)

| Pin | Signal Name |
| --- | --- |
| 1 | Reset |
| 2 | Ground |

## A.5	Keylock Connector (P5)

| Pin | Signal Name |
| --- | --- |
| 1 | +5Vdc |
| 2 | Mechanical Key |
| 3 | Ground |
| 4 | Keyboard Inhibit |
| 5 | Ground |

## A.6	Fan Connector (P6)

| Pin | Signal Name |
| --- | --- |
| 1 | +5/+12dc Power |
| 2 | Ground |

## A.7	External Battery Connector (P9)

| Pin | Signal Name |
| --- | --- |
| 1 | +3.6Vdc |
| 2 | N.C. |
| 3 | Ground |
| 4 | Ground |

## A.8	Sleep  Switch (P10)

| Pin | Signal Name |
| --- | --- |
| 1 | EPMI |
| 2 | Ground |

## A.9	USB Connector (P24)

| Signal Name | Pin | Pin | Signal Name |
| --- | --- | --- | --- |
| VCC | 6 | 1 | VCC |
| Port 1- | 7 | 2 | Port 0- |
| Port 1+ | 8 | 3 | Port 0+ |
| Ground | 9 | 4 | Ground |
| NC | 10 | 5 | NC |

## A.10	Power Connector (P15,P18)

| Pin | Signal Name |
| --- | --- |
| 1 | Power Good |
| 2 | +5Vdc |
| 3 | +12Vdc |
| 4 | -12V dc |
| 5 | Ground |
| 6 | Ground |
| 7 | Ground |
| 8 | Ground |
| 9 | -5Vdc |
| 10 | +5Vdc |
| 11 | +5Vdc |
| 12 | +5Vdc |

## A.11	PS/2 Mouse Connector (P21)

| Pin | Signal Name |
| --- | --- |
| 1 | +5V dc |
| 2 | GND |
| 3 | MDATA |
| 4 | MCLK |

## A.12	Keyboard Connector (KB1)

| Pin | Signal Name |
| --- | --- |
| 1 | Keyboard clock |
| 2 | Keyboard data |
| 3 | Not used |
| 4 | Ground |
| 5 | VCC |

## APPENDIX-B          HARDWARE  SETTINGS

## B.1	Layout of RHINO 9 Main Board

## B.2	Jumper Settings

All factory settings are marked by * in the following sections.

## B.2.1	CPU related settings

CPU Voltage Core Selection

RHINO 9 has on board dual voltage  regulators to directly support  Intel Pentium CPU such as P54C, P54CQS, P54CS, P55C, Cyrix 6x86 CPU and AMD K5 CPU. The voltage selection for Core voltage is done by JP9, JP10, JP11, JP17, JP18 and JP52 as follows :

| CPU Core voltage | JP9 | JP10 | JP11 | JP17 | JP18 | JP52 | CPU TYPE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.5V | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 | OPEN | For future CPU |
| 2.8V | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 | 1-2 | Intel P55C CPU |
| 2.9V | 2-3 | 2-3 | 2-3 | 2-3 | 2-3 | 3-4 | For future CPU |
| 3.3V | 1-2 | 1-2 | 1-2 | 1-2 | 1-2 | 5-6 | Intel P54C, P54CQS or Intel P54CS,P54CTB or AMD K5-PR75,PR90 |
| 3.5V (VRE) | 1-2 | 1-2 | 1-2 | 1-2 | 1-2 | 7-8 | Cyrix 6x86 CPU or AMD K5-PR100 |

NOTE :  Be careful to select the appropriate Core voltage for  different CPU.  Improper Core voltage supplied to CPU may result in “PERMANENT DAMAGE” to CPU !

## CPU CLOCK

| JP2 | JP3 | JP8 | JP25 | JP28 | JP29 | CPU Clock | CPU TYPE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-2 2-3 1-2 | 1-2 1-2 1-2 | 1-2 1-2 1-2 | 1-2 1-2 1-2 | 2-3 2-3 2-3 | 2-3 2-3 2-3 | 50MHz | Intel P54C-75 Cyrix 6x86-P120+ (100Mhz) AMD K5-PR75 (75Mhz) |
| 1-2 2-3 2-3 1-2 2-3 1-2 | 1-2 1-2 2-3 2-3 1-2 1-2 | 1-2 1-2 1-2 1-2 1-2 1-2 | 2-3 2-3 2-3 2-3 2-3 2-3 | 1-2 1-2 1-2 1-2 1-2 1-2 | 2-3 2-3 2-3 2-3 2-3 2-3 | 60MHz | Intel P54C-90 Intel P54C-120 Intel P54C-150 Intel P54C-180 Cyrix 6x86-P150+ (120Mhz) AMD K5-PR90 (90Mhz) |
| 1-2 2-3 2-3 1-2 2-3 1-2 | 1-2 1-2 2-3 2-3 1-2 1-2 | 2-3 2-3 2-3 2-3 2-3 2-3 | 2-3 2-3 2-3 2-3 2-3 2-3 | 2-3 2-3 2-3 2-3 2-3 2-3 | 1-2 1-2 1-2 1-2 1-2 1-2 | 66MHz | Intel P54C-100 Intel P54C-133 Intel P54C-166 Intel P54C-200 Cyrix 6x86-P166+ (133Mhz) AMD K5-PR100 (100Mhz) |

## CPU Fan voltage select

|  | JP1 |
| --- | --- |
| +5V | 1-2 * |
| +12V | 2-3 |

## B.2.2	External cache (L2 cache) setting  Synchronous Cache size

| Total Cache Size | JP4 | JP5 | JP6 | JP7 | JP12 |
| --- | --- | --- | --- | --- | --- |
| 0KB | 1-2 | 1-2 | 1-2 | 1-2 | 1-2 |
| 256KB | 2-3 | 1-2 | 1-2 | 1-2 | 1-2 |
| 512KB | 1-2 | 2-3 | 2-3 | 2-3 | 2-3 |

Synchronous SRAM device requirement

256K Configuration :

1. 256KB cache using on board 256K SRAM

| SRAM | location | type | speed | voltage |
| --- | --- | --- | --- | --- |
| tag RAM | U6,U7 (on board) | 8Kx8 / 32Kx8 | 15ns | 5V I/O |
| data RAM | U1,U4 (on board) | 2 pcs 32Kx32 pipelined burst SRAM | 7ns (Clock to o/p valid) | 3.3V I/O |

2. 256KB cache using 256K Cache Module w/o Tag RAM

| SRAM | location | type | speed | voltage |
| --- | --- | --- | --- | --- |
| tag RAM | U6,U7 (on board) | 8Kx8 /32Kx8 | 15ns | 5V I/O |
| data RAM | DM1 | 256K Cache Module w/o Tag | 7ns (Clock to o/p valid) | 3.3V I/O |

512K Configuration:

512KB cache using 256K Cache Module w/o Tag RAM  and 256K SRAM on board

| SRAM | location | type | speed | voltage |
| --- | --- | --- | --- | --- |
| tag RAM | U6,U7 | 32Kx8 | 15ns | 5V I/O |
| data RAM (bank 0) | U1,U4 (on board) | 2 ps 32Kx32 pipelined burst SRAM | 7ns (Clock to o/p valid) | 3.3V I/O |
| data RAM (bank 1) | DM1 | 256K Cache Module w/o Tag RAM | 7ns (Clock to o/p valid) | 3.3V I/O |

## B.2.3	 Peripheral setup

PS/2 MOUSE support

|  | JP49 | JP43 | JP44 |
| --- | --- | --- | --- |
| Enabled | 1-2 * | 2-3 * | 2-3 * |
| Disabled | 2-3 | 1-2 | 1-2 |

## B.2.4	Miscellaneous

Power Good Signal select

|  | JP42 |
| --- | --- |
| External Power Good | 1-2 * |
| On-board Power Good | 2-3 |

CMOS discharge

|  | JP21 |
| --- | --- |
| Preserve CMOS | 1-2 * |
| Clear CMOS | 2-3 |

Battery select

|  | JP23 |
| --- | --- |
| On-board Battery | 1-2 * |
| External Battery | 2-3 |

Reserved Jumpers

| JP20 | 2-3 |
| --- | --- |
| JP35 | 1-2 |