Update Appendix-C, C.3 CPU related settings by adding AMD5k86,Intel P54C-200Mhz option.

## C.3	Jumper Settings

All factory settings are marked by * in the following sections.

### C.3.1 	CPU related settings

CPU Voltage Core Selection

RHINO 9 has on board 2.5V and 3.3V regulators to directly support  Intel Pentium P55C CPU. The P55C will specially need a 2.5V CPU Core voltage in order to funciton properly. Others Pentium CPU such as P54C, P54CQS, P54CS, Cyrix 6x86 CPU and AMD5k86 P75-P100 will need a Core voltage of 3.3V. The voltage selection for Core voltage is done by JP16 as follows :

| JP16 | CPU Core voltage | CPU Core voltage |
| --- | --- | --- |
| 2-3 | 2.5V | Intel P55C CPU |
| 1-2 | 3.3V | Intel P54C, P54CQS, P54CS,P54CTB or Cyrix 6x86 CPU, AMD5k86 P75-P100 |

NOTE :  Be careful to select the appropriate Core voltage for  different CPU.  Improper Core voltage supplied to CPU may result in “PERMANENT DAMAGE” to CPU !

Add AMD5k86 and  Intel P54C setting, combine CPU clock,clock multiper and CPU type to one table, please update CPU type setting page on your manual.

CPU TYPE

| JP2 | JP3 | JP8 | JP25 | JP28 | JP29 | CPU Clock | CPU TYPE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-2 2-3 1-2 | 1-2 1-2 1-2 | 1-2 1-2 1-2 | 1-2 1-2 1-2 | 2-3 2-3 2-3 | 2-3 2-3 2-3 | 50MHz | Intel P54C-75 Cyrix 6x86-P120+ (100Mhz) AMD 5k86-P75 (75Mhz) |
| 1-2 2-3 2-3 1-2 2-3 1-2 | 1-2 1-2 2-3 2-3 1-2 1-2 | 1-2 1-2 1-2 1-2 1-2 1-2 | 2-3 2-3 2-3 2-3 2-3 2-3 | 1-2 1-2 1-2 1-2 1-2 1-2 | 2-3 2-3 2-3 2-3 2-3 2-3 | 60MHz | Intel P54C-90 Intel P54C-120 Intel P54C-150 Intel P54C-180 Cyrix 6x86-P150+ (120Mhz) AMD 5k86-P90 (90Mhz) |
| 1-2 2-3 2-3 1-2 2-3 1-2 | 1-2 1-2 2-3 2-3 1-2 1-2 | 2-3 2-3 2-3 2-3 2-3 2-3 | 2-3 2-3 2-3 2-3 2-3 2-3 | 2-3 2-3 2-3 2-3 2-3 2-3 | 1-2 1-2 1-2 1-2 1-2 1-2 | 66MHz | Intel P54C-100 Intel P54C-133 Intel P54C-166 Intel P54C-200 Cyrix 6x86-P166+ (133Mhz) AMD 5k86-P100 (100Mhz) |

CPU Fan voltage select

|  | JP35 |
| --- | --- |
| +5V | 1-2 * |
| +12V | 2-3 |