Chapter 1

Main Features

The VA-501 mainboard comes with the following high-performance features:

- Easy Installation

- Award BIOS with support for Plug and Play, auto detection of Hard Drive

- and IDE features, MS Windows 95 and Windows NT compatible.

- Flexible Processor Support

- The onboard 321-pin ZIF socket supports Intel Pentium (P54C) CPU speed

- 75/90/100/120/133/150/166/180/200 MHz processors / P54CTB.

Cyrix 6x86-P120+ (100 MHz) / 6x86-P133+ (110 MHz) / 6x86-P150+

(120 MHz) / 6x86-P166+ (133 MHz)  processors.

IBM 6x86-P120+ (100 MHz) / 6x86-P133+ (110 MHz) / 6x86-P150+

(120 MHz) / 6x86-P166+ (133 MHz) processors.

AMD K5-PR75 (75 MHz) / K5-PR90 (90 MHz) / K5-PR100 (100 MHz) /

K5-PR120 (90 MHz) / K5-PR133 (100 MHz) / K5-PR150 (120 MHz) /

K5-PR166 (133 MHz) processors.

- Leading Edge Chipset

- VIA Apollo 580VP chipset, including a CPU interface controller, advanced

- cache controller, integrated DRAM controller, synchronous ISA bus

- controller, PCI local bus interface, integrated power management unit.

- Ultra-fast Level II Cache

- Supports onboard 256KB/512KB synchronous PBSRAM direct-mapped

- write-back cache memory.

- Versatile Main Memory Support

- Accepts up to 64MB RAM in two banks using 72-pin SIMM modules of

- 4, 8, 16, 32MB with support for EDO and  Fast Page Mode memory.

- ISA & PCI Expansion Slots

- Three 16-bit ISA and four 32-bit PCI expansion slots provide all the

- room you need to install a full range of add-on cards.

- Enhanced PCI Bus Master IDE Controller

- Integrated Enhanced PCI local bus IDE controller with two connectors

supports up to four IDE devices such as Hard Disk, CD-ROM or Tape

Backup drives via two channels for high speed data throughput. This

controller supports PIO Modes 3 and 4, and DMA Mode 2 for optimized

system performance.

- Super Multi I/O

- Integrated UMC 669/LGS Prime 3C chipset features two 16550A UART

- compatible serial  ports, one EPP/ECP capable parallel port, and one Floppy

- Disk Drive connector.