AMENDMENT FOR

GMB-P56IPS V0 VERSION 1.05/1.06/

PAM-0056I E-0 EDITION 1.05/1.06

RELEASE 1.00

- In Chapter 2, Section 2.2 Cache Memory Configuration should be changed as follows:

2.2	CACHE MEMORY CONFIGURATION

2.2.1	256KB SECONDARY CACHE ON BOARD

2 pcs of 32Kx32 Synchronous Pipeline Burst SRAM are installed on U9

and U10.  However, user can expand the secondary cache system to

512KB by installing 256KB SRAM modules to the COASt connector M1.

2.2.2	512KB SECONDARY CACHE ON BOARD

2 pcs of 64Kx32 Synchronous Pipeline Burst SRAM are installed on U9

and U10.  With such configuration, no COASt connector will be installed

on M1.

- In Chapter 2, the jumper settings of AMD-K5-PR133 (REV C) and Cyrix 6x86-P200+ are modified as follows:

| Function |  | Jumper Settings |  |
| --- | --- | --- | --- |
| CPU Speed Selection | For 133MHz Intel Pentium, AMD-K5-PR133 (REV C) and Cyrix 6x86-P166+ CPU | JP6: JP7: S1: S2: S3: | 1-2 short 2-3 short open open open |
|  | For 150MHz Cyrix 6x86-P200+ CPU | JP6: JP7: S1: S2: S3: | 1-2 short 2-3 short open open short |

- The corresponding graphical descriptions of jumper settings in Chapter 3 are changed as follows:

7.	For 133MHz Intel Pentium, AMD-K5-PR133 (REV C) and Cyrix

6x86-P166+ CPU

9.	For 150MHz Cyrix 6x86-P200+ CPU