Jumper settings for V4P895P3/SMT/V5.0 (4+2) Motherboard

| Jumper | Cyrix 5x86/ DX4-100/120 | AMD X5-133 | AMD Enhanced CPU |
| --- | --- | --- | --- |
| JP3 | 1-2 | 1-2 | 1-2 |
| JP4 | OPEN | OPEN | OPEN |
| JP5 | 2-3 | 2-3 | 2-3 |
| JP6 | 2-3 | 1-2 | 1-2 |
| JP7 | OPEN | 1-2 | 1-2 |
| JP8 | 1-2 | 1-2 | 1-2 |
| JP9 | 1-2 | 2-3 | 2-3 |
| JP12 | OPEN | CLOSE | CLOSE |
| JP22 | 1-2 | 1-2 | 1-2 |
| JP23 | OPEN | CLOSE | CLOSE |
| JP24 | OPEN | CLOSE | CLOSE |
| JP25 | OPEN | CLOSE | CLOSE |
| JP27 | OPEN | OPEN | OPEN |
| JP36 | 1-2 | 2-3 | 2-3 |
| JP40 | OPEN | OPEN | OPEN |
| JP43 | OPEN | 1-2 | OPEN |

Note:

1. The jumper setting should use together with BIOS v1.4 which is the same as the V4P895 (4x72) motherboard.

2. When use Cyrix 5x86-120, the sing on message will be Cx5x86 100M Hz, however, the performace will be at 120M Hz.

3.  For 120M Hz CPU, the clock setting should be set to 40M Hz, for 100M Hz CPU and AMD X5-133 CPU, the clock should be set to 33M Hz.

Date: 31/1/96