This publication, including all photographs, illustrations and software, is protected under international copyright laws, with all rights reserved. Neither this manual, nor any of the material contained herein, may be reproduced without the express written consent of the manufacturer.

The information in this document is subject to change without notice. The manufacturer makes no representations or warranties with respect to the contents hereof and specifically disclaims any implied warranties of merchantability or fitness for any particular purpose. Further, the manufacturer reserves the right to revise this publication and to make changes from time to time in the content hereof without obligation of the manufacturer to notify any person of such revision or changes.

### Trademarks

IBM, VGA, and PS/2 are registered trademarks of International Business Machines.

AMD and Athlon are registered trademarks of Advanced Micro Devices Inc.

Intel, Pentium, Pentium-II, and MMX are registered trademarks of Intel Corporation.

Microsoft, MS-DOS and Windows 95/98/NT are registered trademarks of Microsoft Corporation.

Sound Blaster is a trademark of Creative Technology Ltd.

PC-cillin and ChipAwayVirus are trademarks of Trend Micro Inc.

AMI is a trademark of American Megatrends Inc.

A3D is a registered trademark of Aureal Inc.

Gamut is a registered trademark of Formosoft International Inc.

SuperVoice is a registered trademark of Pacific Image Communications Inc.

MediaRing Talk is a registered trademark of MediaRing Inc.

Other names used in this publication may be trademarks and are acknowledged.

Copyright © 2000

All Rights Reserved

M800LMR, V1.1A

AM75X/June 2000

Federal Communications Commission (FCC)

This equipment has been tested and found to comply with the limits for a Class B digital device, pursuant to Part 15 of the FCC Rules. These limits are designed to provide reasonable protection against harmful interference in a residential installation. This equipment generates, uses, and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communications. However there is no guarantee that interference will not occur in a particular installation. If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

- Reorient or relocate the receiving antenna.

- Increase the separation between the equipment and the receiver.

- Connect the equipment onto an outlet on a circuit different from that to which the receiver is connected.

- Consult the dealer or an experienced radio/TV technician for help.

Shielded interconnect cables and shielded AC power cable must be employed with this equipment to insure compliance with the pertinent RF emission limits governing this device. Changes or modifications not expressly approved by the system’s manufacturer could void the user’s authority to operate the equipment.

##### Declaration of Conformity

This device complies with part 15 of the FCC rules. Operation is subject to the following conditions:

- This device may not cause harmful interference, and

- This device must accept any interference received, including interference that may cause undesired operation.

Canadian Department of Communications

This class B digital apparatus meets all requirements of the Canadian Interference-causing Equipment Regulations.

Cet appareil numérique de la classe B respecte toutes les exigences du Réglement sur le matériel brouilieur du Canada.

Table of Contents

Chapter 1: Introduction	1

Key Features	2

Package Contents	5

Static Electricity Precautions	6

Pre-Installation Inspection	6

Chapter 2: Mainboard Installation	7

Mainboard Components	8

I/O Ports	9

Install A CPU	9

Install Memory	11

Setting Jumper Switches	11

Install the Mainboard	14

Install the Extension Brackets	16

Optional Extension Brackets	17

Install Other Devices	19

Expansion Slots	21

Chapter 3: BIOS Setup Utility	22

Introduction	22

Running the Setup Utility	23

Standard CMOS Setup Page	24

Advanced Setup Page	25

Power Management Setup Page	27

PCI / Plug and Play Setup Page	29

Load Optimal Settings	29

Load Best Performance Settings	30

Features Setup Page	30

Hardware Monitor Page	32

Change Password	32

Exit	33

Chapter 4: Software & Applications	34

Introduction	34

Installing Support Software	34

Auto-installing under Windows 98	36

Using the PCI Audio Software	38

The Four Speakers System	39

Appendix A: Corel WordPerfect Suite 8 	A1

Welcome to Corel WordPerfect Suite 8 	A2

Installing Corel WordPerfect Suite 8 	A6

Learning how to use Corel WordPerfect Suite 8 	A9

Support and Services 	A13

Appendix B: Gamut2000 	B1

1. GAMUT2000 Family 	B2

1.1 LifeAmp - Versatile Audio Playback System 	B2

1.2 AudioPort - Audio Transportation System 	B4

2. Installation 	B5

2.1 Before Beginning: System Requirements 	B5

2.2 Uninstalling the Previous of GAMUT2000	B5

3. Main Console 	B7

3.1 Introduction 	B7

3.2 Function and Operation 	B7

3.3 Main Menu 	B9

# Chapter 1

## Introduction

This mainboard has a Slot-A processor socket for an AMD Athlon type CPUs. You can install any one of these processors on the mainboard.

The mainboard supports Slot-A processor front-side bus speeds of 200MHz.

This mainboard uses the AMD 750 chipset which provides CPU Plug & Play through firmware, and has an embedded 10BaseT/100BaseTX Network Interface. The mainboard has a built-in PCI 3D Sound System and a V.90 Fax/Modem DAA module is shipped with the mainboard. In addition, the mainboard has an extended set of ATX I/O Ports including PS/2 keyboard and mouse ports, two USB ports, an RJ-45 LAN port, a parallel port, and two serial ports. Two extra USB ports can be added using the Extended USB Module that connects to the mainboard.

This mainboard has all the features you need to develop a powerful multimedia workstation that is network ready, and has built-in communications. The board is ATX size and has power connectors for an ATX power supply.

### Key Features

The key features of this mainboard include:

#### Slot-A Processor Support

- Athlon support for 500MHz to 850MHz clock rates

- Supports 200 MHz Front-Side Bus

Processors are automatically configured using firmware and a synchronous/asynchronous Host/DRAM Clock Scheme.

#### Memory Support

- Three DIMM slots for 168-pin SDRAM memory modules

- Support for 100MHz memory bus

- Maximum installed memory is 3 x 256MB = 768MB

#### Expansion Slots

- One AGP solt for AGP 2.0-compliant interface.

- Four 32-bit PCI slots for PCI 2.2-compliant bus interface.

#### Onboard IDE channels

- Primary and Secondary PCI IDE channels

- Support for PIO (programmable input/output) modes

- Support for Multiword DMA modes

- Support for Bus Mastering and Ultra DMA 33/66 modes

#### Power Supply and Power Management

- ATX power supply connector

- ACPI and previous PMU support, suspend switch, keyboard power on/off

- Supports Wake on Modem, Wake on LAN and Wake on Alarm

#### Sound System

- Complies with the PC98 audio specification

- 16-bit CODEC for full-duplex playback and recording

- HRTF 3D professional audio supports both Direct Sound 3D® and A3D®-compatible interfaces plus support for 4-channel speakers

- Driver support for MS-DOS, Microsoft Windows 95/98/2000/NT 4.0

- Built-in 32ohm earphone buffer and 3D surround sound

- Provides MPU-401 Game/MIDI port and legacy Sound Blaster 16 support

- Downloadable Wave-table Synthesizer supports Direct Music®

- Stereo Mixer supports analog mixing from CD-Audio and Line In or digital mixing from voice, FM/Wave-table and digital CD-Audio

#### Onboard I/O Ports

- Provides PC99 Color Connectors for easy peripheral device connections

- Floppy disk drive connector with 1Mb/s transfer rate

- Two serial ports with 16550-compatible fast UART

- One parallel port with ECP and EPP support

- Two USB ports, optional two USB ports module

- Two PS/2 ports for keyboard and mouse

- One infrared port connector for optional module

#### Hardware Monitoring

- Built-in hardware monitoring for CPU & System temperatures, fan speeds and mainboard voltages

#### Built-in Ethernet LAN

- Built-in 10BaseT/100BaseTX Ethernet LAN

- LAN controller integrates Fast Ethernet MAC and PHY compliant with IEEE802.3u 100BASE-TX, 10BASE-T and ANSI X3.263 TP-PMD standards

- Compliant with ACPI 1.0 and the Network Device Class Power Management 1.0

- High Performance provided by 100Mbps clock generator and data recovery circuit for 100Mbps receiver

#### Fax/Modem DAA Module

- 56 Kbps Fax/Modem DAA module

- Supports V.90, V.34, V.32bis, V.32, V.22bis, V.22

- Supports Auto Fallback and MNP 5, V.42bis data compression with 115,200-compatible Virtual UART

- Requires 16MB RAM and Microsoft Windows 95/98/NT

#### Onboard Flash ROM

- Automatic CPU and board configuration

- Supports Plug and Play configuration of peripheral devices and expansion cards

- Built-in virus protection using Trend’s Chip Away Virus provides boot process virus protection.

#### Bundled Software

- PC-Cillin provides automatic virus protection under Windows 95/98

- SuperVoice is data, fax and voice communication software

- Gamut provides professional audio features included MP3 encoding/playback

- MediaRing Talk provides PC to PC or PC to Phone internet phone communication

- S-YXG50 is music synthesizer software to playback MIDI files on the system

- Corel WordPerfect Suite 8 is a Microsoft Windows® office application suite (optional)

- WinDVD is a DVD playback application (optional)

#### Dimensions

- ATX form factor (30.5cm x 24.4cm)

### Package Contents

Your mainboard package ships with the following items:

- The mainboard

- This User’s Guide

- 1 UDMA/33 IDE cable

- 1 Floppy disk drive cable

- Fax/Modem DAA module

- Support software on CD-ROM disk

#### Optional Accessories

You can purchase the following optional accessories for this mainboard.

- Extended USB module

- UDMA/66 IDE cable

### Static Electricity Precautions

Components on this mainboard can be damaged by static electricity. Take the following precautions when unpacking the mainboard and installing it in a system.

- Keep the mainboard and other components in their original static-proof packaging until you are ready to install them.

- During installation, wear a grounded wrist strap if possible. If you don’t have a wrist strap, discharge static electricity by touching the bare metal of the system chassis.

- Handle the mainboard carefully by the edges. Avoid touching the components unless it is absolutely necessary. During installation put the mainboard on top of the static-protection packaging it came in with the component side facing up.

### Pre-Installation Inspection

- Inspect the mainboard for damage to the components and connectors on the board.

- If you suspect that the mainboard has been damaged, do not connect power to the system. Contact your mainboard vendor and report the damage.

# Chapter 2

## Mainboard Installation

To install this mainboard in a system, follow the procedures in this chapter:

- Identify the mainboard components

- Install a CPU

- Install one or more system memory modules

- Verify that any jumpers or switches are set correctly

- Install the mainboard in a system chassis (case)

- Connect any extension brackets or cables to the mainboard connector headers

- Install any other devices and make the appropriate connections to the mainboard connector headers.

Note:

- Before installing this mainboard, make sure jumper JBAT is set to Normal setting. See this chapter for information on locating JBAT and the setting options.

- Never connect power to the system during installation. Doing so may damage the mainboard.

### Mainboard Components

Use the diagram below to identify the major components on the mainboard.

DAA

Note: Any jumpers on your mainboard that do not appear in this illustration are for testing only.

### I/O Ports

The illustration below shows a side view of the built-in I/O ports on the mainboard.

### Install A CPU

This mainboard has a Slot-A CPU socket for Athlon processors.

Do not try to install a Slot-1 processor in the Slot-A. A Slot-1 processor such as the Pentium-II/III, or the SEPP Celeron does not fit in the Slot-A.

The following list the processors that are currently supported by this mainboard.

Athlon: 500~850MHz, FSB: 200 MHz

Installing a Slot-A Processor Cartridge CPU

- Locate Slot-A and CPU FAN on the mainboard.

Slot-A has pre-installed retention module. The upright arms are folded down for shipping.

CPU FAN

- The Slot-A is installed with a retention module and a supporter module. The upright arms of the retention module are folded down for shipping, raise the arms so that they are in the upright position. And release the clip of the supporter module as lower left illustration.

- Slide the CPU cartridge into the module arms. Follow the instructions that come with the CPU. The edge connector on the CPU cartridge has a notch so that it only fits into the Slot-A in the correct way. And insert the clip portion of the supporter module so that the heat sink can sit and latch on the top of the supporter.

- Locate the cooling fan power supply CPU FAN. Connect the cable from the processor cartridge cooling fan to CPU FAN.

- On this mainboard, you can configure the processor by entering the correct settings in the BIOS setup utility.

### Install Memory

The mainboard has three DIMM sockets for system memory modules. You must install at least one memory module in order to use the mainboard.

DIMM1

DIMM2

DIMM3

For this mainboard, you must use 168-pin, 3.3V unbuffered SDRAM memory modules. If the installed CPU uses a 100MHz system bus, you must use PC100 or PC133 memory. You can install any size memory module from 8 MB to 256MB, so the maximum memory size is 3 x 256MB = 768MB.

The edge connectors on the memory modules have cut outs, which coincide with spacers in the DIMM sockets so that memory modules can only be installed in the correct orientation.

To install a module, push the retaining latches at either end of the socket outwards. Position the memory module correctly and insert it into the DIMM socket. Press the module down into the socket so that the retaining latches rotate up and secure the module in place by fitting into notches on the edge of the module.

### Setting Jumper Switches

Jumpers are sets of pins which can be connected together with jumper caps. The jumper caps change the way the mainboard operates by changing the electronic circuits on the mainboard. If a jumper cap connects two pins, we say the pins are SHORT. If a jumper cap is removed from two pins, the pins are OPEN.

JP706

1

1

1

JP703

JP702

1

JBAT

#### Jumper JP702: Enable/Disable Onboard Modem

Use this jumper to enable or disable the onboard Fax/Modem connector. If the audio jumper is disabled, the Fax/Modem is also disabled, no matter what JP702 is set to.

| Function | Jumper Setting |
| --- | --- |
| Enable Onboard Modem | Short Pins 1-2 |
| Disable Onboard Modem | Short Pins 2-3 |

#### Jumper JP703: Enable/Disable Onboard Audio

Use this jumper to enable or disable the onboard audio system. If you install another sound card, you must disable the onboard audio subsystem. If you use this jumper to disable the audio subsystem, it automatically disables the onboard Fax/Modem, even if Fax/Modem jumper is enabled.

| Function | Jumper Setting |
| --- | --- |
| Disable Audio/Modem | Short Pins 1-2 |
| Enable Audio | Short Pins 2-3 |

#### Jumper JP706: Enable/Disable Onboard LAN

Use this jumper to enable or disable the onboard Ethernet LAN.

| Function | Jumper Setting |
| --- | --- |
| Enable Onboard LAN | Short Pins 1-2 |
| Disable Onboard LAN | Short Pins 2-3 |

#### Jumper JBAT: Clear CMOS Memory

Use this jumper to clear the contents of the CMOS memory. You may need to clear the CMOS memory if the settings in the Setup Utility are incorrect and prevent your mainboard from operating. To clear the CMOS memory, disconnect all the power cables from the mainboard and then move the jumper cap into the CLEAR setting for a few seconds.

| Function | Jumper Setting |
| --- | --- |
| Normal Operation | Short Pins 1-2 |
| Clear CMOS Memory | Short Pins 2-3 |

### Install the Mainboard

Install the mainboard in a system chassis (case). The board is a ATX size mainboard with a twin-tier of I/O ports. You can install this mainboard in ATX case. Ensure that your case has an I/O cover plate that matches the ports on this mainboard.

Install the mainboard in a case. Follow the instructions provided by the case manufacturer using the hardware and internal mounting points on the chassis.

PWR1

SYSTEM FAN

J50

AUX FAN

Connect the power connector from the power supply to the PWR1 connector on the mainboard.

If there is a cooling fan installed in the system chassis, connect the cable from the cooling fan to the SYSTEM FAN fan power connector on the mainboard.

If there is another +12V cooling fan installed in this system, connect the cable from the cooling fan to the AUX FAN fan power connector on the mainboard.

Connect the case switches and indicator LEDs to the J50 switch and LED connector header. See the illustration below for a guide to the J50 connector pin assignments.

21

22

Power LED

Pins 2-4-6

Speaker

Pins 1-3-5-7

Keylock

Pins 8-10

HDD LED

Pins 15-16

12

1

Reset Switch

Pins 17-18

Power Button

Pins 21-22

Turbo LED

Pins 13-14

Suspend LED

Pins 19-20

Suspend Switch

Pins 11-12

### Install the Extension Brackets

The extension brackets are used to connect features on the mainboard to external connectors that can be attached to the system chassis. Follow the steps below to install the extension brackets.

Note: All the ribbon cables used on the extension brackets have a red stripe on the Pin-1 side of the cable.

#### Fax/Modem Module

The Fax/Modem DAA module plugs directly into the mainboard in line with to an expansion slot opening in the system chassis. When you remove the slot cover from the system chassis, you can access the LINE and TEL RJ11 connectors on the metal edge of the Fax/Modem DAA module.

DAA-Modem Header

2 MCLK
4 FRA-SY
6 SCLK
8 RIN-WAK
 
12 SDO
14 SDI
16 RST

GND 1
GND 3
GND 5
AUX 3V 7
HOOK 9
RIN 11
AUX 5V 13
MUTE 15

- Locate the DAA modem header on the mainboard.

- Plug the Fax/Modem DAA module into the DAA header.

- Remove the modem header slot cover.

Modem DAA Module

Modem Header

Line & Tel

RJ11 Sockets

### Optional Extension Brackets

For this mainboard, you can also obtain a USB module extension bracket. Install them by following the steps below.

#### Extended USB Module

This module bracket has two USB ports for more USB devices.

J2 Header Header

1

- Locate the J2 header on the mainboard.

- Plug the bracket cable onto the J2 header.

- In the system chassis, remove a slot cover from one of the expansion slots and install the extension bracket in the opening. Use the screw that held the slot cover in place to secure the extension bracket to the chassis.

### Install Other Devices

Install and connect any other devices in the system following the steps below.

FDC701

1

1

IDE2

1

IDE1

#### Floppy Disk Drive

The mainboard ships with a floppy disk drive cable that can support one or two drives. Drives can be 3.5” or 5.25” wide, with capacities of 360K, 720K, 1.2MB, 1.44MB, or 2.88MB.

Install your drives and connect power from the system power supply. Use the cable provided to connect the drives to the floppy disk drive header FDC701.

#### IDE Devices

IDE devices include hard disk drives, high-density diskette drives, and CD-ROM or DVD-ROM drives, among others.

The mainboard ships with an IDE cable that can support one or two IDE devices. If you connect two devices to a single cable, you must configure one of the drives as Master and one of the drives as Slave. The documentation of the IDE device will tell you how to configure the device as a Master or Slave device. The Master device connects to the end of the cable.

Install the device(s) and connect power from the system power supply. Use the cable provided to connect the device(s) to the Primary IDE channel connector IDE1 on the mainboard.

If you want to install more IDE devices, you can purchase a second IDE cable and connect one or two devices to the Secondary IDE channel connector IDE2 on the mainboard. If you have two devices on the cable, one must be Master and one must be Slave.

#### Internal Sound Connections

If you have installed a CD-ROM drive or DVD-ROM drive, you can connect the drive audio cable to the onboard sound system.

On the mainboard, locate the two 4-pin connectors J701 and J702. There are two kinds of connector because different brands of CD-ROM drive have different kinds of audio cable connectors. Connect the cable to the appropriate connector.

1

J702

J701

#### Infrared Port

You can connect an infrared port to the mainboard. You can purchase this option from third-party vendors.

1

IR Header

- Locate the infrared port IR header on the mainboard.

- If you are adding an infrared port, connect the ribbon cable from the port to the IR header and then secure the port to an appropriate place in your system chassis.

### Expansion Slots

This mainboard has four 32-bit PCI expansion slots and one AGP slot. The PCI slot PCI1 is shared with the onboard Sound, the PCI slot PCI2 is shared with the onboard Modem, and the PCI slot PCI3 is shared with the onboard LAN. It means that you can use either one of shared both but you cannot use the both at the same time.

PCI2

PCI1

AGP1

PCI4

PCI3

Use the AGP1 slot to install an AGP graphics adapter. Use the PCI slots to install 32-bit PCI expansion cards.

Follow the steps below to install a PCI or AGP expansion card.

- Locate the AGP or PCI slots on the mainboard.

- Remove the slot cover for the expansion slot from the system chassis.

- Insert the expansion card edge connector into the slot and press it firmly down into it so that it is fully inserted.

- Secure the expansion card bracket to the system chassis using the screw that held the slot cover in place.

# Chapter 3

## BIOS Setup Utility

### Introduction

The BIOS Setup Utility records settings and information about your computer such as the date and time, the kind of hardware installed, and various configuration settings. Your computer uses this information to initialize all the components when booting up and functions as the basis for coordination between system components.

If the Setup Utility configuration is incorrect, it may cause the system to malfunction. It can even stop your computer from booting properly. If this happens, you can use the clear CMOS jumper to clear the CMOS memory used to store the configuration information, or you can hold down the Page Up key while you reboot your computer. Holding down the Page Up key also clears the setup information.

You can run the setup utility and manually make changes to the configuration. You might need to do this to configure some of the hardware that you install on or connect to the mainboard, such as the CPU, system memory, disk drives, etc.

### Running the Setup Utility

Each time your computer starts, before the operating system loads, a message appears on the screen that prompts you to “Hit <DEL> if you want to run SETUP”. When you see this message, press the Delete key and the Main menu page of the Setup Utility appears on your monitor.

AMIBIOS SIMPLE SETUP UTILITY – VERSION 1.20 ©1998 American Megatrends, Inc. All Rights Reserved

Standard CMOS Setup Features Setup Advanced Setup Hardware Monitor Power Management Setup Change Password PCI / Plug and Play Setup Exit Load Optimal Settings Load Best Performance Settings ESC: Quit  : Select Item (Shift)F2 : Change Color F5 : Old Values F6: Optimal values F7: Best performance values F10: Save&Exit

You can use the cursor arrow keys to highlight any of the options on the main menu page. Press Enter to select the highlighted option. To leave the setup utility, press the Escape key. To cycle through the Setup Utility’s optional color schemes hold down the Shift key and press F2.

Some of the options on the main menu page lead to tables of items with installed values. In these pages, use the cursor arrow keys to highlight the items, and then use the PgUp and PgDn keys to cycle through the alternate values for each of the items. Other options on the main menu page lead to dialog boxes which require you to answer Yes or No by hitting the Y or N keys.

If you have already made changes to the setup utility, press F10 to save those changes and exit the utility. Press F5 to reset the changes to the original values. Press F6 to install the setup utility with a set of default values. Press F7 to install the setup utility with a set of high-performance values.

### Standard CMOS Setup Page

Use this page to set basic information such as the date and time, the IDE devices, and the diskette drives. If you press the F3 key, the system will automatically detect and configure the hard disks on the IDE channels.

AMIBIOS SETUP – STANDARD CMOS SETUP ©1998 American Megatrends, Inc. All Rights Reserved

Date (mm/dd/yy) : Tue Feb 1, 2000 Time (hh/mm/ss) : 14:26:53 LBA Blk PIO 32Bit Type Size Cyln Head WPcom Sec Mode Mode Mode Mode Pri Master : Auto On Pri Slave : Auto On Sec Master : Auto On Sec Slave : Auto On Floppy Drive A : 1.44MB 3 1/2” Floppy Drive B : Not Installed

Month : Jan – Dec ESC : Exit Day : 01 – 31  : Select Item Year : 1901 – 2099 PU/PD/+/- : Modify (Shift)F2 : Color F3 : Detect All HDD

| Date & Time | Use these items to set the system date and time |
| --- | --- |
| Pri Master Pri Slave Sec Master Sec Slave | Use these items to configure devices connected to the Primary and Secondary IDE channels. To configure an IDE hard disk drive, choose Auto. If the Auto setting fails to find a hard disk drive, set it to User, and then fill in the hard disk characteristics (Size, Cyls, etc.) manually. If you have a CD-ROM drive, select the setting CDROM. If you have an ATAPI device with removable media (e.g. a ZIP drive or an LS-120) select Floptical. |
| Floppy Drive A Floppy Drive B | Use these items to set the size and capacity of the floppy diskette drive(s) installed in the system. |

### Advanced Setup Page

Use this page to set more advanced information about your system. Take some care with this page. Making changes can affect the operation of your computer.

AMIBIOS SETUP – ADVANCED SETUP ©1998 American Megatrends, Inc. All Rights Reserved

Trend ChipAwayVirus Enabled Quick Boot Enabled 1st Boot Device IDE-0 2nd Boot Device Floppy Try Other Boot Devices Yes S.M.A.R.T. for Hard Disks Disabled BootUp Num-Lock On Floppy Drive Swap Disabled Floppy Drive Seek Disabled Primary Display VGA/EGA Password Check Setup Boot To OS/2 Over 64MB No Internal Cache Enabled External Cache Enabled System BIOS Cacheable Disabled Graphic Win Size 64M ESC : Quit  : Select Item DRAM Integrity Mode Disabled F1 : Help PU/PD/+/- : Modify Memory Hole Disabled F5 : Old Values (Shift)F2 : Color DRAM Burst Refresh Enabled F6 : Load Optimal values F7 : Load Best performance values

| Trend ChipAway Virus | This mainboard has built-in virus protection in the firmware. Use this item to enable or disable the built-in virus protection. |
| --- | --- |
| Quick Boot | If you enable this item, the system starts up more quickly be elimination some of the power on test routines. |
| 1st Boot Device 2nd Boot Device | Use these items to determine the device order the computer uses to look for an operating system to load at start-up time. |
| Try Other Boot Device | If you enable this item, the system will also search for other boot devices if it fails to find an operating system from the first two locations. |
| S.M.A.R.T. for Hard Disks | Enable this item if any IDE hard disks support the S.M.A.R.T. (Self-Monitoring, Analysis and Reporting Technology) feature. |
| BootUp Num-Lock | This items determines if the Num Lock key is active or inactive at system start-up time. |
| Floppy Drive Swap | If you have two diskette drives installed and you enable this item, drive A becomes drive B and drive B becomes drive A. |
| Floppy Drive Seek | If you enable this item, your system will check all floppy disk drives at start up. Disable this item unless you are using an old 360KB drive. |
| Primary Display | Use this item to determine the display devices for the primary display of your computer. |
| Password Check | If you have entered a password for the system, use this item to determine if the password is required to enter the Setup Utility (Setup) or required both at start-up and to enter the Setup Utility (Always). |
| Boot to OS/2 Over 64MB | Enable this item if you are booting the OS/2 operating system and you have more than 64MB of system memory installed. |
| Internal Cache | Leave this item enabled since all the processors that can be installed on this board have internal cache memory. |
| External Cache | Leave this item enabled since all the processors that can be installed on this board have external cache memory. |
| System BIOS Cacheable | If you enable this item, a segment of the system BIOS will be cached to main memory for faster execution. |
| Graphic Win Size | This item determines the operation of the onboard graphics adapter. We recommend that you leave this item at the default value. |
| DRAM Integrity Mode | Use this item to define the kind of error correction supported by the memory that you have installed. |
| Memory Hole | This item can be used to reserve memory space for some ISA expansion cards that require it. |
| DRAM Burst Refresh | Use this item set the burst refresh for the installed DRAM. |

### Power Management Setup Page

This page sets some of the parameters for system power management operation.

AMIBIOS SETUP – POWER MANAGEMENT SETUP ©1998 American Megatrends, Inc. All Rights Reserved

Power Management/APM Disabled Green Monitor Power State Suspend Video Power Down Mode Suspend Hard Disk Power Down Mode Suspend Suspend Time Out (Minute) Disabled Ring On Power On Disabled RTC Alarm Power On Disabled RTC Alarm Date 15 RTC Alarm Hour 12 RTC Alarm Minute 30 RTC Alarm Second 30 ESC : Quit  : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift)F2 : Color F6 : Load Optimal values F7 : Load Best performance values

| Power Management/APM | Use this item to enable or disable a power management scheme. If you enable power management, you can use the items below to set the power management operation. Both APM and ACPI are supported. |
| --- | --- |
| Green Monitor Power State | This item defines which power-saving mode is required to trigger the power management operations of a green monitor. |
| Video Power Down Mode | This item defines which power-saving mode is required to power down the monitor. |
| Hard Disk Power Down Mode | This item defines which power-saving mode is required to power down the hard disk drive. |
| Suspend Time Out (Minute) | This sets the timeout for Suspend mode in minutes. If the time selected passes without any system activity, the computer will enter power-saving Suspend mode. |
| Ring On Power On | The system can be turned off with a software command. If you enable this item, the system can automatically resume if there is an incoming call on the Fax/Modem. You must use an ATX power supply in order to use this feature. |
| RTC Alarm Power On | The system can be turned off with a software command. If you enable this item, the system can automatically resume at a fixed time based on the system’s RTC (realtime clock). Use the items below this one to set the date and time of the wake-up alarm. You must use an ATX power supply in order to use this feature. |

### PCI / Plug and Play Setup Page

This page sets some of the parameters for devices installed on the PCI bus and devices that use the system plug and play capability.

| Plug and Play Aware O/S | Enable this item if you are using an O/S that supports Plug and Play such as Windows 95 or 98. |
| --- | --- |
| Primary Graphics Adapter | This item indicates if the primary graphics adapter uses the PCI or the AGP bus. The default PCI setting still lets the onboard display work and allows the use of a second display card installed in a PCI slot. |
| Allocate IRQ to PCI VGA | If this item is enabled, an IRQ will be assigned to the PCI VGA graphics system. You set this value to No to free up an IRQ. |

### Load Optimal Settings

If you select this item and press Enter a dialog box appears. If you press Y, and then Enter, the Setup Utility loads a set of fail-safe default values. These default values are not very demanding and they should allow your system to function with most kinds of hardware and memory chips.

### Load Best Performance Settings

If you select this item and press Enter a dialog box appears. If you press Y, and then Enter, the Setup Utility loads a set of best-performance default values. These default are quite demanding and your system might not function properly if you are using slower memory chips or other low-performance components.

### Features Setup Page

This page sets some of the parameters for peripheral devices connected to the system.

AMIBIOS SETUP – PERIPHERAL SETUP ©1998 American Megatrends, Inc. All Rights Reserved

OnBoard FDC Enabled OnBoard Serial Port1 3F8h/COM1 OnBoard Serial Port2 2F8h/COM2 OnBoard IR Port Disabled IR Mode HPSIR IR Port IRQ N/A OnBoard Parallel Port 378h Parallel Port Mode Normal Parallel Port IRQ 7 Parallel Port DMA N/A OnBoard IDE Both USB Function Enabled USB Function for DOS Disabled Ultra DMA Support Disabled ESC : Quit  : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift)F2 : Color F6 : Load Optimal values F7 : Load Best performance values

| OnBoard FDC | Use this item to enable or disable the onboard floppy disk drive interface. |
| --- | --- |
| OnBoard Serial Port1 | Use this item to enable or disable the onboard COM1 serial port, and to assign a port address |
| OnBoard Serial Port2 | Use this item to enable or disable the onboard COM2 serial port, and to assign a port address |
| OnBoard IR Function | Use this item to define the protocol for an infrared port if you have installed an optional IR port. The choices are IrDA and ASKIR. |
| IR Mode | Use this item to define if the optional infrared port is HSPIR or ASKIR. |
| IR Port IRQ | This item displays the assigned IRQ for the optional infrared port. |
| Onboard Parallel Port | Use this item to enable or disable the onboard LPT1 parallel port, and to assign a port address. The Auto setting will detect and available address. |
| Parallel Port Mode | Use this item to set the parallel port mode. You can select SPP (Standard Parallel Port), ECP (Extended Capabilities Port), EPP (Enhanced Parallel Port), or ECP + EPP. |
| Parallel Port IRQ | Use this item to assign either IRQ 5 or 7 to the parallel port. |
| Parallel Port DMA | Use this item to assign a DMA channel to the parallel port. The options are 0, 1 and 3. |
| Onboard IDE | Use this item to enable or disable either or both of the onboard Primary and Secondary IDE channels. |
| USB Function | Enable this item if you plan to use the USB ports on this mainboard. |
| USB Function for DOS | Enable this item if you plan to use the USB ports on this mainboard in a DOS environment. |
| Ultra DMA Support | Use this item to set Ultra DMA support for IDE devices on the Primary or Secondary IDE channels. You must enable this or UDMA devices will not work at their intended speed. |

### Hardware Monitor Page

This page sets some of the parameters for the hardware monitoring function of this mainboard.

AMIBIOS SETUP – HARDWARE Monitor ©1998 American Megatrends, Inc. All Rights Reserved

--- System Hardware Monitor --- CPU TEMP. 30°C/86°F System TEMP. 24°C/75°F CPU FAN Speed System FAN Speed Vcc 5.000 V Vcc3 3.300 V Vcore 1.650 V +12V 12.000 V ESC : Quit  : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift)F2 : Color F6 : Load Optimal values F7 : Load Best performance values

| CPU / System TEMP. | These items display CPU and system temperature measurement. |
| --- | --- |
| FANs Speed & Voltage Measurements | These items indicate cooling fan speeds in RPM and the various system voltage measurements. |

### Change Password

If you highlight this item and press Enter, a dialog box appears which lets you enter a Supervisor password. You can enter no more than six letters or numbers. Press Enter after you have typed in the password. A second dialog box asks you to retype the password for confirmation. Press Enter after you have retyped it correctly. The password is then required to access the Setup Utility or for that and at start-up, depending on the setting of the  Password Check item in Advanced Setup.

#### Change or Remove the Password

Highlight this item, press Enter and type in the current password. At the next dialog box, type in the new password, or just press Enter to disable password protection.

### Exit

Highlight this item and press Enter to save the changes that you have made in the Setup Utility configuration and exit the program. When the Save and Exit dialog box appears, press Y to save and exit, or press N to exit without saving.

# Chapter 4

## Software & Applications

### Introduction

The support software CD-ROM that is included in the mainboard package contains all the drivers and utility programs needed to properly run our products. Below you can find a brief description of each software program, and the location for your mainboard version. More information on some programs is available in a README file, located in the same directory as the software.

If the operating system used in your system is Windows 98, it will automatically install all the drivers and utilities for your board. See the Auto-Installing under Windows 98 section.

### Installing Support Software

The software on the support CD-ROM is for Windows 95/NT/2000 and Windows 98. The installation procedure differs depending on which Operating System you have, but the automatic installation is now for Win98 only.

#### Installing under Windows 95/NT/2000

To install support software for Windows 95/NT/2000 follow this general procedure:

- Insert the support CD-ROM disc in the CD-ROM drive.

(The system might get an error message from the PnP function. Don’t care the message. You don’t really need that file to install the drivers)

- Use My Computer or Windows Explorer to look at the directory structure. You must use the Open command in the right-button menu. Double-clicking on the drive icon will result in an error message because the disc’s AutoRun feature doesn’t work in Windows 95/NT/2000.

- Execute the EXE file name given in the description below.

Note: The correct path name for each software driver is provided, where D: identifies the CD-ROM drive letter – modify if necessary.

#### Bus Master IDE Driver

The IDE Bus Master Drivers allows the system to properly manage the IDE channels on the mainboard. You need to install two drivers if you are running Windows 9x.

- Windows 9x – D:\IDE\AMD751\AGP\Miniport_445.EXE

and then

- Windows 9x – D:\IDE\AMD751\IDE\eide_124.EXE

#### USB Driver

The USB Driver allows the system to recognize the USB ports on the mainboard. You need to install this driver if you are running Windows 95. Windows 95 OSR2 does not require this driver.

This driver is available for:

- Win95 – D:\USB\EUSBSUPP\USBSUPP.EXE

- Win95 (Chinese) – D:\USB\CUSBSUPP\CUSBSUPP.EXE

#### Audio Driver

The Sound driver allows the system to generate optimal sound effects. This driver is available for:

- DOS & Win3.x – D:\SOUND\Driver\DosDrv\

- Windows 9x – D:\SOUND\Driver\W95-98\Drv

- Windows NT – D:\SOUND\Driver\Nt40\Drv\

There are also two Audio applications available for:

- Windows 9x – D:\SOUND\Gamut\Audio Player

- Windows 9x – D:\Yamaha-XG\S-YXG50

#### Fax/Modem Drivers and Software

Find the fax/modem drivers and software here:

- D:\Modem\Driver\9xNT

- D:\Modem\SuperVoice\ [Telecom management software]

- D:\Modem\MediaRing Talk

#### Network Adapter Driver

Find the network adapter driver here:

- D:\LAN\SiS900

#### BIOS Update Utility

The BIOS Update utility allows you to update the BIOS file on the mainboard to a newer version. You can download the latest version of the BIOS setup available for your mainboard from the website.

- D:\UTILITY\AMIFL816.EXE

#### PC-Cillin Software

The PC-cillin software program provides anti-virus protection for your system.

This program is available for:

- DOS – D:\PC-CILLIN\DOS\PCSCAN.EXE

- Win9x – D:\PC-CILLIN\WIN98\SETUP.EXE

### Auto-installing under Windows 98

The support software CD-ROM disc loads automatically under Windows 98. When you insert the CD-ROM disc in the system CD-ROM drive the Autorun feature will automatically bring up the install screen. The screen has three buttons on it, Setup, Browse CD and Exit. See the following screen illustration.

When you click on the Setup button the software installation program will run and you can select what kind of installation you want to do, as explained later in this section.

The Browse CD button is the standard Windows command that allows you to examine the contents of the disc using the Windows 98 file browsing interface.

The Exit button closes the Auto Setup window. To run the program again, reinsert the CD-ROM disc in the drive or click on AutoRun in the context sensitive menu for the CD-ROM drive icon in a file browser window.

#### Installing Software with Auto Setup

To install support software for the system board follow this procedure:

- Click on the Setup button. The install program will load and display the following screen. Click the Next button.

- Select the items that you want to setup by clicking on it (the default options are recommended). Click the Next button to proceed.

- The support software will automatically install.

Once any of the installation procedures start, software is automatically installed in sequence. You will need to follow the onscreen instructions, confirm commands and allow the computer to restart as few times as is needed to complete installing whatever software you selected to install. When the process is finished, all the support software will be installed and working.

There are some utilities that you have to manually install if you need, check to the above section.

### Using the PCI Audio Software

- Before you install the PCI Sound Pro drivers, make sure your Operating System has been installed, otherwise the onboard PCI audio might be detected as an “Other device” by the OS device manager.

- After the drivers are properly installed, choose the MULTIMEDIA icon in the CONTROL PANEL when you need to use the Software Wave-Table drivers as a MIDI output device. Select the MIDI tab and click on “C-media SoftMidi Synthesis (Win98) / Driver (Win95)”, then click “OK” to confirm.

- A Windows application named Audio Rack is provided with the PCI Sound Pro drivers, which gives you control over all the audio functions through a user interface that is as simple to use as a home stereo system. We recommend that you use the Audio Rack’s System Mixer to control your computer’s audio volume, recording devices and recording gain.

- If the devices you are using require the MIDI port as the control interface, you need to select the MULTIMEDIA icon in the CONTROL PANEL. Select the MIDI tab and click on “CM8738 MPU-401” (Win98) or “CM8738/C3DX PCI Audio External MIDI Port” (Win95), and then click “OK” to confirm.

- For more information, refer to the PCI Sound Pro manual on the CD that ships with this mainboard.

### The Four Speakers System

The onboard PCI Sound Pro audio system supports 2 wave channels (front/rear) known as the 4 speaker system. If you are running applications which use the DirectSound® 3D or A3D® audio interface, your system can simulate realistic 3D sound through a 4 speaker setup. Follow the steps below to install a 4-speaker setup.

#### Speaker Installation

Connect the front two speakers to the Line-out jack on the sound ports extension bracket. Connect the rear two speakers to the Line-in/Rear jack on the sound ports extension bracket. The original Line-in can be moved to Aux.

#### Speaker Position

Set up your speakers similar to the following figure to get the best audio result.

#### Mixer Setup

There is a 4-speakers option in the Volume Control of the Mixer when you are setting up the PCI Audio Application. Click on the 4 SPK icon to enable this option. This means that the output to the rear speakers is sent through the Line-in/Rear jack. In order to avoid hardware conflicts, DO NOT enable this option when the Line-in/Rear jack is connected with a line-in device.  While the 4 speakers mode is enabled, turn on/off the output of the front speakers and adjust the volume of the speakers so that the front/rear speakers have the same volume.

#### Demo

Execute the “Helicopter” demo in the C3D HRTF Positional Audio Demos of the PCI Audio Application. When you hear the helicopter flying behind you, it means that the rear speakers are working properly.