Chapter 1

# Getting Started

This chapter allows you to become familiar with the basic parts of your system and tells you how to set up your Book PC.

## Introduction

The key features of this Book PC include:

- The Book PC uses the VIA MVP4 chipset.

- Supports Socket 7 series processors.

- Supports maximum installed memory up to 512 MB.

- Support for Bus mastering and UltraDMA 33 modes.

- Support for Green PC standard, and Wake on Modem/LAN/Alarm.

- Built-in Graphics System supports up to 1280 x 1024 resolution in 16-bit color at 85Hz-refresh rate. Provides TV encoder for NTSC and PAL TV support (AV/S-Video).

- Built-in Sound System meets PC98 audio specification.

- HRTF 3D professional audio supports both Direct Sound 3D® and A3D®  compatible interface, and support for Four Channel Speakers.

- Optional Digital Audio Interface (SPDIF In/Out) with 24-bit stereo, 44KHz sampling rate and measured 120dB audio quality.

- Optional optic fiber interface which enables communication with MiniDisk or high-end audio systems.

- Provides a floppy disk drive, a hard disk drive, and a CD-ROM or DVD drive.

- Built-in 10/100Base LAN adapter and 56K V.90 fax/modem.

- Built-in Trend’s ChipAwayVirus ensures that the boot process is protected from virusses. The bundled PC-Cillin software provides automatic virus protection under Windows 95/98.

- The Book PC is bundled with the Windows 98 operation system, SuperVoice for fax/modem communications, MediaRing Talk for PC to PC internet phone communication, Gamut for professional audio management, including MP3 encoding and playback, WordPerfect Suite 8, the renowned office application suite, and the optional WinDVD application, for DVD playback.

## The Ideal Environment

Before anything else you should decide on a place to set it up your new computer. Below you can find a few tips to keep in mind when choosing the ideal spot.

Your Book PC computer is engineered to provide years of reliable service. However, give it the same care and protection as you would any other valuable piece of electronic equipment. Most of the good care rules are common sense.

### Where to Put the System

A cleared-off desk or table is a perfect location. Look out for the proximity of power supply, telephone and/or network cables.

You should try to avoid dirt, dust and smoke, since these might cause system failure. Also, keep your computer away from strong magnetic fields. This includes fans, air conditioners, radios, large speaker systems, and other electric motors.

Do not set up your system in a room with extreme conditions of temperature and humidity. Too hot or too cold temperature in a room causes your computer to misbehave. Dampness causes oxidation, and dryness causes static electricity.

Heat is probably the single most damaging environmental element to a computer. Because of this, never push the rear of the unit against a bookcase, wall, or anything that restricts the air from circulating freely around the entire case. If the fan outlet in the back of the computer system is blocked, heat builds up inside the computer. A marginal heat problem can be aggravated by accumulated filth from dirt, dust and smoke.

Likewise, do not stack magazines on top of, under, or behind the computer system or the video monitor.

Do not place the computer in direct sunlight, near a radiator, lamp, or a heat vent. Remember that heat causes electronic circuitry to behave in strange ways. When computer components heat up, they tend to fail. If this happens, it may not be a permanent failure, but could lead to one.

### Extra Protection

Electrical surges and spikes can cause serious damage to your system. Use a power filter/surge protector on the system unit and everything connected to it. If just one device is not connected, damaging surges can enter the system through unprotected peripheral and damage everything connected to it.

A power strip/surge protector provides protection against many types of voltage spikes that can travel on the power line and damage your system, but it isn’t much help against lightning. If severe weather of an electrical storm threatens, it’s a good idea to unplug the entire system from the power, and telephone and/or network lines, before the storm starts.

## Maintaining and Caring for the System

Follow the suggestions below to keep your valuable system in good condition.

### Cleaning the System

Use a damp, lint-free cloth to clean the system unit and monitor. Avoid abrasives or solvents, they can permanently damage the finish. Use a soft cloth and window cleaner to clean the monitor. Turn off the monitor. Then squirt a little cleaner on the cloth and apply it to the screen.

All diskette drives and tape backup units should be cleaned periodically. Cleaning kits are available from a variety of sources. Carefully follow the instructions supplied with the cleaning kit you use.

Subjecting a system to a dusty or smoky environment significantly shortens its life and increases the probability of failure. If you use the system in a dirty environment, open it periodically and vacuum the board and components with a small vacuum designed for this kind of work. Don’t loosen anything in the process.

If the mouse pointer or cursor starts jumping when you move the mouse, it is probably caused by a build-up of dirt on the rollers inside the mouse.

If you spill liquid inside the keyboard, unplug the keyboard immediately and turn it upside down to allow the liquid to drain out. Let the keyboard dry overnight before attempting to use it again.

### Transporting the System

Dropping or jarring the system can damage the hard disk drive, particularly if the drive is reading or writing when it is dropped. Hard disk drives are less vulnerable to shock when they are shut off. Consequently, you should always power down the system unit before moving it.

During transport, pack the system unit in its original packing boxes and cushioning materials.

### Backups

There hasn’t been a hard disk made that is guaranteed not to lose its data. Hard disk reliability has improved significantly in the last few years, and it is unlikely that yours will malfunction and lose its data. Still it is not impossible. It is a good idea to make backup copies of your files on a regular basis. You can obtain a tape or backup unit, or optical storage device, or another device that will allow you to save your whole configuration of files at once. Software can be used to make that process automated. You should try to back up your files on a regular basis.

### Shock Precautions

This list is important to protect yourself from electrical shock and your system from damage.

- Be sure all cables are free from table or desk legs resting on them. Arrange cables out of traffic areas.

- Unplug all power cords from their power sources if severe weather threatens. When you disconnect a cable, always pull on the plug or connector, not on the cable itself.

- Know the static electricity precautions.

- Always disconnect the power before opening the system unit. Just because the power is off does not necessarily mean the computer is harmless. Never take chances with the hazards of electricity.

- Plug all three-prong cable into properly grounded power sources. Computers prefer steady and reliable electricity. Use a power filter/surge protector for protection against electrical surges. Do not use adapter plugs or remove the grounding prong. Try to connect everything without the use an extension cord. If you must use one, use a three-wire cable with grounded plugs.

### Care of the Computer

- Do not set up your system in a room with extreme conditions of temperature and humidity.

- Do not allow heat to build up inside of the system unit.

- Never turn the power button off or move the system while the hard disk drive is reading or writing.

- Do not spray any kind of cleaning liquid directly on the case, keyboard or monitor. Instead apply it to a soft cloth. Always turn the power off and unplug each power cable from its power source before cleaning.

- Use the computer in as clean an environment as possible. Cover the keyboard, monitor and system unit.

- Never push foreign objects into the slots on the riser card. Doing this may short out interior components and could cause a fire.

- Do not open the diskette drive door while the drive is active, the indicator LED will be lit. This damages the disk and/or the drive.

- Avoid dropping or jarring your computer. This loosens memory chips, expansion cards, and can severely damage the hard disk drive. Always shut down the computer when moving it, even if it’s only to the other side of the your desk.

- Keep your computer away from magnetic fields.

- Do not eat or drink while using the computer.

## Unpacking

If your system arrives in very cold or damp weather, you should not apply power to the system unit until it has reached room temperature, and no condensation is present. Exposing a cold computer to a warm room causes condensation that could damage the computer or shorten its life. Allow the box to warm up to room temperature for several hours before opening them. Do not plug in the system until all moisture from any condensation has completely dried.

### Checklist

Some vendors may ship certain components as standard, while other vendors treat the same components as optional. In its most basic configuration, your Book PC should include the following:

- A Book PC computer with built-in processor, system memory, hard disk drive, CD-ROM (or DVD) drive, and diskette drive.

- A system unit power cord and a telephone line for modem connection

- A Windows software package, including manual and CD-ROM

- A support CD-ROM and/or disk(s) with system drivers and bundled software.

- Documentation, including this User’s manual and the WordPerfect manual.

The following are normally optional, but some vendors may include them in the basic package. Some items may not be available in certain countries, or some vendors may choose not to carry all the optional items:

- Keyboard and/or mouse

- Video monitor

- Audio speakers

- Additional peripheral devices and/or cables

Note: Contact your system vendor for information on the availability of optional items.

Until the system is up and running, it’s a good idea to leave all the software packages intact. All of it is pre-installed and automatically starts when you turn on the power. They are provided for backup purposes only.

Arrange all the unpacked items on an uncluttered desk or table when you plan to set up and use the system computer. Each piece should be close enough together so that cables can reach to the appropriate connectors. Be sure there is a wall outlet close to the table or desk.

## Installing the System

Most of the work of setting up your system computer is already done. Before shipping your system, we installed all the software and specified the correct hardware configuration in the setup program.

The part we have left for you is comparatively easy. Anyone who can orient cables to mate with connectors can set up this Book PC computer. You won’t have to open the computer to plug anything in. The steps on the next few pages are easy to follow, and there are lots of pictures to help show you the correct way.

First we introduce you to the system components, and then we will help you set up the system and install all the cables.

Warning: In this section you are instructed to connect several cables. To avoid damage to your computer and peripherals, never plug in or unplug a cable when the system unit power is ON. Always shut off the system first, only then make any necessary cable connections or changes.

## Introducing the System

Before you actually set up your system and plug everything in, it’s a good idea to locate all its features, switches and controls. The next few illustrations familiarize you with the system computer external features.

### Looking at the Front

With your computer in front of you, use the picture below to locate each of the labeled parts. Chapter 2 goes into more detail on some of the labeled parts and their usage.

- Power switch – The power switch is located on the right-hand side of the front panel. Press the power switch once to turn your computer on, and press it again to turn your computer off.

- Reset switch – Use this switch to ‘reboot’ the system instead of using the power switch button.

- Indicator LEDs – The front panel has three indicator lights. The indicator LEDs turn on to show that a particular function is active. From right to left the LEDs indicate the following: the Power LED turns green as the system is powered on; the HDD LED lights up if any hard disk activity is detected; the Suspend LED blinks if the system is suspended.

- Diskette Drive – The diskette drive, on the right side of the front panel, can read from, or write to, 3.5” diskettes with capacities of either 720K or 1.44 MB. The diskette drive provides a convenient way of storing and transferring short files.

- CD-ROM/DVD Drive – The drive bay on the left hand side is either installed with a CD-ROM or DVD drive. This option is factory installed. CD-ROM disks can store over 600 MB of audio, video, or data. DVD disks can store up to 4 GB and are currently commonly used to distribute high-quality video. Usually your system identifies the CD-ROM/DVD drive as drive D:. Access to a CD/DVD-ROM is quite fast. CD and DVD disks are used to distribute large software applications, and audio and video files that require a lot of capacity. You can play audio and video files using the supplied Gamut audio player or the optional WINDVD software.

### Looking at the Back

The back of the system is where most external connectors are.

Digital Audio (optional)

Line/Phone

LAN

###### Game port Connector

AC Power Connector

PS/2 Connectors

USB Connectors

S-Video Connector

Video Connector

Fan Outlet

Audio Jacks

VGA Connector

Parallel port Connector

- AC Power Connector – Plug the main power cord in here. Within the range of 115 or 230V AC, the power adapter will automatically sense and adapt to the input power voltage.

- Fan Outlet – A fan inside the case cools the internal parts. Do not block this outlet.

- LAN Jack – Use this jack to plug a network cable in your system.

- Game port Connector – The 15-pin Game port can be used to connect your computer to an IBM-compatible joystick or similar device, or to a MIDI device such as a synthesizer.

- Audio Jacks – The three audio jacks provide you with the ability to connect an external audio source or an external microphone to the system. From right to left, the Microphone jack is used to input sound from an external microphone into your Book PC; the Line in jack is used to input stereo sound from other devices, such as a radio or tape recorder, into your computer; and the Speaker jack is used to output sound generated by your computer to an external device, such as stereo loudspeakers.

- Parallel Port Connector – The parallel port provides a connection for a parallel printer or other parallel communications device.

- VGA Connector – The external monitor port allows you to connect any standard computer monitor to your system.

- Video Connector – The RCA video jack can be used to output video to devices that use RCA plugs such as most TV receivers.

- S-Video Connector – The S-video jack can be used to output video to devices that use S-video plugs such as camcorders and some TV receivers.

- Digital Audio Jack – The Digital Audio jack provides a digital (SPDIF) interface with an optional external audio device, and at the same time serves as an auxiliary Line in port to import sounds while Four Speakers mode is enabled. An optional device and optic fiber cable are needed in order to make use of the digital interface.

- Line/Phone Jacks – The RJ-11 jacks can be used to connect a phone line to the internal modem device. Use the Line jack to connect a wall socket line to the Book PC, use the Phone jack to export the line to a telephone receiver.

- LAN Jack – The RJ-45 jack can be used to connect the Book PC to an ethernet network.

- USB Connectors – The USB (Universal Serial Bus) port can be used to connect to USB devices.

- PS/2 Connectors – The two PS/2 ports can be used to connect an external pointing device and keyboard.

## Connecting the Cables

This section of setting up your new computer is very important. It talks about attaching all the cables so that you can begin using the system.

The procedure is straightforward and easy to do, however, be sure to follow each step carefully and don’t leave any steps out.

- Connect a keyboard and pointing device through the PS/2 connectors

- Connect any speakers and/or microphones that you might have.

- Connect a computer monitor to the VGA connector.

- Connect phone lines to the Line/Phone jacks.

- Connect any USB devices you want to use with your system.

- Connect any printer or other parallel device you might want to use to the Parallel port.

- Connect a TV receiver to the Video or S-Video Connectors.

- Connect a network cable to the LAN connector.

- Connect any Game/MIDI devices you might want to use.

- Plug in the power cable, one end goes into the back of the system, the other end goes into a grounded outlet or power strip/surge protector.

- Make sure all other peripheral devices are properly connected to a power source if they do not draw their power from the system.

You are now ready to start up your system. Check Chapter 2 for detailed information on starting up your system for the first time.