This publication, including all photographs, illustrations and software, is protected under international copyright laws, with all rights reserved. Neither this manual, nor any of the material contained herein, may be reproduced without the express written consent of the manufacturer.

The information in this document is subject to change without notice. The manufacturer makes no representations or warranties with respect to the contents hereof and specifically disclaims any implied warranties of merchantability or fitness for any particular purpose. Further, the manufacturer reserves the right to revise this publication and to make changes from time to time in the content hereof without obligation of the manufacturer to notify any person of such revision or changes.

### Trademarks

IBM, VGA, and PS/2 are registered trademarks of International Business Machines.

Intel, Pentium, Pentium-II, Pentium-III, MMX, and Celeron are registered trademarks of Intel Corporation.

Microsoft, MS-DOS and Windows 95/98/NT are registered trademarks of Microsoft Corporation.

Sound Blaster is a trademark of Creative Technology Ltd.

PC-cillin and ChipAwayVirus are trademarks of Trend Micro Inc.

AMI is a trademark of American Megatrends Inc.

A3D is a registered trademark of Aureal Inc.

Gamut is a registered trademark of Formosoft International Inc.

SuperVoice is a registered trademark of Pacific Image Communications Inc.

MediaRing Talk is a registered trademark of MediaRing Inc.

Other names used in this publication may be trademarks and are acknowledged.

Copyright © 2000

All Rights Reserved

M583LMR, V1.2

A156X/May 2000

Federal Communications Commission (FCC)

This equipment has been tested and found to comply with the limits for a Class B digital device, pursuant to Part 15 of the FCC Rules. These limits are designed to provide reasonable protection against harmful interference in a residential installation. This equipment generates, uses, and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communications. However there is no guarantee that interference will not occur in a particular installation. If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

- Reorient or relocate the receiving antenna.

- Increase the separation between the equipment and the receiver.

- Connect the equipment onto an outlet on a circuit different from that to which the receiver is connected.

- Consult the dealer or an experienced radio/TV technician for help.

Shielded interconnect cables and shielded AC power cable must be employed with this equipment to insure compliance with the pertinent RF emission limits governing this device. Changes or modifications not expressly approved by the system’s manufacturer could void the user’s authority to operate the equipment.

##### Declaration of Conformity

This device complies with part 15 of the FCC rules. Operation is subject to the following conditions:

- This device may not cause harmful interference, and

- This device must accept any interference received, including interference that may cause undesired operation.

Canadian Department of Communications

This class B digital apparatus meets all requirements of the Canadian Interference-causing Equipment Regulations.

Cet appareil numérique de la classe B respecte toutes les exigences du Réglement sur le matériel brouilieur du Canada.

Table of Contents

Chapter 1: Introduction	1

Key Features	2

Package Contents	6

Static Electricity Precautions	7

Pre-Installation Inspection	7

Chapter 2: Mainboard Installation	7

Mainboard Components	8

I/O Ports	9

Install A CPU	9

Install Memory	11

Setting Jumper Switches	12

Install the Mainboard	14

Install the Extension Brackets	16

Optional Extension Brackets	17

Install Other Devices	18

Expansion Slots	20

Chapter 3: BIOS Setup Utility	21

Introduction	21

Running the Setup Utility	22

Standard CMOS Setup Page	23

Advanced Setup Page	24

Power Management Setup Page	26

PCI / Plug and Play Setup Page	28

Load Optimal Settings	29

Load Best Performance Settings	29

Features Setup Page	29

CPU PnP Setup Page	31

Hardware Monitor Page	32

Change Password	32

Exit	33

Chapter 4: Software & Applications	35

Introduction	35

Installing Support Software	35

Auto-installing under Windows 98	37

Using the PCI Audio Software	39

The Four Speakers System	40

Appendix A: Corel WordPerfect Suite 8 	A1

Welcome to Corel WordPerfect Suite 8 	A2

Installing Corel WordPerfect Suite 8 	A6

Learning how to use Corel WordPerfect Suite 8 	A9

Support and Services 	A13

Appendix B: Gamut 	B1

Introduction 	B2

Before Installation 	B3

Installation 	B4

Produce MP3 fileUse CD-Cashier 	B5

Play MP3 fileUse Musician 	B7

Play music CDUse 3DFS-ACD 	B8

Play MIDI filesUse Midier 	B9

Recording audio dataUse Voice-Catcher 	B10

# Chapter 1

## Introduction

This mainboard supports all Socket 7 processors including newer designs which feature a 100 MHz system bus. The mainboard firmware supports CPU Plug and Play so that the system will automatically adopt the correct configuration for the Socket-7 processor that you install.

This mainboard uses the ALi Art X chipset which integrates a 3D 128-bit AGP Graphics Accelerator with Hardware Geometry Engine and has an embedded 10BaseT/100BaseTX Ethernet Network Interface. The mainboard has a built-in PCI 3D Sound System and a V.90 Fax/Modem DAA module is shipped with the mainboard. In addition, the mainboard has a full set of ATX I/O Ports including PS/2 keyboard and mouse ports, two USB ports, a parallel port, a serial port and a VGA port.

This mainboard has all the features you need to develop a powerful multimedia workstation that is network ready, and has built-in communications. The board is Flex-ATX size and has power connectors for an ATX power supply.

### Key Features

The key features of this mainboard include:

#### Socket-7 Processor Support

- Supports all recent socket-7 processors including the Intel P55C (Pentium MMX), the Cyrix/IBM 6x86L/6x86MX /MII, the AMD K6/K6-2/K6-III, IDT C6, and WinChip 2/2A CPUs

- Supports socket-7 processors with system bus frequencies of 66/100 MHz

- CPU Plug and Play support lets the firmware automatically configure the CPU

#### Memory Support

- Two DIMM slots for 168-pin SDRAM memory modules

- Support for 66MHz, 100MHz memory bus

- Maximum installed memory is 2 x 512MB = 1 GB

- Support Symmetric (128-bit) architecture

Note: For the best performance, please use the same memory

size with the same single/double side on the both two

DIMM slots

#### Expansion Slots

- Two 32-bit PCI slots

#### Onboard IDE channels

- Primary and Secondary PCI IDE channels

- Support for PIO (programmable input/output) modes

- Support for Multiword DMA modes

- Support for Bus Mastering and Ultra DMA 33/66 modes

#### Power Supply and Power Management

- ATX power supply connector

- ACPI and previous PMU support, suspend switch, keyboard power on/off

- Supports Wake on Modem and Wake on Alarm

#### Built-in Graphics System

- Onboard advanced 128-bit 2D/3D AGP Graphics Accelerator

- Supports Hardware Geometry Engine

- Shared SDRAM memory architecture allows a maximum of 32 MB main memory to act as frame buffer

- Supports high resolutions up to 1600x1200 and 32-bit color pipeline, up to 2048x2048 Texture size and Virtual screen up to 4096x4096

- High performance 128-bit 2D GUI/DirectDraw acceleration

- Video Acceleration for Direct7, DirectShow, MPEG-1, MPEG-2 and Indeo

#### Sound System

- Complies with the PC98 audio specification

- 16-bit CODEC for full-duplex playback and recording

- HRTF 3D professional audio supports both Direct Sound 3D® and A3D®-compatible interfaces plus support for 4-channel speakers

- Driver support for MS-DOS, Microsoft Windows 95/98/2000/NT 4.0

- Built-in 32ohm earphone buffer and 3D surround sound

- Provides MPU-401 Game/MIDI port and legacy Sound Blaster 16 support

- Downloadable Wave-table Synthesizer supports Direct Music®

- Digital Audio Interface (SPDIF In/Out) with 24-bit stereo, 48KHz sampling rate and measured 120dB audio quality

- Optional optic fiber interface enables communication with MiniDisc player/recorder or a high-end audio system

- Stereo Mixer supports analog mixing from CD-Audio and Line In or digital mixing from voice, FM/Wave-table and digital CD-Audio

#### Onboard I/O Ports

- Provides PC99 Color Connectors for easy peripheral device connections

- Floppy disk drive connector with 1Mb/s transfer rate

- One serial port with 16550-compatible fast UART

- One parallel port with ECP and EPP support

- Two USB ports

- Two PS/2 ports for keyboard and mouse

- One infrared port header for optional module

#### Built-in Ethernet LAN

- 10BaseT/100BaseTX Ethernet LAN

- LAN controller integrates Fast Ethernet MAC and PHY compliant with IEEE802.3u 100BASE-TX, 10BASE-T and ANSI X3.263 TP-PMD standards

- Compliant with ACPI 1.0 and the Network Device Class Power Management 1.0

- High Performance provided by 100Mbps clock generator and data recovery circuit for 100Mbps receiver

#### Fax/Modem DAA Module

- 56 Kbps Fax/Modem DAA module

- Supports V.90, V.34, V.32bis, V.32, V.22bis, V.22

- Supports Auto Fallback and MNP 5, V.42bis data compression with 115,200-compatible Virtual UART

- Requires 16MB RAM and Microsoft Windows 95/98/NT

#### Onboard Flash ROM

- Automatic CPU and board configuration

- Supports Plug and Play configuration of peripheral devices and expansion cards

- Built-in virus protection using Trend’s ChipAwayVirus provides boot process virus protection.

#### Bundled Software

- PC-Cillin provides automatic virus protection under Windows 95/98

- SuperVoice is data, fax and voice communication software

- Gamut2000 provides professional audio features included MP3 encoding/playback

- MediaRing Talk provides PC to PC or PC to Phone internet phone communication

- S-YXG50 is music synthesizer software to playback MIDI files on the system

- Corel WordPerfect Suite 8 is an office application suite under Windows® (optional)

#### Dimensions

- Flex-ATX form factor (22.9cm x 19cm)

### Package Contents

Your mainboard package ships with the following items:

- The mainboard

- This User’s Guide

- 1 UDMA/33 IDE cable

- 1 Floppy disk drive cable

- 1 Fax/Modem DAA module

- Support software on CD-ROM disk

#### Optional Accessories

You can purchase the following optional accessories for this mainboard.

- Optic module extension bracket

- UDMA/66 IDE cable

### Static Electricity Precautions

Components on this mainboard can be damaged by static electricity. Take the following precautions when unpacking the mainboard and installing it in a system.

- Keep the mainboard and other components in their original static-proof packaging until you are ready to install them.

- During installation, wear a grounded wrist strap if possible. If you don’t have a wrist strap, discharge static electricity by touching the bare metal of the system chassis.

- Handle the mainboard carefully by the edges. Avoid touching the components unless it is absolutely necessary. During installation put the mainboard on top of the static-protection packaging it came in with the component side facing up.

### Pre-Installation Inspection

- Inspect the mainboard for damage to the components and connectors on the board.

- If you suspect that the mainboard has been damaged, do not connect power to the system. Contact your mainboard vendor and report the damage.

# Chapter 2

## Mainboard Installation

To install this mainboard in a system, follow the procedures in this chapter:

- Identify the mainboard components

- Install a CPU

- Install one or more system memory modules

- Verify that any jumpers or switches are set correctly

- Install the mainboard in a system chassis (case)

- Connect any extension brackets or cables to the mainboard connector headers

- Install any other devices and make the appropriate connections to the mainboard connector headers.

Note:

- Before installing this mainboard, make sure jumper JP4 is set to Normal, the default setting is set to Clear CMOS. See this chapter for information on locating JP4 and the setting options.

- Never connect power to the system during installation. Doing so may damage the mainboard.

### Mainboard Components

Use the diagram below to identify the major components on the mainboard.

Note: Any jumpers on your mainboard that do not appear in this illustration are for testing only.

### I/O Ports

The illustration below shows a side view of the built-in I/O ports on the mainboard.

### Install A CPU

This mainboard has a Socket-7 which may be installed with any of the socket-7 processors including the Intel P55C (MMX) series, the Cyrix/IBM 6x86L/6x86MX/MII series, the AMD K6/K6-2/K-III series, the IDT C6/Winchip 2/2A series. The mainboard supports system bus speeds of 66, 100 MHz.

Do not try to install a Socket-370 processor in the Socket-7. A Socket-370 processor such as the PPGA Celeron or FCPGA Pentium III does not fit in the Socket-7.

The board supports CPU plug and play, so the system can automatically run the installed processor with the correct clock speed and the correct system bus frequency. To automatically configure the processor, use the BIOS setup program to select the clock speed and system bus frequency. See chapter three for more information.

To ensure reliability, make sure that your socket-7 processor is fitted with a heatsink/cooling fan assembly.

The socket-7 processor installs into the ZIF (Zero Insertion Force) socket-7 on the mainboard.

- Locate the Socket-7 and FAN1. Pull the locking lever out slightly from the socket and raise it to the upright position.

Socket-7

Pin-1 Corner

FAN1

- On the processor, identify the Pin-1 corner by its beveled edge.

- On the Socket-7, identify the Pin-1 corner. The Pin-1 corner is at the end of the locking lever when it is locked.

- Match the Pin-1 corners and insert the processor into the socket. No force is required and the processor should drop into place freely.

- Swing the locking lever down and hook it under the catch on the side of the socket. This secures the CPU in the socket.

- All processors should be installed with a combination heatsink/cooling fan, connect the cable from the fan to the CPU fan power connector FAN1.

### Install Memory

The mainboard has two DIMM sockets for system memory modules. You must install at least one memory module in order to use the mainboard.

DIMM2

DIMM1

For this mainboard, you must use 168-pin, 3.3V unbuffered SDRAM memory modules. If the installed CPU uses a 100 MHz system bus, you must use PC100 memory. If the installed CPU uses a 66 MHz system bus, you must use PC66 memory. You can install any size memory module from 16 MB to 512MB, so the maximum memory size is 2 x 512MB = 1GB.

The edge connectors on the memory modules have cut outs, which coincide with spacers in the DIMM sockets so that memory modules can only be installed in the correct orientation.

To install a module, push the retaining latches at either end of the socket outwards. Position the memory module correctly and insert it into the DIMM socket. Press the module down into the socket so that the retaining latches rotate up and secure the module in place by fitting into notches on the edge of the module.

### Setting Jumper Switches

Jumpers are sets of pins which can be connected together with jumper caps. The jumper caps change the way the mainboard operates by changing the electronic circuits on the mainboard. If a jumper cap connects two pins, we say the pins are SHORT. If a jumper cap is removed from two pins, the pins are OPEN.

JP3

1

21

JP700

JP4

1

JP5

1

#### Jumper JP3: Modem & Audio System Enable/disable

This jumper consists of two sets of 3-pin jumpers (pins 1,3,5 and pins 2,4,6). Use pins 1,3,5 to enable or disable the onboard sound system. Use pins 2,4,6 to enable or disable the onboard modem.

| Function | Jumper Setting |
| --- | --- |
| Enable Onboard Audio | Short Pins 1-3 |
| Disable Onboard Audio | Short Pins 3-5 |
| Enable Onboard Modem | Short Pins 2-4 |
| Disable Onboard Modem | Short Pins 4-6 |

Note: If you use pins 1,3,5 to disable the onboard sound system, the onboard Fax/Modem is also disabled, even if you have set pins 2,4,6 the Enable Fax/Modem setting.

#### Jumper JP4: Clear CMOS Memory

Use this jumper to clear the contents of the CMOS memory. You may need to clear the CMOS memory if the settings in the Setup Utility are incorrect and prevent your mainboard from operating. To clear the CMOS memory, disconnect all the power cables from the mainboard and then move the jumper cap into the CLEAR setting for a few seconds.

| Function | Jumper Setting |
| --- | --- |
| Normal Operation | Short Pins 1-2 |
| Clear CMOS Memory | Short Pins 2-3 |

#### Jumper JP5: Enable/Disable Onboard LAN

Use this jumper to enable or disable the onboard network adapter.

| Function | Jumper Setting |
| --- | --- |
| Enable Onboard LAN | Short Pins 1-2 |
| Disable Onboard LAN | Short Pins 2-3 |

#### Jumper JP700: Keyboard Power On Selector

If you enable the keyboard power on feature, you can use hot keys on your keyboard as a power on/off switch for the system.

Note: The system must provide 1A on the +5VSB (+5V Standby) signal before using the Keyboard Power On function.

| Function | Jumper Setting |
| --- | --- |
| Enable Keyboard Power On | Short Pins 1-2 |
| Disable Keyboard Power On | Short Pins 2-3 |

### Install the Mainboard

Install the mainboard in a system chassis (case). The board is a flex-ATX size mainboard with a twin-tier of I/O ports. You can install this mainboard in any ATX case. Special micro-ATX cases are also available with a reduced number of expansion slot bays and a smaller power supply unit. Ensure that your case has an I/O cover plate that matches the ports on this mainboard.

Install the mainboard in a case. Follow the instructions provided by the case manufacturer using the hardware and internal mounting points on the chassis.

PWR2

FAN2

J7

Connect the power connector from the power supply to the PWR2 connector on the mainboard.

If there is a cooling fan installed in the system chassis, connect the cable from the cooling fan to the FAN2 fan power connector on the mainboard.

Connect the case switches and indicator LEDs to the J7 switch and LED connector header. See the illustration below for a guide to the J7 connector pin assignments.

21

22

Power LED

Pins 2-4-6

Speaker

Pins 1-3-5-7

Keylock

Pins 8-10

HDD LED

Pins 15-16

12

1

Reset Switch

Pins 17-18

Power Button

Pins 21-22

Suspend LED

Pins 19-20

### Install the Extension Brackets

The extension brackets are used to connect features on the mainboard to external connectors that can be attached to the system chassis. Follow the steps below to install the extension brackets.

Note: All the ribbon cables used on the extension brackets have a red stripe on the Pin-1 side of the cable.

#### Fax/Modem Module

The Fax/Modem DAA module plugs directly into the mainboard in line with to an expansion slot opening in the system chassis. When you remove the slot cover from the system chassis, you can access the LINE and TEL RJ11 connectors on the metal edge of the Fax/Modem DAA module.

J2-MODEM Header

1

- Locate the J2-MODEM header on the mainboard.

- Plug the Fax/Modem DAA module into the MODEM header.

- Remove the modem header slot cover.

Modem DAA Module

Modem Header

Line & Tel

RJ11 Sockets

### Optional Extension Brackets

For this mainboard, you can also obtain an Optic module extension bracket. Install it by following the steps below.

#### Optic Module Extension Bracket

This bracket has two SPDIF jacks for digital audio in and digital audio out, and an auxiliary jack for a stereo Line In device. It also provides a pair of optic fiber interfaces which enable connection to MiniDisk or high-end audio systems.

SPDIF Out

Optic Out

Optic In

SPDIF In

Aux In

Optic Module

Extension Bracket

SPDFI/O Header

1

- On the mainboard, locate the SPDFI/O header for this bracket.

- Plug the cable from the bracket into the header.

- In the system chassis, remove a blanking plate from one of the expansion slots and install the extension bracket in the slot. Use the screw that held the blanking plate in place to secure the extension bracket.

#### Internal Digital Audio

If you have an optional internal digital audio cable, you can also use it to connect the digital audio output connector of a CD-ROM or DVD-ROM drive to the pins 5-6 of SPDFI/O header.

### Install Other Devices

Install and connect any other devices in the system following the steps below.

IDE1

IDE2

FDC1

1

1	1

#### Floppy Disk Drive

The mainboard ships with a floppy disk drive cable that can support one or two drives. Drives can be 3.5” or 5.25” wide, with capacities of 360K, 720K, 1.2MB, 1.44MB, or 2.88MB.

Install your drives and connect power from the system power supply. Use the cable provided to connect the drives to the floppy disk drive header FDC1.

#### IDE Devices

IDE devices include hard disk drives, high-density diskette drives, and CD-ROM or DVD-ROM drives, among others.

The mainboard ships with an IDE cable that can support one or two IDE devices. If you connect two devices to a single cable, you must configure one of the drives as Master and one of the drives as Slave. The documentation of the IDE device will tell you how to configure the device as a Master or Slave device. The Master device connects to the end of the cable.

Install the device(s) and connect power from the system power supply. Use the cable provided to connect the device(s) to the Primary IDE channel connector IDE1 on the mainboard.

If you want to install more IDE devices, you can purchase a second IDE cable and connect one or two devices to the Secondary IDE channel connector IDE2 on the mainboard. If you have two devices on the cable, one must be Master and one must be Slave.

#### Internal Sound Connections

If you have installed a CD-ROM drive or DVD-ROM drive, you can connect the drive audio cable to the onboard sound system.

On the mainboard, locate the two 4-pin connectors CD1 and CD2. There are two kinds of connector because different brands of CD-ROM drive have different kinds of audio cable connectors. Connect the cable to the appropriate connector.

CD1

CD2

1

#### Infrared Port

You can connect an infrared port to the mainboard. You can purchase this option from third-party vendors.

1 VCC

2 FIR

3 IRRX

4 GND

5 IRTX

6 NC

7 GND

IR1

- Locate the infrared port IR1 header on the mainboard.

- If you are adding an infrared port, connect the ribbon cable from the port to the header and then secure the port to an appropriate place in your system chassis.

### Expansion Slots

This mainboard has two 32-bit PCI expansion slots. The PCI slot PCI2 is shared with the USB port. It means that you can use either one of shared both but you cannot use the both at the same time.

PCI2

PCI1

Follow the steps below to install a PCI expansion card.

- Select a free PCI slot.

- Remove the slot cover for the expansion slot from the system chassis.

- Insert the expansion card edge connector into the slot and press it firmly down into it so that it is fully inserted.

- Secure the expansion card bracket to the system chassis using the screw that held the slot cover in place.

# Chapter 3

## BIOS Setup Utility

### Introduction

The BIOS Setup Utility records settings and information about your computer such as the date and time, the kind of hardware installed, and various configuration settings. Your computer uses this information to initialize all the components when booting up and functions as the basis for coordination between system components.

If the Setup Utility configuration is incorrect, it may cause the system to malfunction. It can even stop your computer from booting properly. If this happens, you can use the clear CMOS jumper to clear the CMOS memory used to store the configuration information, or you can hold down the Page Up key while you reboot your computer. Holding down the Page Up key also clears the CPU PnP Setup information, you may need to set the CPU speed again.

You can run the setup utility and manually make changes to the configuration. You might need to do this to configure some of the hardware that you install on or connect to the mainboard, such as the CPU, system memory, disk drives, etc.

### Running the Setup Utility

Each time your computer starts, before the operating system loads, a message appears on the screen that prompts you to “Hit <DEL> if you want to run SETUP”. When you see this message, press the Delete key and the Main menu page of the Setup Utility appears on your monitor.

AMIBIOS SIMPLE SETUP UTILITY – VERSION 1.20 ©1999 American Megatrends, Inc. All Rights Reserved

Standard CMOS Setup Features Setup Advanced Setup CPU PnP Setup Power Management Setup Hardware Monitor PCI / Plug and Play Setup Change Password Load Optimal Settings Exit Load Best Performance Settings ESC: Quit  : Select Item (Shift)F2 : Change Color F5 : Old Values F6: Optimal values F7: Best performance values F10: Save&Exit

You can use the cursor arrow keys to highlight any of the options on the main menu page. Press Enter to select the highlighted option. To leave the setup utility, press the Escape key. To cycle through the Setup Utility’s optional color schemes hold down the Shift key and press F2.

Some of the options on the main menu page lead to tables of items with installed values. In these pages, use the cursor arrow keys to highlight the items, and then use the PgUp and PgDn keys to cycle through the alternate values for each of the items. Other options on the main menu page lead to dialog boxes which require you to answer Yes or No by hitting the Y or N keys.

If you have already made changes to the setup utility, press F10 to save those changes and exit the utility. Press F5 to reset the changes to the original values. Press F6 to install the setup utility with a set of default values. Press F7 to install the setup utility with a set of high-performance values.

### Standard CMOS Setup Page

Use this page to set basic information such as the date and time, the IDE devices, and the diskette drives. If you press the F3 key, the system will automatically detect and configure the hard disks on the IDE channels.

AMIBIOS SETUP – STANDARD CMOS SETUP ©1999 American Megatrends, Inc. All Rights Reserved

Date (mm/dd/yy) : Tue May 16, 2000 Time (hh/mm/ss) : 14:26:53 LBA Blk PIO 32Bit Type Size Cyln Head WPcom Sec Mode Mode Mode Mode Pri Master : Auto On Pri Slave : Auto On Sec Master : Auto On Sec Slave : Auto On Floppy Drive A : 1.44MB 3 1/2” Floppy Drive B : Not Installed

Month : Jan – Dec ESC : Exit Day : 01 – 31  : Select Item Year : 1901 – 2099 PU/PD/+/- : Modify (Shift)F2 : Color F3 : Detect All HDD

| Date & Time | Use these items to set the system date and time |
| --- | --- |
| Pri Master Pri Slave Sec Master Sec Slave | Use these items to configure devices connected to the Primary and Secondary IDE channels. To configure an IDE hard disk drive, choose Auto. If the Auto setting fails to find a hard disk drive, set it to User, and then fill in the hard disk characteristics (Size, Cyls, etc.) manually. If you have a CD-ROM drive, select the setting CDROM. If you have an ATAPI device with removable media (e.g. a ZIP drive or an LS-120) select Floptical. |
| Floppy Drive A Floppy Drive B | Use these items to set the size and capacity of the floppy diskette drive(s) installed in the system. |

### Advanced Setup Page

Use this page to set more advanced information about your system. Take some care with this page. Making changes can affect the operation of your computer.

AMIBIOS SETUP – ADVANCED SETUP ©1999 American Megatrends, Inc. All Rights Reserved

Trend ChipAway Virus Enabled Share Memory Size 8MB 1st Boot Device IDE-0 2nd Boot Device Floppy 3rd Boot Device CDROM Try Other Boot Devices Yes S.M.A.R.T. for Hard Disks Disabled BootUp Num-Lock On Floppy Drive Swap Disabled Floppy Drive Seek Disabled PS/2 Mouse Support Enabled Password Check Setup Boot To OS/2 > 64MB No Internal Cache Enabled System BIOS Cacheable Enabled DRAM Driving Current Auto ESC : Quit  : Select Item SDRAM CAS Latency 3 Clock F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift)F2 : Color F6 : Load Optimal values F7 : Load Best performance values

| Trend ChipAway Virus | This mainboard has built-in virus protection in the firmware. Use this item to enable or disable the built-in virus protection. |
| --- | --- |
| Share Memory Size | This item lets you allocate a portion of the main memory for use by the onboard VGA display. |
| 1st Boot Device 2nd Boot Device 3rd Boot Device | Use these items to determine the device order the computer uses to look for an operating system to load at start-up time. |
| Try Other Boot Device | If you enable this item, the system will also search for other boot devices if it fails to find an operating system from the first three locations. |
| S.M.A.R.T. for Hard Disks | Enable this item if any IDE hard disks support the S.M.A.R.T. (Self-Monitoring, Analysis and Reporting Technology) feature. |
| BootUp Num-Lock | This items determines if the Num Lock key is active or inactive at system start-up time. |
| Floppy Drive Swap | If you have two diskette drives installed and you enable this item, drive A becomes drive B and drive B becomes drive A. |
| Floppy Drive Seek | If you enable this item, your system will check all floppy disk drives at start up. Disable this item unless you are using an old 360KB drive. |
| PS/2 Mouse Support | If this item is set to Enabled, the onboard PS/2 Mouse port will work. Setting this to Disable turns off the port. |
| Password Check | If you have entered a password for the system, use this item to determine if the password is required to enter the Setup Utility (Setup) or required both at start-up and to enter the Setup Utility (Always). |
| Boot to OS/2 > 64MB | Enable this item if you are booting the OS/2 operating system and you have more than 64MB of system memory installed. |
| Internal Cache | Leave this item enabled since all the processors that can be installed on this board have internal cache memory. |
| System BIOS Cacheable | If you enable this item, a segment of the system BIOS will be cached to main memory for faster execution. |
| DRAM Driving Current | Setting the driving current for the used DRAM by this item. The Auto setting is recommended. |
| SDRAM CAS Latency | This item determines the operation of the SDRAM memory CAS (column address strobe). We recommend that you leave this item at the default value. The 2T setting requires faster memory that specifically supports this mode. |

### Power Management Setup Page

This page sets some of the parameters for system power management operation.

AMIBIOS SETUP – POWER MANAGEMENT SETUP ©1999 American Megatrends, Inc. All Rights Reserved

Power Management/APM Disabled Standby Time Out Disabled Suspend Time Out Disabled Ring On Power On Disabled LAN Card Power On Disabled RTC Alarm Power On Disabled RTC Alarm Date 15 RTC Alarm Hour 12 RTC Alarm Minute 30 RTC Alarm Second 30 KeyBoard Power On Disabled HotKey Select N/A ESC : Quit  : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift)F2 : Color F6 : Load Optimal values F7 : Load Best performance values

| Power Management/APM | Use this item to enable or disable a power management scheme. If you enable power management, you can use the items below to set the power management operation. Both APM and ACPI are supported. |
| --- | --- |
| Standby Time Out | This sets the timeout for Standby mode in minutes. If the time selected passes without any system activity, the computer will enter power-saving Standby mode. |
| Suspend Time Out | This sets the timeout for Suspend mode in minutes. If the time selected passes without any system activity, the computer will enter power-saving Suspend mode. |
| Ring On Power On | The system can be turned off with a software command. If you enable this item, the system can automatically resume if there is an incoming call on the Fax/Modem. You must use an ATX power supply in order to use this feature. |
| LAN Card Power On | The system can be turned off with a software command. If you enable this item, the system can automatically resume if there is traffic on the network adapter. You must use an ATX power supply in order to use this feature. |
| RTC Alarm Power On /Date /Hour /Minute /Second | The system can be turned off with a software command. If you enable this item, the system can automatically resume at a fixed time based on the system’s RTC (realtime clock). Use the items below this one to set the date and time of the wake-up alarm. You must use an ATX power supply in order to use this feature. |
| KeyBoard Power On | If you enable this item, you can turn the system on by pressing hot keys (Ctrl + Alt + Back Space) on the keyboard. You must connect an ATX power supply and enable this function jumper in order to use this feature. |
| HotKey Select | If you have enabled the above item KeyBoard Power On, use this item to select the hot keys to power on the system. |

### PCI / Plug and Play Setup Page

This page sets some of the parameters for devices installed on the PCI bus and devices that use the system plug and play capability.

AMIBIOS SETUP – PCI / PLUG AND PLAY SETUP ©1999 American Megatrends, Inc. All Rights Reserved

Plug and Play Aware O/S Yes Primary Graphics Adapter OnBoard AGP Assign IRQ for VGA Yes ESC : Quit  : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift)F2 : Color F6 : Load Optimal values F7 : Load Best performance values

| Plug and Play Aware O/S | Enable this item if you are using an O/S that supports Plug and Play such as Windows 95 or 98. |
| --- | --- |
| Primary Graphics Adapter | This item indicates if the primary graphics adapter uses the PCI or the AGP bus. The default PCI setting still lets the onboard display work and allows the use of a second display card installed in a PCI slot. |
| Assign IRQ for VGA | If this item is enabled, an IRQ will be assigned to the PCI VGA graphics system. You set this value to No to free up an IRQ. |

### Load Optimal Settings

If you select this item and press Enter a dialog box appears. If you press Y, and then Enter, the Setup Utility loads a set of fail-safe default values. These default values are not very demanding and they should allow your system to function with most kinds of hardware and memory chips.

### Load Best Performance Settings

If you select this item and press Enter a dialog box appears. If you press Y, and then Enter, the Setup Utility loads a set of best-performance default values. These default are quite demanding and your system might not function properly if you are using slower memory chips or other low-performance components.

### Features Setup Page

This page sets some of the parameters for peripheral devices connected to the system.

AMIBIOS SETUP – FEATURES SETUP ©1999 American Megatrends, Inc. All Rights Reserved

OnBoard FDC Enabled OnBoard Serial Port 3F8h/COM1 OnBoard Ir Port Disabled Ir Port Mode N/A Ir Port IRQ N/A Ir Port DMA N/A Ir Transceiver Type N/A Ir Half-Duplex TimeOut N/A OnBoard Parallel Port 378h Parallel Port Mode Normal EPP Version N/A Parallel Port IRQ 7 Parallel Port DMA N/A OnBoard IDE Both Ultra DMA Support Disabled OnBoard USB Function Disabled ESC : Quit  : Select Item USB Function for DOS Disabled F1 : Help PU/PD/+/- : Modify OnBoard Sound Enabled F5 : Old Values (Shift)F2 : Color OnBoard Modem Enabled F6 : Load Optimal values F7 : Load Best performance values

| OnBoard FDC | Use this item to enable or disable the onboard floppy disk drive interface. |
| --- | --- |
| OnBoard Serial Port | Use this item to enable or disable the onboard COM1 serial port, and to assign a port address |
| OnBoard Ir Port | Use this item to define the protocol for an infrared port if you have installed an optional IR port. The choices are IrDA and ASKIR. |
| Ir Port Mode Ir Port IRQ Ir Port DMA Ir Transceiver … Ir Half-Duplex … | Use these items to define the settings for an infrared port if you have installed an optional IR port. |
| Onboard Parallel Port | Use this item to enable or disable the onboard LPT1 parallel port, and to assign a port address. The Auto setting will detect and available address. |
| Parallel Port Mode | Use this item to set the parallel port mode. You can select SPP (Standard Parallel Port), ECP (Extended Capabilities Port), EPP (Enhanced Parallel Port), or ECP + EPP. |
| EPP Version | Use this item to set the version of EPP while the parallel port mode is set to EPP. |
| Parallel Port IRQ | Use this item to assign either IRQ 5 or 7 to the parallel port. |
| Parallel Port DMA | Use this item to assign a DMA channel to the parallel port. The options are 0, 1 and 3. |
| Onboard IDE | Use this item to enable or disable either or both of the onboard Primary and Secondary IDE channels. |
| Ultra DMA Support | Use this item to set Ultra DMA support for IDE devices on the Primary or Secondary IDE channels. You must enable this or UDMA devices will not work at their intended speed. |
| OnBoard USB Function | Enable this item if you plan to use the USB ports on this mainboard. |
| USB Function for DOS | Enable this item if you plan to use the USB ports on this mainboard in a DOS environment. |
| Onboard Sound | Use this item to enable or disable the onboard audio chip. |
| Onboard Modem | Use this item to enable or disable the onboard modem chip. |

### CPU PnP Setup Page

This page lets you manually configure the mainboard for the CPU. The system will automatically detect the kind of CPU that you have installed and make the appropriate adjustments to the items on this page.

Note: If you manually set the wrong speed and the system won’t run properly, press the Page Up key while the system is booting and a default setting will replace the incorrect CPU setting.

AMIBIOS SETUP – CPU PnP SETUP ©1999 American Megatrends, Inc. All Rights Reserved

CPU Plug and Play Auto CPU Brand CPU Speed 450 MHz VCCore Voltage 2.0V CPU Base Frequency 100 MHz CPU Multiple Factor X4.5 ESC : Quit  : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift)F2 : Color F6 : Load Optimal values F7 : Load Best performance values

| CPU Plug and Play | Use this item to select the CPU Plug and Play function by auto-detect or user-define. |
| --- | --- |
| CPU Brand; VCCore Voltage | These items display the CPU brand and CPU core voltage by the system automatic detecting. |
| CPU Speed | The item displays the internal clock speed of the CPU, based on the next two items. |
| CPU Base Frequency | Use this item to set the external clock frequency for the CPU. Set the CPU clock based on the requirements of the CPU installed on the board. |
| CPU Multiple Factor | Use this item to set a multiplier for the CPU external frequency. The multiplier times the external CPU frequency sets the internal clock speed of the CPU, e.g. 100 MHz (external clock or “FSB”) x 4.5 (muliplier) = 450 MHz (internal clock speed of the installed CPU). |

### Hardware Monitor Page

This page sets some of the parameters for the hardware monitoring function of this mainboard.

AMIBIOS SETUP – HARDWARE Monitor ©1999 American Megatrends, Inc. All Rights Reserved

--- Hardware Monitor --- System Temperature 30°C/86°F CPU Temperature 30°C/86°F System Fan CPU Fan Vcc 5.000 V Vcc3 3.300 V Vcore 2.000 V +12V 12.000 V ESC : Quit  : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift)F2 : Color F6 : Load Optimal values F7 : Load Best performance values

| CPU / System Temperature | These items display CPU/system temperature measurement. |
| --- | --- |
| FAN Speeds & Voltage Measurements | These items indicate cooling fan speeds in RPM and the various system voltage measurements. |

### Change Password

If you highlight this item and press Enter, a dialog box appears which lets you enter a Supervisor password. You can enter no more than six letters or numbers. Press Enter after you have typed in the password. A second dialog box asks you to retype the password for confirmation. Press Enter after you have retyped it correctly. The password is then required to access the Setup Utility or for that and at start-up, depending on the setting of the  Password Check item in Advanced Setup.

#### Change or Remove the Password

Highlight this item, press Enter and type in the current password. At the next dialog box, type in the new password, or just press Enter to disable password protection.

### Exit

Highlight this item and press Enter to save the changes that you have made in the Setup Utility configuration and exit the program. When the Save and Exit dialog box appears, press Y to save and exit, or press N to exit without saving.

# Chapter 4

## Software & Applications

### Introduction

The support software CD-ROM that is included in the mainboard package contains all the drivers and utility programs needed to properly run our products. Below you can find a brief description of each software program, and the location for your mainboard version. More information on some programs is available in a README file, located in the same directory as the software.

If the operating system used in your system is Windows 98, it will automatically install all the drivers and utilities for your board. See the Auto-Installing under Windows 98 section.

### Installing Support Software

The software on the support CD-ROM is for Windows 95/NT/2000 and Windows 98. The installation procedure differs depending on which Operating System you have, but the automatic installation is now for Win98 only.

#### Installing under Windows 95/NT/2000

To install support software for Windows 95/NT/2000 follow this general procedure:

- Insert the support CD-ROM disc in the CD-ROM drive.

(The system might get an error message from the PnP function. Don’t care the message. You don’t really need that file to install the drivers)

- Use My Computer or Windows Explorer to look at the directory structure. You must use the Open command in the right-button menu. Double-clicking on the drive icon will result in an error message because the disc’s AutoRun feature doesn’t work in Windows 95/NT/2000.

- Execute the EXE file name given in the description below.

Note: The correct path name for each software driver is provided, where D: identifies the CD-ROM drive letter – modify if necessary.

#### Bus Master IDE Driver

The IDE Bus Master Drivers allow the system to properly manage the IDE channels on the mainboard. This driver is available for:

- Win9x – D:\IDE\Artx\Setup.exe

#### USB Driver

The USB Driver allows the system to recognize the USB ports on the mainboard. You need to install this driver if you are running Windows 95. Windows 95 OSR2 does not require this driver.

This driver is available for:

- Win95 – D:\USB\EUSBSUPP\USBSUPP.EXE

- Win95 (Chinese) – D:\USB\CUSBSUPP\CUSBSUPP.EXE

#### Audio Driver

The Sound driver allows the system to generate optimal sound effects. This driver is available for:

- DOS & Win3.x – D:\SOUND\Driver\DosDrv\

- Windows 9x – D:\SOUND\Driver\W95-98\Drv

- Windows NT – D:\SOUND\Driver\Nt40\Drv\

There are also two Audio applications available for:

- Windows 9x – D:\SOUND\Gamut\Setup.exe

- Windows 9x – D:\Yamaha-XG\S-YXG50

#### Display Drivers and Software

Find the Display drivers and software here:

- D:\VGA\Artx\Axinst.EXE

#### Fax/Modem Drivers and Software

Find the fax/modem drivers and software here:

- D:\Modem\Driver\9xNT

- D:\Modem\SuperVoice\ [Telecom management software]

- D:\Modem\MediaRing Talk

#### Network Adapter Driver

Find the network interface driver here:

- D:\LAN\SiS900

#### BIOS Update Utility

The BIOS Update utility allows you to update the BIOS file on the mainboard to a newer version. You can download the latest version of the BIOS setup available for your mainboard from the website.

- D:\UTILITY\AMIFL818.EXE

#### PC-Cillin Software

The PC-cillin software program provides anti-virus protection for your system.

This program is available for:

- DOS – D:\PC-CILLIN\DOS\PCSCAN.EXE

- Win9x – D:\PC-CILLIN\WIN98\SETUP.EXE

### Auto-installing under Windows 98

The support software CD-ROM disc loads automatically under Windows 98. When you insert the CD-ROM disc in the system CD-ROM drive the Autorun feature will automatically bring up the install screen. The screen has three buttons on it, Setup, Browse CD and Exit. See the following screen illustration.

When you click on the Setup button the software installation program will run and you can select what kind of installation you want to do, as explained later in this section.

The Browse CD button is the standard Windows command that allows you to examine the contents of the disc using the Windows 98 file browsing interface.

The Exit button closes the Auto Setup window. To run the program again, reinsert the CD-ROM disc in the drive or click on AutoRun in the context sensitive menu for the CD-ROM drive icon in a file browser window.

#### Installing Software with Auto Setup

To install support software for the system board follow this procedure:

- Click on the Setup button. The install program will load and display the following screen. Click the Next button.

- Select the items that you want to setup by clicking on it (the default options are recommended). Click the Next button to proceed.

- The support software will automatically install.

Once any of the installation procedures start, software is automatically installed in sequence. You will need to follow the onscreen instructions, confirm commands and allow the computer to restart as few times as is needed to complete installing whatever software you selected to install. When the process is finished, all the support software will be installed and working.

There are some utilities that you have to manually install if you need, check to the above section.

### Using the PCI Audio Software

- Before you install the PCI Sound Pro drivers, make sure your Operating System has been installed, otherwise the onboard PCI audio might be detected as an “Other device” by the OS device manager.

- After the drivers are properly installed, choose the MULTIMEDIA icon in the CONTROL PANEL when you need to use the Software Wave-Table drivers as a MIDI output device. Select the MIDI tab and click on “C-media SoftMidi Synthesis (Win98) / Driver (Win95)”, then click “OK” to confirm.

- A Windows application named Audio Rack is provided with the PCI Sound Pro drivers, which gives you control over all the audio functions through a user interface that is as simple to use as a home stereo system. We recommend that you use the Audio Rack’s System Mixer to control your computer’s audio volume, recording devices and recording gain.

- If the devices you are using require the MIDI port as the control interface, you need to select the MULTIMEDIA icon in the CONTROL PANEL. Select the MIDI tab and click on “CM8738 MPU-401” (Win98) or “CM8738/C3DX PCI Audio External MIDI Port” (Win95), and then click “OK” to confirm.

- For more information, refer to the PCI Sound Pro manual on the CD that ships with this mainboard.

### The Four Speakers System

The onboard PCI Sound Pro audio system supports 2 wave channels (front/rear) known as the 4 speaker system. If you are running applications which use the DirectSound® 3D or A3D® audio interface, your system can simulate realistic 3D sound through a 4 speaker setup. Follow the steps below to install a 4-speaker setup.

#### Speaker Installation

Connect the front two speakers to the Line-out jack on the sound ports extension bracket. Connect the rear two speakers to the Line-in/Rear jack on the sound ports extension bracket. The original Line-in can be moved to Aux.

#### Speaker Position

Set up your speakers similar to the following figure to get the best audio result.

#### Mixer Setup

There is a 4-speakers option in the Volume Control of the Mixer when you are setting up the PCI Audio Application. Click on the 4 SPK icon to enable this option. This means that the output to the rear speakers is sent through the Line-in/Rear jack. In order to avoid hardware conflicts, DO NOT enable this option when the Line-in/Rear jack is connected with a line-in device.  While the 4 speakers mode is enabled, turn on/off the output of the front speakers and adjust the volume of the speakers so that the front/rear speakers have the same volume.

#### Demo

Execute the “Helicopter” demo in the C3D HRTF Positional Audio Demos of the PCI Audio Application. When you hear the helicopter flying behind you, it means that the rear speakers are working properly.