# Chapter 3

## BIOS Setup

### Introduction

The BIOS setup utility stores information about your computer such as the date and time, the kind of hardware you have installed, and so on. Your computer uses this information to initialize all the components at boot up time, and make sure that everything runs smoothly.

If the information in the setup utility is incorrect, it may cause your system to malfunction. It can even stop your computer from booting properly. If this happens, you can use the clear CMOS jumper to clear the CMOS memory area that is used to store the setup information, or you can hold down the End key while you reboot your computer, Holding down the End key also clears the setup information.

You can run the setup utility and manually make changes to the setup utility. You might need to do this to configure some of the hardware that you add to the mainboard, such as the CPU, the memory, disk drive, etc.

### Running the Setup Utility

Each time your computer starts, before the operating system is booted, a message appears on the screen that prompts “Hit <DEL> if you want to run SETUP”. When you see this message, press the Delete key and the Mainmenu page of the setup utility appears on your monitor.

You can use the cursor arrow keys to highlight any of the options on the Mainmenu page. Press Enter to select the highlighted option. To leave the setup utility, press the Escape key. Hold down the Shift key and press F2 to cycle through the optional color schemes of the setup utility.

Some of the options on the Mainmenu page lead to tables of items with installed values. In these pages, use the cursor arrow keys to highlight the items, and then use the PgUp and PgDn keys to cycle through the alternate values for each of the items. Other options on the Mainmenu page lead to dialog boxes which require you to answer Yes or No by hitting the Y or N keys.

If you have already made changes to the setup utility, press F10 to save those changes and exit the utility. Press F5 to reset the changes to the original values. Press F6 to install the setup utility with a set of default values. Press F7 to install the setup utility with a set of high-performance values.

### Standard CMOS Setup Page

Use this page to set basic information such as the date and time, the IDE devices, and the diskette drives.

| Date & Time | Use these items to install your system with the correct date and time |
| --- | --- |
| Pri Master Pri Slave Sec Master Sec Slave | Use these items to configure devices on the primary and secondary IDE channels. To configure a hard disk drive, choose Auto. If the Auto setting fails to find a hard disk drive, set it to User, and then fill in the hard disk characteristics (Size, Cyls, etc.) manually. If you have a CD-ROM drive, select the setting CDROM. If you have an ATAPI device with removable media (e.g. a ZIP drive or an LS-120) select FLOPTICAL. |
| Floppy Drive A Floppy Drive B | Use these items to set the size and capacity of the floppy diskette drive(s) installed in the system. |

### Advanced COMS Setup Page

Use this page to set more advanced information about your system. Take some care with this page. Making changes can affect the operation of your computer.

| Quick Boot | If you enable this item, the system starts up more quickly be elimination some of the power on test routines. |
| --- | --- |
| 1st Boot Device 2nd Boot Device 3rd Boot Device | Use these three items to determine the order and priority that your computer follows to load an operating system at start-up time. |
| Try Other Boot Device | If you enable this item, the system will also search for other boot devices if it fails to find an operating system from the first three locations. |
| S.M.A.R.T. for Hard Disks | Enable this item if your hard disk(s) supports SMART (Self-Monitoring, Analysis and Reporting Technology). |
| BootUp Num-Lock | This items determines if the Num Lock key is active or inactive at system start-up time. |
| Floppy Drive Swap | If you have two diskette drives installed and you enable this item, drive A becomes drive B and drive B becomes drive A. |
| Floppy Drive Seek | If you enable this item, your system will check the diskette drives at start up time. Disable this item unless you are using an old 360K diskette drive. |
| PS/2 Mouse Support | Set this item to auto so that it will automatically detect if you are using a mouse with a PS/2 interface. |
| Primary Display | Use this item to determine the display devices for the primary display of your computer. |
| Password Check | If you have installed a password on your system, use this item to determine if the password is required to enter the setup utility (Setup) or required at start-up time and to enter the setup utility (Always). |
| Boot to OS/2 Over 64MB | Enable this item if you are booting the OS/2 operating system and you have more than 64MB of memory installed. |
| Internal Cache | Leave this item enabled since all the processors that can be installed on this board have internal cache memory. |
| System BIOS Cacheable | If you enable this item, a segment of the system BIOS will be cached to main memory for faster execution. |
| C000, 32k Shadow | If you enable this item, 32k of the video BIOS is cached to main memory for faster execution. |
| XXXX, 16k Shadow | These items allow 16k segments of the BIOS of other devices to be cached to main memory for faster execution. |

### Advanced Chipset Setup Page

This page lets you set some of the timing parameters for the system.

| Trend ChipAway Virus | This mainboard has built-in virus protection in the firmware. Use this item to enable or disable the built-in virus protection. |
| --- | --- |
| Bank X SDRAM /DEO/FPM | The three items which define the timing for DRAM memory are pre-installed with fixed values and you cannot change them. |
| SDRAM CAS# Latency | This item determines the operation of the main memory CAS (Column Address Strobe). We recommend that you leave this item at the default value. |
| RAS to CAS Delay | This item sets delay timing for RAS (Row Address Strobe) to CAS (Column Address Strobe). We recommend that you leave this item at the default value. |
| DRAM Integrity Mode | Use this item to define the kind of error correction supported by the memory that you have installed. |
| AGP Aperture Size | This item defines an aperture for the graphics. Leave this item at the default value 64 MB. |
| USWC Memory Gap Size | This item sets the memory gap size for USWC (Un-cacheable Speculative Write Combining) mode. |
| AT Bus Clock | This item sets the clock speed for the AT ISA bus. |
| ISA Line Buffer | Use this item to enable or disable the line buffer for the ISA bus. |
| Delay Transaction | This item can be enabled if the system has an embedded 32-bit write buffer to support delay transaction cycles. Leave this item at the default value. |
| OnBoard USB Function | Enable this item if you plan on using the USB (Universal Serial Bus) ports integrated on this mainboard. |
| USB Function for DOS | Enable this item if you plan on using the USB ports while working in the DOS environment. |

### Power Management Setup Page

This page sets some of the parameters for the system power management operation.

Note: Some of the power management routines are not functional if you have connected the mainboard to an AT power supply, rather than an ATX power supply.

| Power Management/APM | Use this item to enable or disable the power management routines. If you enable the power management, you can use the items below to set the power management operation. |
| --- | --- |
| Green Monitor Power State | This item defines which power-saving mode is required to trigger the power management operations of a green monitor. |
| Video Power Down Mode | This item defines which power-saving mode is required to power down the monitor. |
| Hard Disk Power Down Mode | This item defines which power-saving mode is required to power down the hard disk drive. |
| Standby Time Out (Minute) | This sets the timeout for standby mode in minutes. If the time selected passes without any system activity, the computer will enter the power-saving standby mode. |
| Suspend Time Out (Minute) | This sets the timeout for suspend mode in minutes. If the time selected passes without any system activity, the computer will enter the power-saving suspend mode. |
| Modem Use IRQ | This sets an interrupt request line to assign to the modem. We recommend that you set this value to N/A. |
| Monitor items | If you set these items to monitor, any activity on the devices will reset the timers for the power-saving timeouts. |
| Power Button Function | This item allows you to define a software power off using the power button. If the value is set to Soft Off, the power button will automatically cause a software power off. If the value is set to Suspend, the power button will automatically cause the power-saving suspend mode. |
| Restore on AC/Power Loss | Use this item to set a system power state when power restores after sudden AC power loss. |
| Ring Resume From Soft Off | If you enable this item, incoming calls to the fax/modem can resume the system from a power-saving mode or a software power down. |
| RTC Alarm Resume From Soft Off | If you enable this item you can set an alarm on the system realtime clock that will resume the system from a power-saving mode or a software power down. |
| RTC Alarm Date / Hour / Minute / Second | If you have enabled the RTC Alarm Power on, use these items to set the time and date of the alarm. |

### PCI / Plug and Play Setup Page

This page sets some of the parameters for devices installed on the system PCI bus, and devices that use the system plug and play capability.

| Plug and Play Aware O/S | Enable this item if you are using an O/S that supports Plug and Play such as Windows 95/98. |
| --- | --- |
| Primary Graphics Adapter | Use this item to define if your primary graphics adapter is installed in a PCI slot or on an AGP bus. |
| PCI VGA Palette Snoop | When this item is enabled, multiple VGA devices operating on different buses can handle data from the CPU on each set of palette registers on every video device. |
| Assign IRQ for VGA | If this item is enabled, an IRQ will be assigned to the VGA graphics system. We recommend that you set this value to No. |
| Offboard PCI IDE Card | If you are using an IDE interface add-in card, use this item to define which PCI slot the card is installed in. Leave this item at Auto for automatic detection. |
| Pri. / Sec. Offboard PCI IDE IRQ | If you are using an IDE interface add-in card, use these items to assign an IRQ to the primary and secondary IDE channels. |
| PCI Slot 1/2/3 IRQ Priority | You may install expansion cards in the PCI slots that require the use of an Interrupt Request Line. Use these three items to determine which slot has priority when the plug and play OS/BIOS is dynamically allocating IRQs to competing demands from the expansion cards. |
| DMA Channel X | If you set this to PnP, the system will dynamically allocate DMA channels as they are required. If you set a DMA channel to ISA/EISA, it will be reserved for a non-plug and play ISA or EISA device. |
| IRQ X | If you set this to PCI/PnP, the system will dynamically allocate IRQs as they are required. If you set an IRQ to ISA/EISA, it will be reserved for a non-plug and play ISA or EISA device. |

### Load Optimal Settings

If you select this item and press Enter a dialog box appears. If you press Y, and then Enter, the setup utility is loaded with a set of optimal default values. The optimal default values are not very demanding and they should allow your system to function with most kinds of hardware and memory chips.

### Load Best Performance Settings

If you select this item and press Enter a dialog box appears. If you press Y, and then Enter, the setup utility is loaded with a set of best-performance default values. The optimal default values are quite demanding and your system might not function properly if you are using slower memory chips or other kinds of low-performance components.

### Peripheral Setup Page

This page sets some of the parameters for peripheral devices installed on the system.

| Onboard FDC | Use this item to enable or disable the onboard floppy disk drive interface. |
| --- | --- |
| Onboard Serial Port 1 | Use this item to enable or disable the onboard serial port COM1/3, and to assign a port address |
| Onboard Serial Port 2 | Use this item to enable or disable the onboard serial port COM2/4, and to assign a port address |
| Onboard IR Port IR items | Use this item to determine the allocation of the resources of the second serial port. If you select normal, the resources are assigned to the optional second serial port. If you select a specific address, the resources are assigned to the IR port, and you can use the five items below to determine the operation of the IR port. |
| Onboard Parallel Port | Use this item to enable or disable the onboard parallel port LPT1, and to assign a port address |
| Parallel Port Mode | Use this item to determine the parallel port mode. You can select Normal, ECP (Extended Capabilities Port), EPP (Enhanced Parallel Port), or ECP + EPP. |
| EPP Version | Use this item to determine the EPP version, if you have set the Parallel Port Mode to EPP |
| Parallel Port IRQ | Use this item to assign an IRQ to the parallel port. |
| Parallel Port DMA | Use this item to assign a DMA channel to the parallel port. |
| Onboard IDE | Use this item to enable or disable either of the two onboard IDE channels, Primary or Secondary. |
| Onboard Modem | Use this item to enable or disable the onboard modem device. |
| Onboard Sound | Use this item to enable or disable the onboard sound system. |

### Hardware Monitor Status Page

This page displays the parameters for hardware monitoring.

| Slot 1 / Socket 370 / System Temp. | Use these items to monitor the threshold temperature for the CPU and the system. |
| --- | --- |
| FANs & voltages | Use these items to monitor the parameters for the fan speeds and the mainboard voltages. |

### Change Supervisor Password

If you highlight this item and press Enter, a dialog box appears which lets you enter a Supervisor password. You can enter no more than six letters or numbers. Press Enter after you have typed in the password. A second dialog box asks you to retype the password for confirmation. Press Enter after you have retyped it correctly. The password is required at boot time, or when the user enters the setup utility.

#### Change or Remove the Password

Highlight this item and type in the current password. At the next dialog box, type in the new password, or just press Enter to disable password protection.

### Auto-Detect Hard Disks

This item automatically detects and installs any hard disk drives installed on the primary and secondary IDE channel. Most modern drives can be detected. If you are using a very old drive that can’t be detected, you can install it manually.

Setup will check for two devices on the primary IDE channel and then two devices on the secondary IDE channel. At each device, the system will flash an N in the dialog box. Press Enter to skip the device and proceed to the next device. Press Y, then Enter to tell the system to auto-detect the device.

### Save Settings and Exit

Highlight this item and press Enter to save the changes that you have made in the setup utility and exit the setup program. When the Save and Exit dialog box appears, press Y to save and exit, or press N to return to the setup main menu.

### Exit Without Saving Option

Highlight this item and press Enter to discard any changes that you have made in the setup utility and exit the setup program. When the Exit Without Saving dialog box appears, press Y to discard changes and exit, or press N to return to the setup main menu.