# Chapter 2

## Mainboard Installation

To install this mainboard into your system, follow the procedures in this chapter:

- Identify the mainboard components

- Install the correct processor

- Install one or more memory modules

- Verify that any jumpers or switches are at the correct setting

- Install the mainboard in the system chassis

- Install any extension cables to the mainboard headers

- Install any other devices and make the appropriate connections to the mainboard headers.

Note: Before installing the mainboard, you must ensure that the Clear CMOS jumper JP4 is set to the Normal setting. See this chapter for information locating JP4 and changing the jumper setting.

Note: Please do not use the AC power cord to connect the system case to a power outlet until you have completely installed the mainboard and components. In some circumstances, the power management of the system might damage components and create unsafe conditions by allowing power to flow before the installation is complete.

### Mainboard Components

Use the diagram below to identify the major components on your mainboard.

Note: Any jumpers on your mainboard that do not appear in this illustration are for testing only.

### Install the Processor

This mainboard has a Slot-1 that can be installed with any Slot-1 processor cartridge including the Pentium-III, Pentium-II, and the SEPP Celeron. It also has a Socket-370 that can be installed with the Celeron processor which is shipped in a PPGA (Plastic Pin Grid Array) package. To ensure reliability, ensure that your PPGA Celeron processor has a heatsink/cooling fan assembly.

You cannot install a PPGA and a Slot-1 processor cartridge together. Take care that you do not try to install a Socket-7 processor into the Socket-370. A Socket-7 processor such as the Pentium-MMX, or the AMD K5/K6 does not fit in the socket-370. The following table lists the processors that are currently supported by this mainboard. New processors may be a released after this manual is printed.

| Processor Cartridge | Clock Rate MHz | Processor Socket | System Bus MHz |
| --- | --- | --- | --- |
| Pentium-III | 600 | Slot-1 | 100 |
| Pentium-III | 550 | Slot-1 | 100 |
| Pentium-III | 500 | Slot-1 | 100 |
| Pentium-III | 450 | Slot-1 | 100 |
| Pentium-II | 450 | Slot-1 | 100 |
| Pentium-II | 400 | Slot-1 | 100 |
| Pentium-II | 350 | Slot-1 | 100 |
| Pentium-II | 333 | Slot-1 | 66 |
| Pentium-II | 300 | Slot-1 | 66 |
| Pentium-II | 266 | Slot-1 | 66 |
| Pentium-II | 233 | Slot-1 | 66 |
| SEPP Celeron | 433 | Slot-1 | 66 |
| SEPP Celeron | 400 | Slot-1 | 66 |
| SEPP Celeron | 366 | Slot-1 | 66 |
| SEPP Celeron | 333 | Slot-1 | 66 |
| SEPP Celeron | 300A | Slot-1 | 66 |
| SEPP Celeron | 300 | Slot-1 | 66 |
| SEPP Celeron | 266 | Slot-1 | 66 |
| PPGA Celeron | 533 | Socket-370 | 66 |
| PPGA Celeron | 500 | Socket-370 | 66 |
| PPGA Celeron | 466 | Socket-370 | 66 |
| PPGA Celeron | 433 | Socket-370 | 66 |
| PPGA Celeron | 400 | Socket-370 | 66 |
| PPGA Celeron | 366 | Socket-370 | 66 |
| PPGA Celeron | 333 | Socket-370 | 66 |
| PPGA Celeron | 300 | Socket-370 | 66 |

#### Installing a Slot-1 Processor Cartridge

- Locate Slot-1, and FAN1 on the mainboard.

Slot-1 with pre-installed cartridge holder. The upright arms are folded down for shipping.

FAN1

- The Slot-1 is installed with a cartridge holder. The upright struts of the cartridge holder are folded down for shipping. Pull the struts upwards so that they are in the upright position.

- Insert the processor cartridge into the cartridge holder. Follow the instructions given with your processor cartridge. The edge connector on the cartridge has a notch so that it only fits into the Slot-1 in the correct way.

- Locate the cooling fan power supply FAN1. Connect the cable from the processor cartridge cooling fan to FAN1.

- Use JP7 and JP2 to configure the correct processor speed. See following section for Setting the CPU Speed.

#### Installing a Socket-370 Processor

The Celeron processor installs into the ZIF (Zero Insertion Force) Socket-370 on the mainboard.

- Locate the Socket-370, and FAN1. Pull the locking lever out from the socket and swing it to the upright position.

Pin-1 Corner

Socket-370

FAN1

- On the Celeron processor, identify the pin-1 corner by noting that it has a slight bevel.

- On the Socket-370, identify the pin-1 corner. The pin-1 corner is on the same side as the locking lever, closest to the top of the lever when it is in the locked position.

- Match the pin-1 corners and insert the Celeron processor into the socket. No force is required and the processor should drop into place freely.

- Swing the locking lever down and hook it under the catch on the side of the socket. This locks the Celeron processor in the socket.

- If the Celeron processor is installed with a cooling fan assembly, connect the cable from the fan to the CPU fan power connector FAN1.

#### Setting the CPU Speed Jumpers

There are two jumpers to set the CPU speed, JP7-Bus Frequency selector and JP2-CPU Multiplier selector. The multiplier x the bus frequency must equal the speed of your CPU, e.g. 100 MHz (system bus) x 3.5 (multiplier) = 350 MHz (speed of installed CPU).

JP2

A

B

C

D

Jumper JP7: Bus Frequency Selector

1      1

JP7

B A

This jumper consists of two sets of 3-pin jumpers JP7B and JP7A. Use JP7 to select the following system bus frequency.

| Function | JP7B Setting | JP7A Setting |
| --- | --- | --- |
| 66 MHz | Short Pins 1-2 | Short Pins 2-3 |
| 75 MHz | Short Pins 2-3 | Short Pins 2-3 |
| 100 MHz | Short Pins 1-2 | Short Pins 1-2 |
| 112 MHz | Short Pins 2-3 | Short Pins 1-2 |

#### Jumper JP2: CPU Multiplier Selector

This jumper consists of four sets of 2-pin jumpers JP2A, JP2B, JP2C and JP2D. Use JP2 to select the following multiplier.

| Function | JP2A | JP2B | JP2C | JP2D |
| --- | --- | --- | --- | --- |
| 2.0 x | Short | Short | Short | Short |
| 2.5 x | Open | Short | Short | Short |
| 3.0 x | Short | Short | Open | Short |
| 3.5 x | Open | Short | Open | Short |
| 4.0 x | Short | Short | Short | Open |
| 4.5 x | Open | Short | Short | Open |
| 5.0 x | Short | Short | Open | Open |
| 5.5 x | Open | Short | Open | Open |
| 6.0 x | Short | Open | Short | Short |
| 6.5 x | Open | Open | Short | Short |
| 7.0 x | Short | Open | Open | Short |
| 7.5 x | Open | Open | Open | Short |
| 8.0 x | Short | Open | Short | Open |

### Install Memory

The mainboard has three DIMM slots which can be installed with memory modules. You must install at least one memory module in order to use the mainboard.

DIMM3

DIMM2

DIMM1

For this mainboard, you must use 168-pin, memory modules installed with SDRAM/EDO/FP memory chips. The board supports 3.3V memory and also 5V memory. You can select the memory voltage by using the jumper JP8. See the next section for information on using JP8.

If you are using a processor cartridge that runs on a 100 MHz system bus, you must use memory that operates on a 100 MHz memory bus (PC-100 memory). If you are using a processor cartridge that runs on a 66 MHz system bus, you can use memory that operates on a 66 MHz memory bus. You can install any size of memory module from 16 MB up to 256 MB, so the maximum memory size is 3 x 256 MB = 768 MB.

The edge connectors on the memory modules have cut outs, which coincide with struts in the DIMM slots, so the memory modules can only be installed in the correct way.

On the DIMM slot, pull the locking latches at either end of the slots outwards. Position the memory module correctly and insert it into the DIMM slot. Press the module down into the slot so that the locking latches lever inwards and lock the module in place.

### Set the Jumpers

Jumpers are sets of pins that can be connected together with jumper caps. The jumper caps change the way the mainboard operates by changing the electronic circuits on the mainboard. If a jumper cap connects two pins, we say the pins are SHORT. If a jumper cap is removed from two pins, the pins are OPEN.

JP1

JP4

1

B

A

A B

1      1

JP8

#### Jumper JP1: Fax/Modem & Audio System Enable/disable

This jumper consists of two sets of 2-pin jumpers JP1A and JP1B. Use JP1A to enable or disable the onboard sound system. Use JP1B to enable or disable the onboard Fax/Modem.

| JP1A: Function | Jumper Setting |
| --- | --- |
| Disable Audio System | Short Pins 1-2 |
| Enable Audio System | Open Pins 1-2 |
| JP1B: Function | Jumper Setting |
| Disable Fax/Modem | Short Pins 1-2 |
| Enable Fax/Modem | Open Pins 1-2 |

Note: If you use JP1A to disable the onboard sound system, the Fax/Modem is also disabled, even if you have set JP1B to the Enable Fax/Modem setting.

#### Jumper JP4: Clear CMOS Memory

Use this jumper to clear the contents of the CMOS memory. You may need to clear the CMOS memory if the settings in the setup utility are incorrect and prevent your mainboard from operating. To clear the CMOS memory, disconnect all the power cables from the mainboard and then move the jumper cap into the CLEAR setting for a few seconds.

| Function | Jumper Setting |
| --- | --- |
| Normal Operation | Short Pins 2-3 |
| Clear CMOS Memory | Short Pins 1-2 |

#### Jumper JP8: DIMM Voltage Selector

This jumper has two columns of three pins. Set the two jumper caps on the pins 1-2 to select a voltage of 5V volts for the memory module DIMM slots. Set the two jumper caps to pins 2-3 to select a voltage of 3.3V for the DIMM slots.

| Function | Left Column Setting | Right Column Setting |
| --- | --- | --- |
| 5V DIMM | Short Pins 1-2 | Short Pins 1-2 |
| 3.3V DIMM | Short Pins 2-3 | Short Pins 2-3 |

### Install the Mainboard

Install the mainboard into the system chassis. This mainboard is baby AT-sized and the I/O ports are provided on extension brackets. In addition the mainboard can operate using an AT power supply unit or an ATX power supply unit. This means that you have a wide choice of cases that can be used by this mainboard.

Install the mainboard into the unit case. Follow the instructions provided by the case manufacturer using the screws and mounting points provided in the chassis.

FAN2

ATX-PW1

1

J7

AT-PW2

Connect the power cable from the power supply unit to the power connector on the mainboard. If you are using an AT power unit, connect it to the AT power connector PW2. If you are using an ATX power unit connect it to the ATX power connector PW1.

If the case that you are using has a chassis cooling fan, connect the power cable from the fan to the onboard chassis cooling fan power supply FAN2.

Connect the case switches and indicator LEDs to the bank of switch and LED connectors J7. See the illustration below for a guide to the pin functions of the J7 case connector.

21 22

Power LED

Pins 2-4-6

Speaker

Pins 1-3-5-7

HDD LED Pins 15-16

1  2

Reset Switch Pins 17-18

Power/Suspend Switch Pins 21-22

Suspend LED Pins 19-20

Turbo LED Pins 13-14

Keylock

Pins 8-10

### Install the Extension Brackets

The extension brackets are used to transmit features on the mainboard to external connectors that can be fixed to the system chassis. Follow the steps below to install the extension brackets.

Note: All the ribbon cables used on the extension brackets carry a red stripe on the pin-1 side of the cable.

#### Audio Ports and Game/MIDI Port Extension Bracket

This bracket provides three audio jacks for stereo line in, stereo line out and microphone. In addition it has a 15-pin D-connector which can be used by either a joystick or a MIDI device.

If you are using a four channel speaker system, channel one and two are output through the Stereo Line-out, and the rear speaker channels three and four are output through Stereo Line-in.

Audio Ports & Game/MIDI Extension Bracket

Stereo Line-in /Rear speaker channels 3-4

Microphone

Stereo Line-out /Speaker channels 1-2

Game/MIDI

On the mainboard, locate the J4 audio header for this bracket.

J4-Audio Header

1

- Plug the cable from the bracket into the J4 audio header.

- In the system chassis, remove a blanking plate from one of the expansion slots and install the extension bracket in the slot. Use the screw that held the blanking plate in place to secure the extension bracket.

#### Serial Ports Extension Bracket

This bracket has two serial ports; COM1 (9-pins) and COM2 (25pins).

- On the mainboard, locate the two headers COM1 and COM2 for this bracket.

- Plug the two cables from the bracket into the appropriate  COM1 and COM2 headers.

1

COM2

Header

COM1 Header

1

Serial Port

COM1

Serial Port

COM2

Serial Ports

Extension Bracket

- In the system chassis, remove a blanking plate from one of the expansion slots and install the extension bracket in the slot. Use the screw that held the blanking plate in place to secure the extension bracket.

#### Parallel Port Extension Bracket

This bracket has one parallel port LPT1.

PRN1

1

Parallel Port Extension Bracket

Parallel port LPT1

- On the mainboard, locate the parallel port headers PRN1 for this bracket.

- Plug the cable from the bracket into the PRN1 header.

- In the system chassis, remove a blanking plate from one of the expansion slots and install the extension bracket in the slot. Use the screw that held the blanking plate in place to secure the extension bracket.

#### Fax/Modem DAA Module

The Fax/Modem DAA module plugs directly into the mainboard adjacent to an expansion slot in the system chassis. When you remove the blanking plate from the system chassis, you can access the LINE and TEL RJ11 phone jacks on the metal edge of the Fax/Modem DAA module.

2 MCLK

4 FRA-SY

6 SCLK

8 RIN-WAK

10 GPIO

12 SDO

14 SDI

16 RST

GND 1

GND 3

GND 5

AUX 3V 7

HOOK 9

RIN 11

AUX 5V 13

MUTE 15

J3-MODEM Header

- Locate the J3 modem header on the mainboard.

- Plug the Fax/Modem DAA module into the J3 modem header.

- Remove the blanking plate adjacent to the Fax/Modem DAA module.

Modem DAA Module

Modem Header

Line & Tel

RJ11 phone jacks

### Optional Extension Brackets

For this mainboard, you can also obtain an ATX Form Card and an SPDIF digital audio extension bracket. Install these brackets by following the steps below.

#### ATX Form Card

This ATX Form Card provides a mini-DIN port for infrared, one mini-DIN port for a PS/2 mouse. In addition it has two USB (Universal Serial Bus) ports.

J5-ATX Header

1

Infrared Port

USB Ports

PS/2 Mouse

Port

ATX Form Card

- On the mainboard, locate the J5 ATX header for this bracket.

- Plug the cable from the bracket into the J5 ATX header.

- In the system chassis, remove a blanking plate from one of the expansion slots and install the extension bracket in the slot. Use the screw that held the blanking plate in place to secure the extension bracket.

#### Optic Module Extension Bracket

This bracket has two RCA jacks for digital audio in and digital audio out, and an auxiliary jack for a Stereo Line-in device. It also provides a pair of optic fiber interface which enables the communication with MiniDisk or high-end audio systems.

1

J1-SPDIF Header

SPDIF Out

Optic Out

Optic In

SPDIF In

Aux In

Optic Module

Extension Bracket

- On the mainboard, locate the J1 SPDIF header for this bracket.

- Plug the cable from the bracket into the J1 SPDIF header.

- In the system chassis, remove a blanking plate from one of the expansion slots and install the extension bracket in the slot. Use the screw that held the blanking plate in place to secure the extension bracket.

#### Internal Digital Audio

Perhaps you have an optional internal digital audio cable, you can also use it to connect the digital audio output connector of a CD-ROM or DVD drive to the pins 5-6 of J1.

### Install Other Devices

Install and connect any other devices in the system following the steps below.

#### Floppy Disk Drive

The mainboard ships with a floppy disk drive cable that can support one or two drives. Drives can be 3.5” or 5.25” wide, with capacities of 360K, 720K, 1.2MB, 1.44MB, or 2.88MB.

Install your drives and supply power from the system power unit. Use the cable provided to connect the drives to the floppy disk drive header FDC1.

1

1

IDE1

IDE2

FDC1

1

#### IDE Devices

IDE devices include hard disk drives, high-density diskette drives, and CD-ROM/DVD drives.

The mainboard ships with an IDE cable that can support one or two IDE devices. If you connect two devices to a single cable, you must configure one of the drives as Master and one of the drives as Slave. The documentation of the IDE device will tell you how to configure for Master or Slave.

Install the device(s) and supply power from the system power unit. Use the cable provided to connect the device(s) to the Primary IDE channel connector IDE1 on the mainboard.

If you want to install more IDE devices, you can purchase a second IDE cable and connect one or two devices to the Secondary IDE channel connector IDE2 on the mainboard. If you have two devices on the cable, one must be Master and one must be Slave.

#### Internal Sound Connections

If you have installed a CD-ROM drive or a DVD drive, you can connect the sound output of the drive to the built-in sound system. You can connect the analog audio output to the analog connectors CD1 or CD2.

CD2

CD1

G   L  G  R

L   G  G  R

On the mainboard, locate the two 4-pin connectors for CD1 and CD2. The illustration shows the ground pins (G) and the pins for the left (L) and right (R) audio channels. There are two kinds of connector because different brands of CD-ROM/DVD drives have different kinds of cable connectors on their audio output cable. Connect the cable to the appropriate connector.

### Expansion Slots

This mainboard has one AGP slot, three 32-bit PCI expansion slots, and one 8/16-bit ISA slot. The first PCI slot (PCI1) is shared with the ISA slot (ISA1). This means that you can use either one of these slots but you cannot use both slots at the same time.

ISA1

PCI3

PCI2

PCI1

AGP

Follow the steps below to install an add-in card into one of the slots.

- Determine which slot you need to use. The table below shows the functions of the slots.

| AGP | AGP stands for Accelerated Graphics Port. Use this slot to install a graphics adapter which has an AGP edge connector. |
| --- | --- |
| PCI | PCI stands for Peripheral Components Interconnect. Use this slot to install current add-in cards which have a 32-bit PCI edge connector. |
| ISA | ISA stands for Industry Standard Architecture. Use this slot to install older, legacy add-in cards which have an 8/16-bit ISA edge connector. |

- Locate the ISA or PCI or AGP slot on the mainboard.

- Remove the blanking plate from the appropriate expansion slot on the system chassis.

- Install the edge connector of the expansion card into the slot and press it quite firmly down so that it is seated correctly.

- Secure the bracket of the card into the expansion slot in the system chassis using the screw that held the blanking plate in place.