# Chapter 1

## Introduction

This mainboard has a slot-1 processor socket for an Intel processor cartridge, and it also has a socket-370 for an Intel Socket-370 processor. You can install either one of these processors according to the power and performance requirements that you need from your system. Note that you cannot install two processors on this mainboard.

Slot-1 processors include the SEPP (Single Edge Processor Package) Celeron, the Pentium-II, and the Pentium-III. Socket-370 processors are the PPGA/FCPGA Celeron and FCPGA Pentium III. The mainboard supports a system bus of 66 MHz or 100 MHz.

The mainboard uses the Xcel 2000 chipset which provides CPU Plug & Play through firmware. The mainboard is highly integrated and includes a built-in 64-bit AGP Graphics Accelerator, a built-in PCI 3D Sound System, and a built-in 10BaseT/100BaseTX Network Adapter. A V.90 Fax/Modem DAA module is also shipped with the mainboard. In addition, it has a full set of ATX I/O Ports including two PS/2 ports, two USB ports, a parallel port, a serial port and a VGA port.

This mainboard has all the features you need to develop a powerful multimedia workstation that is network ready, and has built-in communications. The board is micro-ATX sized and has power connectors for ATX power supply units so it can be installed in a micro-ATX case.

### Key Features

This key features of this mainboard include:

#### Slot-1 Processor Support

- Supports Pentium-III, Pentium-II and SEPP Celeron processors

- Support for 66 MHz and 100 MHz FSB (Front Side Bus)

- All processors configured by CPU Plug & Play

#### Socket-370 Processor Support

- Supports PPGA/FCPGA Celeron and FCPGA Pentium III processors

- Supports a 66 MHz and 100 MHz front side system bus

- All Celerons are automatically configured using firmware

#### Memory Support

- Three DIMM slots for SDRAM 168-pin memory modules

- Support for 66 MHz & 100 MHz memory bus

- Maximum installed memory can be 3 x 256 MB = 768 MB

#### Expansion Slots

- One 32-bit PCI slot

- One 8/16-bit ISA slot

#### Onboard IDE channels

- Primary and Secondary PCI IDE channels

- Support for PIO (programmable input/output) modes

- Support for Bus mastering and UltraDMA 33/66 modes

#### Power Supply and Power Management

- Provides ATX power connector

- Support for Green PC standard, suspend switch, keyboard power on/off

- Supports Wake on Modem, Wake on LAN and Wake on Alarm

#### Built-in Graphics System

- Onboard 64-bit 3D AGP Graphics Accelerator

- Complies with AGP Ver. 2.0 with built-in 8-way/16 entry set-associative GART cache for AGP master

- Shared memory architecture allows a maximum of 8 MB main memory to act as frame buffer

- Supports high resolutions up to 1600 x 1200 pixels

#### Sound System

- Meets PC98 audio specification

- Full duplex playback and recording with built-in 16-bit CODEC

- HRTF 3D professional audio supports both Direct Sound 3D® and A3D® compatible interface plus support for 4-channel speakers

- Drivers support DOS/Windows 95/98/2000/NT 4.0

- Built-in 32 ohm earphone buffer and 3D surround

- Provides MPU-401 Game/MIDI port and legacy Sound Blaster 16 support

- Downloadable Wave-table Synthesizer supports Direct Music®

- Stereo Mixer supports analog mixing from CD-Audio, Line-In, and digital mixing from voice, FM/Wave-table and digital CD-Audio

#### Onboard I/O Ports

- Provides PC99 Color Connector for easy identification of peripheral devices

- Floppy disk drive port with 1Mb/s transfer rate

- One serial ports with 16550-compatible fast UART

- One parallel port with support for ECP and EPP

- Two USB ports & two PS/2 ports

- One infrared port

#### Hardware Monitoring

- Built-in hardware monitoring for CPU temperature and fan speeds

#### Built-in LAN Adapter

- Onboard 10BaseT/100BaseTX LAN Adapter

- LAN controller integrates Fast Ethernet MAC and PHY compliant with IEEE802.3u 100BASE-TX, 10BASE-T and ANSI X3T12 TP-PMD standards

- Compliant with ACPI 1.0 and the Network Device Class Power Management 1.0

- High Performance provided by 100 Mbps clock generator and data recovery circuit for 100 Mbps receiver

#### Fax/Modem DAA Module

- 56 Kbps Fax/Modem DAA module

- Supports V.90, V.34, V.32bis, V.32, V.22bis, V.22

- Supports Auto Fallback and MNP 5, V.42bis data compression with 115200 compatible Virtual UART

- Requires 16 MB RAM and WIN 95/98/NT

#### Onboard Flash ROM

- Provides plug and play function for automatic CPU and board configuration

- Supports plug and play configuration of peripheral devices and expansion cards

- Built-in virus protection using Trend’s ChipAway Virus which ensures that the entire boot process is virus protected.

#### Bundled Software

- PC-Cillin2000 provides automatic virus protection under Windows 95/98/NT/2000

- SuperVoice Fax/Modem software

- MediaRing Talk provides PC to PC base internet phone communication

- 3Deep delivers the precise imagery and displays accurate color in your monitor

#### Dimensions

- Micro-ATX form factor (24.4cm x 22cm)

### Package Contents

Attention: This mainboard series includes four different models. They are M741LMRT (LAN/Modem Ready), M741LRT (LAN Ready), M741MRT (Modem Ready) and M741T (without LAN & Modem).

Please contact your local supplier for your purchase model.

Each model will support different specification, list as below:

| Model | Specification |
| --- | --- |
| M741LMRT | Onboard LAN chip (U11), LAN header (J11), LAN extension bracket and a Fax/Modem DAA module |
| M741LRT | Onboard LAN chip (U11), LAN header (J11), LAN extension bracket |
| M741MRT | Support a Fax/Modem DAA module |
| M741T | --- |

Your mainboard package ships with the following items:

- Mainboard

- This User’s guide

- 1 UDMA/66 IDE cable

- 1 Floppy diskette drive cable

- Support software CD-ROM

### Static Electricity Precautions

- Components on this mainboard can be damaged by static electricity. Take the following precautions when unpacking the mainboard and installing it in a system.

- Keep the mainboard, and other components, in their original static-proof packaging until you are ready to install them.

- During an installation, wear a grounded wrist strap if possible. If you don’t have a wrist strap, frequently discharge any static electricity by touching the bare metal of the system chassis.

- Handle the mainboard carefully by the edges. Avoid touching the components unless it is absolutely necessary. During the installation lay the mainboard on top of the static-proof packaging with the component side facing upwards.

- Inspect the mainboard for any damage caused during transit. Ensure that all the components that are plugged into sockets are correctly seated.

- If you suspect that the mainboard has been damaged, do not apply power to the system. Contact your mainboard vendor and report the damage.