# Chapter 1

## Introduction

This mainboard has a slot-1 processor socket for an Intel processor cartridge, and it also has a socket-370 for an Intel PPGA (Plastic Pin Grid Array) Celeron or FCPGA Pentium III processor. You can install either one of these processors according to the power and performance requirements that you need from your system. Note that you cannot install two processors on this mainboard.

Slot-1 processors include the SEPP (Single Edge Processor Package) Celeron, the Pentium-II, and the Pentium-III. The mainboard supports a system bus of 66 MHz, 100 MHz or 133 MHz (i810e chipset only).

The mainboard uses the Intel i810/i810e chipset which provides CPU Plug & Play through firmware. The mainboard is highly integrated and includes a built-in Graphics Accelerator with optional AV/S-Video output, a built-in PCI 3D Sound System, and a built-in 10BaseT/100BaseTX Network Adapter. A V.90 Fax/Modem DAA module is also shipped with the mainboard. In addition, it has a full set of ATX I/O Ports and three expansion slots.

This mainboard has all the features you need to develop a powerful multimedia workstation that is network ready, and has built-in communications. It supports hardware monitoring and built-in trend’s ChipAwayVirus virus protection system. The board adheres to the micro-ATX form factor and ATX power supply which supports all of the ATX power management features including ACPI (Advanced Configuration and Power management Interface), power saving modes and keyboard power on hot keys.

### Key Features

The key features of this mainboard include:

#### Processors Support

- Pentium-III support for 450 MHz to 800 MHz clock rates

- Pentium-II support for 233 MHz to 450 MHz clock rates

- SEPP Celeron support for 266 MHz to 433 MHz clock rates

- PPGA Celeron provides Pentium-II performance with integrated level 1 and level 2 cache memory

- FCPGA Pentium-III CPUs are supported

- Support for 66 MHz, 100 MHz and 133 MHz (i810e chipset only) Front Side Bus

- All processors configured by CPU Plug & Play

#### Memory Support

- Two DIMM slots for SDRAM 168-pin memory modules

- Support for 100 MHz memory bus

- Maximum installed memory can be 2 x 256 MB = 512 MB

#### Expansion Slots

- Three 32-bit PCI slots

- One AMR slot

#### Onboard IDE channels

- Primary and Secondary PCI IDE channels

- Support for PIO (programmable input/output) modes

- Support for Bus mastering and UltraDMA 33/66 modes (UltraDMA 66 is available with 82801AA chip only)

#### Power Supply and Power Management

- Provides ATX power connector

- Support for Green PC standard, suspend switch, keyboard power on/off

- Support Wake on Modem, Wake on LAN and Wake on Alarm

#### Built-in Graphics System

- 3D Hyper Pipelined Architecture: Parallel Data Processing (PDP) and Precise Pixel Interpolation (PPI).

- Full 2D H/W and Motion Video Acceleration

- Up to 1600x1200 in 8-bit color at 85Hz refresh

- H/W Motion Compensation Assistance for S/W MPEG2 decode

- Optional TV encoder for NTSC and PAL TV support (AV/S-Video)

#### Sound System

- Meets PC98 audio specification

- Full duplex playback and recording with built-in 16-bit CODEC

- HRTF 3D professional audio supports both Direct Sound 3D® and A3D® compatible interface plus support for 4-channel speakers

- Drivers support DOS/Windows 95/98/2000/NT 4.0

- Built-in 32 ohm earphone buffer and 3D surround

- Provides MPU-401 Game/MIDI port and legacy Sound Blaster 16 support

- Downloadable Wave-table Synthesizer supports Direct Music®

- Digital Audio Interface (SPDIF In/Out) with 24-bit stereo, 44KHz sampling rate and measured 120dB audio quality

- Optional optic fibre interface which enables communication with MiniDisk or high-end audio systems

- Stereo Mixer supports analog mixing from CD-Audio, Line-In, and digital mixing from voice, FM/Wave-table and digital CD-Audio

#### Onboard I/O Ports

- Floppy disk drive port with 1Mb/s transfer rate

- One serial ports with 16550-compatible fast UART

- One parallel port with support for ECP and EPP

- Two USB ports, two PS/2 ports and one infrared port

#### Hardware Monitoring

- Built-in hardware monitoring for CPU/system temperatures,   fan speeds and mainboard voltages

- Supports AMI Desktop Client Manager (ADCM)

#### Built-in LAN Adapter

- Onboard 10BaseT/100BaseTX LAN Adapter

- LAN controller integrates Fast Ethernet MAC and PHY compliant with IEEE802.3u 100BASE-TX, 10BASE-T and ANSI X3T12 TP-PMD standards

- Compliant with ACPI 1.0 and the Network Device Class Power Management 1.0

- High Performance provided by 100 Mbps clock generator and data recovery circuit for 100 Mbps receiver

#### Fax/Modem DAA Module

- Supports 56 Kbps V.90, V.34, V.32bis, V.32, V.22bis, V.22

- Supports Auto Fallback and MNP 5, V.42bis data compression with 115200 compatible Virtual UART

- Requires 16 MB RAM and WIN 95/98/NT

#### Onboard Flash ROM

- Supports plug and play configuration of peripheral devices and expansion cards

- Built-in virus protection using Trend’s ChipAwayVirus which ensures that the entire boot process is virus protected.

#### Bundled Software

- AMI Desktop Client Manager supports hardware monitoring on stand alone systems or over a network

- PC-Cillin provides automatic virus protection under Windows 95/98

- SuperVoice Fax/Modem and voice software

- MediaRing Talk provides PC internet phone communication

- Gamut provides professional audio applications included MP3 encoding/decoding

- S-YXG50 is a music synthesizer software to play back MIDI files on the system

- WordPerfect Suite 8 is a windows version office application (optional)

- WinDVD is a DVD playback application (optional)

#### Dimensions

- Micro-ATX form factor (24.4cm x 22cm)

### Package Contents

Your mainboard package ships with the following items:

- Mainboard

- This User’s guide

- 1 UDMA/33 IDE cable

- 1 Floppy diskette drive cable

- Fax/Modem DAA module

- Network adapter extension bracket

- Support software CD-ROM

#### Optional Accessories

You can purchase the following optional accessories for this mainboard.

- Optic module extension bracket

- AV/S-Video output extension bracket

- UDMA/66 IDE cable

### Static Electricity Precautions

- Components on this mainboard can be damaged by static electricity. Take the following precautions when unpacking the mainboard and installing it in a system.

- Keep the mainboard, and other components, in their original static-proof packaging until you are ready to install them.

- During an installation, wear a grounded wrist strap if possible. If you don’t have a wrist strap, frequently discharge any static electricity by touching the bare metal of the system chassis.

- Handle the mainboard carefully by the edges. Avoid touching the components unless it is absolutely necessary. During the installation lay the mainboard on top of the static-proof packaging with the component side facing upwards.

- Inspect the mainboard for any damage caused during transit. Ensure that all the components that are plugged into sockets are correctly seated.

- If you suspect that the mainboard has been damaged, do not apply power to the system. Contact your mainboard vendor and report the damage.