# 1.	INTRODUCTION

YEAR2000 COMPLIANCE

This Intel chipset mainboard is Hardware Year2000 Compliance and supports the latest Pentium III Processor.  It achieves the highest performance which based on the advanced Pentium II Microprocessor and featuring PCI Local Bus and Accelerated Graphics Port  feature.  This mainboard offers a high degree of flexibility in configuration and is fully IBM PC/AT compatible.

## 1.1	KEY FEATURES

This manual applies to different models of Intel chipset mainboard.  Please refer to appropriate section and mainboard layout according to the model no.

| Model No. | TL-BX31 | TL-ZX31 | TL-BX21 | TL-ZX21 |
| --- | --- | --- | --- | --- |
| Chipset | i440BX | i440ZX | i440BX | i440ZX |
| Socket | Slot 1 |  |  |  |
| Front-Side Bus | 66 / 100 MHz |  |  |  |
| Processor | - INTEL Pentium III 450 MHz or above Processor (Please download the updated BIOS from our website) - INTEL Pentium II 233 – 450 MHz Processor - INTEL Celeron / Celeron A 266 MHz or above Processor |  |  |  |
| CPU Speed | Jumperless setting |  |  |  |
| Voltage Regulator | Built in switching voltage regulator to support 1.5V to 3.5V |  |  |  |
| Cache | Built in Level 2 cache: 512KB (PII and PIII), 128KB (Celeron A) |  |  |  |
| SDRAM | Support 66MHz and 100MHz (PC100) SDRAM module. Also SPD or Non SPD. Each DIMM slot supports up to 256 MB. |  |  |  |
| 168 pin DIMM Slot | 3 | 2 | 3 | 2 |
| Max. System Memory | 768 MB | 512 MB | 768 MB | 512 MB |
| Expansion Slots | 1 x AGP 1.0 slot (1X/2X mode) 4 x PCI slots 3 x ISA slots |  | 1 x AGP 1.0 slot (1X/2X mode) 3 x PCI slots 2 x ISA slots |  |
| On-borad I/O | - 2 x Ultra DMA/33 Bus Master IDE Ports (support up to 4 IDE devices and PIO Mode 3 & 4, DMA 2 & Ultra DMA 33) - Support HD greater than 8.4GB - 1 x Floppy port (up to 2.88 MB floppy) - 1 x Serial ports (16550 Fast UART) - 1 x Parallet port (ECP/EPP) - 2 x USB ports - 1 x PS/2 mouse port - 1 x Infra-red port |  |  |  |
| System BIOS | Award 1M or 2M Flash BIOS Support ACPI, DMI, PnP, Green, Wake on Modem (external) features |  |  |  |
| Wake on LAN | Yes |  | -- | -- |
| Power Connector | ATX |  | ATX and AT |  |
| Sound on Board | -- | -- | Crystal Sound (4235) |  |
| Form Factor | ATX |  | Baby AT |  |

| Model No. | TL-IBX3-13 | TL-IZX3-13 | TL-IBX2-13 | TL-IZX2-13 |
| --- | --- | --- | --- | --- |
| Chipset | i440BX | i440ZX | I440BX | i440ZX |
| Socket | Socket 370 |  |  |  |
| Front-Side Bus | 66 / 100 MHz |  |  |  |
| Processor | - INTEL Socket 370 Celeron A 300 MHz or abover Processor |  |  |  |
| CPU Speed | Jumperless setting |  |  |  |
| Voltage Regulator | Built in switching voltage regulator to support 1.5V to 3.5V |  |  |  |
| Cache | Built in Level 2 cache: 128 KB |  |  |  |
| SDRAM | Support 66MHz and 100MHz (PC100) SDRAM module. Also SPD or Non SPD. Each DIMM slot supports up to 256 MB. |  |  |  |
| 168 pin DIMM Slot | 3 | 2 | 3 | 2 |
| Max. System Memory | 768 MB | 512 MB | 768 MB | 512 MB |
| Expansion Slots | 1 x AGP 1.0 slot (1X/2X mode) 4 x PCI slots 3 x ISA slots |  | 1 x AGP 1.0 slot (1X/2X mode) 3 x PCI slots 2 x ISA slots |  |
| On-borad I/O | - 2 x Ultra DMA/33 Bus Master IDE Ports (support up to 4 IDE devices and PIO Mode 3 & 4, DMA 2 & Ultra DMA 33) - Support HD greater than 8.4GB - 1 x Floppy port (up to 2.88 MB floppy) - 1 x Serial ports (16550 Fast UART) - 1 x Parallet port (ECP/EPP) - 2 x USB ports - 1 x PS/2 mouse port - 1 x Infra-red port |  |  |  |
| System BIOS | Award 1M or 2M Flash BIOS Support ACPI, DMI, PnP, Green, Wake on Modem (external) features |  |  |  |
| Wake on LAN | Yes |  | -- | -- |
| Power Connector | ATX |  | ATX and AT |  |
| Sound on Board | -- | -- | Crystal Sound (4235) |  |
| Form Factor | ATX |  | Baby AT |  |

### Crystal (4235) Sound on Board

- Compatible with Sound Blaster, Sound Blaster Pro and Windows Sound System

- Advanced MPC3-compliant input and output mixer

- Enhanced stereo full duplex operation

- Dual Type-F DMA support

- Integrated CrystalClear 3D stereo enhancement

- Industry leading Delta-sigma data converters (90 dB DAC)

- Internal default PnP resources

- CS9236 wavetable interface

- CS4610 audio accelerator interface

### Yamaha 128 Voice SoftSynth Software

The Yamaha Software Synthesizer S-YXG50 makes it possible for computers with MMX technology to playback MIDI data with higher sound quality than ever.  Even to CD quality level without using hardware.  As it also supports DirectSound as well, you can also enjoy the sound effects from the latest DirectSound multimedia games.

### Yamaha XG Studio Mixer

This versatile playback software allows you to play MIDI files in tandem with animation, video and still images to create a wonderful multimedia effect.  In addition, it allows you to change all aspects of the MIDI files from its tempo, volume, source, special effects, voice and individual instruments; so that you are in complete control of the sound effects.

## 1.2	CHECKLIST

- Mainboard  x 1

- User’s Guide x 1

- CD Driver  x 1

- FDD & HDD Cable Pack (Sound connector card & ATX form card for Baby AT mainboard only)

## 1.3	STATIC ELECTRICITY PRECAUTIONS

Static electricity can easily damage your mainboard. Following procedures can help you to protect your mainboard from electrostatic discharge :

- Keep the mainboard and other system components in their anti-static packaging until you are ready to install them.

- Ground yourself before removing any system component from its protective anti-static packaging. A grounded surface within easy reach is the expansion slot covers at the rear of the system case or any other unpainted portion of the system chassis.

- Frequently ground yourself to discharge any static electric charge that may build up in your body while working on installation and/or configuration.

- Handle the mainboard by its edges or by the mounting bracket to avoid touching its components.