# 4. DRIVER INSTALLATION

4.1 	Windows 95 SYSTEM INFO. update Installation

(Driver CD or Diskette bundled)

The installation procedures described here were based on the Windows 95 Final  Release version.

- Insert the floppy disk containing the driver file into drive A (or B). (or insert CD in CD-ROM drive)

- Double click the “My Computer” icon on the Desktop screen.

- Double click the drive A (or B) icon. (or CD-ROM drive)

- Run SETUP.EXE in drive A (or B) :\Win95 (or  CD directory: \Intelmb\setup.exe)

- Follow the advise showed on the screen to complete the installation.

- When system prompted to reboot Windows 95, select “Yes”

- Restart Windows 95 will detect Intel 82371AB PCI IDE Controllers and install the primary and secondary IDE controller  in sequence.

- And the System devices, Intel 82371AB PCI to ISA Bridge, Intel 82371AB power Management Controller, Intel 82443BX PentiumÒ Processor to PCI Bridge will be created and recognized.

- Restart Windows 95 again.

4.2 	PCI card ( or  PCI Universal serial devices):

If you are WIN95 OSR 2.0 user (it shows “PCI Universal serial Devices”), you may obtain USBSUPP.exe from Microsoft or our AGP product driver for installing Microsoft USB supplement which create “USB Supplement to OSR2” in the list of ADD/Remove program tool under Control Panel. After USBSUPP.exe update, run WIN95 SYSINFO uupdate (driver  diskette) setup.exe again.

4.3  Crystal Sound  --  Windows 95 / 98 Driver Installation

- During the Windows 95 / 98 boot procedure, new hardware will be detected as shown below.

- Select the “Driver from disk provided by hardware manufacture” item, and click the “OK” button.

- Windows 95 / 98 will then prompt a “Install From Disk” dialog box.

- Insert the CD into CD-ROM Drive and click the “Browse...” button to choose the letter of CD-ROM Drive ( for example “D:\” ), and type “CX4235\Win95 or Win98\ ” ( for example D:\CX4235\Win95\ ), then click the “OK” button to install the Windows 95 / 98 driver. For other languages replace English with the language of your choice.

5.  Follow installation procedure.

Problem:  There is problem in playing audio CD after reinstalling Windows 98 ?

Answer:  During re-installation of Windows 98, Win98 will auto-detect the Crystal sound device on board and use its default driver of Crystal 4235 sound.  However, this default driver is not specifically designed for this Crystal 4235 chipset.   Therefore, whenever you encounter this problem, please delete the default driver and re-install  the correct driver from the CD supplied by the mainboard manufacturer.

Procedures:

- Run Device Manager which is located in the System option under Control Panel.

- Delete the Win98 default driver of Crystal sound.

- Run the Setup program of Crystal sound (directory X:\CX4235\win98, X: - CD-ROM drive )from the CD supplied by the mainboard manufacturer.

- Re-install the correct driver of Crystal 4235 sound device.

##### 4.4 Yamaha 128 Voice (S-YXG50) Installation

- Start Win 95 / 98.

- Insert the CD disk into the CD-ROM drive. (e.g. Drive D)

- Choose ‘”Run” from the Start Menu.

- Type “D:\Yama_128\XG-50\English\setup.exe” on the command line, then click on the “OK” button.

- Note that for S-YXG50 instead of English, you can choose French, German, Italian, Japanese or Spanish.

- A Welcome dialogue box will appear, click the “Next” button to start the installation.

- When the user information dialogue box appears, it  will ask for a serial number.  Please type in CAPITAL letters “G5AAH98P1013”

- The heart of this software can be located in the XG Synth Driver icon under the Control Panel.

##### 4.5 Yamaha XG Studio Mixer Software Installation

- Start Win 95 / 98.

- Insert the CD disk into the CD-ROM drive. (e.g. Drive D)

- Choose ‘”Run” from the Start Menu.

- Type “D:\Yama_128\xgstudio\setup.exe” on the command line, then click on the “OK” button.

- A Welcome dialogue box will appear, click the “Next” button to start the installation.

- Follow the instructions on screen.

- To assess the software, double click on the XGStudio Mixer option in the YamahaXG Studio menu located inside the Program menu.

# 4.6	BIOS Upgrade

# Before upgrading the BIOS of this Intel mainboard, please check the BIOS size whether it is 1M or 2M.  The BIOS size will be displayed on the monitor screen during system boot-up.  Then download the correct size and version of BIOS from supplier’s website.

# Our Website:  www.pine-tech.com

4.7 Installing ATAPI CD-ROM

- This board offers two E-IDE (Enhanced IDE) connectors, each of which can take two devices. Please set the primary channel for E-IDE hard disks and set the secondary channel for slow throughput ATAPI compliant peripherals (such as ATAPI CD-ROM).

- If you are installing a secondary E-IDE drive, you may have difficult getting the two drives to work together since some brands of E-IDE drivers won’t work on the same bus as another brand. You will have to designate one drivers as master and another as slave.

APPENDIX I

On Board I/O Address & IRQ Maps

| System Resource | IRQ | I/O Address |
| --- | --- | --- |
| Timer | IRQ0 | 040-043 |
| Keyboard | IRQ1 | 060-064 |
| Programable INT Controller | IRQ2 | 0020-0021 00A0-00A1 |
| COM2 (B) | IRQ3 | 2F8-2FF |
| COM1 (A) | IRQ4 | 3F8-3FF |
| Floppy | IRQ6 | 3F0-3F7 |
| LPT1 | IRQ7 | 378-37F |
| Real Time Clock | IRQ8 | 070-071 |
| PS/2 Mouse | IRQ12 |  |
| Math Coprocessor | IRQ13 | 0F0-0FF |
| IDE 1 | IRQ14 | 1F0-1F7 |
| IDE 2 | IRQ15 | 170-177 |

Note : IRQ 5,9,10 and 11 will be available for other interface card.

#### Our Website:  www.pine-tech.com