Chapter 4

# Technical Information

I/O & Memory Map

Memory Map

Address Range 	Size	Description

[00000-7FFFF]	512K	Conventional memory

[80000-9FBFF]	127K	Extended Conventional memory

[9FC00-9FFFF]	1K	Extended BIOS data area if PS/2 mouse is installed

[A0000-C7FFF]	160K	Available for Hi DOS memory

[C8000-DFFFF]	96K	Available for Hi DOS memory and adapter ROMs

[E0000-EEFFF]	60K	Available for UMB

[EF0000-EFFFF]	4K	Video service routine for Monochrome & CGA adaptor

[F0000-F7FFF]	32K	BIOS CMOS setup utility

[F8000-FCFFF]	20K	BIOS runtime service routine (2)

[FD000-FDFFF]	4K	Plug and Play ESCD data area

[FE000-FFFFF]	8K	BIOS runtime service routine (1)

I/O Map

[000-01F]	DMA controller (Master)

[020-021]	INTERRUPT Controller. (Master)

[022-023]	Chipset control register. I/O ports.

[040-05F]	Timer control registers.

[060-06F]	Keyboard interface controller. (8042)

[070-07F]	RTC ports & CMOS I/O ports

[080-09F]	DMA register.

[0A0-0BF]	INTERRUPT controller. (Slave)

[0C0-0DF]	DMA controller. (Slave)

[0F0-0FF]	MATH coprocessor.

[1F0-1F8]	HARD DISK controller

[278-27F]	Parallel port 2.

[2B0-2DF]	Graphics adapter controller.

[2F8-2FF]	Serial port 2.

[360-36F]	Network ports

[378-37F]	Parallel port 2

[3B0-3BF]	Monochrome & Parallel port adapter.

[3C0-3CF]	EGA adapter.

[3D0-3DF]	CGA adapter.

[3F0-3F7]	Floppy disk controller.

[3F8-3FF]	Serial port1

TIME & DMA Channel Map

Time Map :

Timer Channel 0	System timer interrupt.

Timer Channel 1	DRAM refresh request.

Timer Channel 2	Speaker tone generator.

DMA Channels:

DMA Channel 0	Available

DMA Channel 1	Onboard ECP (Option).

DMA Channel 2	Floppy disk (Winbond CHIP)

DMA Channel 3	Onboard ECP (default)

DMA Channel 4	Cascade for DMA controller1.

DMA Channel 5	Available

DMA Channel 6	Available

DMA Channel 7	Available

Interrupt Map

NMI:	Parity check error

IRQ (H/W):	0	System Timer interrupt from Timer0

1	Keyboard output buffer full.

2	Cascade for IRQ 8-15

3	Serial port 2.

4	Serial port 1.

5	Parallel port 2.

6	Floppy Disk (Winbond Chip).

7	Parallel port 1.

8	RTC clock.

9	Available

10	Available

11	Available

12	PS/2 Mouse

13 	MATH coprocessor

14 	Onboard Hard Disk (IDE1) Channel

15	Onboard Hard Disk (IDE2) Channel

RTC & CMOS RAM MAP

RTC & CMOS:

00	Second.

01	Second alarm

02	Minutes

03	Minutes alarm

04	Hours

05	Hours alarm

06	Day of week

07	Day of month

08	Month

09	Year

0A	Status register A

0B	Status register B

0C	Status register C

0D	Status register D

0E	Diagnostic status byte.

0F	Shutdown byte.

10	Floppy disk drive type byte.

11	Reserve

12	Hard Disk type byte

13	Reserve

14	Equipment type

15	Base memory low byte.

16	Base memory high byte.

17	Extension memory low byte

18	Extension memory high byte

19-2d

2E-2F

30	Reserved for extension memory low byte.

31	Reserved for extension memory high byte.

34-3F	Reserve.

40-7F	Reserved for Chipset setting data.

Technical Information

PS/2 Mouse connector

|  | Pin | Signal Name |  |
| --- | --- | --- | --- |
|  | 5 | Clock | (Blue wire) |
|  | 4 | VCC | (Yellow wire) |
|  | 3 | GND | (Green wire) |
|  | 2 | NC |  |
|  | 1 | Data | (RED wire) |

COM1, COM2 : Serial Ports connector

|  | Signal Name | Pin | Pin | Signame |  |
| --- | --- | --- | --- | --- | --- |
|  | DCD | 1 | 2 | SIN |  |
|  | SOUT | 3 | 4 | DTR |  |
|  | GND | 5 | 6 | DSR |  |
|  | RTS | 7 | 8 | CTS |  |
|  | RI | 9 | 10 | N.C |  |

LPT : Parallel Port connector

|  | Signal Name | Pin | Pin | Signal Name |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Strobe- | 1 | 14 | Auto feed- |  |  |
|  | Data Bit 0 | 2 | 15 | Error- |  |  |
|  | Data Bit 1 | 3 | 16 | INIT- |  |  |
|  | Data Bit 2 | 4 | 17 | SLCT IN- |  |  |
|  | Data Bit 3 | 5 | 18 | Ground |  |  |
|  | Data Bit 4 | 6 | 19 | Ground |  |  |
|  | Data Bit 5 | 7 | 20 | Ground |  |  |
|  | Data Bit 6 | 8 | 21 | Ground |  |  |
|  | Data Bit 7 | 8 | 22 | Ground |  |  |
|  | ACJ- | 10 | 23 | Ground |  |  |
|  | Busy | 11 | 24 | Ground |  |  |
|  | PE | 12 | 25 | Ground |  |  |
|  | SLCT | 13 | 26 | N.C |  |  |

FDD:Floppy Disk Connector

|  | Ground | 1 | 2 | FDHDIN |  |
| --- | --- | --- | --- | --- | --- |
|  | Ground | 3 | 4 | N.C |  |
|  | N.C | 5 | 6 | N.C |  |
|  | Ground | 7 | 8 | Index- |  |
|  | Ground | 9 | 10 | Motor Enable |  |
|  | Ground | 11 | 12 | Drive Select B- |  |
|  | Ground | 13 | 14 | Drive Select A- |  |
|  | Ground | 15 | 16 | Motor Enable |  |
|  | Ground | 17 | 18 | DIR- |  |
|  | Ground | 19 | 20 | Step- |  |
|  | Ground | 21 | 22 | Write Data |  |
|  | Ground | 23 | 24 | Wirte Data |  |
|  | Ground | 25 | 26 | Track 00- |  |
|  | Ground | 27 | 28 | Write Protect- |  |
|  | Ground | 29 | 30 | Read Data |  |
|  | Ground | 31 | 32 | Side 1 select |  |
|  | Ground | 33 | 34 | Diskette |  |

CN 1,2 USB Connector

|  | CN1 Signal Name | Pin | CN2 Signal Name | Pin |
| --- | --- | --- | --- | --- |
|  | USB GND0 | 1 | USB VCC1 | 1 |
|  | USB D0 + | 2 | USB D1 - | 2 |
|  | USB D0 - | 3 | USB D1 + | 3 |
|  | USB VCC0 | 4 | USB GND1 | 4 |

DE1/IDE2 : Primary, Secondary IDE Connector

|  | Signal Name | Pin | Pin | Signal Name |
| --- | --- | --- | --- | --- |
|  | Reset | 1 | 2 | Ground |
|  | Host Data 7 | 3 | 4 | Host Data 8 |
|  | Host Data 6 | 5 | 6 | Host Data 9 |
|  | Host Data 5 | 7 | 8 | Host Data 10 |
|  | Host Data 4 | 9 | 10 | Host Data 11 |
|  | Host Data 3 | 11 | 12 | Host Data 12 |
|  | Host Data 2 | 13 | 14 | Host Data 13 |
|  | Host Data 1 | 15 | 16 | Host Data 14 |
|  | Host Data 0 | 17 | 18 | Host Data 15 |
|  | Ground | 19 | 20 | N.C |
|  | DRQ | 21 | 22 | Ground |
|  | I/O Write- | 23 | 24 | Ground |
|  | I/O Read- | 25 | 26 | Ground |
|  | IOCHRDY | 27 | 28 |  |
|  | DACK | 29 | 30 | Ground |
|  | IRQ14 | 31 | 32 | N.C |
|  | Addr 1 | 33 | 34 | N.C |
|  | Addr 0 | 35 | 36 | Addr 2 |
|  | Chip select 0- | 37 | 38 | Chip select 1- |
|  | IDE LED | 39 | 40 | Ground |