Chapter 1

# Introduction

The mainboard is a high performance system hardware based on Intel Pentium processor and is equipped with four PCI slots, three standard ISA slots, Super Multi-I/O controller and dual ports PCI-IDE connectors for the future expansion. The hardware dimension is 220mm x 250mm with four layer design technology.

Specification

	Intel 430TX PCIset chipset

	Intel Pentium Processor operating at 75~233MHz and Intel Pentium MMXTM(P54C/P55C), AMD K5,K6, Cyrix 6x86MXTM/6x86L, M2 with 321 ZIF socket 7 and scalability to accept faster Processors in the future.

	Supports up to 256 MegaBytes DRAM (minimum of 8 MB) on board (72-pin SIMM x 4, 168-pin DIMM x 2), and BIOS auto FP DRAM, EDO DRAM and SDRAM configuration.

	Supports Onboard Burst/Pipelined burst synchronous  L2 Write Back Cache. The cache memory combination could be 256KB/512KB (32KB x 32 or 64KB x 32 SRAM respectively).

	Support three 16-bits ISA slots, four 32-bits PCI slots, and provides two independent high performance PCI IDE interface capable of supporting PIO Mode 3 and Mode 4 devices. Support Ultra DMA 33.

	Supports ATAPI (e.g. CD-ROM) devices on both IDE interface.

Intel ® is a registered trademark of the Intel Corporation

	Supports 1 floppy port, 1 parallel port (EPP, ECP port), and 2 serial port(16550 Fast UART compatible).

	Supports a PS/2 style mouse and standard AT style keyboard connectors.

	Supports Award Plug & Play BIOS. The BIOS is stored in Flash EPROM form . It provides better upgradeability for the system.

	Supports CPU Hardware sleep and SMM (System Management Mode).

	Supports Universal Serial Bus port (USB).

	Supports 115.2Kbps IrDA/ASK IR port.

	Supports Switching power supply.

	Utilizes Lithium battery which provides environmental protection and longer life time.

	Provided modem ring on & wake up suspend.