Chapter2

# Hardware Design

Motherboard Layout

The main board is designed with Intel 430TX PCIset chipset  which is developed by Intel Corporation to fully support Pentium Processor PCI/ISA system. The Intel 430TX PCIset chipset provides increased integration and improved performance designs. The chipset provides an integrated IDE controller with two high performance IDE interfaces for up to four IDE devices (hard devices, CD-ROM device, etc).The Super I/O controller provides the standard PC I/O function: floppy interface, two 16Byte FIFO serial ports and EPP/ECP capable parallel port. This main board layout is shown in previous page (left page) for user's reference. Care must be taken when inserting memory modules, inserting CPU or even plugging PCI card into associated slots to avoid damaging any circuits or sockets on board. A cooling fan is strongl y recommended when installing P54C. P55C/K5/K6/6x86/6x86L processor due to possible overheat.

The main board supports minimum of 8MB of system memory and maximum of 256MB while L2 Cache can be 256KB/512KB synchronous SRAM Onboard . The main board supports standard Fast Page, EDO(Extended Data Out or Hyper Page Mode) or synchronous DRAM. The main board provides four 72-pins SIMM and two 168-pins DIMM sites for memory expansion. The socket support 1M x 32 (4MB), 2M x32 (8MB), 4M x 32(16MB), and 8M x 32(32MB) single-sided or double-sided memory modules. The memory timing requires 70 ns Fast Page devices or 60 ns EDO DRAM. Memory parity generation and checking is not supported. (DRAM Modules may be parity (x 36) or non-parity (x32)).

The main board supports Onboard two PCI IDE connectors, and detects IDE harddisk type by BIOS utility automatic.

The main board supports Award Plug & Play BIOS for the ISA and PCI cards. The BIOS can be located in Flash EPROM . The advantage of having Flash EPROM is much easier to replace BIOS code if  necessary.

Mainboard Layout

FS1,FS2 :Bus Clock select             JP6:Single/Dual voltage select

BF0,BF1,BF2,BF3 :for RATIO select    JP7:cpu voltage select

JP5:DIMM voltage Select               J1:ps2/ mouse

JP8:MTX Clock select                  J8:(keylock,spk,ID LED…)

Connectors and Jumpers

This section describes all of the connectors and jumpers equipped in the motherboard. Please refer to board layout page 5 for actual location of each connector and jumper

PS/2 Mouse Connector:

| Pin Desoription |  |  |
| --- | --- | --- |
|  | 1 | RED wire |
|  | 2 | NC |
|  | 3 | Green wire |
|  | 4 | Yellow Wire |
|  | 5 | Blue Wire |

IRDA/ASK IR Connector:

| Pin | Description |
| --- | --- |
| 1 | VCC |
| 2 | NC |
| 3 | IRRX |
| 4 | GND |
| 5 | IRTX |

Intel CPU Installation

Defined: x= don't care, 1=short, 0=open

BF0, BF1, BF2  (for CPU bus/core ratio)

For Intel P54C/AMD K5

| cpu mode | BUS clock | ratio | fs0 | fs1 | bf0 | bf1 | bf2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AMD-K5-PR90 INTEL Pentium 90(P54C) | 60MHZ | 1.5 | 2-3 | 1-2 | 1-2 | 1-2 | 1-2 |
| AMD-K5-PR100 INTEL Pentium 100(P54C) | 66MHZ | 1.5 | 1-2 | 1-2 | 1-2 | 1-2 | 1-2 |
| AMD-K5-PR120 INTEL Pentium 100(P54C) | 60MHZ | 2.0 | 2-3 | 1-2 | 2-3 | 1-2 | 1-2 |
| AMD-K5-PR133 INTEL Pentium 133(P54C) | 66MHZ | 2.0 | 1-2 | 1-2 | 2-3 | 1-2 | 1-2 |
| AMD-K5-PR166 INTEL Pentium 166(P54C) | 66MHZ | 2.5 | 1-2 | 1-2 | 2-3 | 2-3 | 1-2 |
| INTEL Pentium 200(P54C) | 66MHZ | 3.0 | 1-2 | 1-2 | 1-2 | 2-3 | 1-2 |

Cyrix M1(PR+)/M2/L

| cpu mode | BUS clock | ratio | fs0 | fs1 | bf0 | bf1 | bf2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cyrix 6x86-pr133+ | 55MHZ | 2.0 | 2-3 | 2-3 | 2-3 | 1-2 | 1-2 |
| Cyrix 6x86-pr150+ | 60MHZ | 2.0 | 2-3 | 1-2 | 2-3 | 1-2 | 1-2 |
| Cyrix 6x86-pr166+ | 66MHZ | 2.0 | 1-2 | 1-2 | 2-3 | 1-2 | 1-2 |
| Cyrix 6x86-pr200+ | 75MHZ | 2.0 | 1-2 | 2-3 | 2-3 | 1-2 | 1-2 |

For Intel P55C/AMD K6

| cpu mode | BUS clock | ratio | fs0 | fs1 | bf0 | bf1 | bf2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AMD-K6-PR166 INTEL Pentium 166(P55C) | 66MHZ | 2.5 | 1-2 | 1-2 | 2-3 | 2-3 | 1-2 |
| AMD-K6-PR200 INTEL Pentium 200(P55C) | 66MHZ | 3.0 | 1-2 | 1-2 | 1-2 | 2-3 | 1-2 |
| AMD-K6-PR233 INTEL Pentium 233(P55C) | 66MHZ | 3.5 | 1-2 | 1-2 | 1-2 | 1-2 | 1-2 |

***Default Intel P55C 166  Jumper setting

FS0, FS1 (for Bus clock)

| Bus clock FS0 FS1 |  |  |
| --- | --- | --- |
|  | 55MHz | 2-3 2-3 |
|  | 60MHz | 2-3 1-2 |
|  | 66MHz | 1-2 1-2 |
|  | 75MHz | 1-2 2-3 |

BF0,BF1,BF2 (for RATIO select)

| RATIO BF0 BF1 BF2 |  |  |
| --- | --- | --- |
|  | 1.5X 1-2 | 1-2 1-2 |
|  | 2.0X 2-3 | 1-2 1-2 |
|  | 2.5X 2-3 | 2-3 1-2 |
|  | 3.0X 1-2 | 2-3 1-2 |
|  | 3.5X 1-2 | 1-2 1-2 |

JP1 (for 12Vdc CPU fan)

JP5 (for DIMM module power voltage)

|  | JP5 |  |
| --- | --- | --- |
| 5.0V | 1-3,2-4 |  |
| 3.3V | 3-5,4-6 |  |

JP6 (for CPU class single or double voltage)

|  | JP6 |  |
| --- | --- | --- |
| Dual voltage (P55C, K6,M2) | 5-6,7-8 |  |
| Single voltage (P54C,AMDK5) | 1-2,3-4 |  |

JP7 (for CPU Voltage Select)(0:open,1:close)

CPU Core 1-2 3-4 5-6 7-8

2.1Vdc 0 1 0 0 for AMD K6 CPU

2.8Vdc 0 0 0 1 for Intel P55C/Cyrix/L

2.9Vdc 1 0 0 1 for AMD k6 /Cyrix M2 CPU

3.2Vdc 0 0 1 1 for AMD k6 CPU

3.3Vdc 1 0 1 1 for Intel P54C CPU

3.5Vdc 1 1 1 1 for AMD K5/ Cyrix M1 CPU

JP8 (for U12 MTX clock select)

|  |  | JP8 |  |
| --- | --- | --- | --- |
| 60MHz |  | CLOSE |  |
| 66MHz |  | OPEN |  |

Jumper Settings for Cyrix 6x86/6x86L and AMD 5x86 CPUs

Warning :

Before you install a Cyrix 6x86 CPU, the CPU's cooler model must be approved by Cyrix. Otherwise, your system may overheat. Please refer to your Cyrix CPU dealer for details. Besides, the cooler must be installed in a way that air flow from the CPU fan to requlator heat sink must be strong enough.

System Memory Configuration

The mainboard supports different type of settings for the system memory. There is no jumper nor connector needed for memory configuration. Following figures provides all possible memory cominations.

| SIMM1,2 SIMM3,4 DIMM1 DIMM2 BANK0 BANK1 BANK0 BANK1 |  |
| --- | --- |
| Installed | None None None OK |
| None | Installed None None OK |
| None | None Installed None OK |
| None | None None Installed OK |
| Installed | Installed None None OK |
| None | None Installed Installed OK |
| Installed | None None Installed OK |
| None | Installed Installed None OK |

Note:

1. Supports both Fast Page DRAM and EDO DRAM SIMMs, but they can not be mixed in the same memory bank.

*2.Because the DIMM occupies the same memory block(Bank 0). So they cannot be installed at the same time.

3.	DIMM Module Specification:

3.3V/Unbuffered

The mainboard supports 168-pin DIMM module to extend system memory size. You can install (3.3V/UNBUFFERED) Fast Page, EDO or Synchronous DRAM.

When you have a DIMM module to plug into a 168-pin dual readout connector, you must make sure that the DIMM module is 3.3V/Unbuffered and supports Intel Pentium Processor System.