ENGINEERING RELEASE NOTES

S3 Savage XFree86 X Server Version 1.00-15

Recipient

S3

Purpose

This maintenance release of the XF86_SVGA X server adds support for the ProSavage-K133.

Products Supported

S3â Savage3D, Savage4, Savage2000, Savage/MX family, Savage/IX family, and ProSavage family integrated 2D/3D graphics accelerators.

Language Support

English.

Software Requirements

This driver is for the XFree86 XF86_SVGA server for Linux, FreeBSD and OpenBSD.  It has been tested on Intel and Alpha CPUs.

Additions and Changes

- Add support for ProSavage-K133.

- (Developers Notes:) Files changed:

Listed below are all the files in the XFree86 3.3.6 release, with the versions in which they changed.  Unmarked files are as shipped with 3.3.6.  The 0.12 release was an interim release, and there were no release notes produced.

- s3_savage/Imakefile

- s3_savage/lrmi.c	0.10

- s3_savage/lrmi.h

- s3_savage/newmmio.h	0.10

- s3_savage/regs3sav.h	0.10	0.11			0.15

- s3_savage/s3bci.h	0.10	0.11

- s3_savage/s3bitblt.c

- s3_savage/s3bitmap.c	0.10

- s3_savage/s3sav_accel.c	0.10	0.11			0.15

- s3_savage/s3sav_cursor.c	0.10	0.11		0.14

- s3_savage/s3sav_driver.c	0.10	0.11	0.13	0.14	0.15

- s3_savage/s3sav_driver.h	0.10	0.11	0.13	0.14

- s3_savage/s3sav_misc.c	0.10	0.11				0.15

- s3_savage/s3sav_rop.h

- s3_savage/s3sav_vbe.c		0.11	0.13	0.14

- s3_savage/vbe.h

- © Copyright 1999 S3 Incorporated. All rights reserved. If you have received this document from S3 Incorporated in electronic form, you are permitted to make the following copies for business use related to products of S3 Incorporated: one copy onto your computer for the purpose of on-line viewing, and one printed copy. With respect to all documents, whether received in hard copy or electronic form, other use, copying or storage, in whole or in part, by any means electronic, mechanical, photocopying or otherwise, is not permitted without the prior written consent of S3 Incorporated, P.O. Box 58058., Santa Clara CA 95052-8058. S3, True Acceleration, Trio and ViRGE are registered trademarks of S3 Incorporated. The S3 Corporate Logo, S3 on Board, S3 on Board design, S3d design, Plato, S3d, Scenic, Scenic Highway, Sonic, SonicVibes, S3FM, InfiniPatch, InfiniRate, Audio Card on a Chip, QuickRamp, Aurora64V+, DuoView, Streams Processor, Galileo, No Compromise Integration, No Compromise Acceleration and Innovations in Acceleration are trademarks of S3 Incorporated. Other trademarks referenced in this document are owned by their respective companies. The material in this document is for information only and is subject to change without notice. S3 Incorporated reserves the right to make changes in the product design without reservation and without notice to its users.

- Resolutions Supported

| ### Resolution | ### Depth | ### Refresh Rates |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 640x480 | 8 |  | 60 |  | 72 | 75 | 85 | 160 |
| 640x480 | 16 |  | 60 |  | 72 | 75 | 85 | 160 |
| 640x480 | 32 |  | 60 |  | 72 | 75 | 85 | 160 |
| 800x600 | 8 | 56 | 60 |  | 72 | 75 | 85 | 160 |
| 800x600 | 16 | 56 | 60 |  | 72 | 75 | 85 | 160 |
| 800x600 | 32 | 56 | 60 |  | 72 | 75 | 85 | 160 |
| 1024x768 | 8 |  | 60 | 70 |  | 75 | 85 | 130 |
| 1024x768 | 16 |  | 60 | 70 |  | 75 | 85 | 130 |
| 1024x768 | 32 |  | 60 | 70 |  | 75 | 85 | 130 |
| 1152x864 | 8 |  | 60 | 70 |  | 75 | 85 | 100 |
| 1152x864 | 16 |  | 60 | 70 |  | 75 | 85 | 100 |
| 1152x864 | 32 |  | 60 | 70 |  | 75 | 85 | 100 |
| 1280x1024 | 8 |  | 60 |  |  | 75 | 85 | 100 |
| 1280x1024 | 16 |  | 60 |  |  | 75 | 85 | 100 |
| 1280x1024 | 32 |  | 60 |  |  | 75 | 85 | 100 |
| 1600x1200 | 8 |  | 60 |  |  | 75 | 85 |  |
| 1600x1200 | 16 |  | 60 |  |  | 75 | 85 |  |
| 1600x1200 | 32 |  | 60 |  |  | 75 | 85 |  |
| 1920x1440 | 8 |  | 60 |  |  |  |  |  |
| 1920x1440 | 16 |  | 60 |  |  |  |  |  |

## Application/Utility Components

- XF86_SVGA

- s3switch

- Switches

- None.

- Installation Instructions

- In a working XFree86 environment, this server is installed by stopping the X server, copying the XF86_SVGA binary into the appropriate spot (usually /usr/X11R6/bin) and restarting X.  You can make sure the SVGA server is the default one by doing “ls –l /usr/X11R6/bin/X”; if the link does not point to XF86_SVGA, do:

- cd /usr/X11R6/bin

- rm X

- ln –s XF86_SVGA X

- The server must either be setuid root, or you must start the X server as root.  This is normal for X.

- The s3switch utility has its own makefile.  It also must be run as root.

- Included also is a binary tar file (s3switch-1.0-14.tar.gz) which includes the X server itself, the s3switch utility, and a man page for s3switch, along with a short installation script which places the files in the appropriate spots. S3 may simply post this file on its web page, and point Linux users here when they ask about Savage support.  I do not think XFree86 will be interested in supporting the s3switch utility.

- Known Limitations or Issues

- It is still not clear to me how the s3switch utility should be distributed.

- Resolved Issues

- No CR number: Server does not start on ProSavage-K133.
The –K133 is operationally identical to the –133, but it has a different PCI ID.  Added code to recognize this ID and treat them both the same.

- Unresolved Issues

- None at this time.

- (Developer’s Notes) PVCS Label

- I don’t know this.

- (Developer’s Notes)Test Recommendations

Here are some of the applications we have tested thus far.  The following set are all shipped with XFree86:

- fvwm window manager

- mwm window manager

- AnotherLevel window manager

- Icewm window manager

- Gnome environment

- KDE environment

- x11perf –all, which tests essentially every Xlib primitive

- xbench, which also tests xlib primitives

- Many utilities in /usr/X11R6/bin

Be sure to test switching between the virtual consoles:  Ctrl-Alt-F1 should switch you to the main console screen, where you can see the messages from the server.  Ctrl-Alt-F7 switches back to X.  Try going to the other consoles as well (Ctrl-Alt-F2 through F6).

(Developer’s Notes) Developer(s)

Tim Roberts, Providenza & Boekelheide, Inc.