Note: S60828T BIOS version 1.3 is an released BIOS for SUPER P5STE

motherboard only. It will support all types of Intel P5 CPU,

AMD K5 CPU, Cyrix P150+ and P166+ CPU. It allows user to assign

particular IRQ to particular PCI slot. It can be flashed to BIOS

S6Y08T version 2.5 and above but not reverse. Please take

note of your BIOS version as not all versions can be flashed.

Flash BIOS instruction:

1. Download the filename S60828.zip either from BBS# 408-451-1114 or Web Site

www:http://www. supermicro.com.

2. Unzip the zip file and rename S6828T-1.rom to super.rom.

3. Copy super.rom to a formatted DOS bootable diskette.

4. Put the diskette with super.rom file in to drive A.

5. Press and hold "Ctrl"+"Home" in the keyboard and power up system

until system reads floppy drive.

6. BIOS will be flashed and updated and system will reboot.