Note: NA0828.ROM ver 0.9 BIOS is for all Natoma CHIPSET motherboards only.

It will improve the slow performance in some memory configurations.

It can be flashed from BIOS NA0620 version 0.6 and above.

Pleased take note of your BIOS version as not all versions can

be flashed.

Flash BIOS instruction:

1. Download the filename NA0828.ZIP from BBS#408-451-1114 or Web site

www:http://www.supermicro.com.

Note:  DO NOT DOWNLOAD AMIFLASH.COM.  YOU WON'T NEED IT!

2. Unzip the zip file and rename NA0828.ROM to SUPER.ROM.

3. Copy SUPER.ROM to a formatted DOS bootable diskettte.

4. Power off your computer.

5. Put the diskette with SUPER.ROM file in to drive A.

6. Press and hold "Ctrl"+"Home" in the keyboard and power up system until

system reads floppy drive.

7. BIOS will be flashed and updated and system will reboot.