Flash BIOS instruction:

1. Unzip the zip file and rename xxxxxx.ROM to SUPER.ROM.

2. Copy SUPER.ROM to a formatted DOS bootable diskette.

3. Put the diskette with SUPER.ROM file in to drive A.

4. Press and hold "Ctrl"+"Home" on the keyboard and power up system

until system reads floppy drive.

5. BIOS will be flashed and updated and system will reboot.