How to install Intel PIIX4 BusMaster IDE on Windows 95 OSR2

Currently Windows 95  does not support the Intel 82371AB PIIX4 (430TX) chipset, but Windows 95 OSR2 (OEM Service Release II) can support the dual FIFO PCI IDE controller

by adding the following lines to recognize the 82371AB PIIX4 chipset.

You may need change the view option to see the INF directory under Windows directory

Goto Control Pannel-> View ->Options->View->Show all files

Please edit the following two files:

File 1. C:\WINDOWS\INF\MSHDC.INF

In [INTEL_HDC] section add the following line

%PCI\VEN_8086&DEV_7111.DeviceDesc%=DUAL_GOOD_IDE,,PCI\VEN_8086&DEV_7111

In [CONTROLFLAGS] section (after [INTEL_HDC] section) add the following line

CopyFilesOnly=PCI\VEN_8086&DEV_7111

In [STRINGS] section (after [CONTROLFLAGS] section) add the following line

PCI\VEN_8086&DEV_7111.DeviceDesc="Intel 82371AB PCI Bus Master IDE Controller"

File 2. C:\WINDOWS\INF\MACHINE.INF

Note: Each HKLM line is continuous, do not add Return

In [PCI_DRV_AddReg] section add the following lines

HKLM,System\CurrentControlSet\Services\VxD\PCI,71118086,1,04,00,00,00

HKLM,System\CurrentControlSet\Services\VxD\PCI\IRQMiniport\7118086,Name,"Intel 82371AB"

HKLM,System\CurrentControlSet\Services\VxD\PCI\IRQMiniport\7110806,Path,"PCIMP.PCI"

HKLM,System\CurrentControlSet\Services\VxD\PCI\IRQMiniport\711080806,Instance,1,01,00,00,00