TI5TV(A)

Pentium PCI ATX Motherboard

User’s Manual

Version 3.20

Contents

Chapter 1 Specifications 	1

Chapter 2 Hardware Description	3

- Processor and CPU Voltage	5

- L2 Cache Memory	5

- Main Memory	6

- BIOS	7

- I/O Port Address Map	8

- DMA Channels	8

- Interrupt Controllers	9

- Onboard PCI EIDE	9

- Onboard Multi-I/O	9

Chapter 3 Hardware Setting	10

- SW1(1-6): CPU Frequency Selector	12

- SW1(7): CMOS Discharge Select	14

- SW2(1-4): CPU Voltage Selector	14

- J16(7, 17): ATX Power On Switch	15

- JP5: Onboard Audio (ESS1868) Selector (Option)	15

Chapter 4 I/O Ports Connector Setting and Pin out	16

- J1, J6: Serial Ports	18

- J2, J4: PS2 Mouse and Keyboard Connector	18

J2: PS2 Mouse Connector	18

J4: PS2 Keyboard Connector	18

- J5: Parallel Port	19

- J7: ATX Power Supply Connector	19

- JP1: CPU Fan Power Connector	19

- J10: Floppy Driver Connector	20

- J12, J11: IDE Connector	20

J12: Primary IDE Connector	20

J11: Secondary IDE Connector	21

- J16: Front Bezel Connectors	22

Pin 1-4: Speaker	22

Pin 6 and 16: Power Saving Switch Connector	22

Pin 11-15: Power LED and Keylock	22

Pin 8 and 18: Turbo LED Connector	23

Pin 9 and 19: Reset Switch	23

Pin 10 and 20: Hard Disk Drive LED Connector	23

- J17: IrDA. Connector	23

- J8, J9: USB Connector	24

- J13: Audio I/O Connector (Option)	24

- J14: CD-ROM Audio In Connector (Option)	24

Chapter 5 AMI BIOS User Guide	25

- How to Invoke the Setup Program	26

5.1.1	Point and Click Interface	27

5.1.2	Using a Mouse with Setup	27

5.1.3	Using the Keyboard with Setup	28

5.1.4	Setup Main Menu	28

5.1.5	Original	29

5.1.6	Default Value	29

5.1.7	Optimal	30

5.1.8	Fail Safe	30

5.1.9	 Save Changes and Exit	30

- Setup Menu Categories	31

5.2.1	Standard Setup Menu	32

5.2.2	Advanced Setup Menu	33

5.2.3	Chipset Setup Menu	36

5.2.4	Power Management Setup	37

5.2.5	PCI/PnP Setup Menu	39

5.2.6	Peripheral Setup Menu	43

- Password	44

- Anti-Virus	45

Chapter 6 ESS1868 (Audio) Driver Installation (Option)	46

- Audio driver install for Windows 3.1x	47

- Audio driver install for Windows 95	52

- Audio Rack Install for Windows 3.1x	58

- Audio Rack Install for Windows 95	62

Chapter 1 Specifications

The TI5TV(A) is a high-performance, Pentium powered PCI motherboard with an ATX form factor. It is highly flexible in CPU frequency, L2 cache type and size, and main memory type and size. The ATX form factor of the TI5TV(A) allows easy access to main memory sockets and expansion by using full length add-on cards.

The main features are listed as follows:

Main Processor

Intel Pentium/P55C* 75/90/100/120/133/150/166/200/233

Cyrix 6x86/6x86L P120+/P133+/P150+/P166+

Cyrix M2-166/180/200

AMD K5 PR75/PR90/PR100/PR120/PR133/PR150/PR166

AMD K6 PR2-166/PR2-200/PR2-233

* P55C =Pentium MMX

Processor Upgrade

Intel P6 based Over Drive

L2 Cache

Size: 512K

Type: Pipeline Burst Synchronous SRAM on board

Main Memory

Maximum Size: Up to 128 MB of total main memory

Memory Type

4x72-pin SIMM sockets and 2x168-pin DIMM sockets

Up to 128MB of total main memory using FPM or EDO

Up to 64MB using SDRAM

SIMM Size: 4MB, 8MB, 16MB or 32MB

SIMM Type: Fast Page Mode or Extended Data Out (EDO)

DIMM Size: 8MB, 16MB or 32MB

DIMM Type: SDRAM (Synchronous DRAM), EDO, Fast Page Mode DRAM

Chipset

Intel 430VX PCIset with built-in PCI EIDE

Onboard I/O

WINBOND W83877 for two serial, one parallel, and one floppy drive interfaces, support IrDA device.

Onboard Bus Mastering EIDE

Two EIDE interfaces for up to four devices, support Mode 3,4 IDE Hard Disk & ATAPI CD-ROM.

BIOS

Licensed BIOS with additional features:

- FLASH EEPROM (128KB) for BIOS update

- ISA Plug and Play (PnP) extension

- Power management

RTC (Real Time Clock)

DALLAS 12887 compatible

Keyboard & Mouse

PS/2 Keyboard

PS/2 Mouse (using IRQ12)

Onboard Audio (Option)

ESS1868 sound chip support Plug and Play (PnP) function.

CPU Temperature Overheat Protection

When the CPU temperature exceeds 80C, the system automatically enters the Standby Mode where the CPU slows down to one third of the original speed. The system returns to ‘Normal’ Mode once the CPU temperature goes below 80C .

Expansion Slots

Four Master/Slave PCI slots

Three ISA slots

Form Factor

ATX

Chapter 2 Hardware Description

This chapter briefly describes each of the major features of the TI5TV(A) motherboard. The layout of the board is shown in Figure 1 which shows the locations of key components. The topics covered in this chapter are as follows:

2.1 Processor and CPU Voltage	5

2.2 L2 Cache Memory	5

2.3 Main Memory 	6

2.4 BIOS 	7

2.5 I/O Port Address Map 	8

2.6 DMA Channels 	8

2.7 Interrupt Request Lines (IRQ)	9

2.8 Onboard PCI EIDE 	9

2.9 Onboard Multi-I/O 	9

Figure 1: Layout of the TI5TV(A) Motherboard

2.1 Processor and CPU Voltage

The TI5TV(A) is designed to take a Pentium Processor with a bus speed of 50, 60 and 66 MHz. Since the internal clock of the CPU can be multiples of 1.5, 2, 2.5, 3 and 3.5 of the bus clock, the CPU frequency can be 75, 90, 100, 120, 133, 150, 166, 200 or 233MHz.

Onboard CPU voltage provides support for CPUs Vcore which require a voltage other than 3.3V such as 2.5V, and/or power consumption higher than 10 watts. One example is a P6 based Over Drive.

2.2 L2 Cache Memory

The TI5TV(A) supports P.B. (Pipelined Burst) Synchronous Cache. The P.B. Synchronous Cache boosts the system performance 10% higher than regular Asynchronous Cache.

2.3 Main Memory

The TI5TV(A) provides two 64-bit memory banks for up to 128MB. Each bank consists of two SIMM sockets, SIMM1/SIMM2 and SIMM3/SIMM4. Both EDO and page mode DRAMs are supported. The size of the SIMMs can be 1Mx32, 2Mx32, 4Mx32 and 8Mx32.

Each 64-bit memory bank can also be populated by using a  DIMM (Dual In-line Memory Module). The DIMM size can be 1Mx64, 2Mx64, and 4Mx64. The DIMM types can be FPM, EDO or SDRAM (synchronous DRAM). SIMMs are not available in SDRAM.

The following are the important factors for populating the SIMM and DIMM sockets:

1. 	SIMM1/SIMM2, SIMM3/SIMM4, DIMM1 and DIMM2 bank can be populated first.

2. 	Each bank should consist of the same size SIMMs

3. 	Each bank should consist of the same type SIMMs. For example,

SIMM1 and SIMM2 should both be EDO or page mode.

4. 	SIMMs can be mixed by the bank. For example, SIMM1/SIMM2 are page mode and SIMM3/SIMM4 are EDO.

5. 	If SIMM1/SIMM2 or SIMM3/SIMM4 are occupied, then DIMM2 or DIMM1 cannot be populated. The reverse is also true.

(1) 168Pin DIMM (3V)

SDRAM or Fast page Mode/EDO DRAM

| Bank0 (DIMM2) | Bank1 (DIMM1) | Total Memory |
| --- | --- | --- |
| 8MB | ----- | 8MB |
| 16MB | ----- | 16MB |
| 32MB | ----- | 32MB |
| 8MB | 8MB | 16MB |
| 16MB | 8MB | 24MB |
| 32MB | 8MB | 40MB |
| 16MB | 16MB | 32MB |
| 32MB | 16MB | 48MB |
| 32MB | 32MB | 64MB |

(2) 72Pin SIMM (5V)

Fast page mode/EDO DRAM

| Bank0 (SIMM3, SIMM4) | Bank1 (SIMM1, SIMM2) | Total Memory |
| --- | --- | --- |
| 4MB2 | ----- | 8MB |
| 8MB2 | ----- | 16MB |
| 16MB2 | ----- | 32MB |
| 32MB2 | ----- | 64MB |
| 4MB2 | 4MB2 | 16MB |
| 8MB2 | 4MB2 | 24MB |
| 16MB2 | 4MB2 | 40MB |
| 32MB2 | 4MB2 | 72MB |
| 8MB2 | 8MB2 | 32MB |
| 16MB2 | 8MB2 | 48MB |
| 32MB2 | 8MB2 | 80MB |
| 16MB2 | 16MB2 | 64MB |
| 32MB2 | 16MB2 | 96MB |
| 32MB2 | 32MB2 | 128MB |

2.4 BIOS

The BIOS on the TI5TV(A) motherboard provides the standard BIOS functions plus the following additional features:

1. ISA Plug and Play (PnP) Extension

Unlike PCI cards, which are plug and play, ISA cards require setting jumpers to resolve hardware conflict. To make a computer system PnP, an ISA PnP standard is established and supported by new OSes, such as Windows 95. Under Windows 95, the motherboard BIOS must have ISA PnP extension to support the new ISA PnP cards.

2. Power  Management

The power management feature provides power saving by slowing down the CPU clock, turning off the monitor screen, and stopping the HDD spindle motor.

2.5 I/O Port Address Map

Each peripheral device in the system is assigned a set of I/O port addresses which also becomes the identity of the device. There are a total of 1K port address spaces available. The following table lists the I/O port addresses used on the motherboard.

| Address | Device Description |
| --- | --- |
| 000h - 01Fh | DMA Controller #1 |
| 020h - 03Fh | Interrupt Controller #1 |
| 040h - 05Fh | Timer |
| 060h - 06Fh | Keyboard Controller |
| 070h - 07Fh | Real Time Clock, NMI |
| 080h - 09Fh | DMA Page Register |
| 0A0h - 0BFh | Interrupt Controller #2 |
| 0C0h - 0DFh | DMA Controller #2 |
| 0F0h | Clear Math Coprocessor Busy Signal |
| 0F1h | Reset Math Coprocessor |
| 1F0h - 1F7h | IDE Interface |
| 278 - 27F | Parallel Port #2(LPT2) |
| 2F8h - 2FFh | Serial Port #2(COM2) |
| 2B0 - 2DF | Graphics adapter Controller |
| 378h - 3FFh | Parallel Port #1(LPT1) |
| 360 - 36F | Network Ports |
| 3B0 - 3BF | Monochrome & Printer adapter |
| 3C0 - 3CF | EGA adapter |
| 3D0 - 3DF | CGA adapter |
| 3F0h - 3F7h | Floppy Disk Controller |
| 3F8h - 3FFh | Serial Port #1(COM1) |

2.6 DMA Channels

There are seven DMA channels available on the motherboard. Only DRQ2 is used by the floppy controller. In the case that ECP mode on the parallel port is used, DRQ1 or DRQ3 will be used.

2.7 Interrupt Request Lines (IRQ)

There are a total of 15 IRQ lines available on the motherboard. Peripheral devices use interrupt request to notify the CPU for the service required. The following table shows the IRQ used by the devices on the motherboard:

| Level | Function |
| --- | --- |
| IRQ0 | System Timer Output |
| IRQ1 | Keyboard |
| IRQ2 | Interrupt Cascade |
| IRQ3 | Serial Port #2 |
| IRQ4 | Serial Port #1 |
| IRQ5 | Parallel Port #2 |
| IRQ6 | Floppy Disk Controller |
| IRQ7 | Parallel Port #1 |
| IRQ8 | Real Time Clock |
| IRQ9 | Software Redirected to Int 0Ah |
| IRQ10 | Reserved |
| IRQ11 | Reserved |
| IRQ12 | Reserved |
| IRQ13 | 80287 |
| IRQ14 | Primary IDE |
| IRQ15 | Secondary IDE |

2.8 Onboard PCI EIDE

The PCI EIDE controller is part of the 430VX PCIset. It supports PIO mode 3/4 and bus mastering . The peak transfer rate of PIO mode 3/4 can be as high as 17MB/sec. Using HDDs that support bus mastering, the peak transfer rate can reach 22MB/sec.

There are two IDE connectors, primary IDE and secondary IDE. With two devices per connector, up to four IDE drives are supported.

2.9 Onboard Multi-I/O

The onboard multi-I/O chip, Winbond W83877F, provides two serial port, one parallel port and one floppy controller. The serial ports are 16550 UART compatible. The parallel port supports high speed EPP/ECP mode.

The floppy controller supports up to 2.88MB format. The I/O port addresses of the serial and parallel ports are programmable via BIOS set-up. Each I/O can be individually disabled.

Chapter 3 Hardware Setting

The following section describes the necessary procedures and proper jumper settings to configure the TI5TV(A) motherboard.

3.1	SW1(1-6): CPU Frequency Selector	12

3.2	SW1(7): CMOS Discharge Select 	14

3.3	SW2(1-4): CPU Voltage Selector	14

3.4	J16(7, 17): ATX Power On Switch	15

3.5	JP5: Onboard Audio (ESS1868) Selector (Option)	15

For the locations of the jumpers, refer to Figure 2 on the following page.

Remarks:

| = | Jumper Open |
| --- | --- |
| = | Jumper Closed |

|  | Switch 1 and switch 2 are set to OFF. Switch 3 and switch 4 are set to ON. |
| --- | --- |

Figure 2: Jumper Locations of the TI5TV(A)

3.1	SW1(1-6): CPU Frequency Selector

For Intel Pentium CPU

| SW1 | Bus Clock | Mutiplier | CPU FREQ. |
| --- | --- | --- | --- |
|  | 50MHz | 1.5x | P54C-75 |
|  | 60MHz | 1.5x | P54C-90 |
|  | 66MHz | 1.5x | P54C-100 |
|  | 60MHz | 2x | P54C-120 |
|  | 66MHz | 2x | P54C-133 |
|  | 60MHz | 2.5x | P54C-150 |
|  | 66MHz | 2.5x | P54C/P55C-166 |
|  | 66MHz | 3x | P54C/P55C-200 |
|  | 66MHz | 3.5x | P55C-233 |

*P55C = Pentium with MMX technology

For Cyrix 6x86, 6x86L CPU

| SW1 | Bus Clock | Mutiplier | CPU FREQ. |
| --- | --- | --- | --- |
|  | 50MHz | 2x | P120+ (100MHz) |
|  | 55MHz | 2x | P133+ (110MHz) |
|  | 60MHz | 2x | P150+ (120MHz) |
|  | 66MHz | 2x | P166+ (133MHz) |

For Cyrix M2 CPU

| SW1 | Bus Clock | Mutiplier | CPU FREQ. |
| --- | --- | --- | --- |
|  | 66MHz | 2.5x | 166MHz |
|  | 60MHz | 3x | 180MHz |
|  | 66MHz | 3x | 200MHz |

For AMD K5, K6 CPU

| SW1 | Bus Clock | Mutiplier | CPU FREQ. |
| --- | --- | --- | --- |
|  | 50MHz | 1.5x | PR75 |
|  | 60MHz | 1.5x | PR90 |
|  | 66MHz | 1.5x | PR100 |
|  | 60MHz | 1.5x | PR120 |
|  | 66MHz | 1.5x | PR133 |
|  | 60MHz | 1.75x | PR150 |
|  | 66MHz | 1.75x | PR166 / K6-166 |
|  | 66MHz | 3x | K6-200 |
|  | 66MHz | 3.5x | K6-233 |

3.2	SW1(7): CMOS RAM Discharge

| SW1(7) | Function |
| --- | --- |
|  | Clear CMOS Content |
|  | Normal Operation |

3.3	SW2(1-4): CPU Voltage Selector

For Single Voltage CPU: Intel P54C, Cyrix 6x86, AMD K5

| SW2 | VIO | VCORE |
| --- | --- | --- |
|  | 3.5V | 3.5V |

For Dual Voltage CPU: Intel P55C, Cyrix 6x86L/M2, AMD K5, K6

| SW2 | VIO | VCORE | CPU |
| --- | --- | --- | --- |
|  | 3.3V | 3.5V |  |
|  | 3.3V | 3.2V | K6-233 |
|  | 3.3V | 3.1V |  |
|  | 3.3V | 3.0V |  |
|  | 3.3V | 2.9V | K6-166/200 K5-166 |
|  | 3.3V | 2.8V | P55C 6x86L/M2 |
|  | 3.3V | 2.7V |  |
|  | 3.3V | 2.6V |  |
|  | 3.3V | 2.5V |  |

3.4	J16 (7, 17): ATX Power ON Switch

This 2-pin connector is the “Power Supply On/Off Switch” on the motherboard that connects to the power switch on the case. When pressed, the power switch will force the motherboard to power on. When pressed again, it will force the motherboard to power off.

3.5	JP5: Onboard Audio (ESS1868) Selector (Option)

| JP5 | Function |
| --- | --- |
|  | Enable |
|  | Disable |

Chapter 4 I/O Ports Connector

This chapter describes the connectors and interfaces that the TI5TV(A) provides for creating a working system. Refer to Figure 3 for the location of the connectors. The following items are covered in this chapter:

4.1	J1, J6: Serial Ports	18

4.2	J2, J4: PS2 Mouse and Keyboard Connector	18

4.3	J5: Parallel Port	19

4.4	J7: ATX Power Supply Connector	19

4.5	JP1: CPU Fan Power Connector	19

4.6	J10: Floppy Drive Connector	20

4.7	J12, J11: IDE Connectors	21

4.8	J16: Front Bezel Connectors	22

Pin 1-4: Speaker	22

Pin 6 and 16: Power Saving Switch Connector	22

Pin 11-15: Power LED and Keylock	22

Pin 8 and 18: Turbo LED Connector	23

Pin 9 and 19: Reset Switch	23

Pin10 and 20: Hard Disk Drive LED Connector	23

4.9	J17: IrDA Connector	23

4.10	J8, J9: USB Connectors	24

4.11	J13: Audio I/O Connector (Option)	24

4.12	J14: CD-ROM Audio In Connector (Option)	24

Figure 3: Connector Locations of the TI5TV(A)

4.2	J7: PS2 Mouse and Keyboard Connector

J7 is composed of two mini DIN connectors for a PS2 mouse and a PS/2 keyboard. The connector on top is for a PS/2 mouse. The following tables show their respective pin outs.

| PS/2 Mouse PS/2 Keyboard | J2 Pin # | Signal Name |
| --- | --- | --- |
|  | 1 | Mouse data |
|  | 2 | N.C. |
|  | 3 | GND |
|  | 4 | 5V |
|  | 5 | Mouse clock |
|  | 6 | GND |
|  | J4 Pin # | Signal Name |
|  | 1 | Keyboard data |
|  | 2 | N.C. |
|  | 3 | GND |
|  | 4 | 5V |
|  | 5 | Keyboard clock |
|  | 6 | GND |

4.2	J1, J6: Serial Ports

The onboard serial ports of the TI5TV(A) are DB-9 connectors. J1 is COM 1 and J6 is COM2. The following table shows the pin outs of these connectors.

|  | Pin # | Signal Name |
| --- | --- | --- |
|  | 1 | DCD, Data carrier detect |
|  | 2 | RXD, Receive data |
|  | 3 | TXD, Transmit data |
|  | 4 | DTR, Data terminal ready |
|  | 5 | GND, ground |
|  | 6 | DSR, Data set ready |
|  | 7 | RTS, Request to send |
|  | 8 | CTS, Clear to send |
|  | 9 | RI, Ring indicator |

4.3	J5: Parallel Port Connector

J5 is a DB-25 connector. The following table describes the pin outs of this connector.

|  | Signal Name | J5 Pin # | J5 Pin # | Signal Name |
| --- | --- | --- | --- | --- |
|  | Line printer strobe | 1 | 14 | AutoFeed |
|  | PD0, parallel data 0 | 2 | 15 | Error |
|  | PD1, parallel data 1 | 3 | 16 | Initialize |
|  | PD2, parallel data 2 | 4 | 17 | Select |
|  | PD3, parallel data 3 | 5 | 18 | Ground |
|  | PD4, parallel data 4 | 6 | 19 | Ground |
|  | PD5, parallel data 5 | 7 | 20 | Ground |
|  | PD6, parallel data 6 | 8 | 21 | Ground |
|  | PD7, parallel data 7 | 9 | 22 | Ground |
|  | ACK, acknowledge | 10 | 23 | Ground |
|  | Busy | 11 | 24 | Ground |
|  | Paper empty | 12 | 25 | Ground |
|  | Select | 13 | N/A | N/A |

4.4	J7: ATX Power Supply Connector

J7 is a 20-pin ATX power supply connector. Refer to the following for the pin outs.

| 11 1 20 10 | Signal Name | J7 Pin# | J7 Pin # | Signal Name |
| --- | --- | --- | --- | --- |
|  | 3.3V | 11 | 1 | 3.3V |
|  | -12V | 12 | 2 | 3.3V |
|  | Ground | 13 | 3 | Ground |
|  | PS-ON | 14 | 4 | +5V |
|  | Ground | 15 | 5 | Ground |
|  | Ground | 16 | 6 | +5V |
|  | Ground | 17 | 7 | Ground |
|  | -5V | 18 | 8 | Power good |
|  | +5V | 19 | 9 | 5VSB |
|  | +5V | 20 | 10 | +12V |

4.5	JP1: CPU Fan Power Connector

JP1 is a 3-pin header for a CPU fan. The fan must be a 12V fan.

| 1 2 3 | JP1 Pin # | Signal Name |
| --- | --- | --- |
|  | 1 | Ground |
|  | 2 | +12V |
|  | 3 | Ground |

4.6	J10: Floppy Drive Connector

J10 of the TI5TV(A) is a 34-pin header and will support up to 2.88MB floppy drives.

|  | Signal Name | Pin # | Pin # | Signal Name |
| --- | --- | --- | --- | --- |
|  | Ground | 1 | 2 | RM/LC |
|  | Ground | 3 | 4 | No connect |
|  | Ground | 5 | 6 | No connect |
|  | Ground | 7 | 8 | Index |
|  | Ground | 9 | 10 | Motor enable 0 |
|  | Ground | 11 | 12 | Drive select 1 |
|  | Ground | 13 | 14 | Drive select 0 |
|  | Ground | 15 | 16 | Motor enable 1 |
|  | Ground | 17 | 18 | Direction |
|  | Ground | 19 | 20 | Step |
|  | Ground | 21 | 22 | Write data |
|  | Ground | 23 | 24 | Write gate |
|  | Ground | 25 | 26 | Track 00 |
|  | Ground | 27 | 28 | Write protect |
|  | Ground | 29 | 30 | Read data |
|  | Ground | 31 | 32 | Side 1 select |
|  | Ground | 33 | 34 | Diskette change |

4.7	J12, J11: EIDE Connectors

J12 is the primary IDE connector and J11 is the secondary IDE connector.

J12: Primary IDE Connector

|  | Signal Name | J12 Pin # | J12 Pin # | Signal Name |
| --- | --- | --- | --- | --- |
|  | Reset IDE | 1 | 2 | Ground |
|  | Host data 7 | 3 | 4 | Host data 8 |
|  | Host data 6 | 5 | 6 | Host data 9 |
|  | Host data 5 | 7 | 8 | Host data 10 |
|  | Host data 4 | 9 | 10 | Host data 11 |
|  | Host data 3 | 11 | 12 | Host data 12 |
|  | Host data 2 | 13 | 14 | Host data 13 |
|  | Host data 1 | 15 | 16 | Host data 14 |
|  | Host data 0 | 17 | 18 | Host data 15 |
|  | Ground | 19 | 20 | Key |
|  | DRQ0 | 21 | 22 | Ground |
|  | Host IOW | 23 | 24 | Ground |
|  | Host IOR | 25 | 26 | Ground |
|  | IOCHRDY | 27 | 28 | Host ALE |
|  | DACK0 | 29 | 30 | Ground |
|  | IRQ14 | 31 | 32 | No connect |
|  | Address 1 | 33 | 34 | No connect |
|  | Address 0 | 35 | 36 | Address 2 |
|  | Chip select 0 | 37 | 38 | Chip select 1 |
|  | Activity | 39 | 40 | Ground |

J11: Secondary IDE Connector

|  | Signal Name | J11 Pin # | J11 Pin # | Signal Name |
| --- | --- | --- | --- | --- |
|  | Reset IDE | 1 | 2 | Ground |
|  | Host data 7 | 3 | 4 | Host data 8 |
|  | Host data 6 | 5 | 6 | Host data 9 |
|  | Host data 5 | 7 | 8 | Host data 10 |
|  | Host data 4 | 9 | 10 | Host data 11 |
|  | Host data 3 | 11 | 12 | Host data 12 |
|  | Host data 2 | 13 | 14 | Host data 13 |
|  | Host data 1 | 15 | 16 | Host data 14 |
|  | Host data 0 | 17 | 18 | Host data 15 |
|  | Ground | 19 | 20 | Key |
|  | DRQ0 | 21 | 22 | Ground |
|  | Host IOW | 23 | 24 | Ground |
|  | Host IOR | 25 | 26 | Ground |
|  | IOCHRDY | 27 | 28 | Host ALE |
|  | DACK1 | 29 | 30 | Ground |
|  | MIRQ0 | 31 | 32 | No connect |
|  | Address 1 | 33 | 34 | No connect |
|  | Address 0 | 35 | 36 | Address 2 |
|  | Chip select 0 | 37 | 38 | Chip select 1 |
|  | Activity | 39 | 40 | Ground |

4.8	J16: Front Bezel Connectors

The front bezel of the case has a control panel which provides light indication of the computer activities and switches to change the computer status. J16 is a 20-pin header that provides interfaces for the following functions.

Speaker: Pins 1 - 4

This connector provides an interface to a speaker for audio tone generation. An 8-ohm speaker is recommended.

|  | J16 Pin # | Signal Name |
| --- | --- | --- |
|  | 1 | Speaker out |
|  | 2 | No connect |
|  | 3 | Ground |
|  | 4 | +5V |

Power Saving Switch Connector: Pins 6 and 16

This connector is for the "Green Switch" on the control panel, which, when pressed, will force the motherboard immediately into the power saving (sleep) mode.

|  | J16 Pin # | Signal Name |
| --- | --- | --- |
|  | 6 | Sleep |
|  | 16 | Ground |

Power LED and Keylock: Pins 11 - 15

The power LED indicates the status of the main power switch. The keylock switch, when closed, will disable the keyboard function.

|  | J16 Pin # | Signal Name |
| --- | --- | --- |
|  | 11 | Power LED |
|  | 12 | No connect |
|  | 13 | Ground |
|  | 14 | Keylock |
|  | 15 | Ground |

Turbo LED Connector: Pins 8 and 18

There is no turbo/deturbo function on the motherboard. The Turbo LED on the control panel will always be on when attached to this connector.

|  | J16 Pin # | Signal Name |
| --- | --- | --- |
|  | 8 | 5V |
|  | 18 | Ground |

Reset Switch: Pins 9 and 19

The reset switch allows the user to reset the system without turning the main power switch off and then on. Orientation is not required when making a connection to this header.

Hard Disk Drive LED Connector: Pins 10 and 20

This connector connects to the hard drive activity LED on control panel. This LED will flash when the HDD is being accessed.

4.9	J17: IrDA Connector

J17 is an IrDA connector on the TI5TV(A) that is used for infrared wireless communication.

| +5V IRRX IRTX N.C. GND | J17 Pin # | Signal Name |
| --- | --- | --- |
|  | 1 | +5V |
|  | 2 | No connect |
|  | 3 | Ir RX |
|  | 4 | Ground |
|  | 5 | Ir TX |

4.10	J8, J9: USB Connectors

J8 and J9 are optional USB connectors. The following table shows the pin outs of these connectors.

| J8 1 2 3 4 | USB | J9 1 2 3 4 | J8 (USB1) Pin # | J9 (USB2) Pin # | Signal Name |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 1 | 1 | Vcc |
|  |  |  | 2 | 2 | USB- |
|  |  |  | 3 | 3 | USB+ |
|  |  |  | 4 | 4 | Ground |

4.11	J13: Audio I/O Connector (Option)

|  | Signal Name | J12 Pin # | J12 Pin # | Signal Name |
| --- | --- | --- | --- | --- |
|  | N.C. | 1 | 2 | MCLK |
|  | N.C. | 3 | 4 | N.C. |
|  | N.C. | 5 | 6 | MIDI in |
|  | +5V | 7 | 8 | +5V |
|  | Joystick A | 9 | 10 | Joystick C |
|  | Joystick data A | 11 | 12 | Joystick data C |
|  | Ground | 13 | 14 | MIDI out |
|  | Ground | 15 | 16 | Joystick D |
|  | Joystick B | 17 | 18 | Joystick data D |
|  | Joystick data B | 19 | 20 | Joystick data D |
|  | +5V | 21 | 22 | N.C. |
|  | Reset | 23 | 24 | N.C. |
|  | Stereo output R | 25 | 26 | Ground |
|  | Speaker R | 27 | 28 | Ground |
|  | Speaker L | 29 | 30 | N.C. |
|  | Stereo output L | 31 | 32 | Ground |
|  | Stereo input R | 33 | 34 | -12V |
|  | Stereo input L | 35 | 36 | Ground |
|  | Microphone input | 37 | 38 | +12V |
|  | Ground | 39 | 40 | Ground |

4.12	J14: CD-ROM Audio In Connector (Option)

|  | J14 Pin # | Signal Name |
| --- | --- | --- |
|  | 1 | Ground |
|  | 2 | CD left |
|  | 3 | Ground |
|  | 4 | CD right |

Chapter 5 AMI BIOS User Guide

This chapter describes the different settings available in the Award BIOS that comes with the TI5TV motherboard. The topics covered in this chapter are as follows:

# 5.1 How to Invoke the Setup Program	26

# 5.1.1 Point and Click Interface	27

# 5.1.2 Using a Mouse with Setup	27

# 5.1.3 Using the Keyboard with Setup	28

5.1.4 Setup Main Menu	28

# 5.1.5 Original	29

# 5.1.6 Default Values	29

# 5.1.7 Optimal	30

# 5.1.8 Fail Safe	30

# 5.1.9 Save Changes and Exit	30

# 5.2 Setup Menu Categories	31

# 5.2.1 Standard Setup Menu	32

# 5.2.2 Advanced Setup Menu	33

# 5.2.3 Chipset Setup Menu	36

# 5.2.4 Power Management Setup	37

# 5.2.5 PCI/PnP Setup Menu	39

# 5.2.6 Peripheral Setup Menu	42

# 5.3 Password	44

# 5.4 Anti-Virus	45

# 5.1 How to Invoke the Setup Program

You can enter the Setup program by pressing a key (or a combination of keys), when a message for the option to enter the Setup program appears on the monitor screen during the boot-up process. Refer to the message on the screen for the necessary key combination.

A message, similar to the following, will appear on the monitor screen during the boot-up process.

"Hit <Del> if you want to run Setup"

Note:	The BIOS will normally display an option for you to enter the setup program during the boot process. Follow the instruction on the monitor screen to enter Setup.

# 5.1.1 Point and Click Interface

The Setup uses the familiar point and click navigation technique. You can point with mouse anywhere on the screen, click the left mouse button, and Setup control is transferred to the new location.

The previous window is closed. All parameters that have been changed will automatically be saved, pending the selection on the exit screen.

# 5.1.2 Using a Mouse with Setup

Setup can be accessed via a keyboard, mouse or pen. The mouse click functions are:

- Single click to change or select both global and current fields and

- Double-click to perform an operation in the selected field.

# 5.1.3 Using the Keyboard with Setup

Use the following keys to modify and move around in the Setup:

| Keystroke | Function |
| --- | --- |
| <Tab> | Move to the next window or field. |
| , , ,  | Move to the next field to the right, left above, or below. |
| <Enter> | Select in the current field. |
|  | Increments a value. |
|  | Decrements a value. |
| <Esc> | Close the current operation and return to previous level. |
| <PgUp> | Returns to the previous page. |
| <PgDn> | Advances to the next page. |
| <Home> | Returns to the beginning of the text. |
| <End> | Advances to the end of the text. |
| <Alt><H> | Access a help window. |
| <Alt><Spacebar> | Exit WINBIOS Setup. |
| Alphabetic Keys | A to Z are used in the virtual keyboard and are not case-sensitive. |
| Numeric Keys | 0 to 9 are used in the virtual keyboard and numeric keypad. |

- Setup Main Menu

After pressing the <DEL> key, the BIOS Setup Screen will display:

# 5.1.5 Original

Choose the Original icon to return to the system configuration values present in WINBIOS Setup when you first began this WINBIOS Setup session.

# 5.1.6 Default Values

The icons in this section permit you to select a group of settings for all the Setup options. Not only can you use these icons to quickly set system configuration parameters, you can choose a group of settings that have a better chance of working when the system is having configuration-related problems.

# 5.1.7 Optimal

You can load the optimal default settings for the BIOS by selecting the Optimal icon. The Optimal default settings are best-case values that should optimize system performance. If CMOS RAM is corrupted, the Optimal settings are loaded automatically.

Note: The optimal values are for regular use.

# 5.1.8 Fail Safe

You can load the Fail-Safe BIOS Setup option setting by selecting the Fail-Safe icon from the Default section of the Setup main menu.

The Fail-Safe settings provide far from optimal system performance, but are the most stable settings. Use this option as a diagnostic aid if the system is behaving erratically.

# 5.1.9 Save Changes and Exit

Select this option to save into the CMOS RAM all modifications you specify during the current session and to save the configuration changes.

Note: If you want to review the configuration, go into detail or modify, you  can continue on the next page before “Save Changes and Exit”.

# 5.2 Setup Menu Categories

Your Setup program can have up to six separate screens. Different types of system configuration parameters are set on each screen.

| Type | Description |
| --- | --- |
| Standard Setup | Set the time and date. Configure disk drives. |
| Advanced Setup | Configure basic system performance parameters. |
| Chipset Setup | Configure features specific to the chipset used in the computer. |
| Power Management Setup | Configure power conservation features. |
| PCI/PnP Setup | Configure PCI and Plug-and-Play features. |
| Peripheral Setup | Configure I/O support. |

# 5.2.1 Standard Setup Menu

Standard Setup options are displayed by choosing the Standard icon from the Setup menu. All Standard Setup options are described below.

Date/Time

Select the Date/Time option to change the date or time. The current date and time are displayed. Enter new values through the displayed window.

Floppy Drive A, B

Choose the Floppy Drive A or B icon to specify the floppy drive type. The settings are 360KB 5 1/4", 1.2MB 5 1/4”, 720KB 3 1/2", 1.44MB 3 1/2", or 2.88MB 3 1/2"

Pri Master

Pri Slave

Sec Master

Sec Slave

Choose these icons to configure the hard disk named in the option. When you click on an icon, the following parameters are listed: Type, LBA/Large Mode, Block Mode, 32Bit Mode, and PIO Mode. All parameters to IDE drives except Type.

Configuring an MFM Drive

If configuring an old MFM  hard disk drive, you must know the drive parameters (number of heads, number of cylinders, number of sectors the starting write precompensation cylinder, and drive capacity). Choose Type and choose the appropriate hard disk drive type (1-46).

User-Defined Drive

If you are configuring a SCSI drive or an MFM, RLL, ARLL, or ESDI drive with drive parameters that do not match drive types 1-46, you must select User in the Type field. You must then enter the drive parameters on the screen that appears. The drive parameters include:

- Cylinder (number of cylinders)

- Hd (number of heads)

- WP (starting write precompensation cylinder)

- Sec (number of sectors)

- Size (drive capacity)

Configuring IDE Drives

If the hard disk drive to be configured is an IDE drive, select the appropriate drive icon (Pri Master, Pri Slave, Sec Master, or Sec Slave). Choose the Type parameter and select Auto.

The BIOS automatically detects the IDE drive parameters and displays them. Click on the OK button to accept these parameters.

Configuring a CD-ROM Drive

Select the appropriate drive icon (Pri Master, Pri Slave, Sec Master, or Sec Slave). Choose the Type parameter and select CDROM. You can boot the computer from a CD-ROM drive.

# 5.2.2 Advanced Setup Menu

Quick Boot

Set this option to Enabled to instruct AMIBIOS to boot quickly when the computer is powered on. This option replaces the old Above 1 MB Memory Test Advanced Setup option.

1st Boot Device

2nd Boot Device

3rd Boot Device

4th Boot Device

These options set the sequence of boot drives (floppy drive A:, hard disk drive C:, or a CD-ROM drive) that the AMIBIOS attempts to boot from after AMIBIOS POST completes.

- M.A.R.T. (for hard disks)

S.M.A.R.T. stands for Self Monitor Analysis Reporting Technology. The default setting is Disabled.

BootUp NumLock

Set this option to Off to turn the Num Lock key off when the computer is booted so you can use the arrow keys on both the numeric keypad and the keyboard. The settings are On or Off. The default setting is On.

Floppy Drive Swap

Set this option to Enabled to permit drives A: and B: to be swapped. The settings are Enabled or Disabled. The default setting is Disabled.

Floppy Drive Seek

Set this option to Enabled to specify that floppy drive A: will perform a Seek operation at system boot. The settings are Disabled or Enabled. The Optimal and Fail-Safe default settings are Enabled.

PS/2 Mouse Support

When this option is set to Enabled, AMIBIOS supports a PS/2-type mouse. The settings are Enabled or Disabled. The default setting is Enabled.

Password Check

This option enables password checking every time the computer is powered on or every time BIOS Setup is executed. If Always is chosen, a user password prompt appears every time the computer is turned on. If Setup is chosen, the password prompt appears if BIOS is executed.

The Optimal and Power-On defaults are Setup.

OS/2 Compatible Mode

Set this option to Enable to permit AMIBIOS to run with IBM OS/2. The settings are Enabled or Disabled. The default setting is Disabled.

Internal Cache

This option specifies the caching algorithm used for L1 internal cache memory. The settings are:

| Setting | Description |
| --- | --- |
| Disabled | Neither L1 internal cache memory on the CPU or L2 secondary cache memory is enabled. |
| Write Back (default) | Use the write-back caching algorithm. |

External Cache

This option specifies the caching algorithm used for L2 secondary (external) cache memory. The settings are:

| Setting | Description |
| --- | --- |
| Disabled | Neither L1 internal cache memory on the CPU or L2 secondary cache memory is enabled. |
| Enabled (default) | Use the write-back caching algorithm. |

System BIOS Shadow cacheable

When this option is set to Enabled, the contents of the F0000h system memory segment can be read from or written to L2 secondary cache memory. The contents of the F0000h memory segment are always copied from the BIOS ROM to system RAM for faster execution.

The settings are Enabled or Disabled. The Optimal and Fail-Safe default setting is Disabled.

C000, 16K Shadow

C400, 16K Shadow

C800, 16K Shadow

CC00, 16K Shadow

D000, 16K Shadow

D400, 16K Shadow

D800, 16K Shadow

C000, 16K Shadow

These options control the location of the contents of the 16KB of ROM beginning at the specified memory location. If no adapter ROM is using the named ROM area, this area is made available to the local bus. The settings are:

| Setting | Description |
| --- | --- |
| Shadow | The contents of C0000h - C3FFFh are written to the same address in system memory (RAM) for faster execution. |
| Cache | The contents of the named ROM area are written to the same address in system memory (RAM) for faster execution, if an adapter ROM will be using the named ROM area. Also, the contents of the RAM area can be read from and written to cache memory. |
| Disable | The video ROM is not copied to RAM. The contents of the video ROM cannot be read from or written to cache memory. |

The C000~C7FF 32K default setting is Enable, other segment is Disable.

# 5.2.3 Chipset Setup Menu

DRAM Timing Setting

Use this option to specify an area in memory that sets the read/write and pre-charge timing on the memory bus. The default setting is Auto.

8-bit I/O Recovery Time  (SYSCLK)

This option specifies the length of the delay (in SYSCLKs) inserted between consecutive 8-bit I/O operations. The settings are 1, 2, 3, 4, 5, 6, 7, or 8. The Optimal and Fail-Safe default settings are 1.

16-Bit I/O Recovery Time  (SYSCLK)

This Option specifies the length of the delay (in SYSCLKs) inserted between consecutive 16-bit I/O operations. The settings are 1, 2, 3, 4, 5, 6, 7, or 8. The Optimal and Fail-Safe default settings are 1.

# 5.2.4 Power Management Setup

Power Management/APM

Set this option to Enabled to enable the power management and APM (Advanced Power Management) features.

The settings are Enabled or Disabled. The default setting is Enabled.

Instant On Support

Set this option to Enabled to allow the computer to go to full power on mode when leaving a power-conserving state. This option is only available if supported by the computer hardware. AMIBIOS uses the RTC Alarm function to wake the computer at a pre-specified time. The settings are Enabled or Disabled. The default setting is Disabled.

Green PC Monitor Power State

This option specifies the power management state that the Green PC-compliant video monitor enters after the specified period of display inactivity has expired. The settings are Disabled, Off, Standby, or Suspend. The default setting is Off.

Video Power Down Mode

This option specifies the power management state that the video subsystem enters after the specified period of display inactivity has expired. The settings are Disabled, Standby, or Suspend. The default setting is Disabled..

Hard Disk Power Down Mode

This option specifies the power management state that the hard disk drive enters after the specified period of display inactivity has expired. The settings are Disabled, Standby, or Suspend. The default setting is Disabled.

Hard Disk Timeout (Min)

This option specifies the length of a period of hard disk inactivity. When this period expires, the hard disk drive enters the power-conserving mode specified in the Hard Disk Power Down Mode option, described on the previous page. The settings are Disabled, 1Min (minutes), and all one minute intervals up to and including 15 Min. The default setting is Disabled.

Full-On to Standby Timeout

This option specifies the length of the period of system inactivity when the computer is in Full-On mode before the computer is placed in Standby mode. In Standby mode, some power use is curtailed. The settings are Disabled, 1 Min, 2 Min and all one minute intervals up to and including 15 Min. The default setting is Disabled.

Standby to Suspend Timeout

This option specifies the length of the period of system inactivity when the computer is already in Standby mode before the computer is placed in Suspend mode. In Suspend mode, nearly all power use is curtailed. The settings are Disabled, 1 Min, 2 Min and all one minute intervals up to and including 15 Min. The default setting is Disabled.

Slow Clock Ratio

This option specifies the speed at which the system clock runs in power saving modes. The settings are expressed as a ratio between the normal clock speed and the power down clock speed. The settings are 1:1, 1:2 (half as fast as normal), 1:4 (the normal clock speed), 1:8, 1:16, 1:32, 1:64, or 1:128. The default setting is 1:8.

Display Activity

This option specifies if AMIBIOS is to monitor activity on the display monitor for power conservation purposes. When this option is set to Monitor and there is no display activity for the length of time specified in the value in the Fill-On to Standby Timeout (Min) option, the computer enters a power saving state. The settings are Monitor, Wakeup, Ignore or Both. The default setting is Ignore.

| IRQ3 | IRQ4 | IRQ5 | IRQ7 |
| --- | --- | --- | --- |
| IRQ9 | IRQ10 | IRQ11 | IRQ12 |
| IRQ13 | IRQ14 | IRQ15 |  |

These options enable event monitoring. When the computer is in a power saving mode, activity on the named interrupt request line is monitored by AMIBIOS. When any activity occurs, the computer enters Full On mode.

Each of these options can be set to Monitor or Ignore. The default setting for all options is Ignore. Except IRQ3, IRQ4, IRQ14 are Monitor.

# 5.2.5 PCI/PnP Setup Menu

Plug and Play Aware OS

Set this option to Yes if the operating system installed in the computer is Plug and Play-aware. AMIBIOS only detects and enables PnP ISA adapter cards that are required for system boot. The Windows 95 operating system detects and enable all other PnP-aware adapter cards. Windows 95 is PnP-aware.

Set this option to No if the operating system (such as DOS, OS/2, Windows 3.x) does not use PnP. You must set this option correctly or PnP-aware adapter cards installed in your computer will not be configured properly. The settings are No or Yes. The Optimal and Fail-Safe default settings are Yes.

PCI Latency Timer (in PCI Clocks)

This option sets latency of all PCI devices on the PCI bus. The settings are in units equal to PCI clocks. The settings are 32, 64, 96, 128, 160, 192, 244, or 248. The Optimal and Fail-Safe default settings are 32.

PCI VGA Palette Snoop

This option must be set to Enabled if any ISA adapter card installed in the computer requires VGA palette snooping. The settings are Disabled or Enabled. The Optimal and Fail-Safe default settings are Disabled.

PCI IDE BusMaster

Set this option to Enabled to specify that the IDE controller on the PCI local bus has bus mastering capability. The settings are Disabled or Enabled. The Optimal default setting is Enabled. The Fail-safe default settings is Enabled.

Offboard PCI IDE Card

This option specifies if an offboard PCI IDE controller adapter card is used in the computer. You must also specify the PCI expansion slot on the motherboard where the offboard PCI IDE controller card is installed. If an offboard PCI IDE controller is used, the onboard IDE controller on the motherboard is automatically disabled. The settings are Disabled, Auto Slot1, Slot2, Slot3, or Slot4.

If Auto is selected, AMIBIOS automatically determines the correct setting for this option. The Optimal and Fail-Safe default settings are Auto.

Offboard PCI IDE Primary IRQ

The Optimal and Fail-Safe default settings are INTA.

Offboard PCI IDE Secondary IRQ

The Optimal and Fail-Safe default settings are INTB.

| IRQ3 | IRQ4 | IRQ5 | IRQ7 | IRQ9 |
| --- | --- | --- | --- | --- |
| IRQ10 | IRQ11 | IRQ12 | IRQ14 | IRQ15 |

These options specify the bus that the named interrupt request lines (IRQs) are used on. These options allow you to specify IRQs for use by legacy ISA adapter cards.

These options determine if AMIBIOS should remove an IRQ from the pool of available IRQs passed to BIOS configurable devices. The available IRQs pool is determined by reading the ESCD NVRAM. If more IRQs must be removed from the pool, the end user can use these PCI/PnP Setup options to remove the IRQ by assigning the option to the ISA/EISA setting. Onboard I/O is configurable by AMIBIOS. The IRQs used by onboard I/O are configured PCI/PnP.

The settings are PCI/PnP or ISA/EISA. The Optimal and Fail-Safe default settings are PCI/PnP.

Reserved Memory Size

This option specifies the size of the memory area reserved for legacy ISA adapter cards.

The settings are Disabled, 16K, 32K, or 64K. The Optimal and Fail-Safe default settings are Disabled.

Reserved Memory Address

This option specifies the beginning address (in hex) of the reserved memory area. The specified ROM memory area is reserved for use by legacy ISA adapter cards.

The settings are C0000, C4000, C8000, CC000, D0000, D4000, D8000, or DC000. The Optimal and Fail-Safe default settings are C8000.

# 5.2.6 Peripheral Setup Menu

Peripheral Setup options are displayed by choosing the Peripheral Setup icon from the Setup main menu. All Peripheral Setup options are described in this section.

Onboard FDC

This option enables the floppy drive controller on the motherboard. The settings are Enabled or Disabled. The Fail-Safe default setting is Auto.

Onboard Serial Port 1

This option enables serial port 1 on the motherboard and specifies the base I/O port address for serial port 1.

The settings are 3F8h, 3E8h, 2F8h, 2E8h, Auto, or Disabled. The Optimal and Fail-Safe default settings are 3F8h/COM1.

Onboard Serial Port 2

This option enables serial port 2 on the motherboard and specifies the base I/O port address for serial port 2.

The settings are 2F8h, 2E8h, 3F8h, 3E8h, Auto, or Disabled. The Optimal and Fail-Safe default settings are 2F8h/COM2.

IR Duplex Mode

The Optimal and Fail-Safe default settings are Half.

Onboard Parallel port

This option enables the parallel port on the motherboard and specifies the parallel port base I/O port address. The settings are 378h, 278h, 3BCH, Auto, or Disabled. The Optimal and Fail-Safe default settings are 378h/LPT1.

Parallel Port Mode

This option specifies the parallel port mode. ECP and EPP are both

bi-directional data transfer schemes that adhere to the IEEE P1284

specifications. The settings are:

| Setting | Description |
| --- | --- |
| SPP/EPP | The parallel port can be used with devices that adhere to the Enhanced Parallel Port (EPP) specification. EPP uses the existing parallel port signals to provide asymmetric bi-directional data transfer driven by the host device. |
| ECP | The parallel port can be used with devices that adhere to the Extended Capabilities Port (ECP) specification. ECP uses the DMA protocol to achieve transfer rates of approximately 2.5Mbs. ECP provides symmetric bi-directional communications. |

Parallel Port DMA

This option is only available if the setting for the Parallel Port Mode option is ECP.

The settings are Disabled. DMA CH (channel) 0, DMA CH 1, or DMA CH 3. The default setting is None.

Parallel Port IRQ

The Optimal and Fail-Safe default settings are 7.

Onboard IDE

This option specifies the onboard IDE controller channels that will be used. The settings are Primary, Secondary, Both, or Disabled. The Optimal and Fail-Safe default settings are Both.

# 5.3 Password

If You Do Not Want to Use a Password

Just press <Enter> when the password prompt appears.

Setting a Password

The password check option is enabled in Advanced Setup by choosing either Always (the password prompt appears every time the system is powered on) or Setup (the password prompt appears only when BIOS is run). The password is stored in CMOS RAM.

You can enter a password by:

- typing the password on the keyboard,

- selecting each letter via the mouse, or

- select each letter via the pen stylus.

Pen access must be customized for each specific hardware platform

When you select Supervisor or User, AMIBIOS prompts for a password. You must set the Supervisor password before you can set the User password. Enter a one through six character password. The password does not appear on the screen when typed. Make sure you write it down. If you forget it, you must drain CMOS RAM and reconfigure the system.

# 5.4 Anti-Virus

When this icon is selected from the Security section of the Setup main menu, AMIBIOS issues a warning when any program (or virus) issues a Disk Format command or attempts to write to the boot sector of the hard disk drive. The settings are Enabled or Disabled. If enabled, the following appears when a write is attempted to the boot sector. You may have to type N several times to prevent the boot sector write.

Boot Sector Write!!!

Possible VIRUS: Continue (Y/N)? _

The following appears after any attempt to format any cylinder, head, or sector of any hard disk drive via the BIOS INT 13 Hard Disk Drive Service:

Format!!!

Possible VIRUS: Continue (Y/N)? _

Chapter 6 ESS1868 (Audio) Drive Installation

The TI5TV(A) comes with an optional ESS1868 sound chip.

The Setup program provides an easy way for you to install the AudioDrive driver and applications. The topics covered in this chapter are as follows:

6.1 Audio driver install for Windows 3.1x	47

6.2 Audio driver install for Windows 95	52

6.3 Audio Rack Install for Windows 3.1x	58

6.4 Audio Rack Install for Windows 95	62

6.1 Audio driver install for Windows 3.1x

## First-time installation

1.	Start Windows 3.1 or higher.

2.	In Program Manager, choose Run from the File menu.

3.	Place Disk 1 in a floppy drive. In the Run dialog box, type the letter of the drive and setup (for example A:\Win31\ Setup), then click the OK button.  The first AudioDrive Setup window appears.

## Install the AudioDrive Driver

Here is the first AudioDrive Setup window:

Note the instructions, then click Continue to go on, or Exit to leave Setup.

After you click Continue, if you have a previously installed version of an AudioDrive driver, be sure to choose Driver Uninstallation first.  You will then have to restart windows and start the install program again from the beginning.  Once you have removed the driver or if you never had a previously installed version to start with, choose Driver Installation.

If you choose Driver Installation, the following window appears.

The window above shows the factory default settings for an ES1868 based board.  These settings may vary depending on the settings chosen by the sound card manufacturer. ES1868, the interrupt and DMA channel are software configurable.  If you wish to alter these settings, simply click on the down-arrow and choose one of the available options as determined by the driver.  Should there be a conflict with another driver or if the selected option is unavailable, the software will automatically reconfigure itself to use an available interrupt or DMA channel.

In the next window (below), choose Reboot System.

In the next Windows (below), Remove all disks from the disk drives before choosing OK.

## ESSCFG.EXE and ESSVOL.EXE

The installation of the driver will also install three files into the windows directory: ESSCFG.EXE, ESSVOL.EXE and ESSCFG.INI. ESSCFG.EXE configures AudioDrive chips to make them game-ready in DOS. ESSCFG.INI stores all the potential settings available to your hardware. ESSVOL.EXE sets the default mixer volume levels. The following lines are also added to the AUTOEXEC.BAT by the driver installation program:

C:\WINDOWS\ESSCFG.EXE /A:220 /I:7 /D:1

C:\WINDOWS\ESSVOL.EXE /V:8 /L:8 /M:0 /C:8 /S:8 /A:8

Depending on the capabilities of your hardware, the above statements may vary.

The ESSCFG settings are selected during the driver installation.  If you wish to use different settings at boot-up, use a text editor to edit your AUTOEXEC.BAT and alter the settings as you desire.

Note:  Modifying the AUTOEXEC.BAT file does not modify the settings for the Windows driver.  Run the Audio Applications Setup and uninstall and reinstall the driver or use Control Panel in Windows Main to modify the driver settings.  Refer to your Windows User Guide on modifying driver settings.  Modifying the driver settings under Windows via the Control Panel does not update AUTOEXEC.BAT with the new driver settings.

## Using the Configuration Utility in DOS

In DOS you may change the configuration of the hardware without rebooting by using ESSCFG.EXE.  Type ESSCFG at the DOS prompt to run it.  You will then be given configuration options similar to the following:

Choose 1 to select a new interrupt (IRQ) channel.

Choose 2 to select a new DMA channel.

Choose 3 to select a new interrupt channel for the MPU401.

Choose 4 to select a new base address for the MPU401.

Choose 5 to enable or disable the joystick port.

Choose 6 to display the current settings.

ESC will return you to the main menu from one of the functions above or to the DOS prompt from the main menu.

Warning:  Choosing the wrong settings could cause a system failure or a sound malfunction.  Check your hardware manual for the proper settings.

## Using the Volume Control Utility

ESSVOL.EXE allows you to change from the default volume settings of your mixer under DOS.  Note that some programs have their own mixers that will override settings previously set by

ESSVOL.EXE.  Type essvol at the DOS prompt and the current AudioDrive settings will be displayed from 0 (the lowest) to 15 (the highest) for each source.  To change the setting for a source, type essvol, switch, the source letter code, colon, and the new setting.  For example, essvol /W:12 will set the wave volume level to twelve. The following is a list of the letter codes for each source.

V	master volume	M	mic volume

L	line volume	C	CD audio volume

W	wave volume	S	synthesizer volume

A	AuxB volume (ES1688, ES1788, and ES1888 chips only)

Type essvol /? to see a list of commands.

## ESFM

ESFM generates superior-quality FM music systhesis over traditional FM synthesis, producing greater timbre and depth of instrument voices. ESFM is invoked automatically under Windows when you have properly installed hardware capable of supporting ESFM synthesis; namely ES1688, ES1788 and ES1888 chips

6.2 Audio driver install for Windows 95

Please note that the following procedures assume that you already installed Windows 95 into your system.

1. Start Windows 95.

2. Click on ”Start” (Main Menu).

3. Click on ”Settings”.

4. Click on ”Control Panel”.

5. Double click on ”System” icon.

6. Click on ”Device Manager”.

7. Under ”Other Devices” are two items of ”ESS ES1868 Plug and

Play Audio Drive”. This is because Windows 95 does not provide

the driver for ESS1868 AudioDrive. (Windows 95 only provides

those of ESS488, ESS688, ESS688, ESS1488 and ESS1688)

8. 	Choose the first ”ESS ES1868 Plug and Play 	AudioDrive” and 	click ”Properties”. The screen then shows the window below.

9. 	Click on ”Driver”.

10. 	Click on ”Change Driver”.

11.

Choose ”Sound Video and game controllers” in the list box, and 		click on ”OK”.

12.

Click on ”Save Disk”.

13. 	Place ESS1868 Audio Driver in a floppy drive. In the dialog box, 	type the letter of the drive and directory (for example: A:\WIN95), 	then click on ’OK’.

14.

Click on ”OK”.

15.

Click on ”OK”.

16.

Place Windows 95 CD in your CD-ROM.

17. Repeat Step 8 to Step 15.

18. The ESS1868 Audio Driver has been finished.

6.3 Audio Rack Install for Windows 3.1x

1. Start Windows 3.1 or higher.

2.

In Program Manager, choose ”Run” from the file menu.

3.

Place ESS1868 Audio Rack disk in a floppy drive. In the Run dialog box, type the letter of the drive and Setup (for example A:\Win31\Setup), then click the ”OK” button. The first Audio Rack Setup Window appears.

4.

Choose ”Continue” to Setup Audio Rack.

5.	The default directory is AUDRACK. To store the Audio Rack in another directory, type its name and click ”OK”.

6.	If the directory you specify does not exist, Setup will create it for you. If the Audio Rack have been previously installed, the following window appears.

7.	Choose ”Change Directory” to install the files in another directory, preserving the previous installation. If you choose ”Continue”, the following window appears.

To install all the Audio Rack, click the ”Complete Installation” button. To install only some Audio Rack, choose ”Custom Installation”

8. If you choose ”Custom Installation”, the following window appears.

To exclude an application from the installation, click its box. To store the applications in another directory, choose ”Change Directory” and specify the directory. Choose ”Install” to install Audio Rack.

9. 	After Complete Installation or Custom Installation has finished, 	the following window appears.

Audio Rack software installation is complete, click on ”Exit”.

10.	After installation, the Audio Rack icons appear in the Audio Rack 		Program group.

6.4 Audio Rack Install for Windows 95

1. Start Windows 95.

2. Double click on ”My Computer” icon.

3. Insert ESS1868 Audio Rack disk to floppy A or B.

4. Double click on “3 1/2 floppy” (A:) icon.

5. First select ”Win95” subdirectory, then select ”Setup.exe” icon and double click on them.

6. Here is the first Windows 95 Audio Rack Setup Window:

Note the instructions , then click ”Continue” to go on, or ”Exit” to leave Setup.

7.	The default directory is ”AUDRACK.W32”. To store the Audio Rack in another directory, type its name and click ”OK”.

8.	If the directory you specify does not exist, Setup will create it for you. If the Audio Rack have been previously installed, the following window appears.

9. Choose ”Change Directory” to install the files in another directory, preserving the previous installation. If you choose ”Continue”, the following window appears.

To install all the Audio applications, click the ”Complete Installation button”.

To install only some Audio applications, choose ”Custom Installation”.

10.	If you choose Custom Installation, the following window appears.

To exclude an application from the installation, click its box. To store the Audio Rack in another directory, choose change directory and specify the directory. Choose Install to install the selected Audio Applications.

11.	After ”Complete Installation” or ”Custom Installation” has finished, the following window appears.

Audio Rack32 software installation is complete, and click on “Exit”.

12.	After installation, the application icons appear in the Audio Rack32 program group: