TI5VG+

Pentium MVP3

ATX Motherboard

User’s  Manual

Version 1.0F

Contents

Chapter 1  Introduction	1

Chapter 2  Specifications	2

Chapter 3  Hardware Description	5

3.1  Processor and CPU Voltage	7

3.2  L2 Cache Memory	7

3.3  Main Memory	7

3.4  BIOS	9

3.5  Onboard PCI EIDE	10

3.6  Onboard Multi-I/O	10

3.7  Onboard Hardware Monitoring IC	10

3.8  I/O Port Address Map	11

3.9  DMA Channels	11

3.10  Interrupt Request Lines (IRQ)	12

3.11  Accelerated Graphics Port (AGP) Slot	12

Chapter 4  Hardware Settings	13

4.1  SW1(1-8): CPU Frequency Selector	15

4.2  JP1: DRAM Operating Frequency	20

4.3  SW2(1-4): CPU Voltage Selector	20

4.4  JP5: Clear CMOS Selection	22

Chapter 5  Installation	23

5.1  I/O Connectors	25

5.2  J1: PS/2 Keyboard and PS/2 Mouse Connectors	25

5.3  J2: USB Connector	26

5.4  J3, J5 Serial Ports	26

5.5  J4: Parallel Port Connector	27

5.6  J6: ATX Power Supply Connector	27

5.7  J7: Floppy Drive Connector	28

5.8  IDE1, IDE2: EIDE Connectors	29

5.9  J8: CPU Fan Power Connector	30

5.10  J9: IrDA Connector	30

5.11  J10 Wake on LAN Connector	30

5.12  J12: Chassis Fan Power Connector	30

5.13  J13 Front Bezel Connector	31

Chapter 6  BIOS Configuration	33

6.1  BIOS Introduction	36

6.2  BIOS Setup	36

6.3  Standard CMOS Setup	38

6.4  BIOS Features Setup	41

6.5  Chipset Features Setup	44

6.6  Power Management Setup	47

6.7  PNP/PCI Configuration	49

6.8  Load BIOS Defaults	52

6.9  Load Setup Defaults	52

6.10  Integrated Peripherals	53

6.11  Supervisor / User Password	55

6.12  IDE HDD Auto Detection	56

6.13  HDD Low Level Format	56

6.14  Save & Exit Setup	57

6.15  Exit Without Saving	57

Chapter 7  LANDesk User’s Guide	59

7.1  Introduction	60

7.2  Installation	61

Chapter 1  Introduction

This manual is designed to give you information on the TI5VG+ Motherboard. It is divided into the following six sections:

- Introduction

- Specifications

- Hardware Description

- Hardware Settings

- Installation

- BIOS Configuration

Checklist

Please check that your package is complete and contains the items below. If you discover damaged or missing items, please contact your dealer.

- The TI5VG+ Motherboard

- This user’s manual

- 1 IDE ribbon cable

- 1 floppy ribbon cable

- 1 CD containing Bus Master IDE drivers, flash utility,  other drivers and System Monitor utility.  The LANDesk Client Manager software is optional.

Chapter 2  Specifications

Based on VIA’s MVP3 chipset, the TI5VG+ is an ATX Pentium Socket 7 motherboard that supports all the features to make a Microsoft PC’97 compliant PCI/ISA system.

The TI5VG+ comes with an Accelerated Graphics Port (AGP) slot, power management functionality that is compliant with ACPI (Advanced Configuration and Power Interface) and legacy APM requirements. In addition to standard PIO and DMA mode operation, it features Ultra DMA/33 for improved disk I/O throughput. The main features of this motherboard are listed as follows:

Main Processor

Intel Pentium 100/133/166/200

Intel Pentium MMX (P55C) 166/200/233

Cyrix 6x86/6x86L P166+/P200+

Cyrix 6x86MX PR166/PR200/PR233/PR266

Cyrix 6x86MII PR266/PR300/PR333/PR366

AMD K5 PR100/PR133/PR166

AMD K6 166/200/233/266/300

AMD K6-2/300/333/350/366/380/400/450

IDT WinChip C6 180/200/225MHz

IDT WinChip2-3D 200/225/266MHz

L2 Cache

512KB or 1MB Pipelined Burst Synchronous SRAM onboard

Main Memory

Two 72-pin SIMM sockets and three 168-pin DIMM sockets

SIMMs: FPM (Fast Page Mode) or EDO (Extended Data Out)

SIMM Size: 4MB, 8MB, 16MB, 32MB, 64MB

DIMMs: EDO DRAM or SDRAM (Synchronous DRAM)

DIMM Size: 8MB, 16MB, 32MB, 64MB, 128MB

Chipset

VIA’s MVP3 chipset with built-in PCI EIDE

[

Onboard I/O

Winbond W83877 for two serial, one parallel, one floppy drive interface and IrDA support

Onboard Bus Mastering EIDE

Two EIDE interfaces for up to four devices, support PIO Mode 3/4 or Ultra DMA/33 IDE Hard Disk  and ATAPI CD-ROM.

BIOS

Licensed BIOS with additional features:

- FLASH EEPROM (128KB) for BIOS update

- ISA Plug and Play (PnP) extension

- Power management

Hardware Monitoring IC

The Winbond W83781D hardware monitoring IC on the motherboard monitors the system temperature, voltages and fan speed. With the use of a system management utility, the hardware status can be checked and warning message can be displayed when there is a problem.

Windows 95 Shut-Down

This allows the PC to power off via Windows 95 operating system.

PC97 Compliance

The TI5VG+ implements BIOS and hardware designs that meet PC97 standards and is fully compatible with all Windows operating systems.

Year 2000 Compliant BIOS

The onboard Award BIOS is Year 2000 Compliant and will pass software applications that have the tendency to invoke INT1AH function 04H such as year2000.exe utility released by NSTL.

Expansion Slots

Five 32-bit PCI slots

Two 16-bit ISA slots

One AGP slot (Accelerated Graphics Port)

Form Factor

ATX,12”x 7.1” (30.5cm x 18cm)

This page is intentionally left blank.

Chapter 3  Hardware Description

This chapter briefly describes each of the major features of the TI5VG+ motherboard. The layout of the board is shown in Figure 1 which shows the locations of the key components. The topics covered in this chapter are as follows:

3.1  Processor and CPU Voltage	7

3.2  L2 Cache Memory	7

3.3  Main Memory	7

3.4  BIOS	9

3.5  Onboard PCI EIDE	10

3.6  Onboard Multi-I/O	10

3.7  Onboard Hardware Monitoring IC	10

3.8  I/O Port Address Map	11

3.9  DMA Channels	11

3.10  Interrupt Request Lines (IRQ)	12

3.11  Accelerated Graphics Port (AGP) Slot	12

Figure 1: Layout of the TI5VG+ Motherboard

3.1  Processor and CPU Voltage

The TI5VG+ is designed to take a Pentium Processor with a bus speed of  66, 75, 83 and 100 MHz. The internal clock of the CPU can be multiples of 1.5, 2, 2.5, 3, 3.5, 4, 4.5, 5 and 5.5 of the bus clock, the CPU frequency can be 100 to 550MHz.

The onboard CPU voltage regulator provides support for CPUs Vcore which require voltage other than 3.3V such as 2.8V, and/or power consumption higher than 10 watts.

3.2  L2 Cache Memory

The TI5VG+ supports 512KB or 1MB P.B. (Pipelined Burst) Synchronous Cache. The P.B. Synchronous Cache boosts the system’s performance, 10% higher than regular Asynchronous Cache.

3.3  Main Memory

The TI5VG+ motherboard supports two 72-pin SIMMs and three 168-pin DIMMs (Dual In-line Memory Module) sockets. The DRAMs for the SIMM sockets can be 4MB, 8MB, 16MB, 32MB, and 64MB in EDO and FPM types. DIMM modules can be 8MB, 16MB, 32MB, 64MB and 128MB in EDO DRAM or SDRAM type.

The following should be noted when populating the SIMM/DIMM sockets:

1. 	Each bank** should consist of the same size SIMMs.

- Each bank should consist of the same type SIMMs. For example,  SIMM1 and SIMM2 are both be EDO or Page Mode.

- Each bank consists of two SIMM slots or one DIMM slot. For example, SIMM 1 and SIMM 2 are considered as one bank and DIMM 1 as another one.

- 72-pin SIMM (5V) - EDO DRAM

| (SIMM1, SIMM2) | Total Memory |
| --- | --- |
| 4MB2 | 8MB |
| 8MB2 | 16MB |
| 16MB2 | 32MB |
| 32MB2 | 64MB |
| 64MB2 | 128MB |

(2)	168-pin DIMM (3.3V) - SDRAM or EDO DRAM

| DIMM3 | DIMM2 | DIMM1 | Total Memory |
| --- | --- | --- | --- |
| 8MB | ----- | ----- | 8MB |
| 16MB | ----- | ----- | 16MB |
| 32MB | ----- | ----- | 32MB |
| 64MB | ----- | ----- | 64MB |
| 128MB | ----- | ----- | 128MB |
| 8MB | 8MB | ----- | 16MB |
| 16MB | 8MB | ----- | 24MB |
| 32MB | 8MB | ----- | 40MB |
| 64MB | 8MB | ----- | 72MB |
| 128MB | 8MB | ----- | 136MB |
| 8MB | 8MB | 8MB | 24MB |
| 16MB | 8MB | 8MB | 32MB |
| 32MB | 8MB | 8MB | 48MB |
| 64MB | 8MB | 8MB | 80MB |
| 128MB | 8MB | 8MB | 144MB |
| 16MB | 16MB | ----- | 32MB |
| 32MB | 16MB | ----- | 48MB |
| 64MB | 16MB | ----- | 80MB |
| 128MB | 16MB | ----- | 144MB |
| 16MB | 16MB | 8MB | 40MB |
| 32MB | 16MB | 8MB | 56MB |
| 64MB | 16MB | 8MB | 88MB |
| 128MB | 16MB | 8MB | 152MB |
| 16MB | 16MB | 16MB | 48MB |
| 32MB | 16MB | 16MB | 64MB |

168Pin DIMM (3.3V) - SDRAM or EDO DRAM (continued)

| DIMM3 | DIMM2 | DIMM1 | Total Memory |
| --- | --- | --- | --- |
| 64MB | 16MB | 16MB | 96MB |
| 128MB | 16MB | 16MB | 160MB |
| 32MB | 32MB | ----- | 64MB |
| 64MB | 32MB | ----- | 96MB |
| 128MB | 32MB | ----- | 160MB |
| 32MB | 32MB | 8MB | 72MB |
| 64MB | 32MB | 8MB | 104MB |
| 128MB | 32MB | 8MB | 168MB |
| 32MB | 32MB | 16MB | 80MB |
| 64MB | 32MB | 16MB | 112MB |
| 128MB | 32MB | 16MB | 176MB |
| 32MB | 32MB | 32MB | 96MB |
| 64MB | 32MB | 32MB | 128MB |
| 128MB | 32MB | 32MB | 192MB |
| 64MB | 64MB | ----- | 128MB |
| 128MB | 64MB | ----- | 192MB |
| 64MB | 64MB | 8MB | 136MB |
| 128MB | 64MB | 8MB | 200MB |
| 64MB | 64MB | 16MB | 144MB |
| 128MB | 64MB | 16MB | 208MB |
| 64MB | 64MB | 32MB | 160MB |
| 128MB | 64MB | 32MB | 224MB |
| 64MB | 64MB | 64MB | 192MB |
| 128MB | 64MB | 64MB | 256MB |
| 128MB | 128MB | 128MB | 384MB |

3.4  BIOS

The BIOS on the TI5VG+ motherboard provides the standard BIOS functions plus the following additional features:

1.  DMI (Desktop Management Interface)

The motherboard supports DMI to enable computer system management in an enterprise.

2.  ISA Plug and Play (PnP) Extension

Unlike PCI cards which are plug and play, ISA cards require setting jumpers to resolve hardware conflicts. To make a computer system PnP, an ISA PnP standard is established and supported by new OSes, such as Windows 95. Under Windows 95, the motherboard BIOS must have ISA PnP extension to support the new ISA PnP cards.

3.  Power  Management

The power management feature provides power savings by slowing down the CPU clock, turning off the monitor screen and stopping the HDD spindle motor.

3.5  Onboard PCI EIDE

The PCI EIDE controller is part of the MVP3 chipset. It supports PIO mode 3/4 and bus mastering . The peak transfer rate of PIO mode 3/4 can be as high as 17MB/sec. Using HDDs that support bus mastering, the peak transfer rate can reach 22MB/sec. There are two IDE connectors, primary IDE and secondary IDE. With two devices per connector, up to four IDE drives are supported. The MVP3 also supports  Ultra DMA/33, a new “synchronous DMA” protocol for faster data transfers.

3.6  Onboard Multi-I/O

The onboard multi-I/O chip, Winbond W83877, supports two 16550 UART-compatible serial ports, one high speed EPP/ECP parallel port and one floppy controller. The floppy controller supports 2.88MB format. The I/O port addresses of the serial and parallel ports are programmable via BIOS set-up.

3.7  Onboard Hardware Monitoring IC

The Winbond W83781D hardware monitoring IC on the motherboard monitors the system temperature, voltages and fan speed. With the use of a system management utility, the hardware status can be checked and warning message can be displayed when there is a problem.

3.8  I/O Port Address Map

Each peripheral device in the system is assigned a set of I/O port addresses which also becomes the identity of the device. There are a total of 1K port address space available. The following table lists the I/O port addresses used on the motherboard.

| Address | Device Description |
| --- | --- |
| 000h - 01Fh | DMA Controller #1 |
| 020h - 03Fh | Interrupt Controller #1 |
| 040h - 05Fh | Timer |
| 060h - 06Fh | Keyboard Controller |
| 070h - 07Fh | Real Time Clock, NMI |
| 080h - 09Fh | DMA Page Register |
| 0A0h - 0BFh | Interrupt Controller #2 |
| 0C0h - 0DFh | DMA Controller #2 |
| 0F0h | Clear Math Coprocessor Busy Signal |
| 0F1h | Reset Math Coprocessor |
| 1F0h - 1F7h | IDE Interface |
| 278 - 27F | Parallel Port #2 (LPT2) |
| 2F8h - 2FFh | Serial Port #2 (COM2) |
| 2B0 - 2DF | Graphics adapter Controller |
| 378h - 3FFh | Parallel Port #1(LPT1) |
| 360 - 36F | Network Ports |
| 3B0 - 3BF | Monochrome & Printer adapter |
| 3C0 - 3CF | EGA adapter |
| 3D0 - 3DF | CGA adapter |
| 3F0h - 3F7h | Floppy Disk Controller |
| 3F8h - 3FFh | Serial Port #1 (COM1) |

3.9  DMA Channels

There are seven DMA Channels available on the motherboard. Only DMA2 is used by the floppy controller. In the case that ECP mode on the parallel port is used, DMA1 or DMA3 will be used.

3.10  Interrupt Request Lines (IRQ)

There are a total of 15 IRQ lines available on the motherboard. Peripheral devices use interrupt request lines to notify the CPU for the service required. The following table shows the IRQ used by the devices on the motherboard.

| Level | Function |
| --- | --- |
| IRQ0 | System Timer Output |
| IRQ1 | Keyboard |
| IRQ2 | Interrupt Cascade |
| IRQ3 | Serial Port #2 |
| IRQ4 | Serial Port #1 |
| IRQ5 | Parallel Port #2 |
| IRQ6 | Floppy Disk Controller |
| IRQ7 | Parallel Port #1 |
| IRQ8 | Real Time Clock |
| IRQ9 | Software Redirected to Int 0Ah |
| IRQ10 | Reserved |
| IRQ11 | Reserved |
| IRQ12 | Reserved |
| IRQ13 | 80287 |
| IRQ14 | Primary IDE |
| IRQ15 | Secondary IDE |

3.11  Accelerated Graphics Port (AGP) Slot

The TI5VG+ motherboard comes with an AGP slot for an AGP VGA card that supports 133MHz 2X mode for AD and SBA signaling to provide high-performance 3D graphic display.

Chapter 4  Hardware Settings

The following sections describe the necessary procedures and proper jumper settings to configure the TI5VG+ motherboard.

4.1  SW1(1-8): CPU Frequency Selector	15

4.2  JP1: DRAM Operating Frequency	20

4.3  SW2(1-4): CPU Voltage Selector	20

4.4  JP5: Clear CMOS Selection	22

In the case your motherboard TI5VG+-100MHz has a processor supporting 100MHz CPU clock (such as AMD K6-300) and your DIMM module is PC100 compliant, the SDRAM clock can be set to 100MHz. Otherwise, set the SDRAM clock at 66MHz.

For the locations of the jumpers, refer to Figure 2 on the following page.

The following examples show the conventions used in this chapter

|  | Jumper Open |
| --- | --- |
|  | Jumper Closed |

|  | Switch 1 and switch 2 are set to OFF. Switch 3 and switch 4 are set to ON. |
| --- | --- |

Figure 2: Jumper Locations of the TI5VG+

4.1  SW1(1-8): CPU Frequency Selector

For Intel Pentium, IDT WinChip 2-3D / C6

| SW1 | CPU Clock | AGP Clock | Multiplier | CPU FREQ. |
| --- | --- | --- | --- | --- |
| off on on off off off off on | 66MHz | 66MHz | 1.5x | P54C-100 |
| off on on off on off off on | 66MHz | 66MHz | 2x | P54C-133 |
| off on on off on on off on | 66MHz | 66MHz | 2.5x | P54C/P55C-166 |
| off on on off off on off on | 66MHz | 66MHz | 3x | P54C/P55C-200 WinChip 2-3D / C6-200 |
| off on on off off off off on | 66MHz | 66MHz | 3.5x | P55C-233 |
| off off on off off on off on | 75MHz | 75MHz | 3x | WinChip 2-3D / C6-225 |
| off on on off on off on on | 66MHz | 66MHz | 4x | WinChip 2-3D 266 |
| off off off on off on on off | 100MHz | 66MHz | 2.33x | WinChip 2 266 |
| off off off on on on off off | 100MHz | 66MHz | 2.5x | WinChip 2 300 |

For Cyrix 6x86, 6x86L, 6x86MX CPU

| SW1 | CPU Clock | AGP Clock | Multiplier | CPU FREQ. |
| --- | --- | --- | --- | --- |
| off on on off on off off on | 66MHz | 66MHz | 2x | P166+ (133MHz) |
| off off on off on off off on | 75MHz | 75MHz | 2x | P200+ (150MHz) |
| off on on off on on off on | 66MHz | 66MHz | 2.5x | PR200 (166MHz) |
| off on on off off on off on | 66MHz | 66MHz | 3x | PR233 (200MHz) |
| off on on off off off off on | 66MHz | 66MHz | 3.5x | PR266 (233MHz) |
| off off on off on on off on | 75MHz | 75MHz | 2.5x | PR233 (187.5MHz) |
| off on off off on on off off | 83MHz | 66MHz | 2.5x | PR266 (208MHz) |
| off on off off off on off off | 83MHz | 66MHz | 3x | PR300 (250MHz) |
| off off off on on on off off | 100MHz | 66MHz | 2.5x | PR333 (250MHz) |
| off off off on off on off off | 100MHz | 66MHz | 3x | PR366 (300MHz) |
| off off off on on on off off | 100MHz | 66MHz | 2.5x | MII-366 (250MHz) |

| off off off on off on off off | 100MHz | 66MHz | 3x | MII-380 (300MHz) |
| --- | --- | --- | --- | --- |
| on off off on off off off off | 95MHz | 63MHz | 3.5x | MII-400 (333MHz) |
| off off off on off off off off | 100MHz | 66MHz | 3.5x | MII-433 (350MHz) |
| on off off on on off on off | 95MHz | 63MHz | 4x | MII-450 (380MHz) |
| off off off on on off on off | 100MHz | 66MHz | 4x | MII-466 (400MHz) |

For AMD K5, K6, K6-2 CPU

| SW1 | CPU Clock | AGP Clock | Multiplier | CPU FREQ. |
| --- | --- | --- | --- | --- |
| off on on off off off off on | 66MHz | 66MHz | 1.5x | PR100 |
| off on on off on off off on | 66MHz | 66MHz | 2x | PR133 |
| off on on off on on off on | 66MHz | 66MHz | 2.5x | PR166 / K6-166 |
| off on on off off on off on | 66MHz | 66MHz | 3x | K6-200 |
| off on on off off off off on | 66MHz | 66MHz | 3.5x | K6-233 |
| off on on off on off on on | 66MHz | 66MHz | 4x | K6-266 |
| off on on off on on on on | 66MHz | 66MHz | 4.5x | K6-300 |
| off off off on off on off off | 100MHz | 66MHz | 3x | K6-2/300 |
| on off off on off off off off | 95MHz | 63MHz | 3.5x | K6-2/333 |
| off off off on off off off off | 100MHz | 66MHz | 3.5x | K6-2/350 |
| off on on off off off on on | 66MHz | 66MHz | 5.5x | K6-2/366 |

| on off off on on off on off | 95MHz | 64MHz | 4x | K6-2/380 |
| --- | --- | --- | --- | --- |
| off on on off on off off on | 66MHz | 66MHz | 6x | K6-2/400 |
| off off off on on off on off | 100MHz | 66MHz | 4x | K6-2/400 K6-3/400 |
| off off off on on on on off | 100MHz | 66MHz | 4.5x | K6-2/450* K6-3/450* |
| on off off on off on on off | 95MHz | 63MHz | 5x | K6-2/475* K6-3/475* |
| off off off on off on on off | 100MHz | 66MHz | 5x | K6-2/500* K6-3/500* |

|  | AMD K6-2 450 and K6-3 CPUs have 2.4V Vcore voltage requirement. |
| --- | --- |

4.2  JP1: DRAM Operating Frequency

|  | JP1 |  | SDRAM Frequency |  |
| --- | --- | --- | --- | --- |
|  | pin 1-2 | short | Run CPU Clock |  |
|  | pin 2-3 | short | Run AGP Clock |  |

|  | Set the SDRAM Frequency to Run CPU Clock only when the CPU clock is 100MHz and the DIMM modules are PC-100. |
| --- | --- |

4.3  SW2(1-4): CPU Voltage Selector

For Single Voltage CPU:

| SW2 | VIO | VCORE | CPU |
| --- | --- | --- | --- |
| on on on on | 3.5V | 3.5V | Intel P54C, Cyrix 6x86, AMD K5, IDT WinChip C6 |

For Dual Voltage CPU:

Intel P55C, Cyrix 6x86L/MX/MII, AMD K6/K6-2, IDT WinChip 2-3D

| SW2 | VIO | VCORE | CPU |
| --- | --- | --- | --- |
| on on on on | 3.3V | 3.5V | WinChip 2-3D (0.35) |
| off on on on | 3.3V | 3.4V |  |
| on off on on | 3.3V | 3.3V |  |

Remarks: 0.35 and 0.25 refers to CPU manufacturing process.

For Dual Voltage CPU:

Intel P55C, Cyrix 6x86L/MX/MII, AMD K6/K6-2, IDT WinChip 2-3D

| SW2 | VIO | VCORE | CPU |
| --- | --- | --- | --- |
| off off on on | 3.3V | 3.2V | K6-233 (0.35) |
| on on off on | 3.3V | 3.1V |  |
| off on off on | 3.3V | 3.0V |  |
| on off off on | 3.3V | 2.9V | K6-166/200 6x86MX |
| off off off on | 3.3V | 2.8V | WinChip 2-3D (0.35) P55C 6x86L |
| on on on off | 3.3V | 2.7V |  |
| off on on off | 3.3V | 2.6V |  |
| on off on off | 3.3V | 2.5V |  |
| off off on off | 3.3V | 2.4V |  |
| on on off off | 3.3V | 2.3V |  |

For Dual Voltage CPU:

Intel P55C, Cyrix 6x86L/MX/MII, AMD K6/K6-2

| SW2 | VIO | VCORE | CPU |
| --- | --- | --- | --- |
| off on off off | 3.3V | 2.2V | K6, K6-2 (0.25) |
| on off off off | 3.3V | 2.1V |  |
| off off off off | 3.3V | 2.0V |  |

4.4  JP5: Clear CMOS Selection

Use JP5, a 3-pin header, to clear the contents of the CMOS RAM. Note that the ATX-power connector should be disconnected from the motherboard before clearing CMOS. Do not clear the CMOS RAM unless it is absolutely necessary. You will lose your password, etc.

| JP5 | Function |
| --- | --- |
| 1 2 3 | Normal |
| 1 2 3 | Clear CMOS |

Chapter 5  Installation

This chapter describes the connectors and interfaces that the TI5VG+ provides for creating a working system. Refer to Figure 3 for the location of the connectors.

The following items are covered in this chapter:

5.1  I/O Connectors	25

5.2  J1: PS/2 Keyboard and PS/2 Mouse Connectors	25

5.3  J2: USB Connector	26

5.4  J3, J5 Serial Ports	26

5.5  J4: Parallel Port Connector	27

5.6  J6: ATX Power Supply Connector	27

5.7  J7: Floppy Drive Connector	28

5.8  IDE1, IDE2: EIDE Connectors	29

5.9  J8: CPU Fan Power Connector	30

5.10  J9: IrDA Connector	30

5.11  J10 Wake on LAN Connector	30

5.12  J12: Chassis Fan Power Connector	30

5.13  J13 Front Bezel Connector	31

Figure 3: Connector Location on the TI5VG+

5.1  I/O Connectors

The I/O connectors connect the TI5VG+ to the most common peripherals. To attach cables to these connectors, carefully align Pin 1 of the cables to that of the connectors. Refer to Figure 4 for the location and orientation of the connectors.

Figure 4: Orientation of the I/O Connector

5.2  J1: PS/2 Keyboard and PS/2 Mouse Connectors

PS/2 Mouse

PS/2 Keyboard

Below are the pin-out assignments of the connectors.

| Signal Name | Keyboard | Mouse | Signal Name |
| --- | --- | --- | --- |
| Keyboard data | 1 | 1 | Mouse data |
| N.C. | 2 | 2 | N.C. |
| GND | 3 | 3 | Ground |
| 5V | 4 | 4 | 5V |
| Keyboard clock | 5 | 5 | Mouse Clock |
| N.C. | 6 | 6 | N.C. |

5.3  J2: USB Connector

J2 is the standard USB external connector consisting of two ports. USB support allows connections of up to 64 plug and play external peripherals per channel. The following table shows the pin outs of these ports.

| USB0 USB1 |  | J2 Pin # | Signal Name |
| --- | --- | --- | --- |
|  |  | 1/5 | Vcc |
|  |  | 2/6 | USB- |
|  |  | 3/7 | USB+ |
|  |  | 4/8 | Ground |

5.4  J3, J5 Serial Ports

The onboard serial ports of the TI5VG, J3 and J5, are two DB-9 external connectors. The following table shows the pin-out assignments of the these connectors.

| Signal Name | Pin # | Pin # | Signal Name |
| --- | --- | --- | --- |
| DCD, Data carrier detect | 1 | 6 | DSR, Data set ready |
| RXD, Receive data | 2 | 7 | RTS, Request to send |
| TXD, Transmit data | 3 | 8 | CTS, Clear to send |
| DTR, Data terminal ready | 4 | 9 | RI, Ring indicator |
| GND, ground | 5 | 10 | Not Used |

J4 (Parallel Port)

J3 (COM1 )		 J5 (COM2)

5.5  J4: Parallel Port Connector

J4 is a DB-25 external connector. The following table describes the pin- out assignments of this connector. See the figure on the previous page.

|  | Signal Name | Pin # | Pin # | Signal Name |
| --- | --- | --- | --- | --- |
|  | Line printer strobe | 1 | 14 | AutoFeed |
|  | PD0, parallel data 0 | 2 | 15 | Error |
|  | PD1, parallel data 1 | 3 | 16 | Initialize |
|  | PD2, parallel data 2 | 4 | 17 | Select |
|  | PD3, parallel data 3 | 5 | 18 | Ground |
|  | PD4, parallel data 4 | 6 | 19 | Ground |
|  | PD5, parallel data 5 | 7 | 20 | Ground |
|  | PD6, parallel data 6 | 8 | 21 | Ground |
|  | PD7, parallel data 7 | 9 | 22 | Ground |
|  | ACK, acknowledge | 10 | 23 | Ground |
|  | Busy | 11 | 24 | Ground |
|  | Paper empty | 12 | 25 | Ground |
|  | Select | 13 | N/A | N/A |

5.6  J6: ATX Power Supply Connector

J6 is a 20-pin ATX power supply connector. Refer to the following table for the pin out assignments.

| 11 1 20 10 | Signal Name | Pin # | Pin # | Signal Name |
| --- | --- | --- | --- | --- |
|  | 3.3V | 11 | 1 | 3.3V |
|  | -12V | 12 | 2 | 3.3V |
|  | Ground | 13 | 3 | Ground |
|  | PS-ON | 14 | 4 | +5V |
|  | Ground | 15 | 5 | Ground |
|  | Ground | 16 | 6 | +5V |
|  | Ground | 17 | 7 | Ground |
|  | -5V | 18 | 8 | Power good |
|  | +5V | 19 | 9 | 5VSB |
|  | +5V | 20 | 10 | +12V |

5.7  J7: Floppy Drive Connector

J7 of the TI5VG+ is a 34-pin header and will support up to 2.88MB floppy drives.

|  | Signal Name | Pin # | Pin # | Signal Name |
| --- | --- | --- | --- | --- |
|  | Ground | 1 | 2 | RM/LC |
|  | Ground | 3 | 4 | No connect |
|  | Ground | 5 | 6 | No connect |
|  | Ground | 7 | 8 | Index |
|  | Ground | 9 | 10 | Motor enable 0 |
|  | Ground | 11 | 12 | Drive select 1 |
|  | Ground | 13 | 14 | Drive select 0 |
|  | Ground | 15 | 16 | Motor enable 1 |
|  | Ground | 17 | 18 | Direction |
|  | Ground | 19 | 20 | Step |
|  | Ground | 21 | 22 | Write data |
|  | Ground | 23 | 24 | Write gate |
|  | Ground | 25 | 26 | Track 00 |
|  | Ground | 27 | 28 | Write protect |
|  | Ground | 29 | 30 | Read data |
|  | Ground | 31 | 32 | Side 1 select |
|  | Ground | 33 | 34 | Diskette change |

5.8  IDE1, IDE2: EIDE Connectors

IDE1: Primary IDE Connector

|  | Signal Name | Pin # | Pin # | Signal Name |
| --- | --- | --- | --- | --- |
|  | Reset IDE | 1 | 2 | Ground |
|  | Host data 7 | 3 | 4 | Host data 8 |
|  | Host data 6 | 5 | 6 | Host data 9 |
|  | Host data 5 | 7 | 8 | Host data 10 |
|  | Host data 4 | 9 | 10 | Host data 11 |
|  | Host data 3 | 11 | 12 | Host data 12 |
|  | Host data 2 | 13 | 14 | Host data 13 |
|  | Host data 1 | 15 | 16 | Host data 14 |
|  | Host data 0 | 17 | 18 | Host data 15 |
|  | Ground | 19 | 20 | Key |
|  | DRQ0 | 21 | 22 | Ground |
|  | Host IOW | 23 | 24 | Ground |
|  | Host IOR | 25 | 26 | Ground |
|  | IOCHRDY | 27 | 28 | Host ALE |
|  | DACK0 | 29 | 30 | Ground |
|  | IRQ14 | 31 | 32 | No connect |
|  | Address 1 | 33 | 34 | No connect |
|  | Address 0 | 35 | 36 | Address 2 |
|  | Chip select 0 | 37 | 38 | Chip select 1 |
|  | Activity | 39 | 40 | Ground |

IDE2: Secondary IDE Connector

|  | Signal Name | Pin # | Pin # | Signal Name |
| --- | --- | --- | --- | --- |
|  | Reset IDE | 1 | 2 | Ground |
|  | Host data 7 | 3 | 4 | Host data 8 |
|  | Host data 6 | 5 | 6 | Host data 9 |
|  | Host data 5 | 7 | 8 | Host data 10 |
|  | Host data 4 | 9 | 10 | Host data 11 |
|  | Host data 3 | 11 | 12 | Host data 12 |
|  | Host data 2 | 13 | 14 | Host data 13 |
|  | Host data 1 | 15 | 16 | Host data 14 |
|  | Host data 0 | 17 | 18 | Host data 15 |
|  | Ground | 19 | 20 | Key |
|  | DRQ0 | 21 | 22 | Ground |
|  | Host IOW | 23 | 24 | Ground |
|  | Host IOR | 25 | 26 | Ground |
|  | IOCHRDY | 27 | 28 | Host ALE |
|  | DACK1 | 29 | 30 | Ground |
|  | MIRQ0 | 31 | 32 | No connect |
|  | Address 1 | 33 | 34 | No connect |
|  | Address 0 | 35 | 36 | Address 2 |
|  | Chip select 0 | 37 | 38 | Chip select 1 |
|  | Activity | 39 | 40 | Ground |

5.9  J8: CPU Fan Power Connector

J8 is a 3-pin header for the CPU fan power connector. The fan must be a 12V fan.

| 3 2 1 | Pin # | Signal Name |
| --- | --- | --- |
|  | 1 | Fan sensor |
|  | 2 | +12V |
|  | 3 | Ground |

5.10  J9: IrDA Connector

This connector is used for an IrDA connector that supports infrared wireless communication with IrDA devices.

| +5V IRRX IRTX N.C. GND | Pin # | Signal Name |
| --- | --- | --- |
|  | 1 | +5V |
|  | 2 | No connect |
|  | 3 | Ir RX |
|  | 4 | Ground |
|  | 5 | Ir TX |

5.11  J10 Wake on LAN Connector

J10is a 3-pin header for Wake on LAN function on the motherboard.

| 3 2 1 | Pin # | Signal Name |
| --- | --- | --- |
|  | 1 | 5VSB |
|  | 2 | Ground |
|  | 3 | Wake on LAN |

5.12  J12: Chassis Fan Power Connector

J12 is a 3-pin header for the chassis fan power connector. The fan must be a 12V fan.

| 3 2 1 | Pin # | Signal Name |
| --- | --- | --- |
|  | 1 | Fan sensor |
|  | 2 | +12V |
|  | 3 | Ground |

5.13  J13 Front Bezel Connector

The front bezel of the case has a control panel which provides light indication of the computer activities and switches to change the computer status. J13 is a 20-pin header that provides interfaces for the following functions.

| Hard Disk Drive LED |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Reset Switch |  |  |
| Turbo LED Connector |  |  |  |  |  |  |
|  |  |  |  | ATX Power On Switch |  |  |
| CPU Overheat LED |  |  |  |  |  |  |
|  |  |  |  | Power LED and Keylock |  |  |
| Speaker |  |  |  |  |  |  |

Speaker: Pins 1 - 4

This connector provides an interface to a speaker for audio tone generation. An 8-ohm speaker is recommended.

|  | J13 Pin # | Signal Name |
| --- | --- | --- |
|  | 1 | Speaker out |
|  | 2 | No connect |
|  | 3 | Ground |
|  | 4 | +5V |

Power LED and Keylock: Pins 11 - 15

The power LED indicates the status of the main power switch. The keylock switch, when closed, will disable the keyboard function.

|  | J13 Pin # | Signal Name |
| --- | --- | --- |
|  | 11 | Power LED |
|  | 12 | No connect |
|  | 13 | Ground |
|  | 14 | Keylock |
|  | 15 | Ground |

CPU Overheat LED: Pins 6 and 16

This connector connects to the CPU Overheat LED that lights up when CPU temperature exceeds the CPU warning temperature set in the BIOS. When this occurs, the system slows down until the temperature falls to a safe level.

|  | J13 Pin # | Signal Name |
| --- | --- | --- |
|  | 6 | 5V |
|  | 16 | Ground |

ATX Power ON Switch: Pins 7 and 17

This is an “ATX Power Supply On/Off Switch” that is connected to the power switch on the case. When pressed for more than 4 seconds, the system powers off. When pressed for less than 4 seconds, the system enters the Suspend Mode (depending on BIOS pre-settings).

Turbo LED Connector: Pins 8 and 18

The Turbo LED will always be On when linked to this connector.

|  | J13 Pin # | Signal Name |
| --- | --- | --- |
|  | 8 | 5V |
|  | 18 | Ground |

Reset Switch: Pins 9 and 19

The reset switch allows the user to reset the system without turning Off the main power and then On. Orientation is not required when making a connection to this header.

Hard Disk Drive LED Connector: Pins 10 and 20

This connector connects to the hard drive activity LED on the control panel. The LED flashes when HDD is in use.

|  | J13 Pin # | Signal Name |
| --- | --- | --- |
|  | 10 | Ground |
|  | 20 | 5V |

Chapter 6  BIOS Configuration

This chapter describes the different settings available in the Award BIOS that comes with the TI5VG+ motherboard. The topics covered in this chapter are as follows:

6.1  BIOS Introduction	36

6.2  BIOS Setup	36

6.3  Standard CMOS Setup	38

Date

Time

Primary HDDs / Secondary HDDs

Drive A / Drive B

Floppy 3 Mode Support

Video

Halt On

6.4  BIOS Features Setup	41

Virus Warning

CPU Internal Cache / External Cache

Quick Power On Self Test

Boot Sequence

Swap Floppy Drive

Boot Up Floppy Seek

Boot Up NumLock Status

Boot Up System Speed

Gate A20 Option

Memory Parity/ECC Check

Typematic Rate Setting

Typematic Rate (Chars/Sec)

Typematic Delay (Msec)

Security Option

PCI/VGA Palette Snoop

OS Select for DRAM > 64MB

Video BIOS Shadow

C8000 - CBFFF Shadow/DC000 - DFFFF Shadow

6.5  Chipset Features Setup	44

Bank DRAM Timing

SDRAM Cycle Length

SDRAM Bank Interleave

DRAM Read Pipeline

Sustained 3T Write

Cache Rd+CPU Wt Pipeline

Read Around Write

Cache Timing

Video BIOS Cacheable

System BIOS Cacheable

Memory Hole at 15MB Addr.

AGP Aperture Size

Cyrix M2 ADS# delay

CPU/PCI Clock Select

Auto Detect DIMM/PCI Clk

Spread Spectrum

OnChip USB

USB Keyboard Support

CPU Warning Temperature

Temperature/Fan Speed/Voltage

6.6  Power Management Setup	47

Power Management

PM Control by APM

Video Off Method

Modem Use IRQ

Soft-Off by PWRBTN

HDD Power Down

Doze Mode

Suspend Mode

PM Events

6.7  PNP/PCI Configuration	49

PNP OS Installed

Resources Controlled by

Reset Configuration Data

ACPI I/O Device Node

IRQ3/4/5/7/9/10/11/12/14/15, DMA0/1/3/5/6/7 assigned to

CPU to PCI Write Buffer

PCI Dynamic Bursting

PCI Master 0 WS Write

PCI Delay Transaction

PCI Master Read Prefetch

PCI#2 Access #1 Retry

AGP Master 1 WS Write

AGP Master 1 WS Read

PCI IRQ Activated by

Assign IRQ for USB/VGA

6.8  Load BIOS Defaults	52

6.9  Load Setup Defaults	52

6.10  Integrated Peripherals	53

OnChip Primary/Secondary PCI IDE

IDE Prefetch Mode

IDE HDD Block Mode

IDE Primary/Secondary Master/Slave PIO

IDE Primary/Secondary Master/Slave UDMA

Onboard FDD Controller

Onboard Serial/Parallel Port

UART 2 Mode

Onboard Parallel Mode

6.11  Supervisor / User Password	55

6.12  IDE HDD Auto Detection	56

6.13  HDD Low Level Format	56

6.14  Save & Exit Setup	57

6.15  Exit Without Saving	57

6.1  BIOS Introduction

The Award BIOS (Basic Input/Output System) installed in your computer system’s ROM supports Intel/Cyrix/AMD processors in a standard IBM-AT compatible I/O system. The BIOS provides critical low-level support for standard devices such as disk drives, serial and parallel ports. It also adds virus and password protection as well as special support for detailed fine-tuning of the chipset controlling the entire system.

6.2  BIOS Setup

The Award BIOS provides a Setup utility program for specifying the system configurations and settings. The BIOS ROM of the system stores the Setup utility. When you turn on the computer, the Award BIOS is immediately activated. Pressing the <Del> key immediately allows you to enter the Setup utility. If you are a little bit late pressing the <Del> key, POST(Power On Self Test) will continue with its test routines, thus preventing you from invoking the Setup. If you still wish to enter Setup, restart the system by pressing the ”Reset” button or simultaneously pressing the <Ctrl>, <Alt> and <Delete> keys. You can also restart by turning the system Off and back On again. The following message will appear on the screen:

Press  <DEL>  to  Enter  Setup

In general, you press the arrow keys to highlight items, <Enter> to select, the <PgUp> and <PgDn> keys to change entries, <F1> for help and <Esc> to quit.

When you enter the Setup utility, the Main Menu screen will appear on the screen. The Main Menu allows you to select from various setup functions and exit choices.

ROM PCI/ISA BIOS

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | HDD LOW LEVEL FORMAT |
| LOAD BIOS DEFAULTS | SAVE & EXIT SETUP |
| LOAD SETUP DEFAULTS | EXIT WITHOUT SAVING |
| ESC : Quit | á â à ß : Select Item |
| F10 : Save & Exit Setup | (Shift) F2 : Change Color |
| Time, Date, Hard Disk Type |  |

The section below the setup items of the Main Menu displays the control keys for this menu. Another section at the bottom of the Main Menu just below the control keys section displays information on the currently highlighted item in the list.

|  | After making and saving system changes with Setup, you find that your computer cannot boot, the Award BIOS supports an override to the CMOS settings that resets your system to its default. |
| --- | --- |
|  | We strongly recommend that you avoid making any changes to the chipset defaults. These defaults have been carefully chosen by both Award and your system manufacturer to provide the absolute maximum performance and reliability. |

6.3  Standard CMOS Setup

“Standard CMOS Setup” choice allows you to record some basic hardware configurations in your computer system and set the system clock and error handling. If the motherboard is already installed in a working system, you will not need to select this option. You will need to run the Standard CMOS option, however, if you change your system hardware configurations, the onboard battery fails, or the configuration stored in the CMOS memory was lost or damaged.

ROM PCI/ISA BIOS

STANDARD CMOS SETUP

AWARD SOFTWARE, INC.

| Date (mm:dd:yy) : Mon, Aug 19 1996 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Time (hh:mm:ss) : 00 : 00 : 00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HARD DISKS | TYPE |  | SIZE |  | CYLS | HEAD |  | PRECOMP |  | LANDZ |  |  | SECTOR |  | MODE |  |
| Primary Master | Auto |  | 0 |  | 0 | 0 |  | 0 |  | 0 |  |  | 0 |  | Auto |  |
| Primary Slave | None |  | 0 |  | 0 | 0 |  | 0 |  | 0 |  |  | 0 |  | ----- |  |
| Secondary Master | None |  | 0 |  | 0 | 0 |  | 0 |  | 0 |  |  | 0 |  | ----- |  |
| Secondary Slave | None |  | 0 |  | 0 | 0 |  | 0 |  | 0 |  |  | 0 |  | ----- |  |
| Drive A |  | : 1.44M, 3.5in |  |  |  |  |  |  | Base Memory |  |  | : |  | 640K |  |  |
| Drive B |  | : None |  |  |  |  |  |  | Extended Memory |  |  | : |  | 15360K |  |  |
| Floppy 3 Mode Support |  |  |  | : Disabled |  |  |  |  | Other Memory |  |  | : |  | 384K |  |  |
| Video |  | : EGA / VGA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Halt On |  | : All Errors |  |  |  |  |  |  | Total Memory |  |  | : |  | 16384K |  |  |
| ESC : Quit |  |  |  | á â à ß : Select Item |  |  |  |  |  |  | PU / PD / + / - : Modify |  |  |  |  |  |
| F1 : Help |  |  |  | (Shift) F2 : Change Color |  |  |  |  |  |  |  |  |  |  |  |  |

At the bottom of the menu are the control keys for use on this menu. If you need any help in each item field, you can press the <F1> key. It will display the relevant information to help you. The memory display at the lower right-hand side of the menu is read-only. It will adjust automatically according to the memory changed. The following describes each item of this menu.

Date

The date format is:

Day :	Sun to Sat

Month :	1 to 12

Date :	1 to 31

Year :	1994 to 2079

To set the date, highlight the “Date” field and use the PageUp/ PageDown or +/- keys to set the current time.

Time

The time format is: 	Hour    :	00 to 23

Minute :	00 to 59

Second :	00 to 59

To set the time, highlight the “Time” field and use the <PgUp>/ <PgDn> or +/- keys to set the current time.

Primary HDDs / Secondary HDDs

The onboard PCI IDE connectors provide Primary and Secondary channels for connecting up to four IDE hard disks or other IDE devices. Each channel can support up to two hard disks; the first is the “Master” and the second is the “Slave”.

To enter the specifications for a hard disk drive, you must select first a “Type”. There are 45 predefined types and 4 user definable types are for Enhanced IDE BIOS. Type 1 to 45 are predefined. Type “User” is user-definable. For the Primary Master/Slave as well as Secondary Master/Slave, you can select “Auto” under the TYPE and MODE fields. This will enable auto detection of your IDE drives and CD-ROM drive during POST.

Press <PgUp>/<PgDn> to select a numbered hard disk type or type the number and press the <Enter> key. The hard disk will not work properly if you enter incorrect information for this field. If your hard disk drive type is not matched or listed, you can use Type User to define your own drive type manually. If you select Type User, related information is asked to be entered to the following items.

CYLS :	Number of cylinders

HEAD :	Number of read/write heads

PRECOMP :	Write precompensation

LANDZ :	Landing zone

SECTOR :	Number of sectors

SIZE :	Automatically adjust according to the configuration

MODE (for IDE HDD only) :	 Auto

Normal	(HD < 528MB)

Large	(for MS-DOS only)

LBA 	(HD > 528MB and supports Logical Block Addressing)

|  | The specifications of your drive must match with the drive table. The hard disk will not work properly if you enter incorrect information in these fields. If your hard disk drive type is not matched or listed, you can use Type User to define your own drive type manually. |
| --- | --- |

Drive A / Drive B

These fields identify the types of floppy disk drive A or drive B that has been installed in the computer. The available specifications are:

|  | 360KB 5.25 in. | - 2MB 5.25 in. | 720KB 3.5 in. | - 44MB - 3.5 in. | - 88MB 3.5 in. |  |
| --- | --- | --- | --- | --- | --- | --- |

Floppy 3 Mode Support

This is the Japanese standard floppy drive. The standard stores 1.2MB in a 3.5-inch diskette. You have four options to choose:

Disabled		No 3 mode floppy drive installed. (default)

Drive A		Installed 3 mode drive at drive A.

Drive B		Installed 3 mode drive at drive B.

Both		Installed 3 mode drive at drive A and B.

Video

This field selects the type of video display card installed in your system. You can choose the following video display cards:

EGA/VGA		For EGA, VGA, SEGA, SVGA

or PGA monitor adapters. (default)

CGA 40	Power up in 40 column mode.

CGA 80	Power up in 80 column mode.

MONO		For Hercules or MDA adapters.

Halt On

This field determines whether the system will halt if an error is detected during power up.

| No errors | Whenever the BIOS detects a non-fatal error, the system will stop and you will be prompted. |
| --- | --- |
| All errors | The system boot will not be halted for any error that may be detected. (default) |
| All, But Keyboard | The system boot will not be halted for a keyboard error; it will stop for all other errors. |
| All, But Diskette | The system boot will not be halted for a disk error; it will stop for all other errors. |
| All, But Disk/Key | The system boot will not be halted for a key- board or disk error; it will stop for all others. |

6.4  BIOS Features Setup

This section allows you to configure and improve your system and allows you to set up some system features according to your preference.

ROM / PCI ISA BIOS

BIOS FEATURES SETUP

AWARD SOFTWARE, INC.

| Virus Warning | : Disabled | Video BIOS Shadow |  | : Enabled |
| --- | --- | --- | --- | --- |
| CPU Internal Cache | : Enabled | C8000-CBFFF Shadow |  | : Disabled |
| External Cache | : Enabled | CC000-CFFFF Shadow |  | : Disabled |
| Quick Power On Self Test | : Enabled | D0000-D3FFF Shadow |  | : Disabled |
| Boot Sequence | : A, C, SCSI | D4000-D7FFF Shadow |  | : Disabled |
| Swap Floppy Drive | : Disabled | D8000-DBFFF Shadow |  | : Disabled |
| Boot Up Floppy Drive | : Enabled | DC000-DFFF Shadow |  | : Disabled |
| Boot Up Numlock Status | : On |  |  |  |
| Boot Up System Speed | : High |  |  |  |
| Gate A20 Option | : Fast |  |  |  |
| Memory Parity/ECC Check | : Disabled |  |  |  |
| Typematic Rate Setting | : Disabled |  |  |  |
| Typematic Rate (chars/Sec) | : 6 |  |  |  |
| Typematic Delay (Msec) | : 250 |  |  |  |
| Security Option | : Setup |  |  |  |
| PCI /VGA Palette Snoop | : Disabled |  |  |  |
| OS Select For DRAM>64MB | : Non-OS2 | ESC : Quit | á â à ß : Select Item |  |
|  |  | F1 : Help | PU/PD/+/- : Modify |  |
|  |  | F5 : Old Values | (Shift) F2 : Color |  |
|  |  | F6 : Load BIOS Defaults |  |  |
|  |  | F7 : Load Setup Defaults |  |  |

Virus Warning

This item protects the boot sector and partition table of your hard disk against accidental modifications. If an attempt is made, the BIOS will halt the system and display a warning message. If this occurs, you can either allow the operation to continue or run an anti-virus program to locate and remove the problem.

|  | Many disk diagnostic programs which attempt to access the boot sector table can cause the virus warning. If you will run such a program, disable the Virus Warning feature. |
| --- | --- |

CPU Internal Cache / External Cache

These items allow you to enable (speed up memory access) or disable the cache function. By default, these items are Enabled.

Quick Power On Self Test

This choice speeds up the Power On Self Test (POST) after you power up the system. If it is set to Enabled, BIOS will skip some items. By default, this choice is Enabled.

Boot Sequence

This field determines the drive that the system searches first for an operating system. The options are :

| A, C, SCSI | D, A, SCSI | SCSI, C, A |
| --- | --- | --- |
| C, A, SCSI | E, A, SCSI | C only |
| C, CDROM, A | F, A, SCSI | LS/ZIP, C |
| CDROM, C, A | SCSI, A, C |  |

The default value is A, C, SCSI.

Swap Floppy Drive

This item allows you to determine whether to enable Swap Floppy Drive or not. When enabled, the BIOS swaps floppy drive assignments so that Drive A becomes Drive B, and Drive B becomes Drive A. By default, this field is set to Disabled.

Boot Up Floppy Seek

When enabled, the BIOS will seek whether or not the floppy drive installed has 40 or 80 tracks. 360K type has 40 tracks while 760K, 1.2M and 1.44M all have 80 tracks. By default, this field is set to Enabled.

Boot Up NumLock Status

This allows you to activate the NumLock function after you power up the system. By default, the system boots up with NumLock On.

Boot Up System Speed

This has no function and selects the default system speed (High).

Gate A20 Option

This field allows you to select how Gate A20 is worked. The Gate A20 is a device used to address memory above 1 MB. By default, this field is set to Fast.

Memory Parity/ECC Check

Set this option to Enabled, when the memory installed in your system supports ECC (error correction code) or parity. By default, this field is set to Disabled.

Typematic Rate Setting

When disabled, continually holding down a key on your keyboard will generate only one instance. When enabled, you can set the two typematic controls listed next. By default, this field is set to Disabled.

Typematic Rate (Chars/Sec)

When the typematic rate is enabled, the system registers repeated keystrokes speeds. You can select speed range from 6 to 30 characters per second. By default, this item is set to 6.

Typematic Delay (Msec)

When the typematic rate is enabled, this item allows you to set the time interval for displaying the first and second characters. By default, this item is set to 250msec.

Security Option

This field allows you to limit access to the System and Setup. The default value is Setup. When you select System, the system prompts for the User Password every time you boot up. When you select Setup, the system always boots up and prompts for the Supervisor Password only when the Setup utility is called up.

PCI/VGA Palette Snoop

Some non-standard VGA display cards may not show colors properly. This field allows you to set whether MPEG ISA/VESA VGA Cards can work with PCI/VGA or not. When this field is enabled, a PCI/VGA can work with a MPEG ISA/VESA VGA card. When this field is disabled, a PCI/VGA cannot work with a MPEG ISA/VESA Card.

OS Select for DRAM > 64MB

This option allows the system to access greater than 64MB of DRAM memory when used with OS/2 that depends on certain BIOS calls to access memory. The default setting is NON-OS/2.

Video BIOS Shadow

This item allows you to change the Video BIOS location from ROM to RAM. Video Shadow will increase the video speed.

C8000 - CBFFF Shadow/DC000 - DFFFF Shadow

Shadowing a ROM reduces the memory available between 640KB to 1024KB. These fields determine whether optional ROM will be copied to RAM or not.

6.5  Chipset Features Setup

This Setup menu controls the configuration of the motherboard chipset.

ROM PCI/ISA BIOS

CHIPSET FEATURES SETUP

AWARD	SOFTWARE INC.

| Bank 0/1 DRAM Timing | : |  | 60ns |  | OnChip USB |  |  |  |  | : Enabled |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bank 2/3 DRAM Timing | : |  | 60ns |  | USB Keyboard Support |  |  |  |  | : Disabled |  |  |
| Bank 4/5 DRAM Timing | : |  | 60ns |  |  |  |  |  |  |  |  |  |
| SDRAM Cycle Length | : |  | 3 |  | CPU Warning Temperature |  |  |  |  | : 70C/158F |  |  |
| SDRAM Bank Interleave | : |  | Disabled |  | Current System Temp |  |  |  |  | : |  |  |
| DRAM Read Pipeline | : |  | Enabled |  | Current CPU Temperature |  |  |  |  | : |  |  |
| Sustained 3T Write | : |  | Enabled |  | CPU FAN Speed |  |  |  |  | : |  |  |
| Cache Rd+CPU Wt Pipeline | : |  | Enabled |  | CHASSIS FAN Speed |  |  |  |  | : |  |  |
| Read Around write | : |  | Enabled |  | VCORE | : |  | VIO |  |  |  |  |
| Cache Timing | : |  | Fast |  | VCC3 | : |  |  |  |  |  |  |
| Video BIOS Cacheable | : |  | Enabled |  | +12 V | : |  | +5 V |  |  | : |  |
| System BIOS Cacheable | : |  | Disabled |  | - 5V | : |  | -12 V |  |  | : |  |
| Memory Hole At 15MB | : |  | Disabled |  |  |  |  |  |  |  |  |  |
| AGP Aperture Size | : |  | 64M |  |  |  |  |  |  |  |  |  |
| CPU/PCI Clock Select | : |  | Default |  | ESC : Quit |  |  |  | á â à ß : Select Item |  |  |  |
| Auto Detect DIMM/PCI Clk | : |  | Disabled |  | F1 : Help |  |  |  | PU/PD/+/- : Modify |  |  |  |
| Spread Spectrum | : |  | Disabled |  | F5 : Old Values |  |  |  | (Shift) F2 : Color |  |  |  |
| Cyrix M2 ADS# delay |  | : |  | Disabled | F6 : Load BIOS Defaults |  |  |  |  |  |  |  |
|  |  |  |  |  | F7 : Load Setup Defaults |  |  |  |  |  |  |  |

Bank DRAM Timing

These fields define the speed of the DRAM memory onboard. The options are Normal, Medium, Fast, Turbo, 70ns and 60ns. By default, these fields are set to 60ns.

SDRAM Cycle Length

This field sets the SDRAM cycle length to either 2 or 3. The default setting is 3.

SDRAM Bank Interleave

This field enables or disables the SDRAM bank interleave. The default setting is Disabled.

DRAM Read Pipeline

When enabled, this field supports pipelining of DRAM reads The default setting is Enabled.

Sustained 3T Write

This field allows support for PBSRAM sustained 3T write. By default, this field is set Enabled.

Cache Rd+CPU Wt Pipeline

When enabled, this item allows pipelining of cache reads and CPU writes. The default setting is Enabled.

Read Around Write

DRAM optimization feature: If a memory read is addressed to a location whose latest write is being held in a buffer before being written to memory, the read is satisfied through the buffer contents, and the read is not sent to the DRAM. The default setting is Enabled.

Cache Timing

This field sets the timing of the cache in the system. The options are Fast and Fastest. By default, this field is set to Fast.

Video BIOS Cacheable

When enabled, access to video BIOS addressed at C0000H to C7FFFH are cached, provided that the cache controller is enabled.

System BIOS Cacheable

When enabled, access to the system BIOS ROM addressed at F0000H-FFFFFH are cached, provided that the cache controller is enabled.

Memory Hole at 15MB Addr.

In order to improve performance, certain space in memory can be reserved for ISA cards. This field allows you to reserve 15MB to 16MB memory address space to ISA expansion cards. This makes memory from 15MB and up unavailable to the system. Expansion cards can only access memory up to 16MB. By default, this field is set to Disabled.

AGP Aperture Size

The field sets aperture size of the graphics. The aperture is a portion of the PCI memory address range dedicated for graphics memory address space. Host cycles that hit the aperture range are forwarded to the AGP without any translation. The default setting is 64M.

Cyrix M2 ADS# delay

This field is only visible when your system supports a Cyrix M2 processor. It enables or disables the ADS# delay signal for Cyrix M2.

CPU/PCI Clock Select

The default settings for the CPU/PCI clock are 66/33MHz (default) and 100/33MHz, depending on the bus speed of the CPU you have installed. If overclocking causes the system to be unstable, use the defaults. Note that 112/37MHz, 124/41MHz and 133/44MHz settings are only supported if your motherboard has an ICS 9148AF-58 clock generator.

|  | When your system does not boot because the CPU has been changed, turn off the computer by pressing the power button and turn it on again by pressing the Insert key and the power button simultaneously. (Note to press the Insert key until an image appears on the screen.) When the system is turned on, press the Delete key to enter BIOS Setup and configure the CPU speed. |
| --- | --- |

Auto Detect DIMM/PCI Clk

When enabled, the system automatically shuts off clocks of unused DIMM/PCI slots.  This field is for CE testing use only.

Spread Spectrum

This field sets the value of the spread spectrum. Options are Disabled, 1.8% (CNTR), 0.6% (CNTR), 1.8% (DOWN), and 0.6% (DOWN). The default setting is Disabled.  This field is for CE testing use only.

OnChip USB

This field allows you either to enable or disable the USB function.

USB Keyboard Support

This field has a default setting of Disabled.

CPU Warning Temperature

When the CPU reaches the CPU Warning Temperature, the system alarms the user through the speaker.

Temperature/Fan Speed/Voltage

The values for the system/CPU temperature, CPU/chassis fan speed and system voltages are displayed as monitored by the H/W monitoring IC.

6.6  Power Management Setup

ROM PCI/ISA BIOS (2A59IM29)

POWER MANAGEMENT SETUP

AWARD	SOFTWARE,  INC.

| Power Management | : User Define |  | Primary INTR |  | : ON |
| --- | --- | --- | --- | --- | --- |
| PM Control by APM | : Yes |  | IRQ3 (COM2) |  | : Primary |
| Video Off Option | : Suspend -> Off |  | IRQ4 (COM1) |  | : Primary |
| Video Off Method | : V/H SYNC +Blank |  | IRQ5 (LPT 2) |  | : Primary |
| MODEM Use IRQ | : 3 |  | IRQ6 (Floppy Disk) |  | : Primary |
| Soft-Off by PWRBTN | : Instant-Off |  | IRQ7 (LPT 1) |  | : Primary |
| ** PM Timers ** |  |  | IRQ8 (RTC Alarm) |  | : Disabled |
| HDD Power Down | : Disabled |  | IRQ9 (IRQ2 Redir) |  | : Secondary |
| Doze Mode | : Disabled |  | IRQ10 (Reserved) |  | : Secondary |
| Suspend Mode | : Disabled |  | IRQ11 (Reserved) |  | : Secondary |
| ** PM Events ** |  |  | IRQ12 (PS/2 Mouse) |  | : Primary |
| VGA | : OFF |  | IRQ13 (Coprocessor) |  | : Primary |
| LPT & COM | : LPT / COM |  | IRQ14 (Hard Disk) |  | : Primary |
| HDD & FDD | : ON |  | IRQ15 (Reserved) |  | : Primary |
| DMA / master | : OFF |  | ESC : Quit | á â à ß : Select Item |  |
| Modem Ring Resume | : Disabled |  | F1 : Help | PU/PD/+/- : Modify |  |
| RTC Alarm Resume | : Disabled |  | F5 : Old Values | (Shift) F2 : Color |  |
|  |  | F6 : Load BIOS Defaults |  |  |  |
|  |  | F7 : Load Setup Defaults |  |  |  |

Power Management

This field allows you to select the type of power saving management modes. There are fours selections for Power Management.

Disabled	No power management.

Min. Power Saving	Minimum power management.

Max. Power Saving 	Maximum power management. Only available for SL CPU.

User Define	Each of the ranges are from 1 min. to 1hr. Except for HDD Power Down which ranges from 1 min. to 15 min.

(Default)

|  | In order to enable the CPU overheat protection feature, the Power Management field should not be set to Disabled. |
| --- | --- |

PM Control by APM

This field allows you to use the Advanced Power Management device to enhance the Max. Power Saving mode and stop the CPU’s internal clock. If the Max. Power Saving is not enabled, this will be preset to NO.

Video Off Method

This field defines the Video Off features. There are three options.

V/H SYNC + Blank	Default setting, blank the screen and turn off vertical and horizontal scanning.

DPMS	Allows the BIOS to control the video display card if it supports the DPMS feature.

Blank Screen	This option only writes blanks to the video buffer.

Modem Use IRQ

This field defines the IRQ being used by the modem. By default, the IRQ is set to 3.

Soft-Off by PWRBTN

This field defines the power-off mode when using an ATX power supply. The Instant Off mode allows powering off immediately upon pressing the power button. In the Delay 4 Sec mode, the system powers off when the power button is pressed for more than four seconds or enters the suspend mode when pressed for less than 4 seconds. The default value is Instant Off.

HDD Power Down

When enabled, and after the set time of system inactivity, the hard disk drive will be powered down while all other devices remain active.

Doze Mode

When enabled, and after the set time of system inactivity, the CPU clock will run at a slower speed while all other devices still operate at full speed.

Suspend Mode

When enabled, and after the set time of system inactivity, all devices except the CPU will be shut off.

PM Events

The VGA, LPT & COM, HDD & FDD, DMA /master, Modem Ring Resume, RTC Alarm Resume and Primary INTR section are I/O events which can prevent the system from entering a power saving mode or can awaken the system from such a mode. When an I/O device wants to gain the attention of the operating system, it signals this by causing an IRQ to occur. When the operating system is ready to respond to the request, it interrupts itself and performs the service. The default value is Off. When set On, activity will neither prevent the system from going into a power management mode nor awaken it. The IRQ section sets the wake-up call of the system. If activity is detected from any enabled IRQ channels in the left-hand group, the system wakes up from suspended mode.

6.7  PNP/PCI Configuration

This option configures the PCI bus system. All PCI bus systems on the system use INT#, thus all installed PCI cards must be set to this value.

ROM PCI/ISA BIOS

PNP/PCI Configuration

AWARD	SOFTWARE INC.

| PNP OS Installed | : No | CPU to PCI Write Buffer |  | : Enabled |
| --- | --- | --- | --- | --- |
| Resources Controlled by | : Manual | PCI Dynamic Bursting |  | : Enabled |
| Reset Configuration Data | : Disabled | PCI Master 0 WS Write |  | : Enabled |
| ACPI I/O Device Node | : Disabled | PCI Delay Transaction |  | : Enabled |
| IRQ-3 assigned to | : Legacy ISA | PCI Master Read Prefetch |  | : Enabled |
| IRQ-4 assigned to | : Legacy ISA | PCI#2 Access #1 Retry |  | : Disabled |
| IRQ-5 assigned to | : PCI/ISA PnP | AGP Master 1 WS Write |  | : Enabled |
| IRQ-7 assigned to | : Legacy ISA | AGP Master 1 WS Read |  | : Disabled |
| IRQ-9 assigned to | : PCI/ISA PnP |  |  |  |
| IRQ-10 assigned to | : PCI/ISA PnP | PCI IRQ Activated By |  | : Level |
| IRQ-11 assigned to | : PCI/ISA PnP | Assign IRQ For USB |  | : Enabled |
| IRQ-12 assigned to | : PCI/ISA PnP | Assign IRQ For VGA |  | : Enabled |
| IRQ-14 assigned to | : Legacy ISA |  |  |  |
| IRQ-15 assigned to | : Legacy ISA |  |  |  |
| DMA-0 assigned to | : PCI/ISA PnP |  |  |  |
| DMA-1 assigned to | : PCI/ISA PnP | ESC : Quit | á â ß : Select Item |  |
| DMA-3 assigned to | : PCI/ISA PnP | F1 : Help | PU/PD/+/- : Modify |  |
| DMA-5 assigned to | : PCI/ISA PnP | F5 : Old Values | (Shift) F2 : Color |  |
| DMA-6 assigned to | : PCI/ISA PnP | F6 : Load BIOS Defaults |  |  |
| DMA-7 assigned to | : PCI/ISA PnP | F7 : Load Setup Defaults |  |  |

PNP OS Installed

This field allows you to specify if the operating system installed in your system is plug and play aware.

Note: 	Operating systems such as DOS, OS/2, and Windows 3.x do not use PnP.

Resources Controlled by

This PnP BIOS can configure all of the boot and compatible devices automatically. However, this capability needs you to use a PnP operating system such as Windows 95. The default value is Manual.

Reset Configuration Data

This field allows you to determine whether  to reset the configuration data or not. The default value is Disabled.

ACPI I/O Device Node

This field enables or disables the ACPI I/O Device Node. The default setting is Disabled.

IRQ3/4/5/7/9/10/11/12/14/15, DMA0/1/3/5/6/7 assigned to

These fields allow you to determine the IRQ/DMA assigned to the ISA bus and is not available to any PCI slot.

CPU to PCI Write Buffer

When enabled, this option increase the efficiency of  the PCI bus to and speed up the execution in the processor. By default, this field is set to Enabled.

PCI Dynamic Bursting

When enabled, this option combines several PCI cycles into one. By default, this field is set to Enabled.

PCI Master 0 WS Write

When enabled, this option increases the write cycle speed. By default, this field is set to Enabled.

PCI Delay Transaction

When enabled, this option delays PCI data transaction. By default, this field is set to Enabled.

PCI Master Read Prefetch

When this item is enabled, the system is allowed to prefetch the next read and initiate the next process. By default, this field is set to Enabled.

PCI#2 Access #1 Retry

This item enables PC#2 Access #1 attempts. By default, this field is set to Disabled.

AGP Master 1 WS Write

When enabled, writes to the AGP bus are executed with 1 wait states. By default, this field is set to Enabled.

AGP Master 1 WS Read

When enabled, reads to the AGP bus are executed with 1 wait states. By default, this field is set to Disabled.

PCI IRQ Activated by

This field allows you to select the method by which the PCI bus recognizes that an IRQ service is being requested by a device. The default value is Level.

Assign IRQ for USB/VGA

These fields allow you to enable or disable the IRQ for USB and VGA. By default, these fields are enabled.

6.8  Load BIOS Defaults

This option allows you to load the troubleshooting default values permanently stored in the BIOS ROM. These default settings are non-optimal and disable all high-performance features.

ROM PCI/ISA BIOS

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP Load BIOS Defaults (Y/N)? N | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | HDD LOW LEVELF ORMAT |
| LOAD BIOS DEFAULTS | SAVE & EXIT SETUP |
| LOAD SETUP DEFAULTS | EXIT WITHOUT SAVING |
| ESC : Quit | á â à ß : Select Item |
| F10 : Save & Exit Setup | (Shift) F2 : Change Color |
| Load BIOS Defaults except Standard CMOS Setup |  |

To load BIOS defaults value to CMOS SRAM, enter “Y”. If not, enter “N”.

6.9  Load Setup Defaults

This option allows you to load the default values to your system configuration. These default settings are optimal and enable all high performance features.

ROM PCI/ISA BIOS

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP Load Setup Defaults (Y/N)? N | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | HDD LOW LEVEL FORMAT |
| LOAD BIOS DEFAULTS | SAVE & EXIT SETUP |
| LOAD SETUP DEFAULTS | EXIT WITHOUT SAVING |
| ESC : Quit | á â à ß : Select Item |
| F10 : Save & Exit Setup | (Shift) F2 : Change Color |
| Load BIOS Defaults except Standard CMOS Setup |  |

To load SETUP defaults value to CMOS SRAM, enter “Y”. If not, enter “N”.

6.10  Integrated Peripherals

This option sets your hard disk configuration, mode and port.

ROM PCI/ISA BIOS

INTEGRATED PERIPHERALS

AWARD	SOFTWARE INC.

| OnChip IDE First Channel | : Enabled | Onboard Parallel Mode |  | : SPP |
| --- | --- | --- | --- | --- |
| OnChip IDE Second Channel | : Enabled |  |  |  |
| IDE Prefetch Mode | : Disabled |  |  |  |
| IDE HDD Block Mode | : Enabled |  |  |  |
| IDE Primary Master PIO | : Auto |  |  |  |
| IDE Primary Slave PIO | : Auto |  |  |  |
| IDE Secondary Master PIO | : Auto |  |  |  |
| IDE Secondary Slave PIO | : Auto |  |  |  |
| IDE Primary Master UDMA | : Auto |  |  |  |
| IDE Primary Slave UDMA | : Auto |  |  |  |
| IDE Secondary Master UDMA | : Auto |  |  |  |
| IDE Secondary Slave UDMA | : Auto |  |  |  |
| Onboard FDD Controller | : Enabled |  |  |  |
| Onboard Serial Port 1 | : 3F8/IRQ4 |  |  |  |
| Onboard Serial Port 2 | : 2F8/IRQ3 | ESC : Quit | á â ß : Select Item |  |
| Uart 2 Mode | : Standard | F1 : Help | PU/PD/+/- : Modify |  |
|  |  | F5 : Old Values | (Shift) F2 : Color |  |
|  |  | F6 : Load BIOS Defaults |  |  |
| Onboard Parallel Port | : 378H/IRQ7 | F7 : Load Setup Defaults |  |  |

OnChip Primary/Secondary PCI IDE

These fields allow you either to enable or disable the Primary/Secondary controller. You might choose to disable the controller if you were to add a higher performance or specialized controller.

IDE Prefetch Mode

This field allows your hard disk controller to use prefetch mode to transfer data to increase the performance.

IDE HDD Block Mode

This field allows your hard disk controller to use the fast block mode to transfer data to and from your hard disk drive.

IDE Primary/Secondary Master/Slave PIO

These fields allow your system hard disk controller to work faster. Rather than have the BIOS issue a series of commands that transfer to or from the disk drive, PIO (Programmed Input/Output) allows the BIOS to communicate with the controller and CPU directly.

The system supports five modes, numbered from 0 (default) to 4, which primarily differ in timing. When Auto is selected, the BIOS will select the best available mode.

IDE Primary/Secondary Master/Slave UDMA

This field allows your system to improve disk I/O throughput to 33Mb/sec with the Ultra DMA/33 feature. The options are Auto and Disabled.

Onboard FDD Controller

This option allows you to select the onboard FDD port.

Onboard Serial/Parallel Port

These fields allow you to select the onboard serial and parallel ports and their addresses. The default value for these ports are:

Serial Port 1		3F8/IRQ4

Serial Port 2		2F8/IRQ3

Parallel Port		378H/IRQ7

UART 2 Mode

This field determines the UART 2 mode in your computer. The options are Standard, HPSIR, and ASKIR.

Onboard Parallel Mode

This field allows you to determine parallel port mode function.

SPP			Normal Printer Port

EPP			Enhanced Parallel Port

ECP		Extended Capabilities Port

6.11  Supervisor / User Password

These two options set the system password. Supervisor Password sets a password that will be used to protect the system and Setup utility. User Password sets a password that will be used exclusively on the system. To specify a password, highlight the type you want and press <Enter>. The Enter Password: message prompts on the screen. Type the password, up to eight characters in length, and press <Enter>. The system confirms your password by asking you to type it again. After setting a password, the screen automatically returns to the main screen.

To disable a password, just press the <Enter> key when you are prompted to enter the password. A message will confirm the password to be disabled. Once the password is disabled, the system will boot and you can enter Setup freely.

ROM PCI/ISA BIOS

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP Enter Password: | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | HDD LOW LEVEL FORMAT |
| LOAD BIOS DEFAULTS | SAVE & EXIT SETUP |
| LOAD SETUP DEFAULTS | EXIT WITHOUT SAVING |
| ESC : Quit | á â à ß : Select Item |
| F10 : Save & Exit Setup | (Shift) F2 : Change Color |
| Change / Set / Disable Password |  |

6.12  IDE HDD Auto Detection

This option detects the parameters of an IDE hard disk drive, and automatically enters them into the Standard CMOS Setup screen.

ROM PCI/ISA BIOS

STANDARD CMOS SETUP

AWARD SOFTWARE, INC.

| HARD DISKS |  | TYPE |  |  | SIZE |  | CYLS |  | HEAD |  | PRECOMP |  | LANDZ |  | SECTOR |  | MODE |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Primary Master: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Select Primary Master Option (N=SKIP) : N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | OPTIONS |  | TYPE | SIZE |  | CYLS |  | HEAD |  | PRECOMP |  | LANDZ |  | SECTOR |  | MODE |  |  |
|  | 1 (Y) |  | 0 | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | 0 |  | NORMAL |  |  |
| NOTE: Some OSes (like SCO-UNIX) must use ”NORMAL” for installation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ESC: SKIP |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Up to four IDE drives can be detected, with parameters for each appearing in sequence inside a box. To accept the displayed entries, press the “Y” key; to skip to the next drive, press the “N” key. If you accept the values, the parameters will appear listed beside the drive letter on the screen.

6.13  HDD Low Level Format

This option should only be used by a professional. Low-level formatting can cause irreparable damage to your hard disk. The procedures include selecting the drive you want to low-level format, determining the bad tracks, and proceeding with pre-formatting.

6.14  Save & Exit Setup

This option allows you to determine whether to accept the modifications or not. If you type “Y”, you will quit the setup utility and save all changes into the CMOS memory. If you type “N”, you will return to Setup utility.

ROM PCI/ISA BIOS

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP Save to CMOS and Exit (Y/N)? N | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | HDD LOW LEVEL FORMAT |
| LOAD BIOS DEFAULTS | SAVE & EXIT SETUP |
| LOAD SETUP DEFAULTS | EXIT WITHOUT SAVING |
| ESC : Quit | á â à ß : Select Item |
| F10 : Save & Exit Setup | (Shift) F2 : Change Color |
| Save Data to CMOS & Exit Setup |  |

6.15  Exit Without Saving

Select this option to exit the Setup utility without saving the changes you have made in this session. Typing “Y” will quit the Setup utility without saving the modifications. Typing “N” will return you to Setup utility.

ROM PCI/ISA BIOS

CMOS SETUP UTILITY

AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP | INTEGRATED PERIPHERALS |
| --- | --- |
| BIOS FEATURES SETUP | SUPERVISOR PASSWORD |
| CHIPSET FEATURES SETUP | USER PASSWORD |
| POWER MANAGEMENT SETUP Quit Without Saving (Y/N)? N | IDE HDD AUTO DETECTION |
| PNP/PCI CONFIGURATION | HDD LOW LEVEL FORMAT |
| LOAD BIOS DEFAULTS | SAVE & EXIT SETUP |
| LOAD SETUP DEFAULTS | EXIT WITHOUT SAVING |
| ESC : Quit | á â à ß : Select Item |
| F10 : Save & Exit Setup | (Shift) F2 : Change Color |
| Abandon all Data & Exit Setup |  |

Chapter 7  LANDesk User’s Guide

This chapter gives a brief introduction to the optional LANDesk Client Manager (LDCM) utility, as well as the installation procedures.

The following items are covered in this chapter:

7.1  Introduction	60

7.2  Installation	61

7.2.1  Installing the Local Version of LDCM

7.2.2  Installing the Administrator Version of LDCM

LANDesk is a registered trademark of Intel Corporation.

7.1  Introduction

LANDesk Client Manager (LDCM 3.1 ) provides the capability for managing components (network interface cards, memory, printers, software applications, etc.) within a PC system. It uses the Desktop Management Interface (DMI) standard established by the Desktop Management Task Force (DMTF). Manageable components can be viewed, monitored, and administrated across multiple platforms, either locally or remotely on a network.

The LDCM package has been implemented in two different ways: a user (client/local) version and an administrator version (Remote Companion). The user version provides the ability to only manage the local PC. The administrator version allows a network administrator to manage the local PC and other PC nodes on the network. This means that the administrator version has the ability to gather information about remote PCs, as well as remotely controlling the PCs. The remote access is based upon granted rights by the managed client.

LDCM provides the user with self-help diagnostics, including a PC health meter, local alerting of potential problems, and hardware and software inventory. Automatic polling and alerting of memory and hardware conditions and predictive failure mechanisms minimize downtime and increase effective troubleshooting. LDCM can take periodic “snapshots” of critical configuration files for easy change management and restoration when needed.

To use LDCM, your computer must meet the following requirements:

- Operating System: Windows 95, Windows NT 3.51, or Windows NT4.0

- Memory: about 200KB

- Disk Storage Space: 3-5MB

- Hardware System: a DMI BIOS is required for full LDCM functionality

For network computers, the following requirements also apply:

- Protocols: IPX or IP (WinSock-enabled) communication protocol loaded on the client

- Hardware Interfaces: a network card for communication on the network

7.2  Installation

The optional LANDesk utility that comes with the CPU card runs in Windows NT or Windows 95 operating system.

Upon entering the Windows NT 4.0 or Windows 95 environment, insert the CD. Windows will autorun the installation program and show the  following screen.

|  | During Setup, you will be asked to install Internet Explorer 3.02 in order to continue, or else Setup will be aborted. LDCM supports various languages and will default to English if it is unable to load ‘language.dl’.. |
| --- | --- |

7.2.1  Installing the Local Version of LDCM

Double Click on ‘LANDesk Client Manager’ in the initial screen and the following screen will appear. Double click on the local version of  LANDesk Client Manager.

When the Welcome screen appears, click on “Next” to continue with Setup.

Choose the directory location where Setup will install LANDesk Client Manager. Click “Browse” if you want to change the directory suggested. Otherwise, click “Next” to start installing LDCM.

When Setup is finished, changes will have been made to the file AUTOEXEC.BAT. Restart your computer for the changes to take effect.

7.2.2  Installing the Administrator Version of LDCM

After double clicking on ‘LANDesk Client Manager’ in the initial screen, select the administrator version of the LDCM and the Welcome screen below will appear. Click on “Next” to continue.

The screen below allows you to install the documentation in Adobe Acrobat format and the Adobe Acrobat Reader software. Select the options you need and click  on “Next” to start the installation.

After LANDesk Client Manager Setup is complete, restart your computer to be able to use the LANDesk Client Manager.