# BIOS Setup

This Flash ROM BIOS has a built-in Setup program that allows users to modify the basic system configuration.  This information is stored in battery-backed RAM so that it retains the Setup information when the power is turned off.

## Getting Help

The online description of the highlighted setup function is displayed at the bottom of the screen.

## The Main Menu

Once you enter  BIOS CMOS Setup Utility, the Main Menu will appear on the Screen..  Use  arrow keys to select  the desired  items, press <Enter> to select or enter a submenu.

### Standard CMOS Setup

The items in Standard CMOS Setup Menu are divided into several categories.  Each category includes no, one or more than one setup items.  Use the arrow keys to highlight the item and then use the <PgUp> or <PgDn> keys to select the value you want in each item.

| Date & Time | Use these items to set the system date and time |
| --- | --- |
| Pri Master Pri Slave Sec Master Sec Slave | Use these items to configure devices connected to the Primary and Secondary IDE channels. To configure an IDE hard disk drive, choose Auto. If the Auto setting fails to find a hard disk drive, set it to User, and then fill in the hard disk characteristics (Size, Cyls, etc.) manually. If you have a CD-ROM drive, select the setting CDROM. If you have an ATAPI device with removable media (e.g. a ZIP drive or an LS-120) select Floptical. |
| Floppy Drive A Floppy Drive B | Use these items to set the size and capacity of the floppy diskette drive(s) installed in the system. |

### Advanced Setup

Use this page to set more advanced information about your system. Take some care with this page. Making changes can affect the operation of your computer.

AMIBIOS SETUP – ADVANCED SETUP ©1998 American Megatrends, Inc. All Rights Reserved

Trend ChipAwayVirus Enabled Boot From OnBoard LAN Disabled 1st Boot Device IDE-0 2nd Boot Device Fl oppy Try Other Boot Devices Yes S.M.A.R.T. for Hard Disks Disabled BootUp Num-Lock On Floppy Drive Swap Disabled Floppy Drive Seek Disabled Primary Display VGA/EGA Password Check Setup Boot To OS/2 over 64MB No Internal Cache Enabled System BIOS Cacheable Disabled Video BIOS Shadow Enabled Graphic Win Size 64M ESC : Quit  : F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift)F2 : Color F6 : Load BIOS Defaults F7 : Load Setup Defaults

| Trend ChipAway Virus | This mainboard has built-in virus protection in the firmware. Use this item to enable or disable the built-in virus protection. |
| --- | --- |
| Boot From OnBoard LAN | Use this item to enable or disable the function for Boot from onboard LAN. |
| 1st Boot Device 2nd Boot Device | Use these items to determine the device order the computer uses to look for an operating system to load at start-up time. |
| Try Other Boot Device | If you enable this item, the system will also search for other boot devices if it fails to find an operating system from the first two locations. |
| S.M.A.R.T. for Hard Disks | Enable this item if any IDE hard disks support the S.M.A.R.T. (Self-Monitoring, Analysis and Reporting Technology) feature. |

| BootUp Num-Lock | This item determines if the Num Lock key is active or inactive at system start-up time. |
| --- | --- |
| Floppy Drive Swap | If you have two diskette drives installed and you enable this item, drive A becomes drive B and drive B becomes drive A. |
| Floppy Drive Seek | If you enable this item, your system will check all floppy disk drives at start up. Disable this item unless you are using an old 360KB drive. |
| Primary Display | Use this item to determine the display devices for the primary display of your computer. |
| Password Check | If you have entered a password for the system, use this item to determine if the password is required to enter the Setup Utility (Setup) or required both at start-up and to enter the Setup Utility (Always). |
| Boot to OS/2 > 64MB | Enable this item if your system is featured more than 64MB. |
| Internal Cache | Leave this item enabled since all the processors have internal cache memory. |
| System BIOS Cacheable | If you enable this item, a segment of the system BIOS will be cached to main memory for faster execution. |
| Graphics Win Size | This item determines the operation of the onboard graphics adapter. We recommend that you leave this item at the default value. |

### Power Management Setup

This page sets some of the parameters for system power management operation.

AMIBIOS SETUP – POWER MANAGEMENT SETUP ©1998 American Megatrends, Inc. All Rights Reserved

Power Management/APM Disabled Green Monitor Power State Suspend Video Power Down Mode Suspend Hard Disk Power Down Mode Suspend Standby Time Out (Minute) Disabled Standby Time Out (Minute) Disabled FDC/LPT/COM Ports Monitor Ring On Power On Disabled LAN Card Power On Disabled Keyboard Power On Disabled Stroke Keys Selected N/A RTC Alarm Power On Disabled RTC Alarm Date 15 RTC Alarm Hour 12 RTC Alarm Minute 30 ESC : Quit  : Select Item RTC Alarm Second 30 F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift)F2 : Color F6 : Load Optimal values F7 : Load Best performance values

| Power Management/APM | Use this item to enable or disable a power management scheme. Both APM and ACPI are supported. |
| --- | --- |
| Green Monitor Power State | This sets the timeout for monitor Power Down. If the set time past without any activity on the monitor, it will enter into Power Down Mode. |
| Video Power Down Mode | This sets the timeout for graphics sub-system. If the set time past without any activity on the graphics, it will enter into Power Down Mode. |
| Hard Disk Power Down Mode | This sets the timeout for HDD Power Down. If the set time past without any activity on the HDD, it will enter into Power Down Mode. |
| Standby Time Out (Minute) | This sets the timeout for Standby mode in minutes. If the set time past without any system activity, the computer will enter into Standby mode. |
| Suspend Time Out (Minute) | This sets the timeout for Suspend mode in minutes. If the time selected past without any system activity, the computer will enter into Suspend mode. |
| FDC/LPT/COM/ Ports | If you select this item to Monitor, any activity on he floppy disk drive(s)/parallel port/serial port can rresume the system from a power saving mode. |
| Ring On Power On | The system can be turned off with a software command. When it’s enabled , the system can automatically resume for an incoming call on the Fax/Modem. An ATX power supply is required for effecting this feature. |
| LAN Card Power On | The system can be turned off with a software command. If you enable this item, the system can automatically resume if there is traffic on the network adapter. An ATX power supply is required for effecting this feature. |
| Keyboard Power On | If you enable this item as well as the Keyboard Power On jumer setting, you can turn the system on and off by pressing hot keys. |
| Stroke Keys Selected | If you enabled the Keyboard Power On, use this item to select the hot keys. |
| RTC Alarm Power On | When it’s enabled, the system can automatically resume at a set date and time for wake-up alarm. An ATX power supply is required for effecting this feature. |

### PCI / Plug and Play Setup

This page sets some of the parameters for devices installed on the PCI bus and devices that use the system plug and play capability.

AMIBIOS SETUP – PCI / PLUG AND PLAY SETUP ©1998 American Megatrends, Inc. All Rights Reserved

Plug and Play Aware O/S Yes Primary Graphics Adapter OnBoard VGA Assign IRQ for VGA Yes ESC : Quit  : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift)F2 : Color F6 : Load Optimal values F7 : Load Best performance values

| Plug and Play Aware O/S | Enable this item when your O/S is supportive for Plug and Play such as Windows 95 or 98. |
| --- | --- |
| Primary Graphics Adapter | The Graphics OnBoard was set to be Primary. The default PCI onboard display is able to work simultaneously with the second display card installed in a PCI slot. |
| Assign IRQ for VGA | If this item is set to Yes, an IRQ will be assigned to VGA graphics system. If it’s set to be No, an IRQ is released. |

### Load Best Performance Settings

When you press <Enter> on this item, a dialog box appears as below:

Load Best Performance Defaults (Y/N) ? N

Pressing ‘Y’ loads the BIOS default values for the most stable, minimal-

performance system operations.

Load Optimized Defaults

When you press  <Enter> on this item you get a confirmation dialog box

with a message likely to :

Load Optimized Defaults (Y/N) ? N

Pressing  ‘Y’ loads the default values that are factory settings for optimal perfor-

mance  system operation.

### Features Setup

This page sets some of the parameters for peripheral devices connected to the system.

AMIBIOS SETUP – FEATURES SETUP ©1998 American Megatrends, Inc. All Rights Reserved

OnBoard FDC Enabled OnBoard Serial Port1 3F8h/COM1 OnBoard IR Port2 Disabled Ir Function Disabled Ir Duplex Mode N/A OnBoard Parallel Port 378h Parallel Port Mode Normal Parallel Port IRQ 7 Parallel Port DMA N/A OnBoard Midi Port 300h Midi Port IRQ Select 9 OnBoard Game Port 208h OnBoard IDE Both Ultra DMA Support Disabled OnBoard USB Function Enabled USB Function for DOS Disabled ESC : Quit  : Select Item TV OUT Type NTSC F1 : Help PU/PD/+/- : Modify OnBoard Sound/Modem Enabled F5 : Old Values (Shift)F2 : Color OnBoard LAN Enabled F6 : Load Optimal values F7 : Load Best performance values

| OnBoard FDC | Enable or disable the onboard floppy disk drive interface. |
| --- | --- |
| OnBoard Serial Port1 | Enable or disable the onboard COM1 serial port, and to assign a port address |
| OnBoard Serial Port1 | Enable or disable the onboard COM2 serial port, and to assign a port address |
| IR Function | To define the protocol for an infrared port if you have installed an optional IR port. The choices are IrDA and ASKIR. |
| IR Duplex Mode | To define if the optional infrared port is full - duplex or half-duplex. |
| OnBoard Parallel Port | Enable or disable the onboard LPT1 parallel port, and to assign a port address. |
| Parallel Port Mode | For setting the parallel port mode. You can select SPP (Standard Parallel Port), ECP (Extended Capabilities Port), EPP (Enhanced Parallel Port), or ECP + EPP. |
| Parallel Port IRQ | Assign either IRQ 5 or 7 to the parallel port. |
| Parallel Port DMA | Assign a DMA channel to the parallel port. The options are 0, 1 and 3. |
| Onboard Midi Port | Enable or disable the onboard MIDI port, and to assign a port address. |
| Midi Port IRQ | For assigning an IRQ to the MIDI port. |
| OnBoard Game Port | Enable or Disable the Onboard Game Port, and to assign a port address. |
| OnBoard IDE | Enable or Disable either or both of the Onboard Primary and Secondary IDE channels. |
| Ultra DMA Support | For setting DMA to support IDE devices on the Primary or Secondary IDE channels. |
| OnBoard USB Function | Enable this item for using USB ports onboard. |
| USB Function for DOS | Enable this item for using USB onboard under a DOS environment. |
| TV Out type | TV Out kit support NTSC and PAL system. |
| OnBoard Sound/Modem | Enable or Disable onboard audio/modem. |
| OnBoard LAN | Enable or Disable onboard LAN device. |

### CPU PnP Setup

This page lets you manually configure the mainboard for the CPU. The system will automatically detect the kind of CPU that you have installed and make the appropriate adjustments to the items on this page.

Note: If you manually set the wrong speed and the system won’t run properly, press the Page Up key while the system is booting and a default setting will replace the incorrect CPU setting.

AMIBIOS SETUP – CPU PnP SETUP ©1998 American Megatrends, Inc. All Rights Reserved

CPU Type CPU Speed Manual CPU Core Voltage CPU Ratio x4.5 CPU Frequency. 133 MHz ESC : Quit  : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift)F2 : Color F6 : Load Optimal values F7 : Load Best performance values

| CPU Type CPU Core Voltage | These two items display the type and the core voltage of installed CPU |
| --- | --- |
| CPU Speed/Ratio/Frequency | Use this item to set the internal clock speed. When it’s set to Manual, the CPU speed equal to the CPU Ratio (Multiplier) x CPU external Frequency (i.e. CPU clock). |

### Hardware Monitor

This page is for sttting the parameters of hardware monitoring function.

AMIBIOS SETUP – HARDWARE Monitor ©1998 American Megatrends, Inc. All Rights Reserved

--- System Hardware Monitor --- CPU Socket 370 Temp. 30°C/86°F Sytem Temp 24°C/86°F CPU Fan Speed Chassis Fan Speed Vcore 2.000 V Vcc +1.8V 1.800 V Vcc +3.3V 3.3.00 V Vcc +5.0V 5.000 V Board +12V 12.000 V ESC : Quit  : Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift)F2 : Color F6 : Load Optimal values F7 : Load Best performance values

| CPU / System Temp. | Display of CPU and the system temperature measurement. |
| --- | --- |
| FAN Speed & Voltage Measurements | Indication of cooling fan speeds in RPM and the various system voltage measurements. |

### Change Password

If you highlight this item and press Enter, a dialog box appears which lets you enter a Supervisor password. You can enter no more than six letters or numbers. Press Enter after you have typed in the password. A second dialog box asks you to retype the password for confirmation. Press Enter after you have retyped it correctly. The password is then required to access the Setup Utility or for that and at start-up, depending on the setting of the  Password Check item in Advanced Setup.

Highlight this item, press Enter and type in the current password. At the next dialog box, type in the new password, or just press Enter to disable password protection.

### Change or Remove the Password

Highlight this item, press Enter and type in the current password. At the next dialog box, type in the new password, or just press Enter to disable password protection.

### Exit

Highlight this item and press Enter to save the changes that you have made in the Setup Utility configuration and exit the program. When the Save and Exit dialog box appears, press Y to save and exit, or press N to exit without saving.