Block Diagram

AMI BIOS POST Code :

| POST (HEX) | Description |
| --- | --- |
| C2 | NMI is Disabled. Power on delay starting. |
| C5 | Power on delay complete. Going to disable Cache if any. |
| C6 | Calculating ROM BIOS checksum. |
| C7 | ROM BIOS checksum passed. CMOS shutdown register test to be done next. |
| C8 | CMOS shutdown register test done. CMOS checksum calculation to be done next. |
| CA | CMOS checksum calculation is done, CMOS Diag byte written. CMOS status register about to init for Date and Time. |
| CB | CMOS status register init done. Any initialization before keyboard BAT to be done next. |
| CD | BAT command to keyboard controller is to be issued. |
| CE | Keyboard controller BAT result verified. Any initialization after KB controller BAT to be done next. |
| CF | Initialization after KB controller BAT done. Keyboard command byte to be written next. |
| D1 | Keyboard controller command byte is written. Going to check pressing of <INS> key during power-on. |
| D2 | Checking for pressing of <INS> key during power-on done. Going to disable DMA and Interrupt controllers. |
| D3 | DMA controller #1,#2, interrupt controller #1,#2 disabled. Chipset init/ auto memory detection about to begin. |
| D4 | Chipset initialization/ auto memory detection over. To uncompress the RUNTIME code. |
| D5 | RUNTIME code is uncompressed. |
| DD | Transfer control to uncompressed code in shadow ram at F000:FFF0. |
| 03 | NMI is Disabled. To check soft reset/power-on. |
| 05 | Soft reset/power-on determined. Going to disable Cache if any. |
| 06 | POST code to be uncompressed. |
| 07 | POST code is uncompressed. CPU init and CPU data area init to be done next |

| POST (HEX) | Description |
| --- | --- |
| 08 | CPU and CPU data area init done. CMOS checksum calculation to be done next. |
| 09 | CMOS checksum calculation is done, CMOS Diag byte written. CMOS init to begin (If "Init CMOS in every boot” is set). |
| 0A | CMOS initialization done (if any). CMOS status register about to init for Date and Time. |
| 0B | CMOS status register init done. Any initialization before keyboard BAT to be done next. |
| 0C | KB controller I/B free. Going to issue the BAT command to keyboard controller. |
| 0D | BAT command to keyboard controller is issued. Going to verify the BAT command. |
| 0E | Keyboard controller BAT result verified. Any initialization after KB controller BAT to be done next. |
| 0F | Initialization after KB controller BAT done. Keyboard command byte to be written next. |
| 10 | Keyboard controller command byte is written. Going to issue Pin-23,24 blocking/unblocking command. |
| 11 | Pin-23,24 of keyboard controller is blocked/ unblocked. Going to check pressing of <INS> key during power-on. |
| 12 | Checking for pressing of <INS> key during power-on done. Going to disable DMA and Interrupt controllers. |
| 13 | DMA controller #1,#2, interrupt controller #1,#2 disabled. Video display is disabled and port-B is initialized. Chipset init about to begin. |
| 15 | Chipset initialization over. 8254 timer test about to start. |
| 19 | 8254 timer test over. About to start memory refresh test. |
| 1A | Memory Refresh line is toggling. Going to check 15 micro second ON/OFF time. |
| 20 | Memory Refresh period 30 micro second test complete. Base 64K memory to be initialized. |
| 23 | Base 64K memory initialized. Going to set BIOS stack and to do any setup before Interrupt vector init. |
| 24 | Setup required before interrupt vector initialization complete. Interrupt vector initialization about to begin. |

| POST (HEX) | Description |
| --- | --- |
| 25 | Interrupt vector initialization done. Going to read Input port of 9042 for turbo switch (if any) and to clear password if post diag switch is on. |
| 26 | Input port of 8042 is read. Going to initialize global data for turbo switch. |
| 27 | Global data initialization for turbo switch is over. Any initialization before setting video mode to be done next. |
| 28 | Initialization before setting video mode is complete. Going for monochrome mode and color mode setting. |
| 2A | Different BUSes init (system, static, output devices) to start if present. (Please see Appendix for details of different BUSes). |
| 2B | About to give control for any setup required before optional video ROM check. |
| 2C | Processing before video ROM control is done. About to look for optional video ROM and give control. |
| 2D | Optional video ROM control is done. About to give control to do any processing after video ROM returns control. |
| 2E | Return from processing after the video ROM control. If EGA/VGA not found then do display memory R/W test. |
| 2F | EGA/VGA not found. Display memory R/W test about to begin. |
| 30 | Display memory R/W test passed. About to look for the retrace checking. |
| 31 | Display memory R/W test or retrace checking failed. About to do alternate Display memory R/W test. |
| 32 | Alternate Display memory R/W test passed. About to look for the alternate display retrace checking. |
| 34 | Video display checking over. Display mode to be set next. |
| 37 | Display mode set. Going to display the power on message. |
| 38 | Different BUSes init (input, IPL, general devices) to start if present. (Please see Appendix for details of different BUSes). |
| 39 | Display different BUSes initialization error messages. (Please see Appendix for details of different BUSes). |
| 3A | New cursor position read and saved. Going to display the Hit <DEL> message. |

| POST (HEX) | Description |
| --- | --- |
| 3B | Hit <DEL> message displayed. Virtual mode memory test about to start. |
| 40 | Going to prepare the descriptor tables. |
| 42 | Descriptor tables prepared. Going to enter in virtual mode for memory test. |
| 43 | Entered in the virtual mode. Going to enable interrupts for diagnostics mode. |
| 44 | Interrupts enabled (if diagnostics switch is on). Going to initialize data to check memory wrap around at 0:0. |
| 45 | Data initialized. Going to check for memory wrap around at 0:0 and finding the total system memory size. |
| 46 | Memory wrap around test done. Memory size calculation over. About to go for writing patterns to test memory. |
| 47 | Pattern to be tested written in extended memory. Going to write patterns in base 640k memory. |
| 48 | Patterns written in base memory. Going to findout amount of memory below 1M memory. |
| 49 | Amount of memory below 1M found and verified. Going to findout amount of memory above 1M memory. |
| 4B | Amount of memory above 1M found and verified. Check for soft reset and going to clear memory below 1M for soft reset. (If power on, go to check point# 4Eh). |
| 4C | Memory below 1M cleared. (SOFT RESET) Going to clear memory above 1M. |
| 4D | Memory above 1M cleared. (SOFT RESET) Going to save the memory size. (Goto check point# 52h). |
| 4E | Memory test started. (NOT SOFT RESET) About to display the first 64k memory size. |
| 4F | Memory size display started. This will be updated during memory test. Going for sequential and random memory test. |
| 50 | Memory testing/initialization below 1M complete. Going to adjust displayed memory size for relocation/ shadow. |
| 51 | Memory size display adjusted due to relocation/ shadow. Memory test above 1M to follow. |
| 52 | Memory testing/initialization above 1M complete. Going to save memory size information. |

| POST (HEX) | Descriptions |
| --- | --- |
| 53 | Memory size information is saved. CPU registers are saved. Going to enter in real mode. |
| 54 | Shutdown successful, CPU in real mode. Going to disable gate A20 line and disable parity/NMI. |
| 57 | A20 address line, parity/NMI disable successful. Going to adjust memory size depending on relocation/shadow. |
| 58 | Memory size adjusted for relocation/shadow. Going to clear Hit <DEL> message. |
| 59 | Hit <DEL> message cleared. <WAIT...> message displayed. About to start DMA and interrupt controller test. |
| 60 | DMA page register test passed. To do DMA#1 base register test. |
| 62 | DMA#1 base register test passed. To do DMA#2 base register test. |
| 65 | DMA#2 base register test passed. To program DMA unit 1 and 2. |
| 66 | DMA unit 1 and 2 programming over. To initialize 8259 interrupt controller. |
| 67 | 8259 initialization over. |
| 7F | Extended NMI sources enabling is in progress. |
| 80 | Keyboard test started. clearing output buffer, checking for stuck key, About to issue keyboard reset command. |
| 81 | Keyboard reset error/stuck key found. About to issue keyboard controller interface test command. |
| 82 | Keyboard controller interface test over. About to write command byte and init circular buffer. |
| 83 | Command byte written, Global data init done. About to check for lock-key. |
| 84 | Lock-key checking over. About to check for memory size mismatch with CMOS. |
| 85 | Memory size check done. About to display soft error and check for password or bypass setup. |
| 86 | Password checked. About to do programming before setup. |
| 87 | Programming before setup complete. Going to uncompress SETUP code and execute CMOS setup. |
| 88 | Returned from CMOS setup program and screen is cleared. About to do programming after setup. |
| 89 | Programming after setup complete. Going to display power on screen message. |

| POST (HEX) | Description |
| --- | --- |
| 8B | First screen message displayed. <WAIT...> message displayed. About to do Video BIOS shadow. |
| 8C | Video BIOS shadow successful. Setup options programming after CMOS setup about to start. |
| 8D | Setup options are programmed, mouse check and init to be done next. |
| 8E | Mouse check and initialization complete. Going for hard disk controller reset. |
| 8F | Hard disk controller reset done. Floppy setup to be done next. |
| 91 | Floppy setup complete. Hard disk setup to be done next. |
| 94 | Hard disk setup complete. To set base and extended memory size. |
| 95 | Memory size adjusted due to mouse support. Init of different BUSes optional ROMs from C800 to start. (Please see Appendix-I for details of different BUSes). |
| 96 | Going to do any init before C800 optional ROM control. |
| 97 | Any init before C800 optional ROM control is over. Optional ROM check and control will be done next. |
| 98 | Optional ROM control is done. About to give control to do any required processing after optional ROM returns control. |
| 99 | Any initialization required after optional ROM test over. Going to setup timer data area and printer base address. |
| 9A | Return after setting timer and printer base address. Going to set the RS-232 base address. |
| 9B | Returned after RS-232 base address. Going to do any initialization before Coprocessor test. |
| 9C | Required initialization before Coprocessor is over. Going to initialize the Coprocessor next. |
| 9D | Coprocesor initialized. Going to do any initialization after Coprocessor test. |
| 9E | Initialization after Coprocessor test is complete. Going to check extd keyboard, keyboard ID and num-lock. |
| 9F | Extd keyboard check is done, ID flag set, num-lock on/off. Keyboard ID command to be issued. |
| A0 | Keyboard ID command issued. Keyboard ID flag to be reset. |
| A1 | Keyboard ID flag reset. Cache memory test to follow. |
| A2 | Cache memory test over. Going to display any soft errors. |
| A3 | Soft error display complete. Going to set keyboard typematic rate. |

| POST (HEX) | Description |
| --- | --- |
| A4 | Keyboard typematic rate set. To program memory wait states. |
| A5 | Memory wait states programming over. Going to clear the screen and enable parity/NMI. |
| A7 | NMI and parity enabled. Going to do any initialization required before giving control to optional ROM at E000. |
| A8 | Initialization before E000 ROM control over. E000 ROM to get control next. |
| A9 | Returned from E000 ROM control. Going to do any initialization required after E000 optional ROM control. |
| AA | Initialization after E000 optional ROM control is over. Going to display the system configuration. |
| B0 | System configuration is displayed. |
| B1 | Going to copy any code to specific area. |
| 00 | Copying of code to specific area done. Going to give control to INT-19 boot loader. |

APPENDIX:

| CHECK-POINT | Description of CHECK-POINT |
| --- | --- |
| 2A | Different BUSes init (system, static, output devices) to start if present. |
| 38 | Different BUSes init (input, IPL, general devices) to start if present. |
| 39 | Display different BUSes initialization error messages. |
| 95 | Init of different BUSes optional ROMs from C800 to start. |

## Problem Sheet

To conduct correct diagnosis and repairing work, you were requested to fill

this RMA from and attach it with each RMA mainboard before sending back

for repairing.

| Customer |  |  |  |
| --- | --- | --- | --- |
| Name |  | Tel |  |
| address |  | Fax |  |

| Mainboard |  |  |  |
| --- | --- | --- | --- |
| Mode |  | Mainboard Rev |  |
| Serial No. |  | BIOS version |  |

| Peripherals Description |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Brand |  |  | Brand |  |
| CPU | Type |  | RAM | Type & Size |  |
|  | Voltage |  |  | Speed |  |
|  | Brand |  |  | Brand |  |
| Floppy disk | Mode |  | VGA card | Chipset |  |
|  | Size |  |  | RAM Type |  |
|  | Brand |  |  | Brand |  |
| Hard disk | Mode |  | CD-ROM | Mode |  |
|  | Size |  |  | Speed |  |
|  | Brand |  |  | Brand |  |
| Sound card | Mode |  | Lan card | Mode |  |
|  | Remark |  |  | PCI/ISA Bus |  |
|  | Brand |  |  | Brand |  |
| Mouse | Mode |  | O.S | Name |  |
|  | PS/2 or Serial |  |  | Version |  |
| Problem Description |  |  |  |  |  |

MEMO