# Introduction

## Specifications

| System Chipset | VIA 692/693 chipset. |  |
| --- | --- | --- |
| CPU | Intel Pentium II processors, support 233/266/300/333 (Ex. Clk 66MHz) MHz. 300/350/400/ 450/ 500 (Ex. Clk 100MHz) MHz |  |
| Memory | Expandable to 768MB (3 banks) with three 168-pin DIMM socket {support 3.3 V EDO (66MHz only) / SDRAM (66MHz/ 100MHz)}. |  |
| I/O | Winbond 83977, two high speed 16550 compatible serial ports, one Multi-Mode. Parallel Port support SPP/EPP/ECP standard mode. Two onboard PCI IDE Ports (32-bit data transfer). LS-120/ ZIP FDD, IrDA/ ASK IR/ Consumer IR. Dual USB ports Support two 360/720KB/1.2/1.44/2.88MB floppy disk devices. One PS/2 Mouse port. |  |
| BIOS | Award System BIOS installed in socket (Flash and PnP). |  |
| Expansion slots |  | One AGP slot, five PCI Master Slots and two 16-bit ISA Slots. |
| Voltage |  | Auto 1.8V-3.5V |
| Dimension |  | 4-layer PCB, size (22cm x 20cm). |
| Others |  | Support BIOS setting CPU type (Jumper-less), CPU Auto Temperature Sensor & Music Alarm, voltage monitor and CPU Fan monitor, Bus Master/ Ultra DMA/33, ACPI, AGP Bus, Keyboard Power On, PS/2 Mouse Power On, Modem Ring On, LAN wake up, Debug display on board. |

# Setup Guide

## Layout Diagram

## Smart Display On Board

When the CPU, DRAM, Cache RAM, FDD or VGA card have not been properly installed, user can isolate those basic problems through the Debug display and instructions from the manual. To Professional system engineers or maintenance engineers, the Debug display can work as an 80 Port Debug Card.

| Error code | Display | Message | Solution |
| --- | --- | --- | --- |
| C1 | None | Can’t detect DRAM | ### 1. Reinstall or replace the SDRAM. 2. Reinstall or replace the BIOS. |
| C6 | None | Can’t detect DRAM | ### 1. Reinstall or replace the SDRAM. 2. Reinstall or replace the BIOS. |
| OD | None | Can’t detect VGA card | ### 1. Reinstall or replace the VGA card. 2. Replace the BIOS. |
| 4E | Yes | Can’t detect Floppy disk | ### 1. Replace the BIOS. (if no screen) ### 2. Enter the BIOS Setup menu to reset. 3. Check that the FDD cable and the power connector are properly connected. 4. Reconnect the FDD cable or replace the FDD. |
| 61 | Yes | L2 cache problem | 1. Enter BIOS Setup to disable the external cache. |

## CPU Voltage and Frequencies

| ROM PCI/ISA BIOS (2A69KTJA) CHIPSET FEATURE SETUP AWARD SOFTWARE, INC |  |  |  |
| --- | --- | --- | --- |
| Auto Configuration | : Enabled | Auto Detect DIMM/ PCI Clk | : Enabled |
| EDO DRAM Speed Selection | : 60ns | Spread Spectrum | : Disabled |
| EDO CASx# MA Wait State | : 2 | CPU Speed | : Manual |
| EDO RASx# Wait State | : 2 | CPU Ratio | : X 2.5 |
| SDRAM RAS-to CAS Delay | : 3 | CPU Frequency | : 100 MHz |
| SDRAM RAS Precharge Time | : 3 | Spread Spectrum | : Disabled |
| SDRAM CAS latency Time | : Auto | CPU Warning Temperature | : Disabled |
| SDRAM Precharge Control | : Disabled | Current CPU Temperature | : 28℃/ 82℉ |
| DRAM Data Integrity Mode | : Non-ECC | Current SYSFAN Speed | :4285 RPM |
| System BIOS Cacheable | : Enabled | Current CPUFAN Speed | :4000 RPM |
| Video BIOS Cacheable | : Enabled | Current Vin3 (V) | : 2.88V |
| Video RAM Cacheable | : Disabled |  |  |
| 8 Bit I/O Recovery Time | : 1 |  |  |
| 16 Bit I/O Recovery Time | : 1 |  |  |
| Memory Hole At 15M-16M | : Disabled | Esc : Quit  Selection : Item |  |
| Passive Release | : Enabled | F1 : Help PU/PD/+/- : Modify |  |
| Delayed Transaction AGP Aperture Size (MB) | : Disabled : 64 | F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Default F7 : Load Setup Default |  |

Dear Customers:

Thank you for your patronage of our products. The board you bought is a jumper-less mainboard. The ratio and frequency of the CPU shall be set in BIOS and the working voltage for the CPU shall be automatically detected. Please read carefully the following instructions:

1. Power on the installed system and press the "DEL" key to enter BIOS Setup. Select "Chipset Features Setup" and press <Enter>.

2. Select "CPU Speed" and press "PgUp" or "PgDn" to set the CPU ratio and frequency. The available options are Intel PII

233MHz (66X3.5), 266MHz (66X4), 300MHz (66X4.5), 333MHz (66X5), 366MHz (66X5.5), 400MHz(66X6), 433MHz(66X6.5), 466MHz(66X7), 500MHz(66X7.5).

250MHz(100X2.5), 300MHz(100X3), 350MHz(100X3.5), 400MHz(100X4), 450MHz(100X4.5), 500MHz(100X5), 550MHz(100X5.5), 600MHz(100X6) and "Manual".

To set the CPU manually, please note the following:

CPU Speed: "Manual" (you can manually set the CPU ratio and frequency)

CPU Ratios: x3.5, x4, x4.5, x5, x5.5, x6, x6.5, x7, x7.5

CPU Frequency: 66, 68, 75, 83, 100, 103, 112, 133Mhz

Several options are provided for the CPU external clock. You are recommended to restore to the default setting in case of instability when the external clock exceeds 66MHz.

NOTE: System failure may occur if the CPU frequency is set incorrectly. To solve this problem. Press the "Insert" key on the keyboard to clear the previously set frequency (i. c., restore the default frequency), and then reboot the system.

- Switch voltage is applied, making the temperature lower and voltage steadier.

- You don’t need to adjust Voltage in Pentium II mainboard. It will automatically send out one VID (Voltage Identification) to the mainboard power supply to ask for the voltage it needs.

- The CPU type default setting is Intel Pentium II 250MHz=100 MHz＊2.5.

Intel Pentium II CPU family

| CPU | Ext. clk | Ratio | L1 cache | L2 cache | Package |
| --- | --- | --- | --- | --- | --- |
| Intel Pentium II – 450MHz | 100MHz | X4.5 | 32KB | 512KB | SECC 1 |
| Intel Pentium II – 400MHz | 100MHz | X4 | 32KB | 512KB | SECC 1 |
| Intel Pentium II – 350MHz | 100MHz | X3.5 | 32KB | 512KB | SECC 1/ 2 |
| Intel Pentium II – 300MHz | 100MHz | X3 | 32KB | 512KB | SECC 1 |
| Intel Pentium II – 333MHz | 66MHz | X5 | 32KB | 512KB | SECC 1 |
| Intel Pentium II – 300MHz | 66MHz | X4.5 | 32KB | 512KB | SECC 1 |
| Intel Pentium II – 266MHz | 66MHz | X4 | 32KB | 512KB | SECC 1 |
| Intel Pentium II – 233MHz | 66MHz | X3.5 | 32KB | 512KB | SECC 1 |

## EDO/ SDRAM Installation  Procedures:

- A 168-pin DIMM can support up to 768MB 3.3V EDO (66MHz) / SDRAM (66MHz/ 100MHz).

- You are recommended to use SDRAMs. With SPD that are compliant with PC-100. This will enable BIOS to detect the SDRAM speed, thereby fully bring into play the efficiency of the SDRAM.

- To avoid compatibility and reliability problems, you are recommended to test the 168-pin SDRAMs before buying them since the PCB specifications differ.

- First, verify the working voltage of the EDO/ SDRAM module in either DIMM socket.

- P2BXAT only supports 3.3V EDO/ SDRAM module. The following illustration shows you the difference between 3.3V and 5V to ensure your correct selection of 3.3V DIMM module for using.

- You can set up the BIOS “Chipset Feature Setup” to the best working condition basing on the type of EDO/ SDRAM you are using.

- The BIOS DRAM default setting is 60 ns. Change the BIOS “Chipset Feature Setup” default setting to 50ns for better performance, if the chipset is marked 50ns.

- Change nothing if EDO RAM is used. BIOS automatically detect the RAM type.

- MEMO for Installing System:

⊕ Concerning memory setup, you can find how to from “Chipset Feature Setup” under BIOS setup. However, to avoid system unstable or system hang, user without engineering background is not suggested to change BIOS set up.

⊕ If system boot failure, please clean DIMM socket (with clean oil) or polish Gold-Finger of DRAM with soft eraser, and try again.

- The Dual Inline Memory Module (DIMM) must be 3.3 Volt and Unbuffered Synchronous DRAM (SDRAM) 8MB, 16MB, 32MB, 64MB, 128MB or 256MB. The following illustration shows the type of DIMM Module.

168-PIN SDRAM DIMM Notch Key Definitions

## Keyboard/ PS/2 Mouse Power On and MODEM Ring on

- To make sure the 5VSB signal nearly to 750mA (Amperage) from ATX Power Supply, or if your keyboard consuming power than 300mA, it's better to upgrade your ATX Power Supply to 1A for working perfectly.

- If you are going to use the function of keyboard and PS/2 mouse power on, then, the power-switch will be becoming useless automatically (unable to be used).

| ROM PC/ISA BIOS (2A69KTJA) INTEGRATED PERIPHERALS AWARD SOFTWARE, INC. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| IDE HDD Block Mode |  | : Enabled |  | Onboard Serial Port 2 | : 2F8H / IRQ3 |  |
| IDE Primary Master PIO |  | : AUTO |  | UART Mode Select | : Normal |  |
| IDE Primary Slave PIO |  | : AUTO |  |  |  |  |
| IDE Secondary Master PIO |  | : AUTO |  | Onboard Parallel Port | : 378H/IRQ 7 |  |
| IDE Secondary Slave PIO |  | : AUTO |  | Parallel Port Mode | : ECP+EPP |  |
| IDE Primary Master UDMA |  | : AUTO |  | ECP Mode Use DMA | : 3 |  |
| IDE Primary Slave UDMA |  | : AUTO |  | EPP Mode Select | : EPP 1.9 |  |
| IDE Secondary Master UDMA |  | : AUTO |  |  |  |  |
| IDE Secondary Slave UDMA |  | : AUTO |  |  |  |  |
| On-Chip Primary PCI IDE |  | : Enabled |  |  |  |  |
| On-Chip Secondary PCI IDE |  | : Enabled |  |  |  |  |
| USB Keyboard Support |  | : Disabled |  |  |  |  |
| Init Display First |  | : AGP |  |  |  |  |
| POWER ON Function |  | : Hot KEY |  |  |  |  |
|  |  |  |  | Esc : Quit Selection : Item |  |  |
| Hot Key Power ON |  | : Ctrl-F12 |  | F1 : Help PU/PD/+/- : Modify |  |  |
| KBC input clock |  | : 8MHz |  | F5 : Old Values (Shift)F2 : Color |  |  |
| Onboard FDC Controller |  | : Enabled |  | F6 : Load BIOS Default |  |  |
| Onboard Serial Port 1 | : 3F8H / IRQ4 |  | F7 : Load Setup Default |  |  |  |

| Hot KEY | When user select this option, it will show another line lines as Hot Key Power ON: Ctrl-F(1/2/3/4/5/6/7/8/9/10/11/12) select any you like. After power off, if user key in the "Ctrl-F?", it will power on the system. |
| --- | --- |
| PS/2 Mouse Left | It will power on the system by PS/2 mouse left. |
| PS/2 Mouse Right | It will power on the system by PS/2 mouse Right. |
| Button Only | Only the power button can power on the system. |

- Modem Ring On Function Operation:

| ROM PCI / ISA BIOS (2A69KTJA) POWER MANAGEMENT SETUP AWARD SOFTWARE, INC |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Power Management | : Disabled | ** Reload Global Timer Events ** |  |  |  |  |  |
| PM Control by APM | : No | IRQ[3-7, 9-15], NMI | : Enabled |  |  |  |  |
| Video Off Method | : V/H SYNC+Blank | Primary IDE 0 | : Disabled |  |  |  |  |
| Video Off After | : Standby | Primary IDE 1 | : Disabled |  |  |  |  |
| MODEM Use IRQ | : 3 | Secondary IDE 0 | : Disabled |  |  |  |  |
| Doze Mode | : Disabled | Secondary IDE 1 | : Disabled |  |  |  |  |
| Standby Mode | : Disabled | Floppy Disk | : Disabled |  |  |  |  |
| Suspend Mode | : Disabled | Serial Port | : Enabled |  |  |  |  |
| HDD Power Down | : Disabled | Parallel Port | : Disabled |  |  |  |  |
| Throttle Duty Cycle | : 62.5% |  |  |  |  |  |  |
| ZZ Active in Suspend | : Disabled |  |  |  |  |  |  |
| VGA Active Monitor | : Enabled |  |  |  |  |  |  |
| Soft-Off by PWR-BTTN | : Instant-Off |  |  |  |  |  |  |
| CPUFAN Off In Suspend | : Enabled |  |  |  |  |  |  |
| Resume by Ring | : Enabled |  |  |  |  |  |  |
| IRQ 8 Clock Event | : Disabled | Esc : Quit Selection Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Default F7 : Load Setup Default |  |  |  |  |  |

- 1. Have an external MODEM connected to COM 1 or COM 2.

- 2. Enter BIOS setup.

- 3. Select Power Management Setup.

- 4. This number of MODEM use IRQ has to be set as same as the IRQ of

- Serial Port which you are connecting in. Please set in N/A if you are

- not going to use the function of MODEM ring on.

- 5. Resume by Ring: Enable.

- 6. Save BIOS setup and Reboot.

- 7. Booting from DOS, Windows, or Windows 95.

- 8. Turn off the system by:

- a. ATX-Power Switch

- b. Windows 95 Software Power Off

- 9. System Waiting for Modem Ring On

- When Modem Ringing Signal Active, System will wake-up.

## System Health Monitor

- Fan Monitoring:

There are two fan connectors, one is for CPU, the other can be a housing fan. When the fans speed is working abnormal, there will be warning (Speaker Alarm) through application software such as SM10(Small Icon for System Monitoring) to notify user. The fan monitoring function is implemented by connecting fan to 3-pin fan connector FAN1/ FAN2 and installing SM10. Referring to Page 10 (System Health Monitor).

- CPU Thermal Protection:

TM-P2BXAT implements special thermal protection circuits. When temperature is higher than a predefined value, there will be warning (Speaker Alarm) through application software such as SM10 (Small Icon for System Monitor) to notify user. It's automatically implemented by BIOS or SMD10, no hardware installation is needed. Referring to Page10 (System Health Monitor).

- System Voltage Monitoring:

TM-P2BXAT is featured with a voltage monitoring system. When you turn on your system, this smart design will keep on monitoring your system working voltage. If any of voltage is over the component's standard, there will be Speaker Alarm though application software SM10 (Small Icon for System Monitor) for a warning to user. System voltage monitoring function monitors 5V, 12V, 3.3V and CPU voltage. It's automatically implemented by BIOS and SM10, no hardware installation is needed. Referring to Page 10 (System Health Monitor)