TM-S730LMV

All-In-One Mainboard

Version: 1.0

Warning: Never run the processor without the heatsink properly

and firmly installed. Permanent Damage Will result!

# Copyright

Copyright © 2000 by this manufacturer. All rights reserved. No part of this publication may be reproduced, transmitted, transcribed, stored in a retrieval system, or translated into any language or computer language, in any form or by any means, electronic, mechanical, magnetic, optical, manual or otherwise, without the prior written consent of this manufacturer.

# Disclaimer

This manufacturer makes no representations or warranties, either expressed or implied, with respect to the contents hereof and specifically disclaims any warranties, merchantability or fitness for any particular purpose. Any software described in this manual is sold or licensed “as is”. Should the programs prove defective following their purchase, the buyer (and not this manufacturer, its distributor, or its dealer) assumes the entire cost of all necessary servicing, repair, and any incidental or consequential damages resulting from any defect in the software. Further these manufacturer reserves the right to revise this publication and to make changes from time to time in the contents hereof without obligation to notify any person of such revision or changes.

Trademark Acknowledgement

Intel and Pentium are registered trademarks of Intel Corporation. Award is a registered trademark of Award Software Inc. Other brand and product names are trademarks and/or registered trademarks of their respective holders.

Printed in Taiwan

# Overview

The TM-S730LMV is a AMD Athlon based mainboard that utilizes VIA SiS 730S Chipset, a high performance of integrated all-in-one mainboard. The AMD Athlong is a seventh-generation micro architecture with an integrated L2 Cache, which is powerful enough to support bandwidth requirements of a large range of applications, hardware, graphics and memory technologies.

Socket A is the name for AMD’s new socket interface designed to support both AMD Duron and AMD Athlon processors.  This innovation is made possible by integrating the L2 cache memory on chip with the processor. Socket A will help to enable smaller enclosures and ultimately result in a wider variety of solutions in the market.

The AMD Duron processor is driven from the AMD Athlon processor core.  It features full-speed, on-chip cache memory, a 200 MHz front size side system bus, and enhanced 3Dnow! Technology.  Although both processors are related, there are key differences.  The AMD Athlon processor is targeted at the performance segment, and as such will have more cache memory and higher clock speeds.

CONTENT

Copyright	2

Disclaimer	2

Overview	3

CONTENTS	4

Introduction	6

Specifications	6

Setup Guide	8

Layout Diagra………………………………………………….8

Jumper Settings……………………………………………...10

EscoCAM, An Integrated Module For Escorting PC …...……..11

BIOS Update Note…………………………………………...16

BIOS Setup………………………………………………………...17

Standard CMOS Setup	18

Advanced Setup	19

Power Management	21

PCI/PnP Setup	23

Load Optimal Settings	24

Load Best Performance Settings	24

Features Setup	25

CPU PnP Setup 	27

Hardware Monitor	28

Change Password	29

Exit…………	29

DRIVER & SOFTWARE 	30

System Block Diagram……………………………………………36

Post Code.……. …..……………………………………………...37Problem Sheet……..……………………………………………...44

MEMO…....……. …..……………………………………………...37

# Introduction

# Specifications

| System Chipset | SiS 730S Chipset |  |
| --- | --- | --- |
| CPU | Supports Socket A (Socket-462) AMD Duron and Athlon processors at 500 MHz to 1.13GHz. |  |
| Memory | 2 x DIMM supports up to 1024MB of PC133/ VC133 memory (minimum 32MB) on board. |  |
| IDE | Primary and Secondary PCI IDE channels. Support PIO, Multiword DMA, Bus Mastering and Ultra DMA 66/100 modes. |  |
| Graphic On Board | AGP 128 bit 2D/3D 100 MHz AGP Graphics complies with AGP V2.0. |  |
|  | *Shared memory architecture allows a 64MB maximum frame buffer. *Supports high resolutions up to 1920x1200 16M colors, up to 2048x2048 Texture size and Virtual screen up to 4096x4096. *Supports hardware DVD Accelerator. |  |
| Sound On Board | *3D PCI Sound Pro, meets PC98 Spec. *16-bit CODEC for full-duplex playback and record. * HRTF professional Audio supports both Direct Sound 3D and Aureal’s 3D compatible interface plus support for 4-channel speakers out. *Software Wave-Table Synthesizer and Digital Audio Interface(SDPIF) In/Out with 24-bit, 48 KHz sample rate and measured 120db Audio quality. *Built-in 32 ohm earphone buffer and 3D surround sound. Provides PU-401Game/MIDI port and legacy Sound Blaster 16 support. *Stereo Mixer supports analog mixing from CD- Audio and Line In or digital mixing from voice, *FM/Wave-Table and digital CD-Audio. Driver support for MS-DOS, Microsoft Windows 95/98/2000/NT 4.0. |  |
| AMR Fax/Modem enclosed | AMR 56K Fax/Modem card supports V.90 standard for Internet communication. Supports Auto Fallback and MNP5, V.42bis data compression. |  |
| I/O | *2 x PS/2 Mouse and Mouse Ports, 2xUSB, 1x VGA port. *1x Serial Port with 16550-compatible fast UART. *1x Parallel with ECP and EPP. *1x Infrared port, 2 x USB external connectors. |  |
| Power Management | ATX Power Supply, Suspend, Shutdown, Wake Up On LAN, Wake On Modem, Wake On Alarm, Keyboard Power On/Off. |  |
| Hardware Monitor |  | Supports CPU Temperature Voltage and Fan Speed monitor. |
| Expansion slots |  | 2 x PCI 32-bit port, 1x AGP(x4), 1 X AMR |
| Dimension |  | 4-layer PCB, size (25mm x 25mm), Micro-ATX. |
| Bundled Software |  | *Super VB : Provides anti-virus protection. *Super Voice : For data, FAX and voice communication. *Gamut 2000: Provides professional audio features. *Media Ring Talk : Provides PC to PC, or PC to phone internet communication. *Corel WordPerfect Suite 8 : A office application suite under Windows. *3 DEEP : Provides for adjust color, contrast and brightness of monitor. |
| Package Content |  | * Mainboard * Manual & CD Diskette * 1x FDD, 1xDMA66/100 Cables * AMR Fax/Modem Card |

# Setup Guide

# A. Layout Diagram

- JUSB1 : USB Connector

- U11 : Infrered Port (IrDA)

1 – VCC

2 – NC

3 – IRRx

4 – GND

5 - IRTx

## Jumper Settings

When put a jumper cap on two pins to short, we name it ON.  When remove jumper cap from two pins or just put on one pin, we name it OFF.

- CPU Setting : This mainboard support CPU Ratio and Clock automatic detect so that there is no jumper setting requirement.

- JP2 : Keyboard Power On Set Up

When enable Keyboard Power On Set Up, you are able to Power On/Off via hot keys setting. However, the power supply must provide minimum 750mA on the +5VSB (+5V Standby) for working normally.

| JP2 |  |
| --- | --- |
| Pin 1-2 On | Disable Keyboard Power On |
| Pin 2-3 On | Enable Keyboard Power On |

- JP4 : Clear CMOS Memory

| JP4 |  |
| --- | --- |
| Pin 1-2 On | Clear CMOS |
| Pin 2-3 On | Normal Situation |

- JP6 : Onboard LAN LED Header

| JP6 |  |
| --- | --- |
| Pin 1, 2 | Link LED |
| Pin 3, 4 | 10/100Mbps Mode LED |

EscoCAM – An Integrated Module

It’s so valuable that you don't have to pay thrice to buy three fashionable devices of PC Camera, Debugging and IrDA ware now because we have integrated them into our EscoCAM in the shape of a lovely whale to escort your system. The whale is the king of the sea and the symbol of all-mighty power. With PC camera, Debug, CPU heat monitor and IrDA device built into the EscoCAM, we offers you the unbeatable solution to upgrade the value of your PC system.

1. PC Camera:

As video and audio email is gaining popularity, PC cameras are making inroads into PC systems and it will increasingly become a standard PC peripheral. In view of this strong market demand, we has timely launched its state-of-the-art PC cameras. Besides, our PC Camera is allowed to adjust the Focus of Camera via Keyboard

or mouse instead of wheel.  It’s a patent design in PC Camera.  Hope you enjoy

this new technology.

2. External Debug :

When initializing system, LCD will check the peripherals of CPU, Chipset, DRAM, BIOS, Keyboard, CMOS, VGA card, Devices, FDD, HDD and Cache…Once peripherals were not been properly installed or defective, the LCD display will show out the detective place. After system successfully booted up, Time & Date, CPU Temperature, CPU V-Core Voltage, CPU Fan Speed will take turn to show out on the LCD display.

Advantages of Debug Device For User:

In comply with fast development of IT and e-Commerce, the time of people

to stay together with PC is beyond what we can image.  Eventually, the

high density use of PC is unavoidable to cause high defect rate.  Especially,

when warranty is over, user is not in a position to bargain the upraising repair cost. The worse is that users are always overcharged when they have don’t have any sense about defect status.  However, once PC system is equipped with basic debugging devices, user is able to easily understand how far the hardware problem going; thereby drastically minimize the time and cost from RMA.

3. Monitor CPU Temp. And FAN Working Status:

Audio, video and multi-tasking functions call for faster and faster CPUs to improve system efficiency.  However, faster CPUs will generate more heat which must be dissipated in a timely manner. Therefore, a more efficient fans to dissipate heat is as important as watch-out of fan working situation. In this Internet era, people prolonged use of their computers is easily to cause overheat and damage to the CPU. It is therefore necessary to have a device to constantly monitor the CPU temperature, CPU fan speed and V-Core voltage, and timely remind user.

4. IrDA For Data Transmission :

As more and more computers, notebook, mobile phone and PDA are installed

with the IrDA device for wireless transmission, we add in this extra value in the

EscoCAM.

The Connection of EscoCAM

### I. For TM-VAK7 Mainboard :

If you use TM-VAK7P, the most updated mainboard, it’s already built-in all the

circuit of EscoCAM so that user don’t need a extra daughter card to connect it.

The connection drawing as follows:

### II. For All Others Mainboards:

If the mainboard you used is not TM-VAK7, you need A PCI daughter card to

be inserted and make connection as follows:

III. Or, if you just want use the function of PC Camera, the connection

drawing as follows:

BIOS Update Note

- Do not update the BIOS if no abnormalities occur. However, if BIOS update is needed, consult your dealer first. Prior to updating your BIOS, you are recommended to save the original BIOS values.

- 1. Download the AMI BIOS Flash Utility file (AMIFL818.exe)

- 2. Download the BIOS file used by your mainboard(e.g., xxx.BIN)

- 3. Reboot your system (but do not run Himem.sys and Emm386.exe)

- to execute the new BIOS program.

- 4. Execute these commands: AMIFL818 xxx.BIN