# BIOS Setup

This Flash ROM BIOS has a built-in Setup program that allows users to modify the basic system configuration.  This information is stored in battery-backed RAM so that it retains the Setup information when the power is turned off.

## Getting Help

The online description of the highlighted setup function is displayed at the bottom of the screen.

## The Main Menu

Once you enter  BIOS CMOS Setup Utility, the Main Menu will appear on the Screen..  Use  arrow keys to select  the desired  items, press <Enter> to select or enter a submenu.

| CMOS Setup Utility – Copyright (C) 1984 – 2000 Award Software |  |
| --- | --- |
|  Standard CMOS Features  Advanced BIOS Features Advanced Chipset Features  Integrated Peripherals  Power Management Setup  PnP/PCI Configurations  PC Health Status |  Frequency/Voltage Control Load Fail-Safe Defaults Load Optimized Defaults Set Supervisor Password Save & Exit Setup Exit Without Saving |
| Esc : Quit ↑↓→←: Select Item F10 : Save & Exit Setup |  |
| Time, Date, Hard Disk Type... |  |

### Standard CMOS Setup

The items in Standard CMOS Setup Menu are divided into several categories.  Each category includes no, one or more than one setup items.  Use the arrow keys to highlight the item and then use the <PgUp> or <PgDn> keys to select the value you want in each item.

| CMOS Setup Utility – Copyright © 1984 – 2000 Award Software Standard CMOS Features |  |  |
| --- | --- | --- |
|  | Date ( mm: dd: yy ) : Tue, May 15, 2000 |  |
|  | Time ( hh: mm: ss ) : 14 : 01 : 42 |  |
|  | IDE Primary Master : Press Enter 4303 MB |  |
|  | IDE Primary Slave Press Enter None |  |
|  | IDE Secondary Master Press Enter None |  |
|  | IDE Secondary Slave Press Enter None |  |
|  | Drive A 1.44MB, 3.5” |  |
|  | Drive B None |  |
|  | Floppy 3 Mode Support Disabled |  |
|  | Video EGA/VGA |  |
|  | Halt On All Errors |  |

  : Move      Enter : Select             +/-/PU/PD:Value :           F10: Save      ESC: Exit      F1: General

Help       F5: Previous Values                  F6:Fail-Safe Defaults      F7: Optimized Defaults

| Date & Time | Use these items to set the system date and time |
| --- | --- |
| Pri Master Pri Slave Sec Master Sec Slave | If the hard disk Promary Master/Slave and Secondary Master/Slave are set to Auto, then the hard disk size and model will be auto-detcted. If the Auto setting fails to find a hard disk drive, set it to User, and then type in the hard disk . |
| Floppy Drive A Floppy Drive B | Use these items to set the size and capacity of the floppy diskette drive(s) installed in the system. |

### Advanced BIOS Features Setup

Use this page to set more advanced information about your system. Take some care with this page. Making changes can affect the operation of your computer.

| CMOS Setup Utility – Copyright © 1984-2000 Award Software Advanced BIOS Features |  |
| --- | --- |
| Trend ChipAwayVirus Enabled Y2K Monitor Disabled H/W Reset Function Disabled CPU Internal Cache Enabled External Cache Enabled CPU L2 Cache ECC Checking Enabled Quick Power On Self Test Enabled First Boot Device HDD-0 Second Boot Device Floppy Third Boot Device CD-ROM Boot Other Device Enabled Swap Floppy Drive Disabled Boot Up Floppy Seek Disabled Boot Up NumLock Status On Gate A20 Option Fast Typematic Rate Setting Disabled Typematic Rate (Chars/Sec) 6 Typematic Delay (Msec) 250 Security Option Setup |  |

  : Move     Enter : Select     +/-/PU/PD:Value :        F10: Save      ESC: Exit    F1: General

Help       F5: Previous Values         F6:Fail-Safe Defaults   F7: Optimized Defaults

| Trend ChipAway Virus | During and after the system boots up, any attempt to write to the boot sector or partition table of the hard disk drive will halt the system and an error message will appear. Keep in ming that this feature protects only the boot sector, not the entire hard drive. to Enable or Disable the built-in virus protection. |
| --- | --- |
| Y2K Monitor | When Enable this item, the system will monitor for errors generated by Y2K bug. |
| H/W Reset Function | Use this item to Disable or Enable the computer’s hardware reset button. |
| CPU Internal Cache | Use this item to control the status of CPU’s internal Cache. When this item is Enabled, the CPU performance will be increased. |
| CPU External Cache | Use this item to control the status of CPU’s external Cache. When this item is Enabled, the CPU performance will be increased. |
| ECC Checking | This item control if the CPU’s L2 Cache will support Error Checking and Correcting (ECC). When Enabled, the CPU performance will decrease 2%-4%. Enabled is recommended. |
| Quick Power On Self Test | When Enabled this item, system will shorten or skip of the items checked during POST and to speeds up the Power On Self Test. When you are sure that your system hardware is smooth and stable enough, we recommend to enable this item for speeding up POST. |
| 1st /2nd /3rd/ and Other Boot Device | The BIOS trys to load the operating system from the devices in the sequence of 1st, 2nd, 3rd and other devices. The choice : HDD, FDD, CD-ROM, LS120, SCSI. |
| Floppy Drive Swap | This item will swap physical drive A & B if you installed two floppy drive. Enabled: Floppy A & B will be swapped under the O/S. Disabled : will be not swapped. |
| Swap Floppy Drive | This will swap your physical drive letters A & B when you are using two floppy disks. The default is Disabled. |
| Boot Up Floppy Seek | During POST, BIOS will determine if the floppy disk drive installed is 40 or 80 tracks. Only 360K type is 40 tracks while 760K, 1.2MB and 1.44MB are all 80 tracks. The default is Disabled. |
| BootUp Num-Lock | This item control the NumLock key is active or inactive when system boots. On: The keypad Acts as a 10-key pad. Off: The keypad acts like the cursor keys. |
| Gate 20 Option | This refers to the way the system address memory above 1MB(extended memory). Default is Normal. Normal: The A20 signal is controlled by the keyboard controller or chipset hardware. Fast : The A20 signal is controller by Port 92 or chipset specific method. |
| Typematic Rate Setting | Use this item to determine to keystrokes repate rate. Default is Disabled. Enabled: Allows typematic rate and typematic Delay programming. Disabled: The typematic rate and delay will be Controlled by the keyboard controller. |
| Typematic Rate (Chars/Sec.) | The number of characters will be repeated by a keyboard press. The default is 6. 6: 10 characters per second. 24: 24 characters per second. |
| Typematic Delay (msec) | Setting controls the time between the first and the second character displayed by typematic auto-repeat. The default is 250. 250: 250 msec. 750: 750 msec. |
| Security Option | This allows you to limit access to the System and Setup, or just to Setup. The default is Setup. System: The system will not boot and the access To the Setup will be denied if entered password is Incorrect. Setup: The system will boot; but the access to Setup will be denied if entered password is incorrect. |
| OS Select for DRAM > 64MB | Use this option only if system has greater than 64MB. Default is Non-OS2. OS2: Select this if you are running the OS/2 operating system with greater than 64MB. |
| Video BIOS Shadow | This allows video BIOS to be copied into RAM. Video Shadowing will increase the video performance of system. Enabled is defaulted. |
| C8000 - CBFFF CC000 - CFFFF D0000 - 3DFFF D4000 - D7FFF D8000 - DBFFF DC000 - DFFFF Shadow: | When enabled this Optional Shadow, the ROM with the specified address is copied into system DRAM. It will also reduce the size of memory about 16K or 32K byte in the system. The size depends on the chipset of the optional card. |

### Advanced Chipset Features Setup

### This page sets some of the parameters of the mainboard components including memory and the system logic.

| CMOS Setup Utility – Copyright © 1984-2000 Award Software Advanced Chipset Features |  |
| --- | --- |
| Bank 0/1 DRAM Timing SDRAM 8/10 ns Bank 2/3 DRAM Timing SDRAM 8/10ns SDRAM 4/5 DRAM Timing SDRAM 8/10ns SDRAM Cycle Length Auto DRAM Clock Host CLK PCI Master Pipeline Req Enabled P2C/C2P Concurrency Enabled Fast R-W Turn Around Disabled System BIOS Cacheable Disabled Video RAM Cache able Disabled AGP Aperture Size 64MB AGP-4X Mode Enabled AGP Driving Control Auto AGP Driving Value DA AGP Fast Write Disabled K7 CLK_CTL Select Optimal OnChip USB Enabled OnChip USB 2 Disabled USB Keyboard Support Disabled USB Mouse Support Disabled |  |

  : Move     Enter : Select     +/-/PU/PD:Value :        F10: Save      ESC: Exit    F1: General

Help       F5: Previous Values         F6:Fail-Safe Defaults   F7: Optimized Defaults

| Bank 0/1 2/3 4/5 DRAM Timing | The value in this field is set by the system board manufacturer, depending on whether the board has paged DRAMs or EDO DRAMs. |
| --- | --- |
| SDRAM Cycle Length | This setting defines the CAS timing parameter of the SDRAM in terms of clocks. Default is 3. 2: Provides faster memory performance. 3: Provides better memory compatibility. |
| DRAM Clock | Enables the user to select the DRAM Clock. |
| PCI Master Pipeline Req | When enable this item, the system will allows PCI Master pipeline request. |
| P2C/C2P Cncurrency | When this item is disabled, the CPU bus is occupied during the entire PCI operation. en powered down. Default is Enabled. |
| Fast R-W Turn Around | When this item is enabled, the chipset will insert one extra clock to the turn-around of back-back DRAM cycles. |
| System BIOS Cacheable | This allows to copy BIOS code from slow ROM to fast RAM. Default is Enabled.???? Enabled : This option will improve system performance. However, if any program writes to this memory area, a system error may result. |
| Video RAM Cacheable | This allows to copy the video ROM BIOS to fast RAM (C000h to C7FFFh). Enabled : Enabled the Video BIOS Cacheable to speed up the VGA performance. |
| AGP Aperture Size | The amount of system memory that the AGP card is allowed to share. Default is 64MB. |
| AGP 4X Mode | Options : x1, x2, x4 Enable this item can greatly improve the display speed. However, if your graphic card does not support this feature, you need to disable. |
| AGP Driving Control | This item allows you to adjust the AGP driving force. If you select Manual, you can key in a AGP Driving Value in next item. However, we recommend to set in Auto for avoiding any error in your system. |
| AGP Driving Value | This item allows to set the AGP current driving value. |
| K7 CLK_CTL Select | Use this item to specify the clock control for ramp rate. Select Default for a defaulted time value, and optimal for optimum time value which Depends on different CPU ratio. |
| OnChip USB/USB2 | USB Connector (Port 0-1)/(Port 2-3) |
| USB Keyboard Support | Enables this item, when USB keyboard is being Used. Disabled it, when AT keyboard is used. |
| USB Mouse Support | Enables this item, when USB Mouse is being used. Disabled it, when other Mouses are used. |

### Integrated Peripherals Setup

This page sets some of the parameters for peripheral devices connected to the

system.

| CMOS Setup Utility – Copyright © 1984-2000 Award Software Integrated Peripherals |  |
| --- | --- |
| On-Chip IDE Channel 0 Enabled On-Chip IDE Channel 1 Enabled IDE Prefetch Mode Enabled Primary Master PIO Auto Primary Slave PIO Auto Secondary Master PIO Auto Secondary Slave PIO Auto Primary Master UDMA Auto Primary Slave UDMA Auto Secondary Master UDMA Auto Secondary Slave UDMA Auto Init Display First PCI Slot Onboard FDD Controller Enabled Onboard Serial Port 1 3F8/IRQ4 Onboard Serial Port 2 2F8/IRQ3 UART 2 Mode Standard IR Function Duplex Half TX,RX inverting enable No, Yes Onboard Parallel Port 378/IRQ7 |  |

  : Move     Enter : Select     +/-/PU/PD:Value :        F10: Save      ESC: Exit    F1: General

Help       F5: Previous Values         F6:Fail-Safe Defaults   F7: Optimized Defaults

| On-Chip IDE Channel 0, 1 | Channel 0 is to enable On Chip IDE Primary port. Channel 1 is to enable On Chip IDE Secondary Port. |
| --- | --- |
| IDE Prefetch Mode | Enable prefetching for IDE drive interfaces that support its faster access. If you are getting disk drive errors, change the setting to remove the drive interface where the error occur. The error will be appear when the Internal PCI/IDE fiel is Disabled. |
| Primary/ Secondary Master/Slave PIO | This item allows you to assign which kind of PIO is used by IDE devices. Default is Auto. Auto: BIOS will automatically detect the Onboard PCI IDE Accessing mode. Mode 0-4: Manually set the IDE Programmed interrupt mode. |
| Primary/ Secondary Master/Slave UDMA | This item allows you to set Ultra DMA for faster access to IDE HDD which support Ultra DMA function. You may need to install UDMA driver. Default is Auto. Auto: The system will select the optimal setting. Disabled: The HDD will run in normal mode. |
| Init Display First | If two graphic devices used (1xAGP, 1x PCI), this specifies which one will be the primary display. The default is PCI Slot. PCI Slots : PCI graphic will be Primary. AGP : AGP graphic will be Primary. |
| IDE HDD Block Mode | This item allows IDE controller to access blocks of sectors rather than a single sector at a time providing higher HDD transfer rates. Default is Enabled. |
| Onboard FDD Controller | This item controls the state of Onboard floppy controller. Default is Enabled. Enabled: Enable Onboard VIA686A Chip’s FDD interface controller. |
| OnBoard Serial Port 1, 2 | This items allow user to configure the 1st and 2nd serial ports. Default is Auto. Atuo: Enable Onboard Serial Port 1 and address is auto adjusted. COM1: Enable Onboard Serial Port 1 or 2 and address is 3F8H/IRQ4. COM2: Enable Onboard Serial Port 1 or 2 and address is 2F8H/IRQ3. COM3: Enable Onboard Serial Port 1 or 2 and address is 3E8H/IRQ4. COM4: Enable Onboard Serial Port 1 or2 and address si 2E8H/IRQ3. |
| UATR2 Mode | This field is available only if Onboard Serial Port 2 is Disabled. UART mode enables you to select the Infrared protocol – Standard (default), HPSIR or ASKIR. HPSIR is HP’s IR protocol with a maximum baud rate up to 115.2Kbps. ASKIR is Sharp’s IR protocol with a maximum baud rate up to 57.6Kbps. |
| IR Function Duplex | This field is only available when UART2 is set to ASKIR or HPSIR. Full-Duplex means that you can transmit and receive data simultaneously. Half Duplex transmitting data in both direction but only works one direction at a time. |
| TX, RX Inverting enable | Defines the voltage level in accordance to the IR module RxD(receive) and TxD(transmit) mode. |
| OnBoard Parallel Port | This item allows user to configure the LPT port. Default is 378H/IRQ7. 378H: Enable Onboard LPT and address is 378H/IRQ7. 278H: Enable Onboard LPT and address 278H/IRQ5. 3BCH: Enable Onboard LPT and address is 3BCH/IRQ7. Disabled: Disable Onboard I/O chip’s LPT port. |
| Onboard Parallel Port Mode | Use this item to set the parallel port mode. You can select SPP (Standard Parallel Port), ECP (Extended Capabilities Port), EPP (Enhanced Parallel Port), or ECP + EPP. Default is Normal. Normal: Standard mode. IBM PC/AT Compatible bidirectional parallel port. |
| ECP Mode Use DMA | This allows the user to select DAM1 or DMA3 for the ECP mode. Default is DMA3. |
| Parallel Port EPP Type | This allows you to dertermine the IR transfer mode of onboard I/O chip. The Choice is : EPP 1.9, EPP 1.7. |
| OnBoard Legacy Audio | When Enabled legacy audio, following setup items for SB become available. |
| Sound Blaster | Enable Sound Blaster compatible device is usually for DOS environment. |
| SB I/O Base Address | Sound Blaster I/O selection. |
| SB IRQ Select | Legacy audio device IRQ selection |
| SB DMA Select | Sound Blaster DMA channel selection. |
| MPU-401, MPU-401 I/O address | Use this two items to enable MPU-401 function and set the I/O address for the game port. |
| Game Port (200-207H) | Built-in joystick port . Default is Enabled. |

### Power Management Setup

This page sets some of the parameters for system power management operation.

CMOS Setup Utility – Copyright © 1984-2000 Award Software Power Management Setup

Power Management Press Enter ACPI Suspend Type S1(POS) PM Control by APM Yes Video Off Option Suspend  Off Video Off Method DPMS Support MODEM use IRQ 3 Soft-Off by PWRBTN Delay 4 Sec State After Power Failure Off Wake Up Events Press Enter

  : Move     Enter : Select     +/-/PU/PD:Value :        F10: Save      ESC: Exit    F1: General

Help       F5: Previous Values         F6:Fail-Safe Defaults   F7: Optimized Defaults

| Power Management/APM | This field allows you to select your Power Management selection. Default is User Define. Disabled : The system operates in NORMAL conditions. Min. saving : Minimum power savings. Inactivity period is 1 minute in each mode. Max. saving : Maximum power savings. Inactivity period is 1 hour in each mode. |
| --- | --- |
| ACPI Function | This field allows you to select ACPI function. Enabled : Support ACPI function for new O.S. Disabled: No support ACPI function. You can only change the content of Doze, Standby and Suspend Mode when the PM is set to “User Define”. User Define : Allows user to define PM Timers parameters to control power saving mode. |
| ACPI Suspend Type | This item allows to select S1(POS), S3(STR).nutes. If the setup time passes without any system activity, the computer will enter into power-saving.Suspend mode. |
| PM controlled by APM | This option shows weather or not you want the Power Management to be controlled by APM (Advanced Power Management). Default is Yes. |
| Video Off Option | This tells you what time frame that the video will be disabled under current PM setting. The default is Standby. Standby: Video off after time shown in standby mode setting. Doze : Video off after time shown in doze mode setting. Suspend : Video off after time shown in suspend mode setting. N/A: Video off not controlled by PM. |
| Video Off Method | This item shows how the video will be disabled by PM. The default is V/H Sync + Blank. V/H Sync + Blank : System turns off vertical and horizontal synchroniztion ports and writes blanks to the video buffer. DPMS : Select this option if your monitor supports the Display Power Managemetn Signaling (DPMS). Use the software supplied for your video subsystem to select video PM value. Blank Screen : System only writes blanks to the video buffer. |
| MODEM Use IRQ | Use this item to specify the IRQ used by the Modem to awaken the system upon the incoming call on a Modem. Default is IRQ3. |
| Soft-Off By PWRBTN | Use this item to select soft-off function. Default is Delay 4 sec. Instant Off : Turns off the system instantly. Delay 4 Second : Turns off the system after 4 seconds delay (by holding the power button down for 4 seconds). If momentary press of button, the system will go into Suspend Mode. Press the power button again to take system out of Suspend Mode. |
| State After Power Failure | This item allows you to determine the state that system returns after a power failure. If set to Off, the system won’t boot after a power failure, if set to On, the system will restart after a power failure. |
| Wake Up Events | This item opens a submenu enables you to set events that will resume the system from a power saving mode. Press Enter for the availability of the following items: VGA : When set to On (default), any event occurring at a VGA port will awaken a system which has been powered down. LPT & COM, HDD & FDD, PCI Card, Modem Ring Resume & Primary INTR are the same in operation. RTC Alarm Resume: When enable RTC Alarm, you could set the date (of month) and time (hh:mm:ss), and event occurring at will awaken a system which has been powered down. |

### PCI / Plug and Play Setup

This page sets some of the parameters for devices installed on the PCI bus and devices that use the system plug and play capability.

CMOS Setup Utility – Copyright © 1984-2000 Award Software PnP/PCI Configurations

PNP OS Installed Yes Reset Configuration Data Disabled Resources Controlled by Auto (ESCD) IRQ Resources Press Enter DMA Resources Press Enter PCI/VGA Paletter Snoop Disabled Assign IRQ for VGA Enabled Assign IRQ for USB Enabled INT Pin 1 Assignment Auto INT Pin 2 Assignment Auto INT Pin 3 Assignment Auto INT Pin 4 Assignment Auto

  : Move     Enter : Select     +/-/PU/PD:Value :        F10: Save      ESC: Exit      F1: General

Help       F5: Previous Values         F6:Fail-Safe Defaults   F7: Optimized Defaults

| PnP OS Installed | Do you have a PnP OS installed on your system. The default is Yes and this will allows the PnP OS(instead of BIOS) to assign the system resources such as IRQ and I/O address to the ISA PnP device. |
| --- | --- |
| Reset Configuration Data | This item allows you to clear ESCD (Extended System Configuration Data). Default is Disabled. If you have plugged in some Legacy cards to the system and they were recorded into ESCD, you can set here Enabled in order to clear ESCD. |
| Resources Controlled By | Who Controlled the system PNP/PCI resources. The default is Auto. Manual : PnP card’s resources will be controlled manually. You can set which IRQ-X and DMA-X are assigned to PCI/ISA or Legacy ISA cards. Auto: If you ISA card and PCI card are all PnP cards, BIOS will assign the interrupt resource automatically. |
| PCI/VGA Palette Snoop: | This item is set for some non-standard VGA cards. However most of cards include this built-in VGA system do not require palette Snoop so that leave it disabled. |
| Assign IRQ for VGA/USB | This allows BIOS to assign whether IRQ is with VGA/USB or not. If don’t need to connect VGA/USB device, can leave the IRQ for other device. |
| INT Pin 1,2,3,4 Assignment | This field allows user to specify what IRQ will be assigned to PCI devices in the chosen slot. Options available : Auto 3, 4,5,7,9,10,11,12,14 & 15. The default is Auto. |

### Hardware Monitor

This page sets some of the parameters for the monitoring function of this mainboard.

CMOS Setup Utility – Copyright © 1984-2000 Award Software PC Health Status

Shutdown Temperature Disabled Current CPU Temp. Current System Temp. Current CPUFAN1 Speed Current CPUFAN2 Speed Vcore 3.3V 5V 12V

  : Move     Enter : Select     +/-/PU/PD:Value :        F10: Save      ESC: Exit    F1: General

Help       F5: Previous Values         F6:Fail-Safe Defaults   F7: Optimized Defaults

| Shutdown Temperature | This allows you to set the acceptable temperature point. |
| --- | --- |
| Current CPU Temp. | This is the current temperature of CPU. |
| Current System Temp. | This is the current temperature of system. |
| Current CPU FAN Speed | The current CPU fan speed in RPMs. |
| Current Chassis FAN Speed | The current chassis fan speed in RPMs. |
| Vcore | The voltage of the CPU (Vcore) |
| 3.3V | Onboard voltage |
| 5V | Power Supply 5 V |
| 12V | Power Supply 12 V |

### CPU Frequency/Voltage Control

CMOS setup Utility – Copyright © 1984 – 2000 Award Software Frequency/Voltage Control

Auto Detect DIMM/PCI Clk Enabled CPU Host/PCI Clock/Spread Spec. Default CPU Type AMD K7 CPU Speed 650MHz CPU Core Voltage 1.5V CPU Ratio 6.5 CPU Frequency 100 MHz

  : Move     Enter : Select     +/-/PU/PD:Value :        F10: Save      ESC: Exit    F1: General

Help       F5: Previous Values         F6:Fail-Safe Defaults   F7: Optimized Defaults

| Auto Detct DIMM/PCI Clk | When Enabled, the mainboard will automatically disable the clock source for a DIMM socket whick does not have a module in it. The default is Enabled. |
| --- | --- |
| CPU Host/PCI/Spread Spec. | This allows the external clock to be modifed depending upon what FSB has been selected. Should not be used to clock processor faster than it was designed for. Default is default. |
| CPU Ratio, Frequency. | Use this item to set a multiplier for the CPU external frequency. The multiplier times the external CPU frequency sets the internal clock speed of the CPU, e.g. 100 MHz (external clock or “FSB”) x 65 (muliplier) = 60 MHz (internal clock speed of the installed CPU).This show the PU speed |

### Load Best Performance Settings

When you press <Enter> on this item, a dialog box appears as below:

Load Best Performance (Y/N) ? N

Pressing ‘Y’ loads the BIOS default values for the most stable, minimal-

performance system operations.

Load Optimized Defaults

When you press  <Enter> on this item you get a confirmation dialog box

with a message likely to :

Load Optimized Defaults (Y/N) ? N

Pressing  ‘Y’ loads the default values that are factory settings for optimal perfor-

mance  system operation.

### Change Password

Type the password, up to 8 characters in length, and press <Enter>.  The password typed now will clear any previously entered password from CMOS memory.  You will memory. You will be asked to confirm the password once again.  Now, type the password again and press <Enter>.  You may also press <ESC> to abort the password set up.  Once the password is disabled, the system will boot and you can enter Setup freely. If you highlight this item and press Enter, a dialog box appears which lets you enter a Supervisor password. You can enter no more than 8 letters or numbers. Press <Enter> after you have typed in the password. A second dialog box asks you to retype the password for confirmation. Press <Enter> after you have retyped it correctly. The password is then required to access the Setup Utility or for that and at start-up, depending on the setting of the Password Check item in Advanced Setup.

### Change or Remove the Password

For Changing password, you need to press <Enter> and enter the current password. At the next dialog box, type in the new password.   To disable a password, just press <Enter> when you are requested to entered the password .  A message to confirm the password will be disabled.  Once the password is disabled, the system will boot and you can enter setup freely.

### Exit Selecting

Save & Exit Setup

Pressing <Enter> on this item asks for confirmation:

Save to CMOS and EXIT (Y/N)?  Y

Pressing ‘Y’ stores the selections made in the menus in CMOS – a special section

of memory that stays on after you turn your system off.  The next time you boot your

computer, the BIOS configures your system according to the Setup selection stored

in CMOS.  After saving the values the system is restarted again.

Exit Without Saving

Pressing <Enter> on this item asks for confirmation:

Quit without saving (Y/N)?  Y

This allows you to exit Setup without storing in CMOS any change. The previous selection remain in effect.  This exits the Setup utility and restarts your computer.