## F.  BIOS Update Note

- Do not update the BIOS if no abnormalities occur. However, if BIOS update is needed, consult your dealer first. Prior to updating your BIOS, you are recommended to save the original BIOS values.

- 1. Download the AWARD BIOS Flash Utility file (Awdflash.exe)

- 2. Download the BIOS file used by your mainboard(e.g., BXV110N.BIN)

- 3. Reboot your system (but do not run Himem.sys and Emm386.exe) to

- execute the new BIOS program.

- 4. Execute these commands: Awdflash BXV110N.BIN

- 5. When this message displays: "Do you want to save BIOS (Y/N)?"

- Type "N"

- 6. When this message displays: "Are you sure to program (Y/N)?"

- Type "Y"

- 7. Turn off power to your system to clear the CMOS data.

- 8. Turn on the power to test if your system is running normal.

## G. Keyboard/ PS/2 Mouse Power On and MODEM Ring on

- To make sure the 5VSB signal nearly to 750mA (Amperage) from ATX Power Supply, or if your keyboard consuming power than 300mA, it's better to upgrade your ATX Power Supply to 1A for working perfectly.

- If you are going to use the function of keyboard and PS/2 mouse power on, then, the power-switch will be becoming useless automatically (unable to be used).

| ROM PC/ISA BIOS (2A6LGTJE) INTEGRATED PERIPHERALS AWARD SOFTWARE, INC. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| IDE HDD Block Mode |  | : Enabled |  | Onboard Serial Port 2 | : 2F8H / IRQ3 |  |
| IDE Primary Master PIO |  | : AUTO |  | UART Mode Select | : Normal |  |
| IDE Primary Slave PIO |  | : AUTO |  |  |  |  |
| IDE Secondary Master PIO |  | : AUTO |  | Onboard Parallel Port | : 378H/IRQ 7 |  |
| IDE Secondary Slave PIO |  | : AUTO |  | Parallel Port Mode | : ECP+EPP |  |
| IDE Primary Master UDMA |  | : AUTO |  | ECP Mode Use DMA | : 3 |  |
| IDE Primary Slave UDMA |  | : AUTO |  | EPP Mode Select | : EPP 1.9 |  |
| IDE Secondary Master UDMA |  | : AUTO |  |  |  |  |
| IDE Secondary Slave UDMA |  | : AUTO |  |  |  |  |
| On-Chip Primary PCI IDE |  | : Enabled |  |  |  |  |
| On-Chip Secondary PCI IDE |  | : Enabled |  |  |  |  |
| USB Keyboard Support |  | : Disabled |  |  |  |  |
| Init AGP Display First |  | : Enabled |  |  |  |  |
| POWER ON Function |  | : Hot KEY |  |  |  |  |
|  |  |  |  | Esc : Quit Selection : Item |  |  |
| Hot Key Power ON |  | : Ctrl-F12 |  | F1 : Help PU/PD/+/- : Modify |  |  |
| KBC input clock |  | : 8MHz |  | F5 : Old Values (Shift)F2 : Color |  |  |
| Onboard FDC Controller |  | : Enabled |  | F6 : Load BIOS Default |  |  |
| Onboard Serial Port 1 | : 3F8H / IRQ4 |  | F7 : Load Setup Default |  |  |  |

| Hot KEY | When user select this option, it will show another line lines as Hot Key Power ON: Ctrl-F(1/2/3/4/5/6/7/8/9/10/11/12) select any you like. After power off, if user key in the "Ctrl-F?", it will power on the system. |
| --- | --- |
| PS/2 Mouse Left | It will power on the system by PS/2 mouse left. |
| PS/2 Mouse Right | It will power on the system by PS/2 mouse Right. |
| Button Only | Only the power button can power on the system. |

- Modem Ring On Function Operation:

| ROM PCI / ISA BIOS (2A6LGTJE) POWER MANAGEMENT SETUP AWARD SOFTWARE, INC |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Power Management | : Disabled | ** Reload Global Timer Events ** |  |  |  |  |  |
| PM Control by APM | : No | IRQ[3-7, 9-15], NMI | : Enabled |  |  |  |  |
| Video Off Method | : V/H SYNC+Blank | Primary IDE 0 | : Disabled |  |  |  |  |
| Video Off After | : Standby | Primary IDE 1 | : Disabled |  |  |  |  |
| MODEM Use IRQ | : 3 | Secondary IDE 0 | : Disabled |  |  |  |  |
| Doze Mode | : Disabled | Secondary IDE 1 | : Disabled |  |  |  |  |
| Standby Mode | : Disabled | Floppy Disk | : Disabled |  |  |  |  |
| Suspend Mode | : Disabled | Serial Port | : Enabled |  |  |  |  |
| HDD Power Down | : Disabled | Parallel Port | : Disabled |  |  |  |  |
| Throttle Duty Cycle | : 62.5% |  |  |  |  |  |  |
| ZZ Active in Suspend | : Disabled |  |  |  |  |  |  |
| VGA Active Monitor | : Enabled |  |  |  |  |  |  |
| Soft-Off by PWR-BTTN | : Instant-Off |  |  |  |  |  |  |
| CPUFAN Off In Suspend | : Enabled |  |  |  |  |  |  |
| Resume by Ring | : Enabled |  |  |  |  |  |  |
| IRQ 8 Clock Event | : Disabled | Esc : Quit Selection Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Default F7 : Load Setup Default |  |  |  |  |  |

- 1. Have an external MODEM connected to COM 1 or COM 2.

- 2. Enter BIOS setup.

- 3. Select Power Management Setup.

- 4. This number of MODEM use IRQ has to be set as same as the IRQ of

- Serial Port which you are connecting in. Please set in N/A if you are

- not going to use the function of MODEM ring on.

- 5. Resume by Ring: Enable.

- 6. Save BIOS setup and Reboot.

- 7. Booting from DOS, Windows, or Windows 95.

- 8. Turn off the system by:

- a. ATX-Power Switch

- b. Windows 95 Software Power Off

- 9. System Waiting for Modem Ring On

- When Modem Ringing Signal Active, System will wake-up.

## System Health Monitor

- Fan Monitoring:

There are two fan connectors, one is for CPU, the other can be a housing fan. When the fans speed is working abnormal, there will be warning (Speaker Alarm) through application software such as SM10(Small Icon for System Monitoring) to notify user. The fan monitoring function is implemented by connecting fan to 3-pin fan connector FAN1/ FAN2 and installing SM10.

- CPU Thermal Protection:

TM-P2BX370 implements special thermal protection circuits. When temperature is higher than a predefined value, there will be warning (Speaker Alarm) through application software such as SM10 (Small Icon for System Monitor) to notify user. It's automatically implemented by BIOS or SMD10, no hardware installation is needed. Referring to Page19 (System Health Monitor).

This mainboard also reserves an option to use special CPU cooling Fan. With Thermal Sensor on it. The CPU thermal sensor should be connected to J16.

- System Voltage Monitoring:

This board is featured with a voltage monitoring system. When you turn on your system, this smart design will keep on monitoring your system working voltage. If any of voltage is over the component's standard, there will be Speaker Alarm though application software SM10 (Small Icon For System Monitor) for a warning to user. System voltage monitoring function monitors 5V, 12V, 3.3V and CPU voltage. It's automatically implemented by BIOS and SM10, no hardware installation is needed. Referring to Page 19 (System Health Monitor)