TM-VAK7PL

PCI/ AGP/AMR Mainboard

Version: 1.0

Warning: Never run the processor without the heatsink properly

and firmly installed. Permanent Damage Will result!

# Copyright

Copyright © 2000 by this manufacturer. All rights reserved. No part of this publication may be reproduced, transmitted, transcribed, stored in a retrieval system, or translated into any language or computer language, in any form or by any means, electronic, mechanical, magnetic, optical, manual or otherwise, without the prior written consent of this manufacturer.

# Disclaimer

This manufacturer makes no representations or warranties, either expressed or implied, with respect to the contents hereof and specifically disclaims any warranties, merchantability or fitness for any particular purpose. Any software described in this manual is sold or licensed “as is”. Should the programs prove defective following their purchase, the buyer (and not this manufacturer, its distributor, or its dealer) assumes the entire cost of all necessary servicing, repair, and any incidental or consequential damages resulting from any defect in the software. Further these manufacturer reserves the right to revise this publication and to make changes from time to time in the contents hereof without obligation to notify any person of such revision or changes.

Trademark Acknowledgement

AMD Duron and Athlon are registered trademarks of AMD. Award is a registered trademark of Award Software Inc. Other brand and product names are trademarks and/or registered trademarks of their respective holders.

Printed in Taiwan

# Overview

The TM-VAK7PLis a AMD Athlon based mainboard that utilizes VIA KT133 + 686A chipset, a high level of integrated function. The AMD Athlon is a seventh-generation micro architecture with an integrated L2 Cache, which is powerful enough to support bandwidth requirements of a large range of applications, hardware, graphics and memory technologies.

Socket A is the name for AMD’s new socket interface designed to support both AMD Duron and AMD Athlon processors.  This innovation is made possible by integrating the L2 cache memory on chip with the processor. Socket A will help enable smaller enclosures and ultimately result in a wider variety of solutions in the market.

The AMD Duron processor is driven from the AMD Athlon processor core. It features full-speed, on-chip cache memory, a 200 MHz front size side system bus, and enhanced 3Dnow! Technology.  Although both processors are related, there are key differences.  The AMD Athlon processor is targeted at the performance segment, and as such will have more cache memory and higher clock speeds.

# CONTENTS

Copyright	2

Disclaimer	2

Overview	3

CONTENTS	4

Introduction	6

Mainboard Layout……………………………………………….8

Jumper Settings………………………………………………..10

EscoCAM, An Integrated Module For Escorting PC	11

Connection Guide Of EscoCAM..……………………………13

BIOS Update Note	16

Hardware Monitor …………………………………………… 17

AWARD BIOS Setup	18

Standard CMOS Setup	19

BIOS Features Setup	20

Chipset Features Setup	24

Integrated Peripherals Setup	26

Power Management	30

PNP/PCI Configuration Setup	34

Hardware Monitor 	36

Frequency / Voltage Control 	37

Load Setup Default	38

Supervisor Password 	39

Exiting the Setup Program	40

Driver & Software…..………………………………...……..41

System Block Diagram 	46

POST CODE..……………………………….……………….…... 47

PROBLEM SHEET………………………………………………..55

MEMO 	56

# Introduction

## Specifications

| System Chipset | VIA KT133 +686A chipset. |  |
| --- | --- | --- |
| CPU | Supports Socket A AMD Duron and Athlon processors at 500 MHz to 1.13GHz. |  |
| Memory | 2 x DIMM supports up to 1024MB of PC133/ VC133 memory (minimum 32MB) on board. |  |
| Sound On board |  | *AC97 Codec Compliant PC97 2.1 specification. *Supports 18-bit ADC and DAC as well as 18-bit stereo full-duplex codec.. |
| LAN On Board |  | Built-In 10 BaseT/100 BaseTX Ethrenet LAN Complaint with ACPI 1.0 and the Network Device Class Power Management 1.0. |
| Hardware Monitor | Monitoring CPU & System Temp., Fan speeds and mainboard voltages, and music alarm when they are over the set point. |  |
| I/O | *2 x high speed 16550 compatible serial ports, one Multi-Mode. *1 x Parallel Port support SPP/EPP/ECP standard mode. *2 x Onboard PCI IDE Port and support Ultra ATA 66. * LS-120/ ZIP FDD, IrDA/ ASK IR/ Consumer IR. *2 x USB ports, 2 x USB as optional. *2 x 360/720KB/1.2/1.44/2.88MB floppy disk. *1 x PS/2 Mouse port and 1x PS/2 Keyboard Port. |  |
| BIOS | AMI System BIOS installed in socket (Flash and PnP). |  |
| Expansion slots |  | 1 x AGP slot, 2x PCI Master Slots and 1x AMR |
| Voltage |  | Auto |
| Dimension |  | 4-layer PCB, size (24.5mm x 22mm). |
| Other Functions |  | Modem Ring On, LAN Wake Up. |
| Bundled Software |  | *Super VB : Provides anti-virus protection. *Super Voice : For data, FAX and voice communication. *Gamut 2000: Provides professional audio features. *Media Ring Talk : Provides PC to PC, or PC to phone internet communication. *Corel WordPerfect Suite 8 : A office application for Windows. *3 DEEP : Provides for adjust color, contrast and brightness of monitor. |
| Package Contents |  | *The mainboard *The Manual *CD Driver *ATA 66 & FDD cables |

# Mainboard Layout

- USB2 : USB Connector

- SIR1 : Infrered Port (IrDA)

1 – VCC

2 – NC

3 – IRRx

4 – GND

5 - IRTx

## Jumper Settings

When put a jumper cap on two pins to short, we name it ON.  When remove jumper cap from two pins or just put on one pin, we name it OFF.

- JP1: Clear CMOS

| 1-2 On | Normal Operation (Default) |
| --- | --- |
| 2-3 On | For Clearing CMOS Data |

- JP2 : USB Port 1 & 2 Wake Up

| 1-2 On | Disable |  |
| --- | --- | --- |
| 2-3 On |  | Enable |

- JP4: Codec & AMR Option Jumper

| 1-2 On | Enable On Board Codec |
| --- | --- |
| 2-3 On | Enable AMR Slot |

- JP5 : BIOS Write Protection Jumper

Enable this jumper to read BIOS only.

| 1-2 On | Disable |
| --- | --- |
| 2-3 On | Enable (read Only) |

- JP6 : USB Port 3-4 Wake Up

Enable device on USB ports 3-4 to power on PC.

| 1-2 On | Disable |
| --- | --- |
| 2-3 On | Enable |

- JP7 : Define On Board Codec Mode

| 1-2 Off | Master |
| --- | --- |
| 1-2 On | Slave |

EscoCAM – An Integrated Module

It’s so valuable that you don't have to pay thrice to buy three fashionable devices of PC Camera, Debugging and IrDA ware now because we have integrated them into our EscoCAM in the shape of a lovely whale to escort your system. The whale is the king of the sea and the symbol of all-mighty power. With PC camera, Debug, CPU heat monitor and IrDA device built into the EscoCAM, we offers you the unbeatable solution to upgrade the value of your PC system.

1. PC Camera:

As video and audio email is gaining popularity, PC cameras are making inroads into PC systems and it will increasingly become a standard PC peripheral. In view of this strong market demand, we has timely launched its state-of-the-art PC cameras. Besides, our PC Camera is allowed to adjust the Focus of Camera via Keyboard

or mouse instead of wheel.  It’s a patent design in PC Camera.  Hope you enjoy

this new technology.

2. External Debug :

When initializing system, LCD will check the peripherals of CPU, Chipset, DRAM, BIOS, Keyboard, CMOS, VGA card, Devices, FDD, HDD and Cache…Once peripherals were not been properly installed or defective, the LCD display will show out the detective place. After system successfully booted up, Time & Date, CPU Temperature, CPU V-Core Voltage, CPU Fan Speed will take turn to show out on the LCD display.

Advantages of Debug Device For User:

In comply with fast development of IT and e-Commerce, the time of people

to stay together with PC will be beyond what we can image.  Eventually, the

high density use of PC is unavoidable to cause high defect rate.  Especially,

when warranty is over, user will not be in a position to bargain the upraising repair cost. The worse is that users are always overcharged when they don’t have any sense about defect status. However, once PC system is equipped with basic debugging devices, user is able to easily understand how far the hardware problem going; thereby drastically minimize the time and cost from RMA.

3. Monitor CPU Temp. And FAN Working Status:

Audio, video and multi-tasking functions call for faster and faster CPUs to improve system efficiency.  However, faster CPUs will generate more heat which must be dissipated in a timely manner. Therefore, a more efficient fans to dissipate heat is as important as watch-out of fan working situation. In this Internet era, people are using their computers for longer and longer time which can easily cause overheat and damage to the CPU. Prolonged use of the computer will increase CPU temperature.

It is therefore necessary to have a device to constantly monitor the CPU temperature, CPU fan speed and V-Core voltage and timely remind user.

4. IrDA For Data Transmission :

As more and more computers, notebook, mobile phone and PDA are installed

with the IrDA device for wireless transmission, we add in this extra value in the

EscoCAM.

The Connection of EscoCAM

### I. For TM-VAK7 Mainboard :

If you use TM-VAK7P, the most updated mainboard, it’s already built-in all the

circuit of EscoCAM so that user don’t need a extra daughter card to connect it.

The connection drawing as follows:

### II. For All Others Mainboards:

If the mainboard you used is not TM-VAK7, you need A PCI daughter card to

be inserted and make connection as follows:

III. Or, if you just want use the function of PC Camera, the connection

drawing as follows:

BIOS Update Note

- Do not update the BIOS if no abnormalities occur. However, if BIOS update is needed, consult your dealer first. Prior to updating your BIOS, you are recommended to save the original BIOS values.

- 1. Download the AWARD BIOS Flash Utility file (Awdflash.exe)

- 2. Download the BIOS file used by your mainboard(e.g.VAK7PL11.BIN)

- 3. Reboot your system (but do not run Himem.sys and Emm386.exe)

- to execute the new BIOS program.

- 4. Execute these commands: Awdflash VAK7PL11.BIN

- 5. When this message displays: "Do you want to save BIOS (Y/N)?"

- Type "N"

- 6. When this message displays: "Are you sure to program (Y/N)?"

- Type "Y"

Hardware Monitor

- Hardware Monitoring: Installed VIAhm.exe in the start of system

to enable this function.

- CPU Temperature : Display Current CPU temperature and will alarm when CPU temperature is higher than the set point.

- System Temperature : Display Current System temperature and will alarm when System temperature is higher than the set point.

- CPU Fan Speed : Display Current CPU Fan speed and will alarm when Fan speed is lower than the set point

- Chassis Fan Speed : Display Current Chassis Fan speed and will alarm when Chassis Fan speed is lower than the set point.

- CPU voltage level of the CPU V-Core : Display 3.3V, 5V, 12V.