# BIOS Setup

Award BIOS ROM has a built-in Setup program that allows users to modify the basic system configuration.  This information is stored in battery-backed RAM so that it retains the Setup information when the power is turned off.

Entering Setup

To enter the BIOS Setup, press <Del> during POST (Power-On-Self-Test).

Control Keys

Up Arrow 		Move to previous item

Down Arrow		Move to next item

Left Arrow		Move to the left item

Right Arrow		Move to the right item

Esc Key		Main Menu -- Quit and not to save changes to CMOS

Status Page setup menu and Option Page

Setup Menu -- Exit current page and return to Main Menu

PgUp Key		Increase the numeric value or make changes

PgDn Key		Decrease the numeric value or make changes

F1 Key	General help, only for Status Page Setup Menu and Option Setup Menu

F2 Key		Change color from total 16 colors

F3 Key		Calendar, only for Status Page Setup Menu

F4 Key		Reserved

F5 Key	Restore the previous CMOS value from BIOS, only for Option Page Setup Menu

F6 Key	Load the default CMOS value from BIOS default table, only for Option Page Setup Menu

F7 Key		Load the default

F8 Key 		Reserved

F9 Key		Reserved

F10 Key		Save all the CMOS changes, only for Main Menu

## Getting Help

Main Menu

The online description of the highlighted setup function is displayed at the bottom of the screen.

Status Page Setup Menu/Option Page Setup Menu

Press F1 to pop up a small help window that describes the appropriate keys to use and the possible selections for highlighted item.  To exit the Help Window press <Esc>.

## The Main Menu

Once you enter Award BIOS CMOS Setup Utility, the Main Menu will appear on the Screen..  Use  arrow keys to select  the desired  items, press <Enter> to select or enter a submenu.

| CMOS Setup Utility – Copyright (C) 1984 – 2000 Award Software |  |
| --- | --- |
|  Standard CMOS Features  Advanced BIOS Features Advanced Chipset Features  Integrated Peripherals  Power Management Setup  PnP/PCI Configurations  PC Health Status |  Frequency/Voltage Control Load Fail-Safe Defaults Load Optimized Defaults Set Supervisor Password Save & Exit Setup Exit Without Saving |
| Esc : Quit ↑↓→←: Select Item F10 : Save & Exit Setup |  |
| Time, Date, Hard Disk Type... |  |

### Standard CMOS Setup

The items in Standard CMOS Setup Menu are divided into several categories.  Each category includes no, one or more than one setup items.  Use the arrow keys to highlight the item and then press the <PgUp> or <PgDn> keys to select the value you want in each item.

| CMOS Setup Utility – Copyright (C) 1984 – 2000 Award Software Standard CMOS Features |  |  |  |
| --- | --- | --- | --- |
|  | Date ( mm: dd: yy ) Sun, Oct 1, 2000 |  |  |
|  | Time ( hh: mm: ss ) 10 : 52 : 51 |  |  |
|  | IDE Primary Master Press Enter None |  |  |
|  | IDE Primary Slave Press Enter None |  |  |
|  | IDE Secondary Master Press Enter None |  |  |
|  | IDE Secondary Slave HITACHI CDR-7730 |  |  |
|  | Driver A 1.44M, 3.5” |  |  |
|  | Driver B None |  |  |
|  | Video EGA / VGA |  |  |
|  | Halt On All, But Keyboard |  |  |
|  | Base Memory 640K |  |  |
|  | Extended Memory 129024K |  |  |
|  | Total Memory 130048K |  |  |

↑↓→Move   Enter: Select   +/-/PU/PD:Value   F10:Save   ESC:Exit   F1:General Help F5:Previous Values	F6:Fail-Safe Defaults	F7:Optimized Defaults

The settings on the screen are for reference only.  Your version may not be  identical to this one.

| Date & Time | Use these items to set the system date and time |
| --- | --- |
| Pri Master Pri Slave Sec Master Sec Slave | If the hard disk Promary Master/Slave and Secondary Master/Slave are set to Auto, then the hard disk size and model will be auto-detcted. If the Auto setting fails to find a hard disk drive, set it to User, and then type in the hard disk . |
| Floppy Drive A Floppy Drive B | Use these items to set the size and capacity of the floppy diskette drive(s) installed in the system. Note: Floppy 3 Mode support is a mode used to support a special 3.5” drive used in Japan. This is a 3.5” disk that stores only 1.2MB, the default setting is Disabled. |
| Halt On | This field is used to determine when to halt the system by the BIOS if an error occurs. |

BIOS Features Setup

| CMOS Setup Utility – Copyright (C) 1984 – 2000 Award Software Advanced CMOS Features |  |  |  |
| --- | --- | --- | --- |
| Virus Warning CPU L1 Cache | : Disabled : Enabled |  |  |
| CPU L2 Cache | : Enabled |  |  |
| CPU L2 Cache ECC Checking | : Enabled |  |  |
| CPU L2 ECC Checking Processor Serial Number | : Enabled : Disabled |  |  |
| Quick Power On Self Test | : Enabled |  |  |
| First Boot Device | : Floppy |  |  |
| Second Boot Device | : HDD-0 |  |  |
| Third Boot Device | : LS/ZIP |  |  |
| Boot Other Device | : Enabled |  |  |
| Swap Floppy Drive | : Disabled |  |  |
| Boot Up Floppy Seek | : Disabled |  |  |
| Boot Up NumLock Status | : Off |  |  |
| Typematic Rate Setting | : Disabled |  |  |
| Typematic Rate (Chars/Sec) | : 6 |  |  |
| Typematic Delay (Msec) | : 250 |  |  |
| Security Option | : Setup |  |  |
| OS Select For DRAM > 64MB HDD S.M.A.R.T. Capability | : Non-OS2 : Disabled |  |  |

↑↓→Move   Enter: Select   +/-/PU/PD:Value   F10:Save   ESC:Exit   F1:General Help F5:Previous Values	F6:Fail-Safe Defaults	F7:Optimized Defaults

The settings on the screen are for reference only.  Your version may not be identical to this one.

| Virus Warning | During and after the system boots up, any attempt to write to the boot sector or partition table of the hard disk drive, BIOS will show a warning message on screen and alarm beep. Keep in ming that this feature protects only the boot sector, not the entire hard drive. To Enable or Disable the built-in virus protection. |
| --- | --- |
| CPU L1 Cache | Use this item to control the status of CPU’s L1 (internal) Cache. When this item is Enabled, the CPU performance will be increased. |
| CPU L2 Cache | Use this item to control the status of CPU’s L2 (external) Cache. When this item is Enabled, the CPU performance will be increased. |
| CPU L2 ECC Checking | This item control if the CPU’s L2 Cache will support Error Checking and Correcting (ECC). When Enabled, the CPU performance will decrease 2%-4%. Enabled is recommended. |
| Quick Power On Self Test | When Enabled this item, system will shorten or skip of the items checked during POST and to speeds up the Power On Self Test. When you are sure that your system hardware is smooth and stable enough, we recommend to enable this item for speeding up POST. |
| 1st /2nd /3rd/ and Other Boot Device | The BIOS trys to load the operating system from the devices in the sequence of 1st, 2nd, 3rd and other devices. The choice : HDD, FDD, CD-ROM, LS120, SCSI. |
| Swap Floppy Drive | This will swap your physical drive letters A & B when you are using two floppy disks. The default is Disabled. |
| Boot Up Floppy Seek | During POST, BIOS will determine if the floppy disk drive installed is 40 or 80 tracks. Only 360K type is 40 tracks while 760K, 1.2MB and 1.44MB are all 80 tracks. The default is Disabled. |
| BootUp Num-Lock | This item control the NumLock key is active or inactive when system boots. On: The keypad Acts as a 10-key pad. Off: The keypad acts like the cursor keys. |
| Typematic Rate Setting | Use this item to determine to keystrokes repate rate. Default is Disabled. Enabled: Allows typematic rate and typematic Delay programming. Disabled: The typematic rate and delay will be Controlled by the keyboard controller. |
| Typematic Rate (Chars/Sec.) | The number of characters will be repeated by a keyboard press. The default is 6. 6: 10 characters per second. 24: 24 characters per second. |
| Typematic Delay (Msec) | Setting controls the time between the first and the second character displayed by typematic auto-repeat. The default is 250. 250: 250 msec. 750: 750 msec. |
| Security Option | This allows you to limit access to the System and Setup, or just to Setup. The default is Setup. System: The system will not boot and the access To the Setup will be denied if entered password is Incorrect. Setup: The system will boot; but the access to Setup will be denied if entered password is incorrect. |
| OS Select for DRAM > 64MB | Use this option only if system has greater than 64MB. Default is Non-OS2. OS2: Select this if you are running the OS/2 operating system with greater than 64MB. |

### Advanced Chipset Features

| CMOS Setup Utility – Copyright (C) 1984 – 2000 Award Software Advanced Chipset Features |  |  |
| --- | --- | --- |
| SDRAM CAS Latency Time | : 3 |  |
| SDRAM Cycle Time Tras/Trc | : 6/8 |  |
| SDRAM RAS-to-CAS Delay | : 3 |  |
| SDRAM RAS Precharge Time | : 3 |  |
| System BIOS Cacheable | : Disabled |  |
| Video RAM Cacheable | : Disabled |  |
| Delayed Transaction | : Enabled |  |
| System Memory Frequency | : 100 MHz |  |

↑↓→Move   Enter: Select   +/-/PU/PD:Value   F10:Save   ESC:Exit   F1:General Help

F5:Previous Values	F6:Fail-Safe Defaults	F7:Optimized Defaults

The settings on the screen are for reference only.  Your version may not be identical to this one.

| SDRAM CAS Latency Time | This setting defines the CAS timing parameter of the SDRAM in terms of clocks. Default is 3. 2: Provides faster memory performance. 3: Provides better memory compatibility. |
| --- | --- |
| SDRAM Cycle Time trans/Trc | Select the numbers of SCLKs for an access cycle. Default is 6/8. |
| SDRAM RAS-to-CAS Delay | This field allows you insert a timing delay between the RAS and the CAS strobe signals. Used when DRAM is written to, read from, or refreshed. “Fast” give faster performance; and “slow” give stable performance. This filed applies when Synchronous SDRAM is intsalled. Defualt value is 3. |
| SDRAM RAS precharge Time | If an insufficient number of cycle is allowed for the RAS to accumulate its charge before DRAM refresh, the refresh may be complete and DRAM may fail to retain Data. “Fast” gives faster performance and “Slow” gives more stable performance. This filed applies when Synchronous SDRAM is intsalled. Defualt value is 3. |
| System BIOS Cacheable | This allows to copy BIOS code from slow ROM to fast RAM. Default is Disabled. Enabled : This option will improve system performance. However, if any program writes to this memory area, a system error may result. |
| Video BIOS Cacheable | This allows to copy the video ROM BIOS to fast RAM (C000h to C7FFFh). Default is Disabled. Enabled : Enabled the Video BIOS Cacheable to speed up the VGA performance. |
| AGP 4X Mode | This allows to copy BIOS code from slow ROM to fast RAM. Default is Enabled. Enabled : This option will improve system performance. However, if any program writes to this memory area, a system error may result. |
| Delayed Transaction | The chipset has an embedded 32-bit posted write buffer to delay tranactions cycles. Select “Enabled” to support compliance with PCI specification version 2.1. Default is Enabled. |
| System Memory Frequency | 100 MHz of System Memory is defaulted. |

### Integrated Peripherals

| CMOS Setup Utility – Copyright (C) 1984 – 2000 Award Software Integrated Peripherals |  |  |  |
| --- | --- | --- | --- |
| OnChip Primary PCI IDE | : Enabled |  |  |
| OnChip Secondary PCI IDE | : Enabled |  |  |
| IDE Primary Master PIO | : AUTO |  |  |
| IDE Primary Slave PIO | : AUTO |  |  |
| IDE Secondary Master PIO | : AUTO |  |  |
| IDE Secondary Slave PIO | : AUTO |  |  |
| IDE Primary Master UDMA | : Disable |  |  |
| IDE Primary Slave UDMA | : Disable |  |  |
| IDE Secondary Master UDMA | : Disable |  |  |
| IDE Secondary Slave UDMA | : Disable |  |  |
| USB Controller | : Enabled |  |  |
| USB Keyboard Support | : Disabled |  |  |
| Init Display First | : PCI Slot |  |  |
| AC 97 Audio | : Auto |  |  |
| AC 97 Modem | : Auto |  |  |
| IDE HDD Block Mode | : Enabled |  |  |
| KB/Mouse Power On | : Disabled |  |  |
| KB Power On Password | : Enter |  |  |
| KB Power On Hot Key | : Ctrl-F1 |  |  |
| Onboard FDC Controller | : Enabled |  |  |
| Onboard Serial Port 1 | : 3F8/IRQ4 |  |  |
| Onboard Serial Port 2 | : 2F8/IRQ3 |  |  |
| UART2 Mode Select | : Normal |  |  |
| RxD, TxD Active | : Hi, Lo |  |  |
| IR Trnasmission Delay | : Enabled |  |  |
| Onboard Parallel Mode | : 378/IRQ7 |  |  |
| Parallel Port Mode | : ECP+EPP |  |  |
| EPP Mode Select | : EPP1.7 |  |  |
| ECP Mode Use DMA | : 3 |  |  |
| PWR Lost Resume State | : Keep Off |  |  |
| Game Port Address | : 201 |  |  |
| Midi Port Address | : Disabled |  |  |
| Mid Port IRQ | : 10 |  |  |

↑↓→Move   Enter: Select   +/-/PU/PD:Value   F10:Save   ESC:Exit   F1:General Help F5:Previous Values	F6:Fail-Safe Defaults	F7:Optimized Defaults

.

The settings on the screen are for reference only.  Your version may not be identical to this one.

| On-Chip Primary PCI IDE | Channel 0 is to enable On Chip IDE Primary port. |
| --- | --- |
| On-Chip Secondary PCI IDE | Channel 1 is to enable On Chip IDE Secondary Port. |
| Primary/ Secondary Master/Slave PIO | This item allows you to assign which kind of PIO is used by IDE devices. Default is Auto. Auto: BIOS will automatically detect the Onboard PCI IDE Accessing mode. Mode 0-4: Manually set the IDE Programmed interrupt mode. |
| Primary/ Secondary Master/Slave UDMA | This item allows you to set Ultra DMA for faster access to IDE HDD which support Ultra DMA function. You may need to install UDMA driver. Default is Auto. Auto: The system will select the optimal setting. Disabled: The HDD will run in normal mode. |
| USB Controller | Enabled this item when your system contains a Universal Serial Bus (USB) and you have a USB keyboard. Default is Enabled. |
| USB Keyboard Support | Enabled this item when your system contains a Universal Serial Bus (USB) and you have a USB keyboard. Default is Disabled. |
| Init Display First | If two graphic devices used (1xAGP, 1x PCI), this specifies which one will be the primary display. The default is PCI Slot. PCI Slots : PCI graphic will be Primary. AGP : AGP graphic will be Primary. |
| AC97 Audio/Modem | These items allow you to decide to enable or disable the support of AC97 Audio or Modem. Default is Auto |
| IDE HDD Block Mode | This item allows IDE controller to access blocks of sectors rather than a single sector at a time providing higher HDD transfer rates. Default is Enabled. |
| Onboard FDC Controller | This item controls the state of Onboard floppy controller. Default is Enabled. Enabled: Enable Onboard VIA686A Chip’s FDD interface controller. |
| OnBoard Serial Port 1, 2 | This items allow user to configure the 1st and 2nd serial ports. Default is Auto. Atuo: Enable Onboard Serial Port 1 and address is auto adjusted. COM1: Enable Onboard Serial Port 1 or 2 and address is 3F8H/IRQ4. COM2: Enable Onboard Serial Port 1 or 2 and address is 2F8H/IRQ3. COM3: Enable Onboard Serial Port 1 or 2 and address is 3E8H/IRQ4. COM4: Enable Onboard Serial Port 1 or2 and address si 2E8H/IRQ3. |
| UATR2 Mode Select | This field is available only if Onboard Serial Port 2 is Disabled. UART mode enables you to select the Infrared protocol – Normal (default), HPSIR or ASKIR. HPSIR is HP’s IR protocol with a maximum baud rate up to 115.2Kbps. ASKIR is Sharp’s IR protocol with a maximum baud rate up to 57.6Kbps. |
| RxD, TxD Active | Defines the voltage level in accordance to the IR module RxD(receive) and TxD(transmit) mode. |
| OnBoard Parallel Port | This item allows user to configure the LPT port. Default is 378H/IRQ7. 378H: Enable Onboard LPT and address is 378H/IRQ7. 278H: Enable Onboard LPT and address 278H/IRQ5. 3BCH: Enable Onboard LPT and address is 3BCH/IRQ7. Disabled: Disable Onboard I/O chip’s LPT port. |
| Onboard Parallel Port Mode | Use this item to set the parallel port mode. You can select SPP (Standard Parallel Port), ECP (Extended Capabilities Port), EPP (Enhanced Parallel Port), or ECP + EPP. Default is Normal. Normal: Standard mode. IBM PC/AT Compatible bidirectional parallel port. |
| EPP Mode Select | The onboard parallel is EPP spec. compliant, so if you select on parallel port mode with EPP function, the “EPP Mode Select” will display on The screen. At this time, either EPP 1.7 spec. or EPP 1.9 spec. can be choosen. |
| ECP Mode Use DMA | The ECP mode has to use DMA channel, so choose on board Parallel port with ECP feature. After selecting, the “ECP Mode Use DMA” will appear and allow the user to select DAM1 or DMA3 for the ECP mode. Default is 3. |
| PWR Lost Resume State | Default is Keep Off |
| Game Port Address | This determine which Address of Game Port will use. Default Value is 201. |
| Midi Port Address | This determine w hich Address of Midi Port will use. Default Value is Disabled. |
| Midi Port IRQ | Default is 10. |

### Power Management

CMOS Setup Utility – Copyright © 1984-2000 Award Software Power Management Setup

ACPI Function Enabled ACPI Suspend Type S1(POS) Power Management User Define Video Off Method DPMS Video Off In Suspend Yes Suspend Mode Disabled HDD Power Down Disabled Soft-Off By PWR-BTTN Instant-Off Resume on PCI Event Disabled Resume on Ring Disabled USB KB Wake-Up From S3 Disabled Resume on LAN Disabled Resume on Alarm Disabled X Date (of Month) Alarm 0 X Time (hh:mm:ss) Alarm 0 : 0 : 0

  : Move     Enter : Select     +/-/PU/PD:Value :        F10: Save      ESC: Exit    F1: General

Help       F5: Previous Values         F6:Fail-Safe Defaults   F7: Optimized Defaults

The settings on the screen are for reference only.  Your version may not be identical

to this one.

| ACPI Function | This field allows you to select ACPI function. Enabled : Support ACPI function for new O.S. Disabled: No support ACPI function. You can only change the content of Doze, Standby and Suspend Mode when the PM is set to “User Define”. User Define : Allows user to define PM Timers parameters to control power saving mode. |
| --- | --- |
| ACPI Suspend Type | This item allows to select S1(POS), S3(STR).nutes. If the setup time passes without any system activity, the computer will enter into power-saving.Suspend mode. |
| Power Management | This option shows weather or not you want to set up Power Management. |
| Video Off Method | This item shows how the video will be disabled by PM. The default is V/H Sync + Blank. V/H Sync + Blank : System turns off vertical and horizontal synchroniztion ports and writes blanks to the video buffer. DPMS is defaulted Select this option if your monitor supports the Display Power Managemetn Signaling (DPMS). Use the software supplied for your video subsystem to select video PM value. Blank Screen : System only writes blanks to the video buffer. |
| Video Off In Suspend | This determins the manner in which the monitor is blanked. Default is Yes. |
| Suspend Mode | This allows to select Suspend type. The settings are PWRON Suspend, Stop Grant. Default is disabled. |
| HDD Power Down | When this item is Enabled and after the set time of system inactivity, the HDD will be Powered Down when other Drives are remain active. The setttings are /2/3/4/5/6/7/8/9/10/11/12/13/14/15 Min. Default is Disabled. |
| Soft-Off By PWR BTTN | Use this item to select soft-off function. Default is Delay 4 sec. Instant Off : Turns off the system instantly. Delay 4 Second : Turns off the system after 4 seconds delay (by holding the power button down for 4 seconds). If momentary press of button, the system will go into Suspend Mode. Press the power button again to take system out of Suspend Mode. |
| Resume On PCI Event | This will enable the system to wake up via PCI card peripheral. Default is Disabled. |
| Resume On Ring | This enables system to boot up if there is an incoming call from the modem. Default is Disabled. |
| USB KB Wake-Up From S3 | This item allows to use USB Keyboard to wake-up system with Suspend To RAM. |
| Resume On LAN | To use this function, you need a LAN add-on card which support power on functions. It also Supports wake LALN on jumper. Default is Disabled. |
| Resume On Alarm | This item is for setting time and date for your system to boot up. You could set the date (of month) Alarm : You can choose which month will boot up. When set to “0”, system can boot up everyday. time (hh:mm:ss) Alarm: You can choose which hour, minute and second the system will boot up. Note: Once the setting is changed, you must let the system boot up untill it goes to operating system before this function will work. |

### PNP/PCI Configuration Setup

| CMOS Setup Utility – Copyright (C) 1984 – 2000 Award Software PnP/ PCI Configurations |  |  |  |
| --- | --- | --- | --- |
| Reset Configuration Data | : Disabled |  |  |
| Resources Controlled By | : Auto (ESCD) |  |  |
|  IRQ Resources | : Press Enter |  |  |
| PCI/VGA Palette Snoop | : Disabled |  |  |
| * PCI IRQ Assignment * |  |  |  |
| Slot 1, 5 | : Auto |  |  |
| Onboard AC97/Slot 2 | : Auto |  |  |
| Slot 3 | : Auto |  |  |
| Onboard USB/Slot 4 | : Auto |  |  |

↑↓→Move   Enter: Select   +/-/PU/PD:Value   F10:Save       ESC:Exit     F1:General Help

F5:Previous Values	F6:Fail-Safe Defaults	F7:Optimized Defaults

The settings on the screen are for reference only.  Your version may not be identical

to this one.

| Reset Configuration Data | This item allows you to clear ESCD (Extended System Configuration Data). Default is Disabled. If you have plugged in some Legacy cards to the system and they were recorded into ESCD, you can set here Enabled in order to clear ESCD. |
| --- | --- |
| Resources Controlled By | Who Controlled the system PNP/PCI resources. The default is Auto. Manual : PnP card’s resources will be controlled manually. You can set which IRQ-X and DMA-X are assigned to PCI/ISA or Legacy ISA cards. Auto(ESCD): If you ISA card and PCI card are all PnP cards, BIOS will assign the interrupt resource automatically. |
| PCI/VGA Palette Snoop: | This item is set for some non-standard VGA cards. However most of cards include this built-in VGA system do not require palette Snoop so that leave it disabled. |

### PC Health Status

This page sets some of the parameters for monitoring function of this mainboard.

CMOS Setup Utility – Copyright © 1984-2000 Award Software PC Health Status

Current System Temperature 27°C/80°F Current CPU Temperature 36°C/90°F Current Chassis FAN Speed 0 RPM Current CPU FAN Speed 0 RPM Current AGP FAN Speed 0 CPU (V) 2.06V +1.5 V 1.53V +3.3 V 3.31V +5 V 5.05V +12 V 12.03V -12 V -11.37V VBAT (V) 3.21V 5VSB (V) 5.40V

  : Move  Enter : Select   +/-/PU/PD:Value :    F10: Save   ESC: Exit    F1: General

Help       F5: Previous Values    F6:Fail-Safe Defaults   F7: Optimized Defaults

The settings on the screen are for reference only.  Your version may not be

identical to this one.

| Current System perature | These items display the system temperature measurement. The system will alert you if a safe temperature is exceeded. |
| --- | --- |
| Currrent CPU Temperature | These items display CPU temperature measurement. The system will alert you if a safe temperature is exceeded. |
| Current Chassis FAN Speed | These items indicate chassis cooling fan speeds in RPM. If the values deviate beyond certain limits, the hardware monitoring feature will alert you with a warning. |

Frequency/Voltage Control

CMOS Setup Utility – Copyright © 1984-2000 Award Software CPU Frequency Control

Open Slight Adjust Disabled Clock By Slight Adjust 66 Spread Spectrum Disabled CPU Clock Ratio x 3

  : Move  Enter : Select  +/-/PU/PD:Value :   F10: Save   ESC: Exit  F1: General Help

### F5: Previous Values                   F6:Fail-Safe Defaults               F7: Optimized Defaults

The settings on the screen are for reference only.  Your version may not be

identical to this one.

| Current Chassis FAN Speed | These items indicate chassis cooling fan speeds in RPM. If the values deviate beyond certain limits, the hardware monitoring feature will alert you with a warning. |
| --- | --- |

### Load Best Performance Settings

When you press <Enter> on this item, a dialog box appears as below:

Load Best Performance (Y/N) ? N

Pressing ‘Y’ loads the BIOS default values for the most stable, minimal-

performance system operations.

Load Optimized Defaults

When you press  <Enter> on this item you get a confirmation dialog box

with a message likely to :

Load Optimized Defaults (Y/N) ? N

Pressing  ‘Y’ loads the default values that are factory settings for optimal perfor-

mance  system operation.

### Change Password

Type the password, up to 8 characters in length, and press <Enter>.  The password typed now will clear any previously entered password from CMOS memory. You will be asked to confirm the password once again.  Now, type the password again and press <Enter>.  You may also press <ESC> to abort the password set up.  Once the password is disabled, the system will boot and you can enter Setup freely. If you highlight this item and press Enter, a dialog box appears which lets you enter a Supervisor password. You can enter no more than six letters or numbers. Press Enter after you have typed in the password. A second dialog box asks you to retype the password for confirmation. Press Enter after you have retyped it correctly. The password is then required to access the Setup Utility or for that and at start-up, depending on the setting of the  Password Check item in Advanced Setup.

### Change or Remove the Password

For changing password, you need to press <Enter> and enter the current password. At the next dialog box, type in the new password.   To disable a password, just press <Enter> when you are requested to entered the password .  A message to confirm the password will be disabled.  Once the password is disabled, the system will boot and you can enter setup freely.

### Exit Selecting

Save & Exit Setup

Pressing <Enter> on this item asks for confirmation:

Save to CMOS and EXIT (Y/N)?  Y