TM-815EP

PCI/ AGP/CNR Mainboard

Version: 1.0

Warning: Never run the processor without the heatsink proper

and firm installation. Permanent Damage Will result!

# Copyright

Copyright ©2001 by this manufacturer. All rights reserved. No part of this publication may be reproduced, transmitted, transcribed, stored in a retrieval system, or translated into any language or computer language, in any form or by any means, electronic, mechanical, magnetic, optical, manual or otherwise, without the prior written consent of this manufacturer.

# Disclaimer

This manufacturer makes no representations or warranties, either expressed or implied, with respect to the contents hereof and specifically disclaims any warranties, merchantability or fitness for any particular purpose. Any software described in this manual is sold or licensed “as is”. Should the programs prove defective following their purchase, the buyer (and not this manufacturer, its distributor, or its dealer) assumes the entire cost of all necessary servicing, repair, and any incidental or consequential damages resulting from any defect in the software. Further these manufacturer reserves the right to revise this publication and to make changes from time to time in the contents hereof without obligation to notify any person of such revision or changes.

Trademark Acknowledgement

Intel and Pentium are registered trademarks of Intel Corporation. Award is a registered trademark of Award Software Inc. Other brand and product names are trademarks and/or registered trademarks of their respective holders.

Printed in Taiwan

# Overview

The TM-815EP Intel’s i815EP chipset removes i752 graphics in the current GMCH chip of 815E; the rest of the function are the same as of 815E. Both support PC 100/133 SDRAM, ultra ATA100 HDD and 4x USB ports.  The cost-effective multiple network connectivity CNR (Communication and Networking Riser) expansion slot upgrades the computer system to a six-channel sound output. With an optionally purchased CNR interface Modem

or Network card such as Home PNA or 10/100 Ethernet connection, it enables network functions and provides players with a great variety of choices.

In addition to complying with the new generation FCPGA-C CPU fan design and supporting Intel 100/133 MHz FSB (front side bus) Pentium III FCPGA processor to speed of 1GHz or higher, it also supports 66 MHz FSB Celeron FCPGA/PPGA processor. TM-815EP has three built-in 168-pin DIMM memory slots; the main memory can be expanded to a maximum of 1.5GB.

The ICH2 also has an enhanced AC’97 interface supporting full surround-sound for Dolby Digital audio found on DVD and simultaneously supports a soft modem connection.  With two USB controllers, the ICH2 offers 24 Mbps of bandwidth across for ports, supporting Intel’s push for a true “ease-of-use PC”.

It also deliver comprehensive platform software for networking storage, security, audio and video to ensure platform stability and performance. ATA 100 IDE bus is available and added performance using Intel’s storage driver.

# CONTENTS

Copyright	2

Disclaimer	2

Overview	3

CONTENTS	3

Introduction	6

Specifications	6

Setup Guide	8

Layout Diagra.…………………………………………………...8

Connector (I/O, USB & IR)..…………………………………...9

Jumper Settings……..…………………………………………10

Important Functions ………………………………………….12

EscoCAM - An Integrated Module	16

Connection Guide of EscoCAM ……………………………..18

Driver &Software …………………………………………….20

BIOS Updated Note…………….. …………………………….23

AWARD BIOS Setup	24

Getting Help	25

Standard CMOS Setup	26

BIOS Features Setup	28

Advanced Chipset Features Setup	31

Integrated Peripherals 	33

Power Management	37

PNP/PCI Configuration Setup	40

PC Health Status ..…………………..……………………… 42

Frequency / Voltage Control………………………………. 44

Load BIOS Default	45

Load Setup Default	45

Supervisor Password 	46

Exiting the Setup Program	47

Technical information	48

System Block Diagram	48

POST Code	49

Problem Sheet	57

MEMO….	58

# Introduction

## Specifications

| System Chipset | Intel i815EP chipset. |  |
| --- | --- | --- |
| CPU | Intel PPGA-Celeron processors, support 300-766 MHz (Ex. Clk 66 MHz); CuMine FC-PGA 450-800MHz (Ex. Clk 100 MHz); CuMine FC-PGA 533-1 GHz (Ex.Clk 133 MHz) CPU. |  |
| Memory | Available with 32MB to 512MB unbuffered DIMMs expandable to 1.5GB (3 banks) with three 168-pin DIMM socket {support 3.3 V EDO (66MHz only) / SDRAM (66/100/133)}. |  |
| ATX Double Deck Ports | • 2 x USB ports • 2 x NS16C550A-compatible DB-9 serial ports • 1 x DB-25 Parallel Port support SPP/EPP/ECP mode. • 1 x mini-DIN-6 PS/2 Mouse Port • 1 x mini-DIN-6 PS/2 Keyboard Port • 1 x game/MIDI port • 3 x audio jacks: line-out, line-in and mic-in. |  |
| Connectors | • 1 x connector for 2 external USB ports • 1 x connector for IrDA or ASKIR interface • 2 x IDE connectors • 1 x FDD interface supports up to 2 x 2.88MB FDDs. • 1 x ATX P.S. connector • 1 x Wake-On-LAN connector • 1 x Wake-On-Ring connector • CPU, chassis and AGP fan connectors • 1 x opened chassis alarm connector • 3 x internal audio connectors( AUX-in, CD-in and TAD) |  |
| BIOS | Award Flash BIOS, Windows 95/98/2000/ME PnP compatible. |  |
| System Health Monitor |  | This mainboard is able to monitor following healthy status: • Monitors CPU/system temperature & overheat alarm • Monitors speed of CPU fan, chassis fan and AGP fan; controls the fan speed of the CPU and chassis fans; and failure alarm. • Automatic CPU and chassis fans on/off control • Opened chassis alarm • Monitors 5VSB/VBAT/1.5V/3/3V/5V/+-12V/CPU voltage and failure alarm • Read back capability that displays temperature, voltage and fan speed. • Supports Intel processor thermal diode output (real processor temperature) If you want a warning message to pop-up or a warning alarm to sound when an abnormal condition occurs, you must install the Hardware Doctor utility available from the CD driver. |
| Desktop Management Interface (DMI) |  | This mainboard comes with a DMI 2.0 built into the BIOS. The DMI utility in the BIOS automatically records various information about your system configuration and stores these information in the DMI pool, which is a part of the system board’s PnP BIOS. DMI, along with the appropriately networked software, is designed to make inventory, maintenance and troubleshooting of computer system easier. |
| Expansion slots |  | X4 AGP x 1, 5x PCI and 1x CNR Slots. |
| Voltage |  | Automatically detects 1.3V to 2.05V. |
| Dimension |  | 4-layer PCB, size (350mm x 220mm). |
| Others |  | Bus Master/ Ultra ATA100, ACPI, AGP Bus, KB Power On, Modem Ring On, LAN Wake Up, Wake On KB/Mouse, Wake On USB KB, AC’97 compliant Sound. |

# Setup Guide

# Layout Diagram

## Connectors

I/O Connectors:

# J14 . Universal Serial Bus Ports

# J12 . IrDA Connector

- VCC

- CIRRX

- IRRX

- GND

- IRTX

## Jumper Setting

- JP1 : Wake-On USB (1 & 2) Keyboard

| JP1 |  |
| --- | --- |
| 1-2 On | Disable (default) |
| 2-3 On | Enable |

- JP2: Wake-On Keyboard / Mouse

| JP2 |  |
| --- | --- |
| 1-2 On | Disable (default) |
| 2-3 On | Enable |

- JP4: Wake-On USB (3 & 4) Keyboard

| JP4 |  |
| --- | --- |
| 1-2 On | Disable (default) |
| 2-3 On | Enable |

- JP5 : Clear CMOS Data

| JP5 |  |
| --- | --- |
| 1-2 On | Normal (default) |
| 2-3 On | Clear CMOS Data |

- JP6 : Boot Block Lock / Unlock

| JP6 |  |
| --- | --- |
| 1-2 On | Unlock Boot Block (default) |
| 2-3 On | Lock Boot Block |

- JP7 : PC Speaker or Buzzer Select

| JP7 |  |
| --- | --- |
| 1-2 On | Buzzer (default) |
| 2-3 On | PC Speaker |

- JP9 / JP10: CPU Front Side Bus

| CPU/DIMM | JP9 | JP10 |
| --- | --- | --- |
| Auto* | 1-2 On | 1-2 On |
| 66/100 MHz | 2-3 On | 2-3 On |
| 100/100 MHz | All Off | 2-3 On |
| 133/100 MHz | All Off | All Off |
| 133/133 MHz | 2-3 On | All Off |

- Important Functions

CPU and Chassis Fan Automatic Off

The CPU and chassis fans will automatically turn off once the system

Enters into Suspend Mode.

Dual Function Power Button

Depending on the setting in the “Soft-Off By PWR-BTTN” field of

The Power Management Setup, this switch will allow the system to

Enter the Soft-Off or Suspend mode.

Wake-On-Ring

This feature allows the system that is in the Suspend mode or Soft Power Off mode to wake-up upon the calls through an internal or external Modem.

- Notice:

If you are using a Modem add-on card, the 5VSB power source of

your power supply must support≧720mA.

Wake-On-LAN

The Wake-On-LAN function allows the network to remotely wake up a Soft Power Down (Soft-Off) PC. Your LAN card must support the remote wakeup function.

- Notice:

The 5VSB power source of your power supply must support≧720mA.

Wake-On-Keyboard / Wake-On-Mouse

This function allows you to use the keyboard or PS/2 mouse to power-on the

system.

- Notice:

The power button will not function once a keyboard password has been set

in the “KB Power On Password” from the BIOS.  You must type the correct

password to power-on the system.  If you forgot the password, power-off

the system and remove the battery.  Wait for a few seconds and install it

back before powering-on the system.

Wake-On-USB Keyboard

This function allows you to use a USB keyboard to wake up a system that is in the S3 (STR – Suspend To RAM) state.

- Notice:

•If you are using this function for 2 USB ports, the 5VSB power source of

your power supply must support ≧1.5A.

•If you are using this function for 4 USB ports, the 5VSB power source of

your power supply must support ≧2A.

ACPI STR

This board is designed to meet the ACPI specification. ACPI has energy saving

Features that enables PCs to implement Power Management and PnP with operating systems support OS Direct Power Management. Currently, only Windows 98/2000 supports the ACPI function.  When ACPI is enabled from

Power Management, it allows you to use the Suspend to RAM function.

With the Suspend to RAM function enabled, you can power-off the system at

Once by pressing the power button or selecting “Standby” when you shut down

Windows 98/2000 without having to go through the sometimes tiresome process of closing files, applications and operating system.  This is because the system is capable of storing all programs and data files during the entire operating session into RAM (Random Access Memory) when it powers-off.

The operating session will resume exactly where you left off the next time you

Power-on the system.

- Notice:

The 5VSB power source of your power supply must support≧1A.

RTC Timer To Power-On The System:

The RTC installed on the system board allows your system to automatically power-on on the date and time.

AC Power Failure Recovery :

When power returns after an AC power failure, you may choose to either

power-on the system manually or let the system power-on automatically

and return to the state where you left off before power failure occurs.

How to use Suspend to RAM Function

1. Select “Power Management Setup” in the Main Menu.

2. Select “Enabled” in the “ACPI Function”.

3. Select “S3(STR)” in the ACPI Suspend Type.

4. Press <ESC> to return to the Main Menu.

5. Select “Save & Exit Setup” and Press <Enter>.  Type <Y>

and press <Enter>.

6. Install Windows 98 by typing the following parameter.

This is to ensure that the ACPI function is supported.

[drive]:>setup /p j

If you have previously installed Windows 98, you need to upgrade the

System in order to support ACPI.  Please contact Microsoft for upgrade

information.

7. Boot Windows 98. In the Windows 98 desktop, click the Start button.

Move the cursor to Settings, then click Control Panel.

To check whether ACPI was properly installed, double-click the System

Icon.  In the System Properties dialog box, click the “Device Manager”

Tab.  In “View devices by type”, click “System Devices”.

8. Double-click the System icon.  In the System Properties dialog box,

click the performance tab.

9. Click File System.  In the “Typical role of this computer” field, select

“Mobile or docking system”.  Click Apply, then click OK.  Restart the PC.

10. Repeat step 7 to open the Control Panel dialog box.  Double-click the

Power Management icon.

11. Click the Advanced tab.  In the  “When I press the power button on my

computer” field, select “Standby”.

12. After completing the steps above and you want to power-off the computer,

you do not need to go through the process of closing files, applications and

operating system.  You can power-off the computer at once by pressing the

power button or selecting “Standby” when you shut down Windows 98.

To power-on the computer, just press the power button.  The operating

session where you left off when you power-off the computer will resume

in not more than 8 seconds.  However, the power button will not function

if a keyboard password has been set in the “KB Power On Password” field

of the integrated Peripherals submenu.  You must type the password to

power-on the computer.

If you have changed the color or resolution (in the Display Properties

Dialog box), do not apply the settings without restarting.  You must re-

start the computer.

EscoCAM – An Integrated Module

It’s so valuable that you don't have to pay thrice to buy three fashionable devices of PC Camera, Debugging and IrDA ware now because we have integrated them into our EscoCAM in the shape of a lovely whale to escort your system. The whale is the king of the sea and the symbol of all-mighty power. With PC camera, Debug, CPU heat monitor and IrDA device built into the EscoCAM, we offers you the unbeatable solution to upgrade the value of your PC system.

1. PC Camera:

As video and audio email is gaining popularity, PC cameras are making inroads into PC systems and it will increasingly become a standard PC peripheral. In view of this strong market demand, we has timely launched its state-of-the-art PC cameras. Besides, our PC Camera is allowed to adjust the Focus of Camera via Keyboard

or mouse instead of wheel.  It’s a patent design in PC Camera.  Hope you enjoy

this new technology.

2. External Debug :

When initializing system, LCD will check the peripherals of CPU, Chipset, DRAM, BIOS, Keyboard, CMOS, VGA card, Devices, FDD, HDD and Cache…Once peripherals were not been properly installed or defective, the LCD display will show out the detective place. After system successfully booted up, Time & Date, CPU Temperature, CPU V-Core Voltage, CPU Fan Speed will take turn to show out on the LCD display.

3. Monitor CPU Temp. And FAN Working Status:

Audio, video and multi-tasking functions call for faster and faster CPUs to improve system efficiency.  However, faster CPUs will generate more heat which must be dissipated in a timely manner. Therefore, a more efficient fans to dissipate heat is as important as watch-out of fan working situation. In this Internet era, people are using their computers for longer and longer time which can easily cause overheat and damage to the CPU. Prolonged use of the computer will increase CPU temperature.

It is therefore necessary to have a device to constantly monitor the CPU temperature, CPU fan speed and V-Core voltage and timely remind user.

4. IrDA For Data Transmission :

As more and more computers, notebook, mobile phone and PDA are installed

with the IrDA device for wireless transmission, we also developed its own

IrDA device for this purpose.

The Connection of EscoCAM

### For Connecting All Mainboards:

A PCI daughter card is attached with EscoCAM to connect:

Or, if you just want use the function of PC Camera, the connection

drawing as follows:

# Driver & Software

### Installation Guide

1. This driver is for Windows 95/NT/2000 and Win. 98.  Aside from the

installation procedure differs from the Operating System used, the

automatic installation is only available for Windows 98.  For Windows

2000/ME/NT, you must manually install.

2. Improper driver installation may cause system unstable. Pls go to Totem’s

Web site at http://www.totem.com.tw for the latest version for drivers and

Software applications, or you can also go to Chipset Vendors

### Quick Installation under Windows 98

When insert the CD-ROM disc in the system CD-ROM drive, the set up feature will automatically come up the screen with buttons for mainboard models name as follows:

# Browse All The Drivers in CD

After selecting mainboard model name, all the available drivers will appears on the screen as follows:

How to install Windows 95/NT/ME/2000 ?

- Insert the support CD-ROM disc in the CD-ROM drive.

Just bypass the error message from PnP function.

- Since quick installation is not supported by Windows 95/NT/ME/2000,

you have to select correct drivers and EXE given file names as follows:

Bus Master IDE Driver

\IDE\815EP\SETUP.EXE

USB Driver

Win95 - \OTHERS\USB\EUSBSUPP\USBSUPP.EXE

Win95(Chinese) - \OTHERS\USB\CUSBSUPP\CUSBSUPP.EXE

Audio Driver

Windows 95 - \SOUND\815EP\WIN95\SETUP.EXE

Windows 98 - \SOUND\815EP\WIN98\SETUP.EXE

Windows ME - \SOUND\815EP\ME\SETUP.EXE

Windows NT - \SOUND\815EP\WINNT\SETUP.EXE

Windows 2000 - \SOUND\815EP\WIN2000\SETUP.EXE

BIOS Update

\UTILITY\AWDFL779.EXE

Super VB Software

This software provides anti-virus protection PC.

\SUPERVB\AUTO.EXE

Hardware Doctor

Windows 9X - \UTILITY\HWDOCTOR\WIN9X\SETUP.EXE

All steps or procedure to install software drivers are subject to change without notice as the software are occassionally updated.

BIOS Update Note

- Do not update the BIOS if no abnormalities occur. However, if BIOS update is needed, consult your dealer first. Prior to updating your BIOS, you are recommended to save the original BIOS values.

- 1. Download the AWARD BIOS Flash Utility file (Awdflash.exe)

- 2. Download the BIOS file used by your mainboard(e.g., XXXXXXX.BIN)

- 3. Reboot your system (but do not run Himem.sys and Emm386.exe)

- to execute the new BIOS program.

- 4. Execute these commands: Awdflash XXXXXXX.BIN

- 5. When this message displays: "Do you want to save BIOS (Y/N)?"

- Type "N"

- 6. When this message displays: "Are you sure to program (Y/N)?"

- Type "Y"