## System Block Diagram

Award BIOS 6.0 POST Code :

| POST (HEX) | Description |
| --- | --- |
| CFh | Test CMOS R/W functionality. |
| C0h | Early chipset initialization: -Disable shadow RAM -Disable L2 cache (socket 7 or below) -Program basic chipset registers |
| C1h | Detect memory -Auto-detection of DRAM size, type and ECC. -Auto-detection of L2 cache (socket 7 or below) |
| C3h | Expand compressed BIOS code to DRAM |
| C5h | Call chipset hook to copy BIOS back to E000 & F000 shadow RAM. |
| 01h | Expand the Xgroup codes locating in physical address 1000:0 |
| 02h | Reserved |
| 03h | Initial Superio_Early_Init switch. |
| 04h | Reserved |
| 05h | 1. Blank out screen 2. Clear CMOS error flag |
| 06h | Reserved |
| 07h | 1. Clear 8042 interface 2. Initialize 8042 self-test |
| 08h | - Test special keyboard controller for Winbond 977 series Super I/O chips. - Enable keyboard interface. |
| 09h | Reserved |
| 0Ah | - Disable PS/2 mouse interface (optional). - Auto detect ports for keyboard & mouse followed by a port & interface swap (optional). - Reset keyboard for Winbond 977 series Super I/O chips. |
| 0Bh | Reserved |

| POST (HEX) | Description |  |
| --- | --- | --- |
| 0Ch | Reserved |  |
| 0Dh | Reserved |  |
| 0Eh | Test F000h segment shadow to see whether it is R/W-able or not. If test fails, keep beeping the speaker. |  |
| 0Fh | Reserved |  |
| 10h | Auto detect flash type to load appropriate flash R/W codes into the run time area in F000 for ESCD & DMI support. |  |
| 11h | Reserved |  |
| 12h | Use walking 1’s algorithm to check out interface in CMOS circuitry. Also set real-time clock power status, and then check for override. |  |
| 13h | Reserved |  |
| 14h | Program chipset default values into chipset. Chipset default values are MODBINable by OEM customers. |  |
| 15h | Reserved |  |
| 16h | Initial onboard clock generator if Early_Init_Onboard_Generator is defined. See also POST 26h. |  |
| 17h | Reserved |  |
| 18h | Detect CPU information including brand, SMI type (Cyrix or Intel) and CPU level (586 or 686). |  |
| 19h | Reserved |  |
| 1Ah | Reserved |  |
| 1Bh | Initial interrupts vector table. If no special specified, all H/W interrupts are directed to SPURIOUS_INT_HDLR & S/W interrupts to SPURIOUS_soft_HDLR. |  |
| 1Ch | Reserved |  |
| 1Dh | Initial EARLY_PM_INIT switch. |  |
| 1Eh | Reserved |  |
| 1Fh | Load keyboard matrix (notebook platform) |  |
| 20h | Reserved |  |
| 21h | HPM initialization (notebook platform) |  |
| 22h | Reserved |  |
| 23h | - Check validity of RTC value: e.g. a value of 5Ah is an invalid value for RTC minute. 2. Load CMOS settings into BIOS stack. If CMOS checksum fails, use default value instead. |  |
| 24h | Prepare BIOS resource map for PCI & PnP use. If ESCD is valid, take into consideration of the ESCD’s legacy information. |  |
| 25h | Early PCI Initialization: -Enumerate PCI bus number. -Assign memory & I/O resource -Search for a valid VGA device & VGA BIOS, and put it into C000:0 |  |
| 26h | - If Early_Init_Onboard_Generator is not defined Onboard clock generator initialization. Disable respective clock resource to empty PCI & DIMM slots. - Init onboard PWM - Init onboard H/W monitor devices |  |
| 27h | Initialize INT 09 buffer |  |
| 28h | Reserved |  |
| 29h | - Program CPU internal MTRR (P6 & PII) for 0-640K memory address. - Initialize the APIC for Pentium class CPU. - Program early chipset according to CMOS setup. Example: onboard IDE controller. - Measure CPU speed. |  |
| 2Ah | Reserved |  |
| 2Bh | Invoke Video BIOS |  |
| 2Ch | Reserved |  |
| 2Dh | - Initialize double-byte language font (Optional) - Put information on screen display, including Award title, CPU type, CPU speed, full screen logo. |  |
| 2Eh | Reserved |  |
| 2Fh | Reserved |  |
| 30h | Reserved |  |
| 31h | Reserved |  |
| 32h | Reserved |  |
| 33h | Reset keyboard if Early_Reset_KB is defined e.g. Winbond 977 series Super I/O chips. See also POST 63h. |  |
| 34h | Reserved |  |
| 35h | Test DMA Channel 0 |  |
| 36h | Reserved |  |
| 37h | Test DMA Channel 1. |  |
| 38h | Reserved |  |
| 39h | Test DMA page registers. |  |
| 3Ah | Reserved |  |
| 3Bh | Reserved |  |
| 3Ch | Test 8254 |  |
| 3Dh | Reserved |  |
| 3Eh | Test 8259 interrupt mask bits for channel 1. |  |
| 3Fh | Reserved |  |
| 40h | Test 8259 interrupt mask bits for channel 2. |  |
| 41h | Reserved |  |
| 42h | Reserved |  |
| 43h | Test 8259 functionality. |  |
| 44h | Reserved |  |
| 45h | Reserved |  |
| 46h | Reserved |  |
| 47h | Initialize EISA slot |  |
| 48h | Reserved |  |
| 49h | - Calculate total memory by testing the last double word of each 64K page. - Program write allocation for AMD K5 CPU. |  |
| 4Ah | Reserved |  |
| 4Bh | Reserved |  |
| 4Ch |  | Reserved |
| 4Dh |  | Reserved |
| 4Eh |  | - Program MTRR of M1 CPU - Initialize L2 cache for P6 class CPU & program CPU with proper cacheable range. - Initialize the APIC for P6 class CPU. - On MP platform, adjust the cacheable range to smaller one in case the cacheable ranges between each CPU are not identical. |
| 4Fh |  | Reserved |
| 50h |  | Initialize USB Keyboard & Mouse. |
| 51h |  | Reserved |
| 52h |  | Test all memory (clear all extended memory to 0) |
| 53h |  | Clear password according to H/W jumper (Optional) |
| 54h |  | Reserved |
| 55h |  | Display number of processors (multi-processor platform) |
| 56h |  | Reserved |
| 57h |  | - Display PnP logo - Early ISA PnP initialization -Assign CSN to every ISA PnP device. |
| 58h |  | Reserved |
| 59h |  | Initialize the combined Trend Anti-Virus code. |
| 5Ah |  | Reserved |
| 5Bh |  | (Optional Feature) Show message for entering AWDFLASH.EXE from FDD (optional) |
| 5Ch |  | Reserved |
| 5Dh |  | - Initialize Init_Onboard_Super_IO - Initialize Init_Onbaord_AUDIO. |
| 5Eh |  | Reserved |
| 5Fh |  | Reserved |
| 60h |  | Okay to enter Setup utility; i.e. not until this POST stage can users enter the CMOS setup utility. |
| 61h |  | Reserved |
| 62h |  | Reserved |
| 63h |  | Reset keyboard if Early_Reset_KB is not defined. |
| 64h |  | Reserved |
| 65h |  | Initialize PS/2 Mouse |
| 66h |  | Reserved |
| 67h |  | Prepare memory size information for function call: INT 15h ax=E820h |
| 68h |  | Reserved |
| 69h |  | Turn on L2 cache |
| 6Ah |  | Reserved |
| 6Bh |  | Program chipset registers according to items described in Setup & Auto-configuration table. |
| 6Ch |  | Reserved |
| 6Dh |  | - Assign resources to all ISA PnP devices. - Auto assign ports to onboard COM ports if the corresponding item in Setup is set to “AUTO”. |
| 6Eh |  | Reserved |
| 6Fh |  | - Initialize floppy controller - Set up floppy related fields in 40:hardware. |
| 70h |  | Reserved |
| 71h |  | Reserved |
| 72h |  | Reserved |
| 73h |  | (Reserved |
| 74h |  | Reserved |
| 75h |  | Detect & install all IDE devices: HDD, LS120, ZIP, CDROM….. |
| 76h |  | (Optional Feature) Enter AWDFLASH.EXE if: -AWDFLASH.EXE is found in floppy drive. -ALT+F2 is pressed. |
| 77h |  | Detect serial ports & parallel ports. |
| 78h |  | Reserved |
| 79h |  | Reserved |
| 7Ah |  | Detect & install co-processor |
| 7Bh |  | Reserved |
| 7Ch |  | Init HDD write protect. |
| 7Dh |  | Reserved |
| 7Eh |  | Reserved |
| 7Fh |  | Switch back to text mode if full screen logo is supported. -If errors occur, report errors & wait for keys -If no errors occur or F1 key is pressed to continue: Clear EPA or customization logo. |
| 80h |  | Reserved |
| 81h |  | Reserved |
| E8POST.ASM starts |  |  |
| 82h |  | - Call chipset power management hook. - Recover the text fond used by EPA logo (not for full screen logo) - If password is set, ask for password. |
| 83h |  | Save all data in stack back to CMOS |
| 84h |  | Initialize ISA PnP boot devices |
| 85h |  | - USB final Initialization - Switch screen back to text mode |
| 86h |  | Reserved |
| 87h |  | NET PC: Build SYSID Structure. |
| 88h |  | Reserved |
| 89h |  | - Assign IRQs to PCI devices - Set up ACPI table at top of the memory. |
| 8Ah |  | Reserved |
| 8Bh |  | - Invoke all ISA adapter ROMs - Invoke all PCI ROMs (except VGA) |
| 8Ch |  | Reserved |
| 8Dh |  | - Enable/Disable Parity Check according to CMOS setup - APM Initialization |
| 8Eh |  | Reserved |
| 8Fh |  | Clear noise of IRQs |
| 90h |  | Reserved |
| 91h |  | Reserved |
| 92h |  | Reserved |
| 93h |  | Read HDD boot sector information for Trend Anti-Virus code |
| 94h |  | - Enable L2 cache - Program Daylight Saving - Program boot up speed - Chipset final initialization. - Power management final initialization - Clear screen & display summary table - Program K6 write allocation - Program P6 class write combining |
| 95h |  | Update keyboard LED & typematic rate |
| 96h |  | - Build MP table - Build & update ESCD - Set CMOS century to 20h or 19h - Load CMOS time into DOS timer tick - Build MSIRQ routing table. |
| FFh |  | Boot attempt (INT 19h) |

## Problem Sheet

| Customer |  |  |  |
| --- | --- | --- | --- |
| Name |  | Tel |  |
| address |  | Fax |  |

| Mainboard |  |  |  |
| --- | --- | --- | --- |
| Mode |  | Mainboard Rev |  |
| Serial No. |  | BIOS version |  |

| Configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Brand |  |  | Brand |  |
| CPU | Type |  | RAM | Type & Size |  |
|  | Voltage |  |  | Speed |  |
|  | Brand |  |  | Brand |  |
| Floppy disk | Mode |  | VGA card | Chipset |  |
|  | Size |  |  | RAM Type |  |
|  | Brand |  |  | Brand |  |
| Hard disk | Mode |  | CD-ROM | Mode |  |
|  | Size |  |  | Speed |  |
|  | Brand |  |  | Brand |  |
| Sound card | Mode |  | Lan card | Mode |  |
|  | Remark |  |  | PCI/ISA Bus |  |
|  | Brand |  |  | Brand |  |
| Mouse | Mode |  | O.S | Name |  |
|  | PS/2 or Serial |  |  | Version |  |
| Autoexec.bat |  |  | Config.sys |  |  |
| Problem Description |  |  |  |  |  |

MEMO