# Technical information

## Block Diagram

| ## I/O Connector Map |  |  |  |  | ISA Bus |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | GND | 1 |  | 1 | -I/OCH CHK |
|  |  |  |  |  | RESET | 2 |  | 2 | SD 07 |
|  |  |  |  |  | +5V | 3 |  | 3 | SD 06 |
| Floppy Disk Connector |  |  |  |  | IRQ 9 | 4 |  | 4 | SD 05 |
|  |  |  |  |  | -5V | 5 |  | 5 | SD 04 |
|  |  |  |  |  | DRQ2 | 6 |  | 6 | SD 03 |
|  |  |  |  |  | -12V | 7 |  | 7 | SD 02 |
| Ground | 1 |  | 2 | FDHDIN | OWS | 8 |  | 8 | SD 01 |
| Ground | 3 |  | 4 | Reserved | +12V | 9 |  | 9 | SD 00 |
| Ground | 5 |  | 6 | FDEDIN | GND | 10 |  | 10 | -I/O CH RDY |
| Ground | 7 |  | 8 | -Index | -SMEMW | 11 |  | 11 | AEN |
| Ground | 9 |  | 10 | Motor Enable | -SMEMR | 12 |  | 12 | SA 19 |
| Ground | 11 |  | 12 | -Driver selectB | -IOW | 13 |  | 13 | SA 18 |
| Ground | 13 |  | 14 | -Driver selectA | -IOR | 14 |  | 14 | SA 17 |
| Ground | 15 |  | 16 | Motor Enable | -DACK3 | 15 |  | 15 | SA 16 |
| Ground | 17 |  | 18 | -DIR | -DRQ3 | 16 |  | 16 | SA 15 |
| Ground | 19 |  | 20 | -STEP | -DACK1 | 17 |  | 17 | SA 14 |
| Ground | 21 |  | 22 | Write Data | -DRQ1 | 18 |  | 18 | SA 13 |
| Ground | 23 |  | 24 | Write Gate | -REFRESH | 19 |  | 19 | SA 12 |
| Ground | 25 |  | 26 | -Track 00 | BCLK | 20 |  | 20 | SA 11 |
| Ground | 27 |  | 28 | -Write Protect | IRQ 7 | 21 |  | 21 | SA 10 |
| Ground | 29 |  | 30 | -Read Data | IRQ 6 | 22 |  | 22 | SA 09 |
| Ground | 31 |  | 32 | -Side 1 select | IRQ 5 | 23 |  | 23 | SA 08 |
| Ground | 33 |  | 34 | Diskette | IRQ 4 | 24 |  | 24 | SA 07 |
|  |  |  |  |  | IRQ 3 | 25 |  | 25 | SA 06 |
|  |  |  |  |  | -DACK2 | 26 |  | 26 | SA 05 |
| IDE Connector |  |  |  |  | T/C | 27 |  | 27 | SA 04 |
|  |  |  |  |  | BALE | 28 |  | 28 | SA 03 |
|  |  |  |  |  | +5V | 29 |  | 29 | SA 02 |
|  |  |  |  |  | OSC | 30 |  | 30 | SA 01 |
| Reset IDE | 1 |  | 2 | Ground | GND | 31 |  | 31 | SA 00 |
| HOST Data 7 | 3 |  | 4 | HOST Data 8 |  |  |  |  |  |
| HOST Data 6 | 5 |  | 6 | HOST Data 9 | -MEMCS 16 | 1 |  | 1 | SBHE |
| HOST Data 5 | 7 |  | 8 | HOST Data10 | -I/OCS16 | 2 |  | 2 | LA 23 |
| HOST Data 4 | 9 |  | 10 | HOST Data11 | IRQ 10 | 3 |  | 3 | LA 22 |
| HOST Data 3 | 11 |  | 12 | HOST Data12 | IRQ 11 | 4 |  | 4 | LA 21 |
| HOST Data 2 | 13 |  | 14 | HOST Data13 | IRQ 12 | 5 |  | 5 | LA 20 |
| HOST Data 1 | 15 |  | 16 | HOST Data14 | IRQ 15 | 6 |  | 6 | LA 19 |
| HOST Data 0 | 17 |  | 18 | HOST Data15 | IRQ 14 | 7 |  | 7 | LA 18 |
| Grund | 19 |  | 20 | Key | -DACK 0 | 8 |  | 8 | LA 17 |
| DRQ 3 | 21 |  | 22 | Ground | DRQ 0 | 9 |  | 9 | -MEMR |
| -I/O Write | 23 |  | 24 | Ground | -DACK 5 | 10 |  | 10 | -MEMW |
| -I/O Read | 25 |  | 26 | Ground | DRQ 5 | 11 |  | 11 | SD 08 |
| IOCHRDY | 27 |  | 28 | BALE | -DACK 6 | 12 |  | 12 | SD 09 |
| -DACK 3 | 29 |  | 30 | Ground | DRQ 6 | 13 |  | 13 | SD 10 |
| IRQ 14 | 31 |  | 32 | -IOCS16 | -DACK 7 | 14 |  | 14 | SD 11 |
| Addr 1 | 33 |  | 34 | Ground | DRQ 7 | 15 |  | 15 | SD 12 |
| Addr 2 | 35 |  | 36 | Addr2 | +5V | 16 |  | 16 | SD 13 |
| -Chip select 0 | 37 |  | 38 | -Chip select 1 | -MASTER | 17 |  | 17 | SD 14 |
| Activity | 39 |  | 40 | Ground | GND | 18 |  | 18 | SD 15 |

| ATX POWER SUPPLY Connector |  |  |  |  | Parallel port connector |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.3 V | 11 |  | 1 | 3.3 V |  |  |  |  |  |
| -12 V | 12 |  | 2 | 3.3 V | -STROBE | 1 |  | 14 | -AUTO FEED |
| GND | 13 |  | 3 | GND | Data Bit 0 | 2 |  | 15 | -ERROR |
| PS-ON | 14 |  | 4 | 5 V | Data Bit 1 | 3 |  | 16 | -INIT |
| GND | 15 |  | 5 | GND | Data Bit 2 | 4 |  | 17 | -SLCT IN |
| GND | 16 |  | 6 | 5 V | Data Bit 3 | 5 |  | 18 | Ground |
| GND | 17 |  | 7 | GND | Data Bit 4 | 6 |  | 19 | Ground |
| -5 V | 18 |  | 8 | PW-OK | Data Bit 5 | 7 |  | 20 | Ground |
| 5 V | 19 |  | 9 | 5 V-SB | Data Bit 6 | 8 |  | 21 | Ground |
| 5 V | 20 |  | 10 | 12 V | Data Bit 7 | 9 |  | 22 | Ground |
| Serial port connector |  |  |  |  | -ACJ | 10 |  | 23 | Ground |
|  |  |  |  |  | BUSY | 11 |  | 24 | Ground |
|  |  |  |  |  | PE | 12 |  | 25 | Ground |
|  |  |  | 1 | DCD | SLCT | 13 |  |  |  |
| DSR | 6 |  | 2 | SIN |  |  |  |  |  |
| RTS | 7 |  | 3 | SOUT | PS/2 mouse connector signal line |  |  |  |  |
| CTS | 8 |  | 4 | DTR |  |  |  |  |  |
| RI | 9 |  | 5 | GND | Data(Red) | 1 |  | 4 | NC |
|  |  |  |  |  | Clock(Blue) | 2 |  | 5 | VCC(Yellow) |
|  |  |  |  |  | GND(Green) | 3 |  |  |  |

## The transfer rate of IDE PIO and DMA modes.

| Mode | PCI Bus Clock | Cycle time | Data transfer rate |
| --- | --- | --- | --- |
| PIO Mode 0 | 33 MHz | 600 ns | 3.3 MB/s |
| PIO Mode 1 | 33 MHz | 383 ns | 5.2 MB/s |
| PIO Mode 2 | 33 MHz | 240 ns | 8.3 MB/s |
| PIO Mode 3 | 33 MHz | 180 ns | 11.1 MB/s |
| PIO Mode 4 | 33 MHz | 120 ns | 16.6 MB/s |
| PIO Mode 5 | 33 MHz | 90 ns | 20 MB/s |
| DMA Mode 0 | 33 MHz | 480 ns | 4.16 MB/s |
| DMA Mode 1 | 33 MHz | 150 ns | 13.3 MB/s |
| DMA Mode 2 | 33 MHz | 120 ns | 16.6 MB/s |
| DMA/33 | 33 MHz | 60 ns | 33 MB/s |

When IORDY signal is used, PIO Mode 3/4 is in ATA-2 format while PIO Mode 0/1/2 is in ATA format. PIO Mode5 is unlikely to happen.

## The Computer BOOT flow. chart

## The Features Introduction

|  | INTEL 440 BX | INTEL i810e | VIA 693A+596B |
| --- | --- | --- | --- |
| CPU support | Pentium II / III | Pentium II / III | Pentium II / III |
| Memory size | 1GB | 1GB | 1 GB |
| Memory | SDRAM | SDRAM | SDRAM |
| Memory Clock | 100 | 100 | 100/133MHz |
| Ex. Clock | 66/100MHz | 66/100/133MHz | 66/100/133 MHz |
| ACPI | Yes | Yes | ACPI |
| AGP Bus | X 2 | X 2 | X 4 |
| Ultra ATA/66 | No | Yes | Yes |

- Ultra ATA/66

It’s an extension of Ultra ATA33 interface.  This new high-speed interface has doubled the Ultra ATA/33 burst data transfer rate to 66.6 Mbytes/sec and maximized disk performance under current PCI local bus environment.

- ACPI (Advanced Configuration and Power Interface)

- This Advanced Configuration and Power Interface set up by Intel, Microsoft, Compaq, IBM, HP….etc, offers the functions below:

- . Automatically stops offering power to CD-ROM, FDD or HDD when              any of them is not in use.

- . Offers the “OnNow” function; when you start the system, what is seen on the screen is in the condition same as the last time before the system was shut down.

- . Enhances the system configuration like PnP, DMI….

- USB(Univer Serial Bus)

- The new Bus specification defined by Compaq, DEC, IBM, Microsoft, NEC, Intel ….etc, is connected by USB, making it possible for the peripheral to have the “plug and play” function without interface card.

- There are at most 127 peripherals that can be connected at the same time.

- Printer Modes /SPP /ECP /EPP

SPP (Standard Parallel Port)

The current commonly used standard mode.

ECP (Extended Capabilities Port)

Jointly set by Microsoft and HP. Its main feature is using high-performance half-duplex bi-directional channel to achieve faster transmission speed. Its 16-bit FIFO (First-In-First-Out) buffer makes high-speed transmission more stable and reliable. DMA function is included in its controller.

EPP (Enhanced Parallel Port)

Jointly set by Intel, Zenith and Xircom. Bi-directional block transmission makes transmission speed to reach 2MB per second. It is compatible with the standard parallel port interface. For printers that do not support the EPP mode in Windows 95, including the Canon BJ Series and the Epson LQ Series, you can set the Parallel Mode in BIOS to Normal (SPP) Mode to enable EPP.

- IrDA (Infrared Data Association)

This organization sets the infrared transmission standards. The IrDA Protocol sets transmission speed at 115KB per second and a transmission angle of 30 degree. Its Serial Port shall have 16550 UARTs and its maximum transmission distance is one meter.

- S. M. A. R. T (Self  Monitoring Analysis Report Technology)

It is jointly set by Conner, IBM, Quantum, Seagate and Western Digital.

Most hard disks on the market have this function. It issues a warning

message to the computer user prior to the "actual" failure of the user can

have sufficient time to backup data or to replace the hard disk.

## TIME and DMA CHANNELS MAP

TIME MAP:

TIMER Channel 0   	System timer interrupt.

TIMER Channel 1	DRAM REFRESH request.

TIMER Channel 2	SPEAKER tone generator.

DMA CHANNELS:	DMA Channel 0	Available.

DMA Channel 1	Audio.

DMA Channel 2	FLOPPY DISK.

DMA Channel 3	Onboard ECP (default).

DMA Channel 4	Cascade.                                                          	DMA Channel 5 	PCMCIA DMA.

DMA Channel 6       	MPEG.                                                          	DMA Channel 7       	Available.

## INTERRUPT  MAP

NMI:	Parity check error.

IRQ (H/W):		0  System TIMER interrupt form TIMER 0.

1  KEYBOARD output buffer full.

2  Cascade for IRQ 8-15.

3  SERIAL port 2.

4  SERIAL port 1.

5  Audio/MPU-401 or PARALLEL port 2.

6  FLOPPY DISK.

7  PARALLEL port 1.

8  RTC clock.

9  Available.

10  PCMCIA.

11  MPEG.

12  Trackpad (PS/2 Mouse).

13  MATH coprocessor.

14  Primary IDE interface (HDD).

15  Secondary IDE interrface (CD-ROM).

## RTC and CMOS RAM MAP

RTC and CMOS:	00	 Seconds

01	 Seconds alarm

02	 Minutes

03	 Minutes alarm

04	 Hours

05	 Hours alarm

06	 Day of week

07	 Day of month

08	 Month

09	 Year

0A	 Status register A

0B 	Status register B

0C	Status register C

0D 	Status register D

0E	Diagnostic status byte

0F 	Shutdown byte

10	FLOPPY DISK drive type byte

11	Reserve

12	HARD DISK type byte

13	Reserve

14	Equipment type

15	Base memory low byte

16	Base memory high byte

17	Extension memory low byte

18	Extension memory high byte

19-2d

2E-2F

30	Reserved for extension memory low byte

31	Reserved for extension memory high byte

32	DATE CENTURY byte

33	INFORMATION FLAG

35-3F	Reserve

40-7F 	Reserved for CHIPSET SETTING DATA

## POST Code

| POST (HEX) | Description |
| --- | --- |
| C0 ●●○○ ○○○○ | 1. Turn off OEM specific cache, shadow... 2. Initialize all the standard devices with default values standard devices including: - DMA controller (8237) - Programmable Interrupt Controller (8259) - Programmable Interval Timer (8254) |
| C1/C6 - ●○○○○○●╱ ●●○○○●●○ | Auto-detection of onboard DRAM and Cache |
| C3 ●●○○ ○○●● | 1. Test the first 256K DRAM 2. Expand the compressed codes into temporary DRAM area including the compressed System BIOS and Option ROMs |
| C5 ●●○○ ○●○● | Copy the BIOS from ROM into E0000-FFFFF shadow RAM so that POST will go faster |
| 01-02 | Reserved |
| 03 ○○○○ ○○●● | Initialize EISA registers (EISA BIOS ONLY) |
| 04 | Reserved |
| 05 ○○○○ ○●○● | 1. Keyboard Controller Self-Test 2. Enable Keyboard Interface |
| 06 ○○○○ ○●●○ | F000 shadow R/W test |
| 07 ○○○○ ○●●● | Verifies CMOS‘s basfic R/W functionality |
| BE ●○●● ●●●○ | Program defaults values into chipset according to the MODBINable Chipset Default Table |
| 09 ○○○○ ●○○● | 1. Issue CPU ID instruction to idedtify CPU type 2. Program the configuration register of Cyrix CPU according to the MODBINable Cyrix Register Table 3. OEM specific cache initialization |
| 0A ○○○○ ●○●○ | 1. Initialize the first 32 interrupt vectors with correspoinding interrupt handlers Initialize INT no from 33-120 with Dummy (Spurious) Interrupt Handler 2. Early Power Management initialization (OEM specific) |

| POST (HEX) | Description |
| --- | --- |
| 0B ○○○○ ●○●● | 1. Verify whether RTC time is valid or not 2. Detect bad battery 3. Read CMOS data into BIOS stack area 4. PNP initializations including (PNP BIOS ONLY) - Assign CSN to PNP ISA card - Create resource map from ESCD 5. Update the P6 CPU‘s micro code (P6 Only) 6. Assign IO and Memory for PCI devices (PCI BIOS ONLY) |
| 0C ○○○○ ●●○○ | Initialization of the BIOS Data Area (40:0-40:FF) |
| 0D ○○○○ ●●○● | 1. P5 Multi-P BIOS Only Init IO and Local APIC 2. Program some of the Chipset‘s value according to Setup (Early Setup Value Program) 3. Measure CPU speed for display and decide system clock speed 4. Video initialization including Monochrome, CGA, EGA/VGA. If no display device found, the speaker will beep |
| 0E ○○○○ ●●●○ | 1. Initialize the APIC (Multi-Processor BIOS ONLY) 2. Test video RAM (If Monochrome display device found) 3. Show messages including: - Award Logo, Copyright String, BIOS Date code and Part No. - OEM specific sign on messages - Energy Star Logo (Green BIOS Only) - CPU brand, type and speed |
| 0F ○○○○ ●●●● | DMA channel 0 test |
| 10 ○○○● ○○○○ | DMA channel 1 test |
| 11 ○○○● ○○○● | DMA page registers test |
| 12-13 | Reserved |
| 14 ○○○● ○●○○ | Test 8254 Timer 0 Counter 2 |
| 15 ○○○● ○●○● | Test 8259 interrupt mask bits for channel 1 |
| 16 ○○○● ○●●○ | Test 8259 interrupt mask bits for channel 2 |
| 17 | Reserved |

| POST (HEX) | Description |
| --- | --- |
| 19 ○○○● ●○○● | Test 8259 functionality |
| 1A-1D | Reserved |
| 1E ○○○● ●●●○ | If EISA NVM checksum is good,, execute EISA initialization |
| 1F-29 | Reserved |
| 30 ○○●● ○○○○ | 1. Get Base Memory and Extended Memory Size 2. P6 Multi-P BIOS Only Init IO & Local APIC 3. Program K5 CPU‘s Write Allocation |
| 31 ○○●● ○○○● | 1. Get Base Memory and Extended Memory Size 2. P6 Multi-P BIOS Only Init IO and Local APIC 3. Program K5 CPU‘s Write Allocation |
| 32 ○○●● ○○●○ | 1. Display the Award Plug and Play BIOS Extension message (PNP BIOS ONLY) 2. Program all onboard super I/O chips (if any) including COM ports, LPT ports, FDD port... according to setup value 3. Program onboard audio devices |
| 33-3B | Reserved |
| 3C ○○●● ●●○○ | Set flag to allow users to enter CMOS Setup Utility |
| 3D ○○●● ●●○● | 1. Initialize Keyboard 2. Install PS/2 mouse 3. Build the INT 15h function E820H table 4. Build the PnP Device Node for total memory size |
| 3E ○○●● ●●●○ | Try to turn on Level 2 cache Note: Some chipset may need to turn on the L2 cache in this stage. But usually, the cache is turned on later in POST 61h |
| 3F-40 | Reserved |
| BF ●○●● ●●●● | 1. Program the rest of the Chipset‘s value according to setup 2. If auto-configuration is enabled, program the chipset with predefined values in the MODBIN able Auto-Table |
| 41 ○●○○ ○○○● | Initialize floppy disk drive controller |

| POST (HEX) | Description |  |
| --- | --- | --- |
| 42 ○●○○ ○○●○ | 1. Cut IRQ 12 connection if PS/2 mouse is not installed 2. Install IDE Hard Drives - Auto-detect HDDs - Build the AT compatible HDD table for Type 47 - Set PIO timing 3. Detect CD ROM on IDE Bus 4. Detect LS120 drive |  |
| 43 ○●○○ ○○●● | If it is a PNP BIOS, initialize serial and parallel ports |  |
| 44 | Reserved |  |
| 45 ○●○○ ○●○● | Initialize math coprocessor |  |
| 46-4D | Reserved |  |
| 4E ○●○○ ●●●○ | 1. If there is any error detected (such as video, kb.....), show all the error messages on the screen and wait for the user to 2. Enable “Far Hit” for Cyrix 6x86 CPU |  |
| 4F ○●○○ ●●●● | 1. If password is needed, ask for password 2. Clear the Energy Star Logo (Green BIOS ONLY) |  |
| 50 ○●○● ○○○○ | Write all the CMOS values currently in the BIOS stack area back into the CMOS |  |
| 51 | Reserved |  |
| 52 ○●○● ○○●○ |  | 1. Initialize all ISA ROMs 2. Later PCI initializations (PCI BIOS ONLY) - assign IRQ to PCI devices - initialize all PCI ROMs 3. Program shadows RAM according to setup settings 4. Program Parity according to Setup setting 5. Power Management Initialization - Enable/Disable global PM - APM interface initializtion |
| 53 ○●○● ○○●● |  | 1. If it is NOT a PNP BIOS, initialize serial and parallel port 2. Initialize time value in BIOS data area by translating the RTC time value into a timer tick value |
| 54-5F |  | Reserved |
| 60 ○●●○ ○○○○ |  | Setup Virus Protection (Boot Sector Protection) functionality according to setup setting |

| POST (HEX) | Descriptions |
| --- | --- |
| 61 ○●●○ ○○○● | 1. Try to turn on Level 2 cache Note: if L2 cache is already turned on in POST 3D, this part will be skipped 2. Set the boot up speed according to setup setting 3. Last chance for chipset initialization 4. Last chance for power Management initialization (Green BIOS only) 5. Show the system configuration table |
| 62 ○●●○ ○○●○ | 1. Setup daylight saving according to setup value 2. Program the NUM Lock, typmatic rate and typmatic speed according to setup setting |
| 63 ○●●○ ○○●● | 1. If there is any changes in the hardware configuration, update the ESCD information (PNP BIOS ONLY) 2. If there is any changes in the hardware configuration, update the DMI data pool (DMI BIOS ONLY) 3. Clear memory that have been used 4. Boot system via INT 19h |
| FF ●●●● ●●●● | System Booting. This means that the BIOS already pass the control right to the operating system |

Unexpected Errors:

| POST (HEX) | Description |
| --- | --- |
| B0 ●○●● ○○○○ | If interrupt occurs in protected mode |
| B1 ●○●● ○○○● | Unclaimed NMI occurs |

## Problem Sheet

| Customer |  |  |  |
| --- | --- | --- | --- |
| Name |  | Tel |  |
| address |  | Fax |  |

| Mainboard |  |  |  |
| --- | --- | --- | --- |
| Mode |  | Mainboard Rev |  |
| Serial No. |  | BIOS version |  |

| Configuration |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Brand |  |  | Brand |  |
| CPU | Type |  | RAM | Type & Size |  |
|  | Voltage |  |  | Speed |  |
|  | Brand |  |  | Brand |  |
| Floppy disk | Mode |  | VGA card | Chipset |  |
|  | Size |  |  | RAM Type |  |
|  | Brand |  |  | Brand |  |
| Hard disk | Mode |  | CD-ROM | Mode |  |
|  | Size |  |  | Speed |  |
|  | Brand |  |  | Brand |  |
| Sound card | Mode |  | Lan card | Mode |  |
|  | Remark |  |  | PCI/ISA Bus |  |
|  | Brand |  |  | Brand |  |
| Mouse | Mode |  | O.S | Name |  |
|  | PS/2 or Serial |  |  | Version |  |
| Autoexec.bat |  |  | Config.sys |  |  |
| Problem Description |  |  |  |  |  |