TM-P2BX  TM-P2VAP

ISA/ PCI/ AGP Mainboard

with onboard PCI IDE and super Multi-I/O.

Version: 2.0

# Copyright

Copyright © 1998 by this manufacturer. All rights reserved. No part of this publication may be reproduced, transmitted, transcribed, stored in a retrieval system, or translated into any language or computer language, in any form or by any means, electronic, mechanical, magnetic, optical, manual or otherwise, without the prior written consent of this manufacturer.

# Disclaimer

This manufacturer makes no representations or warranties, either expressed or implied, with respect to the contents hereof and specifically disclaims any warranties, merchantability or fitness for any particular purpose. Any software described in this manual is sold or licensed “as is”. Should the programs prove defective following their purchase, the buyer (and not this manufacturer, its distributor, or its dealor) assumes the entire cost of all necessary servicing, repair, and any incidental or consequential damages resulting from any defect in the software. Further these manufacturer reserves the right to revise this publication and to make changes from time to time in the contents hereof without obligation to notify any person of such revision or changes.

Trademark Acknowledgement

Intel and Pentium are registered trademarks of Intel Corporation. Award is a registered trademark of Award Software Inc. Other brand and product names are trademarks and/or registered trademarks of their respective holders.

Printed in Taiwan

# Overview

The TM-P2BX\VA are a Pentium II\III based mainboard that utilizes Intel 82440BX and VIA chipset on ATX PCI/ ISA platform. This mainboard is designed for Pentium II\III CPU, and supports new architectures such as high speed AGP graphic port, Ultra DMA/33/66, Bus Master IDE, SDRAM memory and expandable to a maximum 1GMB. There is no second level cache onboard since the cache is on the Pentium II\III CPU card.

In addition to above features, TM-P2BX\VA implements most advanced technology such as Synchronous switching regulator, CPU thermal protection, CPU fan monitoring, System voltage monitoring, Over current protection, Modem Wake Up, Keyboard power on, PS/2 mouse power on, Debug sensor on board and user-friendly Jumper-less.

The most unique feature of the TM-P2BX\VA is its capability to LED and Voice debug onboard. When the CPU, DRAM, FDD, or VGA cards have not been properly installed, a DIY user can isolate problems through reading the Debug Sensor LED and instructions in the manual or hearing the voice Dedug. To professional system test engineers or maintenance engineers, the Debug Sensor can work as an 80 Port Debug Card. In other words, they can use this debug sensor function to do testing and maintenance in lieu of the 80 Port Debug card.

# Main Features

- Debug sensor on board.

- Support the newest specifications of ACPI.

- Support Ultra DMA/33/ 66 (VIA)-highest rate IDE (must be applied with driver).

- Support current CPUs and future higher speed CPUs that are compatible with SECC Socket.

- Support IR/Consumer IR/USB and PS/2 device.

- Support modem ring on.

- Compatible with PCI 2.1 Spec.

- With the “power saving” feature, the CPU cooler automatically stops when the system enters the green mode state.

- CPU cooler connector built in.

- Supports Intel LDCM Network Manageability.

- Supports ECC or Non-ECC type DRAM module.

- Support Keyboard Power On.

- Support PS/2 Mouse Power On.

- Supports BIOS in setting CPU type.

- Supports automatic closing of the unused DIMM/ PCI clock to reduce noise.

# CONTENTS

Copyright	2

Disclaimer	2

Overview	3

Main Features	4

CONTENTS	5

Introduction	8

A.Specifications	7

Setup Guide	9

A.Layout Diagram	9

B.Smart Debug On Borad & S1 set up…………….…………..9

C.CPU Voltage and Frequencies…………………………….10

D.EDO/ SDRAM Installation  Procedures:	13

E.Other Jumper Settings	15

F.Note to BIOS Update	17

G.Keyboard/ PS/2 Mouse Power On and MODEM Ring on	18

H.System Health Monitor	20

I.  EDO/ SDRAM ConfigurationTable:	22

Award BIOS Setup	24

A.Getting Help	24

B.The Main Menu	25

1.Standard CMOS Setup	26

2.BIOS Features Setup	27

3.Chipset Features Setup	31

4.Power Management	35

5.PNP/PCI Configuration Setup	37

6.Load BIOS Default	39

7.Load Setup Default	39

8.Integrated Peripherals Setup	39

9.Password Setting	42

10.IDE HDD Auto Detection	43

11.Hard Disk Low Level Format Utility	43

12.Exiting the Setup Program	44

Technical information	44

A.Block Dlagram	44

B.I/O Connector Map	 45

C.The transfer rate of IDE PIO and DMA modes.	46

D.The Computer BOOT  flow.chart	47

E. Features Introduction	48

F.TIME and DMA CHANNELS MAP	50

G.INTERRUPT  MAP	50

H.RTC and CMOS RAM MAP	51

I.POST Code	52

J.Problem Sheet	57

Utility guide	58

External 3.5" drive bay 	59

# Introduction

## Specifications

| System Chipset | Intel 82440 BX and VIA 693A+596B (or 686A in Q1, 00) chipset. |  |
| --- | --- | --- |
| CPU | Intel Pentium II\III processors, support 233/266/300/333 (Ex. Clk 66 MHz). 300/350/400/450/500 550/600 (Ex. Clk 100MHz). 533/600 (Ex. Clk 133 MHz) |  |
| Memory | Expandable to 768MB (3 banks) with three 168-pin DIMM socket {support 3.3 V EDO (66MHz only) / SDRAM (66MHz/ 100MHz)}. |  |
| I/O | Winbond 83977, two high speed 16550 compatible serial ports, one Multi-Mode. Parallel Port support SPP/EPP/ECP standard mode. Two onboard PCI IDE Ports (32-bit data transfer). LS-120/ ZIP FDD, IrDA/ ASK IR/ Consumer IR. Dual USB ports Support two 360/720KB/1.2/1.44/2.88MB floppy disk devices. One PS/2 Mouse port. |  |
| BIOS | Award System BIOS installed in socket (Flash and PnP). |  |
| Expansion slots |  | One AGP slot, five PCI Master Slots and two 16-bit ISA Slots. |
| Voltage |  | Auto 1.5V-3.5V |
| Dimension |  | 4-layer PCB, size (300mm x 190mm). |
| Others |  | Support BIOS setting CPU type (Jumper-less), CPU Auto Temperature Sensor & Music Alarm, voltage monitor and CPU Fan monitor, Bus Master/ Ultra DMA/33, ACPI, AGP Bus, Keyboard Power On, PS/2 Mouse Power On, Modem Ring On, LAN wake up, Voice & Debug sensor on board. |

# Setup Guide

## Layout Diagram

JP2 : Ex.Debug display connector.

Jp3 : Short=On board debug display

Open=Ex.Debug display

## Voice / LED Debug On Board & S1 set up

When the CPU, DRAM, Cache RAM, FDD or VGA card have not been properly installed, user can isolate those basic problems through the Debug Sensor LED and instructions from the manual.

| Error Message | LED Error code | Voice Debug |
| --- | --- | --- |
| DRAM defect or improper connection | C1\C6 | ### Please check memory. |
| CPU defect or improper connection | FF | ### Please check CPU. |
| Display card defect or improper connection | OD | ### Please check display card or memory. |
| Cache RAM Defect | 61 | Please check Cache RAM |
| Keyboard defect | 05 | Please check keyboard. |
| Floppy defect or improper connection | 4E | Please check Floppy Disk Drive connector or BIOS. |
| When CPU temperature over the set point. |  | Your CPU temperature is too high. |
| When Mainboard was seriously demaged in some of components. | Refer to page 52-56 for other Error Codes. | The Mainboard components may be demaged, please sent it bck to your supplier for repairing. |

S1 Set up

In case the CPU you use is belonged old style - Multiplier type, you may refer to following set up. All the new frequency-fixed CPU are Auto-Detect so that

don’t need to set up S1.

S1

| Pin 1 | Pin 2 | Pin 3 | Pin 4 | Multiplier |
| --- | --- | --- | --- | --- |
| Off | Off | Off | Off | X2 |
| On | On | On | Off | X2.5 |
| On | On | Off | On | X3 |
| On | On | Off | Off | X 3.5 |
| On | Off | On | On | X 4 |
| On | Off | On | Off | X 4.5 |
| On | Off | Off | On | X 5 |
| On | Off | Off | Off | X 5.5 |

| ROM PCI/ISA BIOS (2A69KTJ9) CHIPSET FEATURE SETUP AWARD SOFTWARE, INC |  |  |  |
| --- | --- | --- | --- |
| Auto Configuration | : Enabled | Auto Detect DIMM/ PCI Clk | : Enabled |
| EDO DRAM Speed Selection | : 60ns |  |  |
| EDO CASx# MA Wait State | : 2 |  |  |
| EDO RASx# Wait State | : 2 |  |  |
| SDRAM RAS-to CAS Delay | : 3 | Spread Spectrum | : Disabled |
| SDRAM RAS Precharge Time | : 3 | CPU Warning Temperature | : Disabled |
| SDRAM CAS latency Time | : Auto | Current CPU Temperature | : 28℃/ 82℉ |
| DRAM Data Integrity Mode | : Non-ECC | Current SYSFAN Speed | :4285 RPM |
| System BIOS Cacheable | : Enabled | Current CPUFAN Speed | :4000 RPM |
| Video BIOS Cacheable | : Enabled | Current Vin3 (V) | : 2.88V |
| Video RAM Cacheable | : Disabled |  |  |
| 8 Bit I/O Recovery Time | : 1 |  |  |
| 16 Bit I/O Recovery Time | : 1 |  |  |
| Memory Hole At 15M-16M | : Disabled |  |  |
| Passive Release | : Enabled | Esc : Quit  Selection : Item |  |
| Delayed Transaction | : Disabled | F1 : Help PU/PD/+/- : Modify |  |
| AGP Aperture Size (MB) | : 64 | F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Default F7 : Load Setup Default |  |

Dear Customers:

Thank you for your patronage of our products. The board you bought is a jumper-less mainboard. The ratio and frequency of the CPU shall be set in BIOS and the working voltage for the CPU shall be automatically detected. Please read carefully the following instructions:

1. Power on the installed system and press the "DEL" key to enter BIOS Setup. Select

"Chipset Features Setup" and press <Enter>.

2. Select "CPU Speed" and press "PgUp" or "PgDn" to set the CPU ratio and frequency. The available options are Intel PII

233MHz (66X3.5), 266MHz (66X4), 300MHz (66X4.5), 333MHz (66X5), 366MHz (66X5.5), 400MHz(66X6), 433MHz(66X6.5), 466MHz(66X7), 500MHz(66X7.5).

250MHz(100X2.5), 300MHz(100X3), 350MHz(100X3.5), 400MHz(100X4), 450MHz(100X4.5), 500MHz(100X5), 550MHz(100X5.5), 600MHz(100X6) and "Manual".

To set the CPU manually, please note the following:

CPU Speed: "Manual" (you can manually set the CPU ratio and frequency)

CPU Ratios: x3.5, x4, x4.5, x5, x5.5, x6, x6.5, x7, x7.5

CPU Frequency: 66, 68, 75, 83, 100, 103, 112, 133Mhz

Several options are provided for the CPU external clock. You are recommended to restore to the default setting in case of instability when the external clock exceeds 66MHz.

NOTE: System failure may occur if the CPU frequency is set incorrectly. To solve this problem. Press the "Insert" key on the keyboard to clear the previously set frequency (i. c., restore the default frequency), and then reboot the system.

- Switch voltage is applied, making the temperature lower and voltage steadier.

- You don’t need to adjust Voltage in Pentium II mainboard. It will automatically send out one VID (Voltage Identification) to the mainboard power supply to ask for the voltage it needs.

- The CPU type default setting is Intel Pentium II 250MHz=100 MHz＊2.5.

Intel Pentium II\III CPU family

| CPU | Ext. clk | Ratio | L1 cache | L2 cache | Voltage |
| --- | --- | --- | --- | --- | --- |
| Intel Pentium III – 600MHz | 133MHz | X4.5 | 32KB | 512KB | 2.8V |
| Intel Pentium III – 533MHz | 133MHz | X4 | 32KB | 512KB | 2.8V |
| Intel Pentium III – 600MHz | 100MHz | X6 | 32KB | 512KB | 2.8V |
| Intel Pentium III – 550MHz | 100MHz | X5.5 | 32KB | 512KB | 2.8V |
| Intel Pentium III – 500MHz | 100MHz | X5 | 32KB | 512KB | 2.8V |
| Intel Pentium III – 450MHz | 100MHz | X4.5 | 32KB | 512KB | 2.8V |
| Intel Pentium II – 450MHz | 100MHz | X4.5 | 32KB | 512KB | 2.8V |
| Intel Pentium II – 400MHz | 100MHz | X4 | 32KB | 512KB | 2.8V |
| Intel Pentium II – 350MHz | 100MHz | X3.5 | 32KB | 512KB | 2.8V |
| Intel Pentium II – 300MHz | 100MHz | X3 | 32KB | 512KB | 2.8V |
| Intel Pentium II – 333MHz | 66MHz | X5 | 32KB | 512KB | 2.8V |
| Intel Pentium II – 300MHz | 66MHz | X4.5 | 32KB | 512KB | 2.8V |

## EDO/ SDRAM Installation  Procedures:

- A 168-pin DIMM can support up to 768MB 3.3V EDO (66MHz) / SDRAM (66MHz/ 100MHz).

- You are recommended to use SDRAMs. With SPD that are compliant with PC-100. This will enable BIOS to detect the SDRAM speed, thereby fully bring into play the efficiency of the SDRAM.

- To avoid compatibility and reliability problems, you are recommended to test the 168-pin SDRAMs before buying them since the PCB specifications differ.

- The following table shows the memories that have passed the PC-100 specifications.

| Brand | Memory size | Number |
| --- | --- | --- |
| TOSHIBA | 8M X 08=64MB | TC59S6408BFT-80 |
| TOSHIBA | 4M X 16=64MB | TC59S6416BFT-80 |
| HITACHI | 8M X 08=64MB | HM5264805TT-B60 |
| HITACHI | 4M X 16=64MB | HM5264165TT-B60 |
| MITSUBISHI | 8M X 08=64MB | M5M4V64S30ATP-8 |
| MITSUBISHI | 4M X 16=64MB | M5M4V64S40ATP-8 |
| MITSUBISHI | 2M X 08=16MB | M5M4V16S30DTP-8 |
| FUJITSU | 8M X 08=64MB | 81F64842B-103FN |
| FUJITSU | 4M X 16=64MB | 81F641642B-103FN |
| TI | 8M X 08=64MB | TMX664814A81A7ET |
| TI | 2M X 08=16MB | TMX626812BDGE5M |
| SAMSUNG | 8M X 08=64MB | KM48S8030BT-GH |
| SAMSUNG | 4M X 16=64MB | KM416S4030BT-G10 |
| SAMSUNG | 2M X 08=16MB | KM48S2020CT-GL |
| HYUNDAI | 8M X 08=64MB | HY57V658020ALTC-10P |
| HYUNDAI | 2M X 08=16MB | HY57V168010C TC-10S |
| LG | 8M X 08=64MB | GM72V66841CT7J |
| LG | 4M X 16=64MB | GM72V661641CT-7J |

- First, verify the working voltage of the EDO/ SDRAM module in either DIMM socket.

- P2BX only supports 3.3V EDO/ SDRAM module. The following illustration shows you the difference between 3.3V and 5V to ensure your correct selection of 3.3V DIMM module for using.

- You can set up the BIOS “Chipset Feature Setup” to the best working condition basing on the type of EDO/ SDRAM you are using.

- The BIOS DRAM default setting is 60 ns. Change the BIOS “Chipset Feature Setup” default setting to 50ns for better performance, if the chipset is marked 50ns.

- Change nothing if EDO RAM is used. BIOS automatically detect the RAM type.

- MEMO for Installing System:

⊕ Concerning memory setup, you can find how to from “Chipset Feature Setup” under BIOS setup. However, to avoid system unstable or system hang, user without engineering background is not suggested to change BIOS set up.

⊕ If system boot failure, please clean DIMM socket (with clean oil) or polish Gold-Finger of DRAM with soft eraser, and try again.

- The Dual Inline Memory Module (DIMM) must be 3.3 Volt and Unbuffered Synchronus DRAM (SDRAM) 8MB, 16MB, 32MB, 64MB, 128MB or 256MB. The following illustration shows the type of DIMM Module.

168-PIN SDRAM DIMM Notch Key Definitions

## Other Jumper Settings

- Speaker: Connect to the system’s speaker for beeping.

- Keylock: Keyboard lock switch and Power LED connector.

- Reset: Short to restart system.

- HDD LED: LED ON when on board PCI IDE hard disk activates.

- POWER SW (FOR ATX POWER SUPPLY):

- The button should be a momentary switch that is normally open. Pushing the ATX Power Switch will immediately change the system status. Before or during “POST”, you need to hold the button for four seconds in order to turn off the system.

- J19: Clear CMOS

- Turn off the system and short pins 2-3 to clear CMOS. Then short pins 1-2 before turning it on.

| - J19 |  |
| --- | --- |
| - 1-2 | - Normal operation(Default). |
| - 2-3 | - for clearing CMOS Data. |

- CPU Cooler Fan connector

- This is the connector for CPU cooler. Never use the jumper to short the connector. Serious damages caused this way will not be warrantied.

- Creative SB-Link

It is used to connect the AWE64 or other compatible sound cards so that they are made compatible with the ISA-compatible SB16 sound cards.

## Note to BIOS Update

- Do not update the BIOS if no abnormalities occur. However, if BIOS update is needed, consult your dealer first. Prior to updating your BIOS, you are recommended to save the original BIOS values.

- 1. Download the AWARD BIOS Flash Utility file (Awdflash.exe)

- 2. Download the BIOS file used by your mainboard(e.g., BXV110N.BIN)

- 3. Reboot your system (but do not run Himem.sys and Emm386.exe) to

- execute the new BIOS program.

- 4. Execute these commands: Awdflash BXV110N.BIN

- 5. When this message displays: "Do you want to save BIOS (Y/N)?"

- Type "N"

- 6. When this message displays: "Are you sure to program (Y/N)?"

- Type "Y"

- 7. Turn off power to your system to clear the CMOS data.

- 8. Turn on the power to test if your system is running normal.

## Keyboard/ PS/2 Mouse Power On and MODEM Ring on

- To make sure the 5VSB signal nearly to 750mA (Amperage) from ATX Power Supply, or if your keyboard consuming power than 300mA, it's better to upgrade your ATX Power Supply to 1A for working perfectly.

- If you are going to use the function of keyboard and PS/2 mouse power on, then, the power-switch will be becoming useless automatically (unable to be used).

| ROM PC/ISA BIOS (2A69KTJ9) INTEGRATED PERIPHERALS AWARD SOFTWARE, INC. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| IDE HDD Block Mode |  | : Enabled |  | Onboard Serial Port 2 | : 2F8H / IRQ3 |  |
| IDE Primary Master PIO |  | : AUTO |  | UART Mode Select | : Normal |  |
| IDE Primary Slave PIO |  | : AUTO |  |  |  |  |
| IDE Secondary Master PIO |  | : AUTO |  | Onboard Parallel Port | : 378H/IRQ 7 |  |
| IDE Secondary Slave PIO |  | : AUTO |  | Parallel Port Mode | : ECP+EPP |  |
| IDE Primary Master UDMA |  | : AUTO |  | ECP Mode Use DMA | : 3 |  |
| IDE Primary Slave UDMA |  | : AUTO |  | EPP Mode Select | : EPP 1.9 |  |
| IDE Secondary Master UDMA |  | : AUTO |  |  |  |  |
| IDE Secondary Slave UDMA |  | : AUTO |  |  |  |  |
| On-Chip Primary PCI IDE |  | : Enabled |  |  |  |  |
| On-Chip Secondary PCI IDE |  | : Enabled |  |  |  |  |
| USB Keyboard Support |  | : Disabled |  |  |  |  |
| Init AGP Display First |  | : Enabled |  |  |  |  |
| POWER ON Function |  | : Hot KEY |  |  |  |  |
|  |  |  |  | Esc : Quit Selection : Item |  |  |
| Hot Key Power ON |  | : Ctrl-F12 |  | F1 : Help PU/PD/+/- : Modify |  |  |
| KBC input clock |  | : 8MHz |  | F5 : Old Values (Shift)F2 : Color |  |  |
| Onboard FDC Controller |  | : Enabled |  | F6 : Load BIOS Default |  |  |
| Onboard Serial Port 1 | : 3F8H / IRQ4 |  | F7 : Load Setup Default |  |  |  |

| Hot KEY | When user select this option, it will show another line lines as Hot Key Power ON: Ctrl-F(1/2/3/4/5/6/7/8/9/10/11/12) select any you like. After power off, if user key in the "Ctrl-F?", it will power on the system. |
| --- | --- |
| PS/2 Mouse Left | It will power on the system by PS/2 mouse left. |
| PS/2 Mouse Right | It will power on the system by PS/2 mouse Right. |
| Button Only | Only the power button can power on the system. |

- Modem Ring On Function Operation:

| ROM PCI / ISA BIOS (2A69KTJ9) POWER MANAGEMENT SETUP AWARD SOFTWARE, INC |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Power Management | : Disabled | ** Reload Global Timer Events ** |  |  |  |  |  |
| PM Control by APM | : No | IRQ[3-7, 9-15], NMI | : Enabled |  |  |  |  |
| Video Off Method | : V/H SYNC+Blank | Primary IDE 0 | : Disabled |  |  |  |  |
| Video Off After | : Standby | Primary IDE 1 | : Disabled |  |  |  |  |
| MODEM Use IRQ | : 3 | Secondary IDE 0 | : Disabled |  |  |  |  |
| Doze Mode | : Disabled | Secondary IDE 1 | : Disabled |  |  |  |  |
| Standby Mode | : Disabled | Floppy Disk | : Disabled |  |  |  |  |
| Suspend Mode | : Disabled | Serial Port | : Enabled |  |  |  |  |
| HDD Power Down | : Disabled | Parallel Port | : Disabled |  |  |  |  |
| Throttle Duty Cycle | : 62.5% |  |  |  |  |  |  |
| ZZ Active in Suspend | : Disabled |  |  |  |  |  |  |
| VGA Active Monitor | : Enabled |  |  |  |  |  |  |
| Soft-Off by PWR-BTTN | : Instant-Off |  |  |  |  |  |  |
| CPUFAN Off In Suspend | : Enabled |  |  |  |  |  |  |
| Resume by Ring | : Enabled |  |  |  |  |  |  |
| IRQ 8 Clock Event | : Disabled | Esc : Quit Selection Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Default F7 : Load Setup Default |  |  |  |  |  |

- 1. Have an external MODEM connected to COM 1 or COM 2.

- 2. Enter BIOS setup.

- 3. Select Power Management Setup.

- 4. This number of MODEM use IRQ has to be set as same as the IRQ of

- Serial Port which you are connecting in. Please set in N/A if you are

- not going to use the function of MODEM ring on.

- 5. Resume by Ring: Enable.

- 6. Save BIOS setup and Reboot.

- 7. Booting from DOS, Windows, or Windows 95.

- 8. Turn off the system by:

- a. ATX-Power Switch

- b. Windows 95 Software Power Off

- 9. System Waiting for Modem Ring On

- When Modem Ringing Signal Active, System will wake-up.

## System Health Monitor

- Fan Monitoring:

There are two fan connectors, one is for CPU, the other can be a housing fan. When the fans speed is working abnormal, there will be warning (Speaker Alarm) through application software such as SM10(Small Icon for System Monitoring) to notify user. The fan monitoring function is implemented by connecting fan to 3-pin fan connector FAN1/ FAN2 and installing SM10. Referring to Page 19 (System Health Monitor).

- CPU Thermal Protection:

TM-P2BX implements special thermal protection circuits. When temperature is higher than a predefined value, there will be warning (Speaker Alarm) through application software such as SM10 (Small Icon for System Monitor) to notify user. It's automatically implemented by BIOS or SMD10, no hardware installation is needed. Referring to Page19 (System Health Monitor).

This mainboard also reserves an option to use special CPU cooling Fan. With Thermal Sensor on it. The CPU thermal sensor should be connected to J16.

- System Voltage Monitoring:

TM-P2BX is featured with a voltage monitoring system. When you turn on your system, this smart design will keep on monitoring your system working voltage. If any of voltage is over the component's standard, there will be Speaker Alarm though application software SM10 (Small Icon For System Monitor) for a warning to user. System voltage monitoring function monitors 5V, 12V, 3.3V and CPU voltage. It's automatically implemented by BIOS and SM10, no hardware installation is needed. Referring to Page 19 (System Health Monitor)

## EDO/ SDRAM Configuration Table:

| DIMM1 | DIMM2 | DIMM3 | TOTAL |
| --- | --- | --- | --- |
| 8MB | 8MB | --- | 16MBytes |
| --- | 8MB | 8MB | 16MBytes |
| 8MB | --- | 8MB | 16MBytes |
| --- | 16MB | --- | 16MBytes |
| 8MB | 8MB | 8MB | 24MBytes |
| 16MB | 8MB | --- | 24MBytes |
| --- | 16MB | 8MB | 24MBytes |
| 16MB | --- | 8MB | 24MBytes |
| 16MB | 16MB | --- | 32MBytes |
| --- | 16MB | 16MB | 32MBytes |
| 16MB | --- | 16MB | 32MBytes |
| --- | 32MB | --- | 32MBytes |
| 16MB | 16MB | 8MB | 40MBytes |
| 8MB | 16MB | 16MB | 40MBytes |
| 32MB | 8MB | --- | 40Mbytes |
| --- | 32MB | 8MB | 40MBytes |
| 16MB | 16MB | 16MB | 48MBytes |
| 32MB | 8MB | 8MB | 48MBytes |
| 32MB | 16MB | --- | 48MBytes |
| --- | 32MB | 16MB | 48MBytes |
| 32MB | 32MB | --- | 64MBytes |
| --- | 32MB | 32MB | 64MBytes |
| --- | 64MB | --- | 64MBytes |
| 32MB | 16MB | 16MB | 64MBytes |
| 32MB | 32MB | 8MB | 72MBytes |
| 8MB | 32MB | 32MB | 72MBytes |
| 64MB | 8MB | --- | 72Mbytes |
| --- | 64MB | 8MB | 72MBytes |
| 32MB | 32MB | 16MB | 80MBytes |
| 16MB | 32MB | 32MB | 80MBytes |
| 64MB | 16MB | --- | 80MBytes |
| --- | 64MB | 16MB | 80MBytes |

| 32MB | 32MB | 32MB | 96MBytes |
| --- | --- | --- | --- |
| 64MB | 32MB | --- | 96MBytes |
| --- | 64MB | 32MB | 96MBytes |
| 16MB | 64MB | 16MB | 96MBytes |
| 64MB | 64MB | --- | 128MBytes |
| --- | 64MB | 64MB | 128MBytes |
| 64MB | 32MB | 32MB | 128MBytes |
| --- | 128MB | --- | 128MBytes |
| 64MB | 64MB | 8MB | 136MBytes |
| 8MB | 64MB | 64MB | 136MBytes |
| 128MB | 8MB | --- | 136MBytes |
| --- | 128MB | 8MB | 136MBytes |
| 64MB | 64MB | 16MB | 144MBytes |
| 16MB | 64MB | 64MB | 144MBytes |
| 128MB | 16MB | --- | 144Mbytes |
| --- | 128MB | 16MB | 144MBytes |
| 64MB | 64MB | 32MB | 160MBytes |
| 32MB | 64MB | 64MB | 160MBytes |
| 128MB | 32MB | --- | 160MBytes |
| --- | 128MB | 32MB | 160MBytes |
| 64MB | 64MB | 64MB | 192MBytes |
| 128MB | 64MB | --- | 192MBytes |
| --- | 128MB | 64MB | 192MBytes |
| 32MB | 128MB | 32MB | 192MBytes |
| 128MB | 128MB | --- | 256MBytes |
| --- | 128MB | 128MB | 256MBytes |
| 64MB | 128MB | 64MB | 256MBytes |
| --- | 256MB | --- | 256MBytes |
| 128MB | 128MB | 8MB | 264MBytes |
| 256MB | --- | 8MB | 264MBytes |
| 256MB | 8MB | --- | 264MBytes |
| --- | 256MB | 8MB | 264MBytes |

| 128MB | 128MB | 32MB | 288MBytes |
| --- | --- | --- | --- |
| 256MB | --- | 32MB | 288MBytes |
| 256MB | 32MB | --- | 288MBytes |
| --- | 256MB | 32MB | 288MBytes |
| 128MB | 128MB | 64MB | 320MBytes |
| 256MB | --- | 64MB | 320MBytes |
| 256MB | 64MB | --- | 320MBytes |
| --- | 256MB | 64MB | 320MBytes |
| 128MB | 128MB | 128MB | 384MBytes |
| 256MB | --- | 128MB | 384MBytes |
| 256MB | 128MB | --- | 384MBytes |
| --- | 256MB | 128MB | 384MBytes |
| 128MB | 128MB | 256MB | 512MBytes |
| 256MB | --- | 256MB | 512MBytes |
| 256MB | 256MB | --- | 512Mbytes |
| --- | 256MB | 256MB | 512MBytes |
| 256MB | 256MB | 8MB | 520MBytes |
| 256MB | 8MB | 256MB | 520MBytes |
| 8MB | 256MB | 256MB | 520MBytes |
| 256MB | 256MB | 16MB | 528MBytes |
| 256MB | 16MB | 256MB | 528MBytes |
| 16MB | 256MB | 256MB | 528MBytes |
| 256MB | 256MB | 32MB | 544MBytes |
| 256MB | 32MB | 256MB | 544MBytes |
| 32MB | 256MB | 256MB | 544MBytes |
| 256MB | 256MB | 64MB | 576MBytes |
| 256MB | 64MB | 256MB | 576MBytes |
| 64MB | 256MB | 256MB | 576MBytes |
| 256MB | 256MB | 128MB | 640MBytes |
| 256MB | 128MB | 256MB | 640MBytes |
| 128MB | 256MB | 256MB | 640MBytes |
| 256MB | 256MB | 256MB | 768MBytes |