# Technical information

| ## I/O Connector Map |  |  |  |  | ISA Bus |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | GND | 1 |  | 1 | -I/OCH CHK |
|  |  |  |  |  | RESET | 2 |  | 2 | SD 07 |
|  |  |  |  |  | +5V | 3 |  | 3 | SD 06 |
| Floppy Disk Connector |  |  |  |  | IRQ 9 | 4 |  | 4 | SD 05 |
|  |  |  |  |  | -5V | 5 |  | 5 | SD 04 |
|  |  |  |  |  | DRQ2 | 6 |  | 6 | SD 03 |
|  |  |  |  |  | -12V | 7 |  | 7 | SD 02 |
| Ground | 1 |  | 2 | FDHDIN | OWS | 8 |  | 8 | SD 01 |
| Ground | 3 |  | 4 | Reserved | +12V | 9 |  | 9 | SD 00 |
| Ground | 5 |  | 6 | FDEDIN | GND | 10 |  | 10 | -I/O CH RDY |
| Ground | 7 |  | 8 | -Index | -SMEMW | 11 |  | 11 | AEN |
| Ground | 9 |  | 10 | Motor Enable | -SMEMR | 12 |  | 12 | SA 19 |
| Ground | 11 |  | 12 | -Driver selectB | -IOW | 13 |  | 13 | SA 18 |
| Ground | 13 |  | 14 | -Driver selectA | -IOR | 14 |  | 14 | SA 17 |
| Ground | 15 |  | 16 | Motor Enable | -DACK3 | 15 |  | 15 | SA 16 |
| Ground | 17 |  | 18 | -DIR | -DRQ3 | 16 |  | 16 | SA 15 |
| Ground | 19 |  | 20 | -STEP | -DACK1 | 17 |  | 17 | SA 14 |
| Ground | 21 |  | 22 | Write Data | -DRQ1 | 18 |  | 18 | SA 13 |
| Ground | 23 |  | 24 | Write Gate | -REFRESH | 19 |  | 19 | SA 12 |
| Ground | 25 |  | 26 | -Track 00 | BCLK | 20 |  | 20 | SA 11 |
| Ground | 27 |  | 28 | -Write Protect | IRQ 7 | 21 |  | 21 | SA 10 |
| Ground | 29 |  | 30 | -Read Data | IRQ 6 | 22 |  | 22 | SA 09 |
| Ground | 31 |  | 32 | -Side 1 select | IRQ 5 | 23 |  | 23 | SA 08 |
| Ground | 33 |  | 34 | Diskette | IRQ 4 | 24 |  | 24 | SA 07 |
|  |  |  |  |  | IRQ 3 | 25 |  | 25 | SA 06 |
|  |  |  |  |  | -DACK2 | 26 |  | 26 | SA 05 |
| IDE Connector |  |  |  |  | T/C | 27 |  | 27 | SA 04 |
|  |  |  |  |  | BALE | 28 |  | 28 | SA 03 |
|  |  |  |  |  | +5V | 29 |  | 29 | SA 02 |
|  |  |  |  |  | OSC | 30 |  | 30 | SA 01 |
| Reset IDE | 1 |  | 2 | Ground | GND | 31 |  | 31 | SA 00 |
| HOST Data 7 | 3 |  | 4 | HOST Data 8 |  |  |  |  |  |
| HOST Data 6 | 5 |  | 6 | HOST Data 9 | -MEMCS 16 | 1 |  | 1 | SBHE |
| HOST Data 5 | 7 |  | 8 | HOST Data10 | -I/OCS16 | 2 |  | 2 | LA 23 |
| HOST Data 4 | 9 |  | 10 | HOST Data11 | IRQ 10 | 3 |  | 3 | LA 22 |
| HOST Data 3 | 11 |  | 12 | HOST Data12 | IRQ 11 | 4 |  | 4 | LA 21 |
| HOST Data 2 | 13 |  | 14 | HOST Data13 | IRQ 12 | 5 |  | 5 | LA 20 |
| HOST Data 1 | 15 |  | 16 | HOST Data14 | IRQ 15 | 6 |  | 6 | LA 19 |
| HOST Data 0 | 17 |  | 18 | HOST Data15 | IRQ 14 | 7 |  | 7 | LA 18 |
| Grund | 19 |  | 20 | Key | -DACK 0 | 8 |  | 8 | LA 17 |
| DRQ 3 | 21 |  | 22 | Ground | DRQ 0 | 9 |  | 9 | -MEMR |
| -I/O Write | 23 |  | 24 | Ground | -DACK 5 | 10 |  | 10 | -MEMW |
| -I/O Read | 25 |  | 26 | Ground | DRQ 5 | 11 |  | 11 | SD 08 |
| IOCHRDY | 27 |  | 28 | BALE | -DACK 6 | 12 |  | 12 | SD 09 |
| -DACK 3 | 29 |  | 30 | Ground | DRQ 6 | 13 |  | 13 | SD 10 |
| IRQ 14 | 31 |  | 32 | -IOCS16 | -DACK 7 | 14 |  | 14 | SD 11 |
| Addr 1 | 33 |  | 34 | Ground | DRQ 7 | 15 |  | 15 | SD 12 |
| Addr 2 | 35 |  | 36 | Addr2 | +5V | 16 |  | 16 | SD 13 |
| -Chip select 0 | 37 |  | 38 | -Chip select 1 | -MASTER | 17 |  | 17 | SD 14 |
| Activity | 39 |  | 40 | Ground | GND | 18 |  | 18 | SD 15 |

| ATX POWER SUPPLY Connector |  |  |  |  | Parallel port connector |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3.3 V | 11 |  | 1 | 3.3 V |  |  |  |  |  |
| -12 V | 12 |  | 2 | 3.3 V | -STROBE | 1 |  | 14 | -AUTO FEED |
| GND | 13 |  | 3 | GND | Data Bit 0 | 2 |  | 15 | -ERROR |
| PS-ON | 14 |  | 4 | 5 V | Data Bit 1 | 3 |  | 16 | -INIT |
| GND | 15 |  | 5 | GND | Data Bit 2 | 4 |  | 17 | -SLCT IN |
| GND | 16 |  | 6 | 5 V | Data Bit 3 | 5 |  | 18 | Ground |
| GND | 17 |  | 7 | GND | Data Bit 4 | 6 |  | 19 | Ground |
| -5 V | 18 |  | 8 | PW-OK | Data Bit 5 | 7 |  | 20 | Ground |
| 5 V | 19 |  | 9 | 5 V-SB | Data Bit 6 | 8 |  | 21 | Ground |
| 5 V | 20 |  | 10 | 12 V | Data Bit 7 | 9 |  | 22 | Ground |
| Serial port connector |  |  |  |  | -ACJ | 10 |  | 23 | Ground |
|  |  |  |  |  | BUSY | 11 |  | 24 | Ground |
|  |  |  |  |  | PE | 12 |  | 25 | Ground |
|  |  |  | 1 | DCD | SLCT | 13 |  |  |  |
| DSR | 6 |  | 2 | SIN |  |  |  |  |  |
| RTS | 7 |  | 3 | SOUT | PS/2 mouse connector signal line |  |  |  |  |
| CTS | 8 |  | 4 | DTR |  |  |  |  |  |
| RI | 9 |  | 5 | GND | Data(Red) | 1 |  | 4 | NC |
|  |  |  |  |  | Clock(Blue) | 2 |  | 5 | VCC(Yellow) |
|  |  |  |  |  | GND(Green) | 3 |  |  |  |

## The transfer rate of IDE PIO and DMA modes.

| Mode | PCI Bus Clock | Cycle time | Data transfer rate |
| --- | --- | --- | --- |
| PIO Mode 0 | 33 MHz | 600 ns | 3.3 MB/s |
| PIO Mode 1 | 33 MHz | 383 ns | 5.2 MB/s |
| PIO Mode 2 | 33 MHz | 240 ns | 8.3 MB/s |
| PIO Mode 3 | 33 MHz | 180 ns | 11.1 MB/s |
| PIO Mode 4 | 33 MHz | 120 ns | 16.6 MB/s |
| PIO Mode 5 | 33 MHz | 90 ns | 20 MB/s |
| DMA Mode 0 | 33 MHz | 480 ns | 4.16 MB/s |
| DMA Mode 1 | 33 MHz | 150 ns | 13.3 MB/s |
| DMA Mode 2 | 33 MHz | 120 ns | 16.6 MB/s |
| DMA/33 | 33 MHz | 60 ns | 33 MB/s |

When IORDY signal is used, PIO Mode 3/4 is in ATA-2 format while PIO Mode 0/1/2 is in ATA format. PIO Mode5 is unlikely to happen.

## The Computer BOOT flow. chart

## The difference of Chipsets

|  | INTEL 440 BX | INTEL i810e | VIA 693A+596B |
| --- | --- | --- | --- |
| CPU support | Pentium II / III | Pentium II / III | Pentium II / III |
| Memory size | 1GB | 1GB | 1 GB |
| Memory | SDRAM | SDRAM | SDRAM |
| Memory Clock | 100 | 100 | 100/133MHz |
| Ex. Clock | 66/100MHz | 66/100/133MHz | 66/100/133 MHz |
| ACPI | Yes | Yes | ACPI |
| AGP Bus | X 2 | X 2 | X 4 |
| Ultra ATA/66 | No | Yes | Yes |

- Ultra ATA/66

It’s an extension of Ultra ATA33 interface.  This new high-speed interface has doubled the Ultra ATA/33 burst data transfer rate to 66.6 Mbytes/sec and maximized disk performance under current PCI local bus environment.

- ACPI (Advanced Configuration and Power Interface)

- This Advanced Configuration and Power Interface set up by Intel, Microsoft, Compaq, IBM, HP….etc, offers the functions below:

- . Automatically stops offering power to CD-ROM, FDD or HDD when              any of them is not in use.

- . Offers the “OnNow” function; when you start the system, what is seen on the screen is in the condition same as the last time before the system was shut down.

- . Enhances the system configuration like PnP, DMI….

- USB(Univer Serial Bus)

- The new Bus specification defined by Compaq, DEC, IBM, Microsoft, NEC, Intel ….etc, is connected by USB, making it possible for the peripheral to have the “plug and play” function without interface card. There are at most 127 peripherals that can be connected at the same time.

- Printer Modes /SPP /ECP /EPP

SPP (Standard Parallel Port)

The current commonly used standard mode.

ECP (Extended Capabilities Port)

Jointly set by Microsoft and HP. Its main feature is using high-performance half-duplex bi-directional channel to achieve faster transmission speed. Its 16-bit FIFO (First-In-First-Out) buffer makes high-speed transmission more stable and reliable. DMA function is included in its controller.

EPP (Enhanced Parallel Port)

Jointly set by Intel, Zenith and Xircom. Bi-directional block transmission makes transmission speed to reach 2MB per second. It is compatible with the standard parallel port interface. For printers that do not support the EPP mode in Windows 95, including the Canon BJ Series and the Epson LQ Series, you can set the Parallel Mode in BIOS to Normal (SPP) Mode to enable EPP.

- IrDA (Infrared Data Association)

This organization sets the infrared transmission standards. The IrDA Protocol sets transmission speed at 115KB per second and a transmission angle of 30 degree. Its Serial Port shall have 16550 UARTs and its maximum transmission distance is one meter.

- S. M. A. R. T (Self  Monitoring Analysis Report Technology)

It is jointly set by Conner, IBM, Quantum, Seagate and Western Digital.

Most hard disks on the market have this function. It issues a warning

message to the computer user prior to the "actual" failure of the user can

have sufficient time to backup data or to replace the hard disk.

## TIME and DMA CHANNELS MAP

TIME MAP:

TIMER Channel 0   	System timer interrupt.

TIMER Channel 1	DRAM REFRESH request.

TIMER Channel 2	SPEAKER tone generator.

DMA CHANNELS:	DMA Channel 0	Available.

DMA Channel 1	Audio.

DMA Channel 2	FLOPPY DISK.

DMA Channel 3	Onboard ECP (default).

DMA Channel 4	Cascade.                                                          	DMA Channel 5 	PCMCIA DMA.

DMA Channel 6       	MPEG.                                                          	DMA Channel 7       	Available.

## INTERRUPT  MAP

NMI:	Parity check error.

IRQ (H/W):		0  System TIMER interrupt form TIMER 0.

1  KEYBOARD output buffer full.

2  Cascade for IRQ 8-15.

3  SERIAL port 2.

4  SERIAL port 1.

5  Audio/MPU-401 or PARALLEL port 2.

6  FLOPPY DISK.

7  PARALLEL port 1.

8  RTC clock.

9  Available.

10  PCMCIA.

11  MPEG.

12  Trackpad (PS/2 Mouse).

13  MATH coprocessor.

14  Primary IDE interface (HDD).

15  Secondary IDE interrface (CD-ROM).

## RTC and CMOS RAM MAP

RTC and CMOS:	00	 Seconds

01	 Seconds alarm

02	 Minutes

03	 Minutes alarm

04	 Hours

05	 Hours alarm

06	 Day of week

07	 Day of month

08	 Month

09	 Year

0A	 Status register A

0B 	Status register B

0C	Status register C

0D 	Status register D

0E	Diagnostic status byte

0F 	Shutdown byte

10	FLOPPY DISK drive type byte

11	Reserve

12	HARD DISK type byte

13	Reserve

14	Equipment type

15	Base memory low byte

16	Base memory high byte

17	Extension memory low byte

18	Extension memory high byte

19-2d

2E-2F

30	Reserved for extension memory low byte

31	Reserved for extension memory high byte

32	DATE CENTURY byte

33	INFORMATION FLAG

35-3F	Reserve

40-7F 	Reserved for CHIPSET SETTING DATA