## POST Code

| POST (HEX) | Description |
| --- | --- |
| C0 | 1. Turn off OEM specific cache, shadow... 2. Initialize all the standard devices with default values standard devices including: - DMA controller (8237) - Programmable Interrupt Controller (8259) - Programmable Interval Timer (8254) |
| C1/C6 | Auto-detection of onboard DRAM and Cache |
| C3 | 1. Test the first 256K DRAM 2. Expand the compressed codes into temporary DRAM area including the compressed System BIOS and Option ROMs |
| C5 | Copy the BIOS from ROM into E0000-FFFFF shadow RAM so that POST will go faster |
| 01-02 | Reserved |
| 03 | Initialize EISA registers (EISA BIOS ONLY) |
| 04 | Reserved |
| 05 | 1. Keyboard Controller Self-Test 2. Enable Keyboard Interface |
| 06 | F000 shadow R/W test |
| 07 | Verifies CMOS‘s basfic R/W functionality |
| BE | Program defaults values into chipset according to the MODBINable Chipset Default Table |
| 09 | 1. Issue CPU ID instruction to idedtify CPU type 2. Program the configuration register of Cyrix CPU according to the MODBINable Cyrix Register Table 3. OEM specific cache initialization |
| 0A | 1. Initialize the first 32 interrupt vectors with correspoinding interrupt handlers Initialize INT no from 33-120 with Dummy (Spurious) Interrupt Handler 2. Early Power Management initialization (OEM specific) |
| 0B | 1. Verify whether RTC time is valid or not 2. Detect bad battery 3. Read CMOS data into BIOS stack area 4. PNP initializations including (PNP BIOS ONLY) - Assign CSN to PNP ISA card - Create resource map from ESCD 5. Update the P6 CPU‘s micro code (P6 Only) 6. Assign IO and Memory for PCI devices (PCI BIOS ONLY) |

| POST (HEX) | Description |
| --- | --- |
| 0C | Initialization of the BIOS Data Area (40:0-40:FF) |
| 0D | 1. P5 Multi-P BIOS Only Init IO and Local APIC 2. Program some of the Chipset‘s value according to Setup (Early Setup Value Program) 3. Measure CPU speed for display and decide system clock speed 4. Video initialization including Monochrome, CGA, EGA/VGA. If no display device found, the speaker will beep |
| 0E | 1. Initialize the APIC (Multi-Processor BIOS ONLY) 2. Test video RAM (If Monochrome display device found) 3. Show messages including: - Award Logo, Copyright String, BIOS Date code and Part No. - OEM specific sign on messages - Energy Star Logo (Green BIOS Only) - CPU brand, type and speed |
| 0F | DMA channel 0 test |
| 10 | DMA channel 1 test |
| 11 | DMA page registers test |
| 12-13 | Reserved |
| 14 | Test 8254 Timer 0 Counter 2 |
| 15 | Test 8259 interrupt mask bits for channel 1 |
| 16 | Test 8259 interrupt mask bits for channel 2 |
| 17 | Reserved |
| 19 | Test 8259 functionality |
| 1A-1D | Reserved |
| 1E | If EISA NVM checksum is good,, execute EISA initialization |
| 1F-29 | Reserved |
| 30 | 1. Get Base Memory and Extended Memory Size 2. P6 Multi-P BIOS Only Init IO & Local APIC 3. Program K5 CPU‘s Write Allocation |
| 31 | 1. Get Base Memory and Extended Memory Size 2. P6 Multi-P BIOS Only Init IO and Local APIC 3. Program K5 CPU‘s Write Allocation |
| 32 | 1. Display the Award Plug and Play BIOS Extension message (PNP BIOS ONLY) 2. Program all onboard super I/O chips (if any) including COM ports, LPT ports, FDD port... according to setup value 3. Program onboard audio devices |

| POST (HEX) | Description |
| --- | --- |
| 33-3B | Reserved |
| 3C | Set flag to allow users to enter CMOS Setup Utility |
| 3D | 1. Initialize Keyboard 2. Install PS/2 mouse 3. Build the INT 15h function E820H table 4. Build the PnP Device Node for total memory size |
| 3E | Try to turn on Level 2 cache Note: Some chipset may need to turn on the L2 cache in this stage. But usually, the cache is turned on later in POST 61h |
| 3F-40 | Reserved |
| BF | 1. Program the rest of the Chipset‘s value according to setup 2. If auto-configuration is enabled, program the chipset with predefined values in the MODBIN able Auto-Table |
| 41 | Initialize floppy disk drive controller |
| 42 | 1. Cut IRQ 12 connection if PS/2 mouse is not installed 2. Install IDE Hard Drives - Auto-detect HDDs - Build the AT compatible HDD table for Type 47 - Set PIO timing 3. Detect CD ROM on IDE Bus 4. Detect LS120 drive |
| 43 | If it is a PNP BIOS, initialize serial and parallel ports |
| 44 | Reserved |
| 45 | Initialize math coprocessor |
| 46-4D | Reserved |
| 4E | 1. If there is any error detected (such as video, kb.....), show all the error messages on the screen and wait for the user to 2. Enable “Far Hit” for Cyrix 6x86 CPU |
| 4F | 1. If password is needed, ask for password 2. Clear the Energy Star Logo (Green BIOS ONLY) |
| 50 | Write all the CMOS values currently in the BIOS stack area Back into the CMOS |
| 51 | Reserved |

| POST (HEX) | Description |  |
| --- | --- | --- |
| 52 |  | 1. Initialize all ISA ROMs 2. Later PCI initializations (PCI BIOS ONLY) - assign IRQ to PCI devices - initialize all PCI ROMs 3. Program shadows RAM according to setup settings 4. Program Parity according to Setup setting 5. Power Management Initialization - Enable/Disable global PM - APM interface initializtion |
| 53 |  | 1. If it is NOT a PNP BIOS, initialize serial and parallel port 2. Initialize time value in BIOS data area by translating the RTC time value into a timer tick value |
| 54-5F |  | Reserved |
| 60 |  | Setup Virus Protection (Boot Sector Protection) functionality According to setup setting |
| 61 |  | 1. Try to turn on Level 2 cache Note: if L2 cache is already turned on in POST 3D, this part will be skipped 2. Set the boot up speed according to setup setting 3. Last chance for chipset initialization 4. Last chance for power Management initialization (Green BIOS only) 5. Show the system configuration table |
| 62 |  | 1. Setup daylight saving according to setup value 2. Program the NUM Lock, typmatic rate and typmatic speed according to setup setting |
| 63 |  | 1. If there is any changes in the hardware configuration, update the ESCD information (PNP BIOS ONLY) 2. If there is any changes in the hardware configuration, update the DMI data pool (DMI BIOS ONLY) 3. Clear memory that have been used 4. Boot system via INT 19h |
| FF |  | System Booting. This means that the BIOS already pass the control right to the operating system |