# Award BIOS Setup

Award BIOS ROM has a built-in Setup program that allows users to modify the basic system configuration.  This information is stored in battery-backed RAM so that it retains the Setup information when the power is turned off.

Entering Setup

To enter the BIOS Setup, press <Del> during POST (Power-On-Self-Test).

Control Keys

Up Arrow 		Move to previous item

Down Arrow		Move to next item

Left Arrow		Move to the left item

Right Arrow		Move to the right item

Esc Key		Main Menu -- Quit and not to save changes to CMOS

Status Page setup menu and Option Page

Setup Menu -- Exit current page and return to Main Menu

PgUp Key		Increase the numeric value or make changes

PgDn Key		Decrease the numeric value or make changes

F1 Key	General help, only for Status Page Setup Menu and Option Setup Menu

F2 Key		Change color from total 16 colors

F3 Key		Calendar, only for Status Page Setup Menu

F4 Key		Reserved

F5 Key	Restore the previous CMOS value from BIOS, only for Option Page Setup Menu

F6 Key	Load the default CMOS value from BIOS default table, only for Option Page Setup Menu

F7 Key		Load the default

F8 Key 		Reserved

F9 Key		Reserved

F10 Key		Save all the CMOS changes, only for Main Menu

## Getting Help

Main Menu

The online description of the highlighted setup function is displayed at the bottom of the screen.

Status Page Setup Menu/Option Page Setup Menu

Press F1 to pop up a small help window that describes the appropriate keys to use and the possible selections for highlighted item.  To exit the Help Window press <Esc>.

## The Main Menu

Once you enter Award BIOS CMOS Setup Utility, the Main Menu will appear on the Screen..  Use  arrow keys to select  the desired  items, press <Enter> to select or enter a submenu.

| ROM PC/ISA BIOS (2A6LGTJE) CMOS SETUP UTILITY AWARD SOFTWARE, INC. |  |
| --- | --- |
| STANDARD CMOS SETUP BIOS FEATURES SETUP CHIPSET FEATURES SETUP POWER MANAGEMENT SETUP PNP/PCI CONFIGURATION LOAD BIOS DEFAULTS LOAD SETUP DEFAULTS | INTEGRATED PERIPHERALS SUPERVISOR PASSWORD USER PASSWORD IDE HDD AUTO DETECTION SAVE & EXIT SETUP EXIT WITHOUT SAVING |
| Esc : Quit  : Select Item F10 : Save & Exit Setup (Shift) F2 : Change Color |  |
| Time, Data, Hard Disk Type... |  |

### Standard CMOS Setup

The items in Standard CMOS Setup Menu are divided into several categories.  Each category includes no, one or more than one setup items.  Use the arrow keys to highlight the item and then use the <PgUp> or <PgDn> keys to select the value you want in each item.

| ROM PCI/ISA BIOS (2A6LGTJE) STANDARD CMOS SETUP AWARD SOFTWARE, INC |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date ( mm: dd: yy ) : Mon, Dec 6 1999 |  |  |  |  |
| Time ( hh: mm: ss ) : 17 : 59 : 44 |  |  |  |  |
| HARD DISKS TYPE SIZE CYLS HEAD PRECOMP LANDZ SECTOR MODE |  |  |  |  |
| ---------------------------------------------------------------------------------------------------------------------------- |  |  |  |  |
| Primary Master | : AUTO 0 0 0 0 0 0 AUTO |  |  |  |
| Primary Slave | : AUTO 0 0 0 0 0 0 AUTO |  |  |  |
| Secondary Master | : AUTO 0 0 0 0 0 0 AUTO |  |  |  |
| Secondary Slave | : AUTO 0 0 0 0 0 0 AUTO |  |  |  |
| Driver A : 1.44M, 3.5 in. |  |  |  |  |
| Driver B : None |  |  | Base Memory: 640K |  |
|  |  |  | Extended Memory: 130048K |  |
| Video : EGA/VGA |  |  | Other Memory: 384K |  |
| Halt On : All, But Keyboard |  |  | ----------------------------------------- |  |
|  |  |  | Total Memory: 131072K |  |
| Esc : Quit Selection Item PU/PD/+/- : Modify |  |  |  |  |
| F1 : Help |  | (Shift) F2 : Color |  |  |

### BIOS Features Setup

| ROM PCI/ISA BIOS (2A6LGTJE) BIOS FEATURE SETUP AWARD SOFTWARE, INC |  |  |  |  |
| --- | --- | --- | --- | --- |
| Virus Warning |  | : Disabled | Video BIOS Shadow | : Enabled |
| CPU Internal Cache |  | : Enabled | C8000-CBFFF Shadow | : Disabled |
| External Cache |  | : Enabled | CC000-CFFFF Shadow | : Disabled |
| CPU L2 Cache Checking |  | : Enabled | D0000-D3FFF Shadow D4000-D7FFF Shadow | : Disabled : Disabled |
| Quick Power On Self Test |  | : Disabled | D8000-DBFFF Shadow | : Disabled |
| Boot Sequence |  | : A, C, SCSI | DC000-DFFFF Shadow | : Disabled |
| Swap Floppy Drive |  | : Disabled |  |  |
| Boot Up Floppy Seek |  | : Enabled |  |  |
| Boot Up NumLock Status |  | : On |  |  |
| IDE HDD Block Mode |  | : Enabled |  |  |
| Gate A20 Option |  | : Fast |  |  |
| Memory Parity/ECC Check Typematic Rate Setting |  | : Disabled : Disabled |  |  |
| Typematic Rate (Chars/Sec) |  | : 6 |  |  |
| Typematic Delay (Msec) |  | : 250 | ESC: Quit elect Item |  |
| Security Option |  | : Setup | F1 : Help PU/PD/+/- : Modify |  |
| PCI/VGA Palette Snoop OS Select For DRAM > 64MB |  | : Disabled : Non-OS2 | F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Defaults F7 : Load Setup Defaults |  |

Virus Warning

This category flashes on the screen.  During and after system boots up, any attempt to write to the boot sector or partition table of the hard disk drive will halt the system and the following error message will appear, in the mean time , you can run anti-virus program to locate the problem.

!WARNING!

Disk boot sector is to be modified

Type "Y" to accept write or "N" to abort write

Award Software, Inc.

| Enabled: | Activate automatically when the system boots up causing a warning message to appear when anything attempts to access the boot sector or hard disk partition table. |
| --- | --- |
| Disabled: | No warning message to appear when anything attempt to access the boot sector or hard disk partition table. |

CPU Internal Cache/External Cache

These two categories speed up memory access.  However,  it depends on CPU/chipset design.  The default value is Enabled.

| Enabled: | Enabled cache |
| --- | --- |
| Disabled: | Disabled cache |

Quick Power On Self Test

This category speeds up Power-On-Self-Test (POST) after you power on the computer.  If it is set to Enable, BIOS will shorten or skip some check items during POST.

| Enabled: | Enable quick POST |
| --- | --- |
| Disabled: | Normal POST |

Boot Sequence

This category determines which drive computer searches first for the hard disk operation system (i.e., DOS).

A, C, SCSI / C, A, SCSI / C, CDROM, A / CDROM, C, A / D, A, SCSI / E, A, SCSI / SCSI, A, C / SCSI, C, A / C only / LS120, C: System will first search drive for BOOT, and then next other drive, if first driver does not boot.

Swap Floppy Drive

Users can enable this item so that the BIOS will see the hardware "Drive A:" as "Drive B:"” , and hardware "Drive B:"” as  "Drive A:"”.

Boot Up Floppy Seek

During POST, BIOS will determine if the floppy disk drive installed is 40 or 80 tracks.  360K type is 40 tracks while 720K, 1.2M and 1.44M drive types are all 80 tracks.

| Enabled: | BIOS searches for floppy disk drive to determine if it is 40 or 80 tracks. Note that BIOS cannot tell from 720K, 1.2M or 1.44M drive type as they are all 80 tracks. |
| --- | --- |
| Disabled: | BIOS will not search for the type of floppy disk drive by track number. Note that there will not be any warning message if the drive installed is 360KB. |

Boot Up NumLock Status: The default value is On.

| On: | Keypad is number keys |
| --- | --- |
| Off: | Keypad is arrow keys |

IDE HDD Block Mode:

This allows your hard disk controller to use the fast block mode to transfer data to and from your hard disk drive (HDD).

| Enabled: | IDE controller uses block mode. |
| --- | --- |
| Disabled | IDE controller uses standard mode. |

Gate A20 Option

The  Gate A20 Option default setting is “fast.”.  This is the optimum setting for this mainboard.

Typematic Rate Setting

This determines the typematic rate.

| Enabled: | Enable typematic rate |
| --- | --- |
| Disabled: | Disable typematic rate |

Typematic Rate (Chars/Sec)

6:   6 characters per second         	  8:  8 characters per second

10: 10 characters per second      	12: 12 characters per second

15: 15 characters per second             	20: 20 characters per second

24: 24 characters per second            	30: 30 characters per second

Typematic Delay (Msec)

It  indicates the time between the first and second character displayed  when you hold a key.

250	: 250 msec      	     500	: 500 msec

750	: 750 msec                               1000	: 1000 msec

Security Option

This category allows you to limit access to the system and Setup, or just to Setup.

| System: | The system will not boot and access to Setup will be denied if the correct password is not entered at the prompt. |
| --- | --- |
| Setup: | The system will boot, but access to Setup will be denied if the correct password is not entered at the prompt. |

NOTE: To disable security, select PASSWORD SETTING at Main Menu and then you will be asked to enter password.  Do not type anything and  press <Enter> to disable security.  Once the security is disabled, the system will boot and you can enter Setup freely.

PCI/ VGA Palette Snoop

It determines whether the MPEG ISA/ VESA VGA cards can work with PCI/ VGA or not.

| Enabled: | When PCI/ VGA working with MPEG ISA/ VESA Card. |
| --- | --- |
| Disabled: | When PCI/ VGA not working with MPEG ISA/ VESA Card. |

OS Select for DRAM>64MB

This item allows you to access the memory that is over  64MB in OS/2.

The default value is Non-OS2.

Video BIOS Shadow

It determines whether video BIOS will be copied to RAM, however, it is optional from chipset  design.  Video shadow will increase the video speed.

| Enabled: | Video shadow is enabled |
| --- | --- |
| Disabled: | Video shadow is disabled |

C8000-CBFFF Shadow/DC000-DFFFF Shadow

These categories determine whether optional ROM will be copied to RAM by 16K byte.

| Enabled: | Optional shadow is enabled |
| --- | --- |
| Disabled: | Optional shadow is disabled |

### Chipset Features Setup

| ROM PCI/ISA BIOS (2A6LGTJE) CHIPSET FEATURES SETUP AWARD SOFTWARE, INC |  |  |  |
| --- | --- | --- | --- |
| Bank 0/1 DRAM Timing | : SDRAM 10ns | Auto Detect DIMM/PCI CLK | : Enabled |
| Bank 2/3 DRAM Timing | : SDRAM 10ns | Spread Spectrum | : Disabled |
| Bank 4/5 DRAM Timing | : SDRAM 10ns | CPU Host Clock (CPU/PCI) | : Default |
| SDRAM Cycle Length | : 3 | CPU Warning Temperature | : Disabled |
| DRAM Clock | : HCLK-33M | Current CPU Temperature | : 54°C/129°F |
| Memory Hole | : Disabled | Current CPUFAN Speed | : 3529RPM |
| Read Around Write | : Disabled | Current SYSFAN Speed | : 3529RPM |
| Concurrent PCI/Host | : Disabled | Current Vcore (V) | : 2.37V |
| System BIOS Cacheable | : Enabled |  |  |
| Video RAM Cacheable | : Enabled | Shutdown Temperature | : 60°C/140°F |
| AGP Aperture Size | : 64M |  |  |
| AGP-2X Mode | : Enabled | Esc : Quit Selection Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Defaults F7 : Load Setup Defaults |  |
| OnChip USB | : Enabled |  |  |
| USB Keyboard Support | : Disabled |  |  |
| Flash BIOS | : Enabled |  |  |

 This setup menu is optimized for this mainboard by your computer vendor.  Unless you are a qualified engineer and know the item functions you are going to modify, we do not recommend you to change the default setting.

Bank 0/1, 2/3, 4/5 DRAM Timing :

The DRAM timing of Bank 0/1, 2/3, 4/5, 6/7 in this field is set by the system board manufacturer, depending on whether the board has fast paged DRAMs or EDO (extended data output) DRAMs. The Choice: Normal, Medium, Fast, Turbo, FP/EDO 60ns, FP/EDO 70ns.

SDRAM Cycle Length:

This Field sets the CAS latency timing.  The Choice: 2, 3.

Memory Hole : In order to improve performance, certain space in memory can be reserved for ISA cards.  This memory must be mapped into the memory space below 16 MB.

| Enabled: | Memory hole supported. |
| --- | --- |
| Disabled | Memory hole not supported. |

Read Around Write :

DRAM optimization feature:  If a memory read is addressed to a location whose latest write is being held in a buffer before being written to memory, the read is satisfied through the buffer contents, and the read is not sent to the

DRAM.  The choice: Enabled, Disabled.

Concurrent PCI / Host :

When disable, CPU bus will be occupied during the entire PCI operation period.

The choice: Enabled, Disabled.

System BIOS Cacheable:

| Enabled: | Allows caching of the system BIOS ROM at F0000h-FFFFFh, Resulting in better system performance. However, if any program Writes to this memory area, a system error may result. |
| --- | --- |
| Disabled: | System BIOS non-Cacheable. |

Video RAM Cacheable:

Select Enabled allows caching of the video RAM, resulting in better system  prformance.  However, if any program writes to this memory area, a system error may result.

The default value is Disabled.

| Enabled: | Enabled this function to get better VGA performance; while some brands of VGA must be disabled this function. |
| --- | --- |
| Disabled: | Disabled this function. |

AGP Aperture Size: The default value is 64 MB.

Select the size of the AGP aperture. The aperture is a portion of the PCI memory address range dedicated for graphics memory address space. Host cycle that hit the aperture range are forwarded to the AGP without any translation. See www.agpforum.org for AGP information. The choice 4, 8, 16, 32, 64, 128, 256.

OnChip USB :

This should be enabled if your system has a USB installed on the system board and you wish to use it. Even when so equipped, if you add a higher performance controller, you will need to disable this feature.

The choice: Enabled, Disabled

USB Keyboard Support :

Select Enabled if your system contains a Universal Serial Bus (USB) controller and you have a USB keyboard.

The choice: Enabled, Disabled.

Auto Detect DIMM/ PCI Clk: The default value is Enabled.

| Enabled: | The unused DIMM/ PCI slot clock will be disabled. |
| --- | --- |
| Disabled: | The unused DIMM/ PCI slot clock will still get the clock signal. |

Spread Spectrum: The default value is Disabled.

| Enabled: | The clock generator spread spectrum will be Enabled. |
| --- | --- |
| Disabled: | The clock generator spread spectrum will be Disabled. |

CPU Warning Temperature

When this item is enabled, we can set the CPU warning temperature. If the CPU temperature is higher than the setting temperature, the system will beep.

Current CPU Temperature

It shows the current system temperature.

Current CPUFAN Speed

It shows the running speed of the CPU fan.

It shows the running speed of the system fan.

Current Vin3 (3)

It shows the Vcore valtage.

### Power Management

| ROM PCI / ISA BIOS (2A6LGTJE) POWER MANAGEMENT SETUP AWARD SOFTWARE, INC |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ACPI function | : Disabled | Primary INTR | : ON |  |  |
| Power Management | : User Define | IRQ3 (COM 2) | : Primary |  |  |
| PM Control by APM | : Yes | IRQ4 (COM 1) | : Primary |  |  |
| Video Off After | : Suspend | IRQ5 (LTP 2) | : Primary |  |  |
| Video Off Method | : V/H SYNC + Blank | IRQ6 (Floppy Disk) | : Primary |  |  |
| MODEM Use IRQ | : 3 | IRQ7 (LPT 1) | : Primary |  |  |
| Soft-Off by PWRBTN | : Instant-Off | IRQ8 (RTC Alarm) | : Disabled |  |  |
| HDD Power Down | : Disable | IRQ9 (IRQ2 Redir) | : Secondary |  |  |
| Doze Mode | : Disable | IRQ10 (Reserved) | : Secondary |  |  |
| Suspend Mode | : Disabled | IRQ11 (Reserved ) | : Secondary |  |  |
| ** PM Events ** |  | IRQ12 (PS/2 Mouse) | : Primary |  |  |
| VGA | : Off | IRQ13 (Coprocessor) | : Primary |  |  |
| LPT & COM | : LPT/COM | IRQ14 (Hard Disk) | : Primary |  |  |
| HDD & FDD | : ON | IRQ15 (Reserved) | : Disabled |  |  |
| DMA/Master | : Off |  |  |  |  |
| Modem Ring Resume | : Disabled | Esc : Quit Selection Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Defaults F7 : Load Setup Defaults |  |  |  |
| RTC Alarm Resume | : Disabled |  |  |  |  |
| Wake Up On Lan | : Disabled |  |  |  |  |

This category determines how much power consumption for system after selecting items mentioned below.  Default value is Disabled.  The following pages tell you the options of each item and describe the meanings of each option.

ACPI Funtion :

This item allows you to enable/disable the Advanced Configuration and Power      Management(ACPI).                                                                                                                         The choice:  Enabled, Disabled.

Power Management: The default value is Disabled.

Disable: The system operation in NORMAL conditions (Non-GREEN), and the Power Management function is disabled.

Max. Saving: Pre-defined timer values are used such that all timers are in their maximum value.

Min Saving: Pre-defined timer values are used such that all timers are in their minimum value.

User Define: Users can configure their own power management values.

PM Control by APM: The default value is No.

| Yes : | System BIOS will wait for APM’s prompt before it enters any PM mode e.g. DOZE, STANDBY or SUSPEND. |
| --- | --- |
| No: | System BIOS will ignore APM when power managing the system. |

NOTE:  If APM is installed, and if there is a task running, even the timer is timeout, the APM will not prompt the BIOS to put the system into any power saving mode

NOTE:  if APM is not installed, this option has no effect.

To make the APM function work, users have to install power.exe (supported by MS-DOS 5.0 or higher) in Config.exe.  To make Windows 3.1 work regularly, in " Windows Setup", users have to set the "Computer" item to " MS-DOS System with APM"

Video Off After :

Selects the power-saving modes during which the monitor goes blank:

Always On	Monitor remains on during power-saving modes.

Suspend --> Off	Monitor blanked when system enters Suspend mode.

All Modes --> Off	Monitor blanked when system enters any power saving mode.

Video Off  Method: The default value is V/H SYNC+Blank.

Blank Screen: The system BIOS will only blank off the screen when disabling video.

V/H SYNC+Blank: In addition to (1), BIOS will also turn off the  V-SYNC & H-SYNC signals form VGA cards to monitor.

DPMS: This function is enabled for only the VGA card supporting DPM.

Soft-Off  by PWRBTN : The default value is Instant-Off

When Enabled, turning the system off with the on/off button places the system in a very low-power-usage state, with only enough circuitry receiving power to detect power button activity or Resume by Ring activity.

The Choice: Instant-Off, Delay 4 Sec.

The following four modes are Green PC power saving functions which are only user configurable when User Defined Power Management has been selected.  See above for available selections.

HDD Power Down : The default value is Disabled

When enabled and after the set time of system inactivity, the hard disk drive will be powered down while all other devices remain active.

Doze Mode

When enabled and after the set time of system inactivity, the CPU clock will run at slower speed while all other devices still operate at full speed.

Suspend Mode: The default value is Disabled

Disabled: System will never enter SUSPEND mode.

10 Sec / 20 Sec / 30 Sec / 40 Sec / 1 Min / 3 Min / 5 Min / 10 Min / 15 Min / 20 Min

30 Min / 40 Min / 1 Hr / 2 Hr / 3 Hr : Defines the continuous idle time before the system

entering SUSPEND mode. If any item defined in (J) is enabled and active, SUSPEND

timer will be reloaded.

PM Events :

PM events are I/O events whose occurrence can prevent the system from entering a power saving mode or can awaken the system from such a mode.  In effect, the system remains alert for anything which occurs to a device which is configured as On, even when the system is in a power down mode.

VGA :

When set to On (default), any event occurring at a VGA port will awaken a system which has been powered down.

LPT & COM :

When set to On (default), any event occurring at a COM(serial)/LPT (printer) port will awaken a system which has been powered down.

HDD & FDD :

When set to On (default), any event occurring at a hard or floppy drive port will awaken a system which has been powered down.

DMA/Master :

When set to On, any event occurring at will awaken a system which has been powered down.

Modem Ring Resume :  The default value is Disabled

When set to Enabled, any event occuring to the Modem Ring will awaden a sysstem

which has been powered down.

The Choice : Enable, Disable

RTC Alarm Resume :

When Enabled, you can set the date and time at which the RTC (real-time clock)

Alarm awakens the system from Suspend mode.

The Choice : Enabled, Disabled.

Primary INTR :

When set to On (default), any even occurring at will awaken a system which has been powered down.

The following is a list of IRQ’s, Interrupt ReQuests, which can be exempted much as the COM ports and LPT ports above can.  When an I/O device wants to gain the attention of the operating system, it signals this by causing an IRQ to occur.  When the operating system is ready to respond to the request, it interrupts itself and performs the service.

### 5. PNP/PCI Configuration Setup

| ROM PCI/ISA BIOS (2A6LGTJE) PNP/PCI CONFIGURATION AWARD SOFTWARE, INC. |  |  |  |
| --- | --- | --- | --- |
| PNP OS Installed | : No | CPU to PCI Write Buffer | : Enabled |
| Resources Contorlled By | : Manual | PCI Dynamic Bursting | : Enabled |
| Reset Configuration Data | : Enabled | PCI Master 0 WS Write | : Enabled |
|  |  | PCI Delay Transaction | : Enabled |
| IRQ-3 assigned to : | : Legacy ISA | PCI#2 Access #1 Retry | : Disabled |
| IRQ-4 assigned to : | : Legacy ISA | AGP Master 1 WS Write | : Enabled |
| IRQ-5 assigned to : | : PCI/ISA PnP | AGP Master 1 WS Read | : Disabled |
| IRQ-7 assigned to : | : PCI/ISA PnP |  |  |
| IRQ-9 assigned to : | : PCI/ISA PnP |  |  |
| IRQ-10 assigned to : | : PCI/ISA PnP | Assign IRQ for USB | : Enabled |
| IRQ-11 assigned to : | : PCI/ISA PnP | Assign IRQ for VGA | : Enabled |
| IRQ-12 assigned to : | : PCI/ISA PnP |  |  |
| IRQ-14 assigned to : | : PCI/ISA PnP |  |  |
| IRQ-15 assigned to : | : PCI/ISA PnP |  |  |
| DMA-0 assigned to : | : PCI/ISA PnP |  |  |
| DMA-1 assigned to : PCI/ISA PnP |  | ESC: Quit : Select Item |  |
| DMA-3 assigned to : PCI/ISA PnP |  | F1 : Help PU / PD / + / - : Modify |  |
| DMA-5 assigned to : PCI/ISA PnP |  | F5 : Old Values (Shift)F2 : Color |  |
| DMA-6 assigned to : PCI/ISA PnP DMA-7 assigned to : PCI/ISA PnP |  | F6 : Load BIOS Defaults F7 : Load Setup Defaults |  |

The following pages tell you the options of each item and describe the meanings of each option.

PNP OS Installed: The default value is No.

Select Yes if the system operating environment is Plug-and-Play aware (e.g.,

Windows 95).

The Choice: Yes and No.

Resources Controlled By:  The default value is Manual.

The Award Plug and Play BIOS can automatically configure all the boot and Plug and Play-compatible devices. If you select Auto, all the interrupt request (IRQ) and DMA assignment fields disappear, as the BIOS automatically assigns them.

The choice:  Auto and Manual.

Manual: PNP Card‘s resources will be controlled manually. You can set which IRQ-X and DMA-X are assigned to PCI/ISA PNP or Legacy ISA Cards.

Auto: If your ISA card and PCI card are all PNP cards, BIOS will assign the interrupt resources automatically.

Reset Configuration Data: The default value is Disabled.

Normally, you leave this field Disabled. Select Enabled to reset Extended System Configuration Data (ESCD) when you exit Setup if you have installed a new add-on and the system reconfiguration has caused such a serious conflict that the operating system cannot boot.

The choice: Enabled and Disabled.

CPU to PCI Write Buffer :

When enabled, up to four Dwords of data can be written to the PCI bus without interrupting the CPU.  When disabled, a write buffer is not used and the CPU read cycle will not be completed until the PCI bus signals that it is ready to receive the data..

Choices are Enabled, Disabled.

PCI Dynamic Bursting :

When  Enabled, every write transaction goes to the write buffer.  Burstable transactions then burst on the PCI bus and nonburstable transactions don’t.

Choices are Enabled, Disabled.

PCI Master 0 WS Write :

When Enabled, writes to the PCI bus are executed with zero wait states.

Choices are Enabled, Disabled.

PCI Delay Transaction :

The chipset has an embedded 32-bit posted write buffer to support delay transactions cycles. Select Enabled to support compliance with PCI specification version 2.1.

The choice: Enabled, Disabled.

## PCI #2 Access #1 Retry

This item allows you enable/disable the PCI #2 Access #1 Retry.

The choice: Enabled, Disabled.

## AGP Master 1 WS Write

This implements a single delay when writing to the AGP Bus. By default, two-wait states are used by the system, allowing for greater stability.

The choice: Enabled, Disabled.

## AGP Master 1 WS Read

This implements a single delay when reading to the AGP Bus. By default, two-wait states are used by the system, allowing for greater stability.

The choice: Enabled, Disabled.

## Assign IRQ For USB/VGA/ACPI

Name the interrupt request (IRQ) line assigned to the USB/VGA/ACPI (if any) on your system.  Activity of the selected IRQ always awakens the system.

### Load BIOS Default

When you access "Load BIOS Default", the following message appears:

Load BIOS Default (Y/N) ?N

The BIOS Default values are the "worst case" default, and are the most stable values for the system.  Use them if the system is performing erratically due to hardware problems.  To load the Setup Default values, press <Y> then <Enter>.

### Load Setup Default

When you access "Load Setup Default", you are shown the following message:

Load Setup Default (Y/N) ?N

The Setup Default values represent the "best case" default, and should provide optimum system performance.  To load the BIOS Default values, press <Y> then

<Enter>.

### Integrated Peripherals Setup

| ROM PC/ISA BIOS (2A6LGTJE) INTEGRATED PERIPHERALS AWARD SOFTWARE, INC. |  |  |  |
| --- | --- | --- | --- |
| OnChip IDE Channel 0 | : Enabled | UART Moe Select | : Normal |
| OnChip IDE Channel 1 | : Enabled |  |  |
| IDE Prefetch Mode | : Enabled |  |  |
| Primary Master PIO | : AUTO |  |  |
| Primary Slave PIO | : AUTO |  |  |
| Secondary Master PIO | : AUTO | Onboard Parallel Port | : 378 / IRQ7 |
| Secondary Slave PIO | : AUTO | Parallel Port Mode | : ECP+EPP |
| Primary Master UDMA | : AUTO | ECP Mode Use DMA | : 3 |
| Primary Slave UDMA | : AUTO | EPP Mode Select | : EPP1.7 |
| Secondary Master UDMA | : AUTO |  |  |
| Secondary Slave UDMA | : AUTO |  |  |
| Init Display First | : AGP |  |  |
| POWER ON Function | : Hot KEY |  |  |
| Hot Key Power On | : Ctrl-F12 |  |  |
| KBC input clock | : 8 MHz | Esc : QuitSelection | :Item |
| Onboard FDC Controller | : Enabled | F1 : Help PU/PD/+/- : | Modify |
| Onboard Serial Port 1 | : 3F8/IRQ4 | F5 : Old Values (Shift)F2 : | Color |
| Onboard Serial Port 2 | : 2F8/IRQ3 | F6 : Load BIOS Default |  |
|  |  | F7 : Load Setup Default |  |

- This setup menu is optimized for this mainboard by your computer vendor.  Unless you are a qualified engineer and know the item functions you are going to modify, we do not recommend you to change the default setting.

OnChip IDE Channel 0 :

The chipset contains a PCI IDE interface with support for two IDE channels.  Select Enabled to activate the first and/or second IDE interface.  Select Disabled to deactivate an interface, if you install a primary and/or secondary add-in IDE interface.  The choice: Enabled, Disabled.

OnChip IDE Channel 1 :

The onboard IDE drive interfaces supports IDE prefetching, for faster drive accesses.  If you install a primary and/or secondary add-in IDE interface, set this field to Disabled if the interface does not support prefetching.

The choice: Enabled, Disabled.

IDE Prefetch Mode :

The onboard IDE drive interfaces supports IDE prefetching, for faster drive accesses.  If you install a primary and/or secondary add-in IDE interface, set this field to Disabled if the interface does not support prefetching.

## IDE Primary/Secondary Master/Slave PIO

The four IDE PIO (Programmed Input/Output) fields let you set a PIO mode (0-4) for each of the four IDE devices that the onboard IDE interface supports. Modes 0 through 4 provide successively increased performance. In Auto mode, the system automatically determines the best mode for each device.

## IDE Primary/Secondary Master/Slave UDMA

Ultra ATA/66 implementation is possible only if your IDE hard drive supports it and the operating environment includes a DMA driver (Windows 95 OSR2 or a third-party IDE bus master driver). If your hard drive and your system software both support Ultra ATA/66, select Auto to enable BIOS support.

The Choice: Auto, Disabled

## Init Display First

This item allows you to decide to active PCI Slot or AGP first

The choice: PCI Slot, AGP.

Onboard FDC Controller : The default value is Enabled

| Enabled : | Enable the Onboard floppy driver interface controller. |
| --- | --- |
| Disabled: | Disable the Onboard floppy driver interface controller when use On- Card card ISA FDC’s controller. |

Onboard Serial Port 1 : The default value is 3F8 / IRQ4.

| COM 1: | Enable Onboard Serial port 1 and address is 3F8 / IRQ4 |
| --- | --- |
| COM 2: | Enable Onboard Serial port 1 and address is 2F8 / IRQ3 |

Onboard Serial Port 2 : The default value is 2F8 / IRQ3.

| COM 1: | Enable Onboard Serial port 2 and address is 3F8 / IRQ4 |
| --- | --- |
| COM 2: | Enable Onboard Serial port 2 and address is 2F8 / IRQ3 |

UART Mode Select:

This item allows you to determine which Infra Red (IR) function of onboard I/O chip. The Choice: Standard, ASKIR, HPSIR.

Onboard Parallel port 2:  The default value is 378H / IRQ 7.

| 378H: | Enable Onboard LPT port and address is 378H and IRQ 7. |
| --- | --- |
| 278H: | Enable Onboard LPT port and address is 278H and IRQ 5. |
| 3BCH: | Enable Onboard LPT port and address is 3BCH and IRQ 7. |
| Disabled: | Disabled Onboard LPT port. |

NOTE: Parallel Port address is 378H / 3BCH that selects routing of IRQ 7 for LPT1. Parallel Port address is 278H that selects the routing of IRQ 5 for LPT 1.

Parallel port Mode: This field allows the user to select the parallel port mode. The default value is Standard mode.

| SPP: | Standard mode. IBM PC / AT Compatible bidirectional parallel port. |
| --- | --- |
| EPP: | Enhanced Parallel Port mode. |
| ECP: | Extended Capabilities Port mode. |
| EPP+ECP: | ECP Mode and EPP Mode. |

ECP Mode USE DMA: This field allows the user to select DMA 1 or DMA 3 for the ECP mode. The default value is DMA 3.

| DMA 1: | The field selects the routing of DMA 1 for the ECP mode. |
| --- | --- |
| DMA 3: | The field selects the routing of DMA 3 for the ECP mode. |

EPP Mode Select : The default value is EPP1.7

| EPP 1.7: | The field selects the routing of EPP1.7 for the EPP mode. |
| --- | --- |
| EPP 1.9: | The field selects the routing of EPP1.9 for the EPP mode. |

### Supervisor\User Password

When you select this function, the following message will appear at the center of the screen to assist you in creating a password.

ENTER PASSWORD

Type the password, up to eight characters, and press <Enter>.  The password typed now will clear any previously entered password from CMOS memory.  You will be asked to confirm the password.  Type the password again and press <Enter>. You may also press <Esc> to abort the selection and not enter a password.

If you select System at Security Option of BIOS Features Setup Menu, you will be prompted for the password everytime the system is rebooted or anytime you try to enterSetup.  If you select Setup at Security Option of BIOS Features Setup Menu, you will be prompted only when you try to enter Setup.

### IDE HDD Auto Detection

This feature allows you to check all the information on your hard disk formation.  When you access "IDE HDD Auto Detection", the system executes auto detection.

At the prompt, it represents all the information on your HDD, and you are asked:

Do you accept this drive C: (Y/N) ?

1. If you accept the test result, press [Y] then [Enter] and the result  is saved, then the system continues to detect another HDD.

2. If not, press [N] then [enter] and the system continues to detect another HDD.

### Hard Disk Low Level Format Utility

This Award Low-Level-Format Utility is designed as a tool to save your time formatting your hard disk.  The Utility automatically looks for the necessary information of the drive you select.  The Utility also searches for bad tracks and lists them for your reference.

Shown below is the Main Menu after you enter into the Award Low-Level-Format Utility.

Control Keys

Use the Up and Down arrow keys to move around the selections displayed on the upper screen.  Press [Enter] to accept the selection.  Press Esc to abort the selection or exit the Utility.

SELECT DRIVE

Select from installed hard disk drive C or D.  List at the bottom of the screen is the drive automatically detected by the utility.

BAD TRACK LIST

Auto scan bad track

The utility will automatically scan bad tracks and  list the bad tracks in the window at the right side of the screen.

Add bad track

Directly type in the information of the known bad tracks in the window at the right side of the screen.

Modify bad track

Modify the information of the added bad tracks in the window at the right side of  the screen.

Delete bad track

Delete the added bad tracks in the window at the right side of the screen.

Clear bad track table

Clear the whole bad track list in the window at the right side of the screen.

PREFORMAT

Interleave

Select the interleave number of the hard disk drive you wish to perform low level format.  You may select from 1 to 8.  Check the documentation that came with the drive for the correct interleave number, or select 0 for utility automatic detection.

Auto scan bad track

This allows the utility to scan first then format by each track.

Start

Press <Y> to start low level format.

### Exiting the Setup Program

To exit the Setup program, do the following:

If you want to save your change:

a.  At the Main menu, select "Save & Exit Setup", then press [Enter]

b.  Press [Y] then [Enter] to confirm.  The system will boot with your new BIOS setting in effect .

If you want to abandon your changes:

a.  At the Main Menu, select "Exit Without Saving", then press [Enter].

- Press [Y] then [Enter] to confirm.  The system will reboot with the original BIOS setting in effect.