Thank you for purchasing one of the finest Computer Main Boards on the market today, the Genoa Turbo Express 586 HX.

The Turbo Express 586 HX now comes in two variations.  There are several physical differences you have to check for to be sure of your product before you make any jumper settings.  Check the layout diagram that comes with this addendum and carefully compare it to the one in the manual and the actual Main Board.

If you use the jumper settings in the manual and you have the revised edition Main Board, you might damage the system, CPU, or other components, and VOID THE WARRANTY!!  IF you have any doubts, contact us for assistance.  Please note the Contact Information section of the Manual.

First, and for your convenience, instead of a VRM header, support for these functions is now built into the Main Board.

The jumper at JP9 is now a 5 pin strip instead of a 4 pin strip.  This is the Clock Speed Multiplier Selector and it is critical that you make sure which one you have.  If you have the 5 pin strip at the edge of the Main Board next to the CPU socket, then use the jumper settings in THIS ADDENDUM.

Also, note the absence of the Pipelined Burst Cache Module Socket.  Instead of being upgradable, they come with 512K of PB Cache on board ending the hassle of future upgrades, as well as cutting the cost.

There is an additional Voltage Regulator for support of the Cyrix 6x86 and the AMD 5k86 CPU’s.

THE BOTTOM LINE...  If you DO NOT have the Pipelined Burst Cache Ram Upgrade Slot, then you have the NEW variation of the HX and need to use the jumper settings in THIS ADDENDUM.