YAMAHA Corporation June 6th, 1997

OPL3-SAx driver  technical document

How to change default values of mixer

Default values of mixer can be changed without compiling. This document describes how we can do that. Follow  the instructions as below.

1. Execute resource editor. Generally resource editor is included in the software development package. (ex. Developer Studio in Visual C++ 4.0,  App Studio in Visual C++ 1.xx,  Resource Workshop in Borland C++)

2. Open “OPL3SA.DRV” . We can edit resources (such as dialog box, bitmap and string table) included in the EXE or DLL using resource editor.

3. See string  table. There are various error messages and names in the string table. Each string has ID number. Mixer defalut values are from ID 200 through ID 271. The following table shows the connections between each ID number and mixer setting.

| ID number | mixer setting | Value |
| --- | --- | --- |
| 200 | Left Master Volume | 0 - 65535 |
| 201 | Right Master Volume | 0 - 65535 |
| 202 | Left Wave Volume | 0 - 65535 |
| 203 | Right Wave Volume | 0 - 65535 |
| 204 | Left CD Volume | 0 - 65535 |
| 205 | Right CD Volume | 0 - 65535 |
| 206 | Left Synth Volume | 0 - 65535 |
| 207 | Right Synth Volume | 0 - 65535 |
| 208 | Left Line Volume | 0 - 65535 |
| 209 | Right Line Volume | 0 - 65535 |
| 210 | Mono In Volume | 0 - 65535 |
| 211 | Mic Output Volume | 0 - 65535 |
| 212 | Left Soft WT Volume | 0 - 65535 |
| 213 | Right Soft WT Volume | 0 - 65535 |
| 214 | Left CD Recording Volume | 0 - 65535 |
| 215 | Right CD Recording Volume | 0 - 65535 |
| 216 | Mic Recording Volume | 0 - 65535 |
| 217 | Left Line Recording Volume | 0 - 65535 |
| 218 | Right Line Recording Volume | 0 - 65535 |
| 219 | Left Loop Recording Volume | 0 - 65535 |
| 220 | Right Loop Recording Volume | 0 - 65535 |
| 221 | Left CD Voice Volume | 0 - 65535 |
| 222 | Right CD Voice Volume | 0 - 65535 |
| 223 | Mic Voice Volume | 0 - 65535 |
| 224 | Left Line Voice Volume | 0 - 65535 |
| 225 | Right Line Voice Volume | 0 - 65535 |
| 226 | Left Loop Voice Volume | 0 - 65535 |
| 227 | Right Loop Voice Volume | 0 - 65535 |
| 228 | Recording Monitor Volume | 0 - 65535 |
| 229 | Master Mute | 0 (not mute), 1(mute) |
| 230 | Wave Mute | 0 (not mute), 1(mute) |
| 231 | CD Mute | 0 (not mute), 1(mute) |
| 232 | Synth Mute | 0 (not mute), 1(mute) |
| 233 | Line Mute | 0 (not mute), 1(mute) |
| 234 | Mono In Mute | 0 (not mute), 1(mute) |
| 235 | Mic Output Mute | 0 (not mute), 1(mute) |
| 236 | Soft WT Mute | 0 (not mute), 1(mute) |
| 237 | Recording Source Select | Aux , LineIn , MIC , Feedback |
| 238 | Voice Source Select | Aux , LineIn , MIC , Feedback |
| 239 | Recording Monitor Mute | 0 (not mute), 1(mute) |
| 240 | SRS Select | 0 (SRS off), 1(SRS on) |
| 243 | BASS Volume | 0 - 65535 |
| 245 | TREBLE Volume | 0 - 65535 |
| 247 | Ymersion Volume | 0 - 65535 |
| 267 | Performance of Ymersion | 0 (DeskTop), 1 (NotePC1) 2 (NotePC2), 3 (HiFi) |
| 268 | Mic +20dB | 0 (OFF), 1(ON) |
| 269 | Quality of SoftSynthesizer | 0 (Normal),1 (Good) 2 (Very Good),3 (Excellent) |
| 270 | Reverb of SoftSynthesizer | 0 (OFF), 1(ON) |
| 271 | MPU401 Out | 0 (External),1 (Soft GM) |

4. Change values and save. You will get a modified driver with new default settings.

But when using ID number except the above, the driver’s action is not guaranteed.