YAMAHA Corporation April 1st, 1996

OPL3-SAx driver technical document

How to change maximum levels of volumes

History

November 8th , 1997 First release

January    23rd, 1997 Maximum value of microphone has changed.

April 1st, 1997 Wave output volume and Soft WT volume have been supported.

May 29st, 1997 FM volume have been supported.

Maximum levels of volumes can be changed without compiling. This document describes how we can do that. Follow the instructions as below.

1. Execute resource editor. Generally resource editor is included in the software development package. (ex. Developer Studio in Visual C++ 4.0,    App Studio in Visual C++ 1.xx,    Resource Workshop in Borland C++)

2. Open “OPL3SA.DRV” . We can edit resources (such as dialog box, bitmap and string table) included in the EXE or DLL using resource editor.

3. See string    table. There are various error messages and names in there. Each string has ID number. Maximum levels of volumes are from ID 250 through ID 266. The following table shows the connections between each ID number and maximum level setting. Maximum value means maximum level which can be set to the hardware.

| ID number | mixer setting | Value |
| --- | --- | --- |
| 250 | Output Master Volume | 0 - 63 |
| 251 | Wave Output Volume | 0 - 63 |
| 252 | CD Volume | 0 - 31 |
| 253 | Synth Output Volume | 0 - 31 |
| 254 | Line Volume | 0 - 31 |
| 255 | Mono In Volume | 0 - 15 |
| 256 | Mic Output Volume | 0 - 31 |
| 257 | Soft WT Volume | 0 - 63 |
| 258 | CD Recording Volume | 0 - 15 |
| 259 | Mic Recording Volume | 0 - 15 |
| 260 | Line Recording Volume | 0 - 15 |
| 261 | Loop Recording Volume | 0 - 15 |
| 262 | CD Voice Volume | 0 - 15 |
| 263 | Mic Voice Volume | 0 - 15 |
| 264 | Line Voice Volume | 0 - 15 |
| 265 | Loop Voice Volume | 0 - 15 |
| 266 | Recording Monitor Volume | 0 - 63 |
| 273 | FM Volume | 0 - 31 |
| 274 | BASS | 0 - 7 |
| 275 | TREBLE | 0 - 7 |
| 276 | 3D WIDE | 0 - 7 |

4.Change values and save. You will get a modified driver with new maximum level    settings.

ID 250 (Output Master Volume)

| Value | MAX Level |
| --- | --- |
| 63-60 | 0dB |
| 59-56 | -2dB |
| 55-52 | -4dB |
| 51-48 | -6dB |
| 47-44 | -8dB |
| 43-40 | -10dB |
| 39-36 | -12dB |
| 35-32 | -14dB |
| 31-28 | -16dB |
| 27-24 | -18dB |
| 23-20 | -20dB |
| 19-16 | -22dB |
| 15-12 | -24dB |
| 11-8 | -26dB |
| 7-4 | -28dB |
| 3-0 | -30dB |

ID 252 (CD Volume), ID 253 (Synth Output Volume), ID 254 (Line Volume), ID256 (Mic Output Volume)

| Value | MAX Level |
| --- | --- |
| 31 | 12.0dB |
| 30 | 10.5dB |
| 29 | 9.0dB |
| 28 | 7.5dB |
| 27 | 6.0dB |
| 26 | 4.5dB |
| 25 | 3.0dB |
| 24 | 1.5dB |
| ... | ... |
| 6 | -25.5dB |
| 5 | -27.0dB |
| 4 | -28.5dB |
| 3 | -30.0dB |
| 2 | -31.5dB |
| 1 | -33.0dB |
| 0 | -34.5dB |

ID 255 (Mono In Volume)

| Value | MAX Level |
| --- | --- |
| 15 | 0dB |
| 14 | -3.0dB |
| 13 | -6.0dB |
| 12 | -9.0dB |
| 11 | -12.0dB |
| 10 | -15.0dB |
| 9 | -18.0dB |
| 8 | -21.0dB |
| 7 | -24.0dB |
| 6 | -27.0dB |
| 5 | -30.0dB |
| 4 | -33.0dB |
| 3 | -36.0dB |
| 2 | -39.0dB |
| 1 | -42.0dB |
| 0 | -45.0dB |

ID 258 (CD Recording Volume), ID 259 (Mic Recording Volume), ID 260 (Line Recording Volume), ID 261 (Loop Recording Volume), ID 262 (CD Voice Volume), ID 263 (Mic Voice Volume), ID264 (Line Voice Volume), ID 265 (Loop Voice Volume)

| Value | MAX Level |
| --- | --- |
| 15 | 22.5dB |
| 14 | 21.0dB |
| 13 | 19.5dB |
| 12 | 18.0dB |
| 11 | 16.5dB |
| 10 | 15.0dB |
| 9 | 13.5dB |
| 8 | 12.0dB |
| 7 | 10.5dB |
| 6 | 9.0dB |
| 5 | 7.5dB |
| 4 | 6.0dB |
| 3 | 4.5dB |
| 2 | 3.0dB |
| 1 | 1.5dB |
| 0 | 0.0dB |

ID 266 (Recording Monitor Volume)

| Value | MAX Level |
| --- | --- |
| 63 | 0dB |
| 62 | -1.5dB |
| 61 | -3.0dB |
| 60 | -4.5dB |
| 59 | -6.0dB |
| 58 | -7.5dB |
| 57 | -9.0dB |
| 56 | -10.5dB |
| ... | ... |
| 6 | -85.5dB |
| 5 | -87.0dB |
| 4 | -88.5dB |
| 3 | -90.0dB |
| 2 | -91.5dB |
| 1 | -93.0dB |
| 0 | -94.5dB |