YAMAHA Corporation June 6th, 1997

OPL3-SAx driver technical document

How to change maximum levels of volumes

History

November 8th , 1997 First release

January  23rd, 1997 Maximum value of microphone has changed.

April 1st, 1997 Wave output volume and Soft WT volume have been supported.

May 29st, 1997 FM volume have been supported.

Maximum levels of volumes can be changed without compiling. This document describes how we can do that. Follow the instructions as below.

1. Execute resource editor. Generally resource editor is included in the software development package. (ex. Developer Studio in Visual C++ 4.0,  App Studio in Visual C++ 1.xx,  Resource Workshop in Borland C++)

2. Open “OPL3SA.DRV” . We can edit resources (such as dialog box, bitmap and string table) included in the EXE or DLL using resource editor.

3. See string  table. There are various error messages and names in there. Each string has ID number. Maximum levels of volumes are from ID 250 through ID 266. The following table shows the connections between each ID number and maximum level setting. Maximum value means maximum level which can be set to the hardware.

ID number

mixer setting

Value

250

Output Master Volume

0 - 63

251

Wave Output Volume

0 - 63

252

CD Volume

0 - 31

253

Synth Output Volume

0 - 31

254

Line Volume

0 - 31

255

| -30dB 29 9.0dB 28 ID 252 (CD Volume), ID 253 (Synth Output Volume), ID 254 (Line Volume), ID256 (Mic Output Volume) Value MAX Level 31 12.0dB 30 10.5dB Mono In Volume 0 - 15 256 Mic Output Volume 0 - 31 257 Soft WT Volume 0 - 63 258 CD Recording Volume 0 - 15 259 Mic Recording Volume 0 - 15 260 Line Recording Volume 0 - 15 261 Loop Recording Volume 0 - 15 262 CD Voice Volume 0 - 15 263 Mic Voice Volume 0 - 15 264 Line Voice Volume 0 - 15 265 Loop Voice Volume 0 - 15 266 Recording Monitor Volume 0 - 63 273 FM Volume 0 - 31 |  | 4.4. values and save. You will get a modified driver with new maximum level settings. But when using ID number except the above, the driver’s action is not guaranteed. ID 250 (Output Master Volume) Value MAX Level 63-60 0dB 59-56 -2dB 55-52 -4dB 51-48 -6dB |
| --- | --- | --- |
| 7.5dB-20dB 27 6.0dB 26 4.5dB 19-16 -22dB 15-12 -24dB 11-8 -26dB 7-4 -28dB 3-0 47-44 -8dB 43-40 -10dB 39-36 -12dB 35-32 -14dB 31-28 -16dB 27-24 -18dB 23-20 |  |  |
| 25 3.0dB |  | 24 1.5dB ... ... 6 |
| -28.5dB-25.5dB 3 -30 5 -27.0dB 4 |  |  |

.0dB

2

-31.5dB

1

-33.0dB

0

-34.5dB

ID 255 (Mono In Volume)

Value

MAX Level

15

0dB

14

-3.0dB

13

-6.0dB

12

-9.0dB

11

-12.0dB

10

-15.0dB

9

-18.0dB

8

-21.0dB

7

-24.0dB

6

-27.0dB

5

-30

| .0dB 4 -33.0dB 3 -36.0dB 2 -39.0dB 1 -42.0dB 0 -45.0dB ID 258 (CD Recording Volume), ID 259 (Mic Recording Volume), ID 260 (Line Recording Volume), ID 261 (Loop Recording Volume), ID 262 (CD Voice Volume), ID 263 (Mic Voice Volume), ID264 (Line Voice Volume), ID 265 (Loop Voice Volume) Value MAX Level 15 22.5dB 14 21.0dB 13 19.5dB 12 18.0dB 11 16.5dB 10 1 |  |  |
| --- | --- | --- |

5.0dB

9

13.5dB

8

12.0dB

7

10.5dB

6

9.0dB

5

7.5dB

4

6.0dB

3

4.5dB

2

3.0dB

1

1.5dB

0

0.0dB

ID 266 (Recording Monitor Volume)

Value

MAX Level

63

0dB

62

-1.5dB

61

-3.0dB

60

-4.5dB

59

-6.0dB

58

-7.5dB

57

-9.0dB

56

-10.5dB

...

...

6

-85.5dB

5

-87.0dB

4

-88.5dB

3

-90.0dB

2

-91.5dB

1

-93.0dB

0

-94.5dB