YAMAHA Corporation May 28th, 1997

OPL3-SAx driver technical document

How to change default values of hardware resource

YAMAHA Corporation

System Development Dept.

Default values of hardware resource can be changed without compiling  in this OPL3-SAx driver. This document describes how we can do that. Following the instructions as below.

1. Execute resource editor. Generally resource editor is included in the software development package. (ex. Developer Studio in Visual C++ 4.0,  App Studio in Visual C++ 1.xx,  Resource Workshop in Borland C++)

2. Open “OPL3SA32.DLL” . We can edit resources (such as dialog box, bitmap and string table) included in the EXE or DLL using resource editor.

3. See string table. There are numbers in string table (Neutral). Each string has ID number. Driver sets hardware resource default values as default value, but you can use ID number from 407 to 423 to change it in value range.  The following table shows the connections between each ID number and hardware resource setting.

| ID number | Hardware resource setting | Value range |
| --- | --- | --- |
| 407 | Interrupt3 | 0(disable),1(enable) |
| 408 | DMA channel of SA2 and higher than SA2 board | 0, 1, 3 |
| 409 | DMAIn channel of SA2 and higher than SA2 board | 0, 1, 3 |
| 410 | Interrupt channel of SA2 and higher than SA2 board | 3, 5, 7, 9, 10, 11 |
| 411 | I/O port of SA2 and higher than SA2 board | 530, 608, E80, F40, |
| 412 | Use dual DMA of SA2 and higher than SA2 board | FALSE(disable), TRUE(enable) |
| 413 | MPU enable of SA2 and higher than SA2 board | FALSE(disable), TRUE(enable) |
| 414 | MPU I/O port of SA2 and higher than SA2 board | 300, 330, 332, 334 |
| 415 | MPU interrupt of SA2 and higher than SA2 board | 3, 5, 7, 9, 10, 11 |
| 416 | DMA channel of SA board | 0 (DMAIn 1) 1 (DMAIn 0) 3 (DMAIn 0) |
| 417 | Interrupt of SA board | 7, 9, 10, 11, |
| 418 | I/O port of SA board | 530, 604, E80, F40 |
| 419 | Use dual DMA of SA board | FALSE(disable), TRUE(enable) |
| 421 | MPU enable of SA board | FALSE(disable), TRUE(enable) |
| 422 | MPU I/O port of SA board | 300, 330, 332, 334 |
| 423 | MPU Interrupt of SA board | 5, 7, 9, 10 |

- B.: If you set value of DMAIn channel of SA2 and higher than SA2 board,

the value must be different from value of DMA channel of SA2 and higher than SA2 board.

4. Change values and save. You will get a modified driver with new default settings.