YAMAHA Corporation May. 28th, 1997

OPL3-SAx driver technical document

How to change maximum levels of volumes

YAMAHA Corporation

System Development Dept.

Maximum levels of volumes can be changed without compiling in this driver. This document describes how we can do that. Follow  the instructions as below.

1. Execute resource editor. Generally resource editor is included in the software development package. (ex. Developer Studio in Visual C++ 4.0,  App Studio in Visual C++ 1.xx,  Resource Workshop in Borland C++)

2. Open “OPL3SA32.DLL” . We can edit resources (such as dialog box, bitmap and string table) included in the EXE or DLL using resource editor.

3. See string  table. There are numbers in string table (Neutral).. Each string has ID number. Driver sets maximum levels of volumes as default value, but you can use ID number from 390 to 405 to change it in value range. The following table shows the connections between each ID number and maximum level setting. Maximum value means maximum level which can be set to the hardware.

4. Change values and save. You will get a modified driver with new maximum level  settings.

| ID number | Maximum levels of volumes | Value range |
| --- | --- | --- |
| 390 | Output Master Volume | 0 - 63 |
| 391 | Line Output Volume | 0 - 31 |
| 392 | Wave Output Volume | Not implemented yet |
| 393 | Synth Output Volume | 0 - 31 |
| 394 | CD Volume | 0 - 31 |
| 395 | Mic Output Volume | 0 - 31 |
| 396 | Mono In Volume | 0 - 15 |
| 397 | Recording Monitor Volume | 0 - 63 |
| 398 | Line Recording Volume | 0 - 15 |
| 399 | Mic Recording Volume | 0 - 15 |
| 400 | CD Recording Volume | 0 - 15 |
| 401 | Loop Recording Volume | 0 - 15 |
| 402 | Line Voice Volume | 0 - 15 |
| 403 | Mic Voice Volume | 0 - 15 |
| 404 | CD Voice Volume | 0 - 15 |
| 405 | Loop Voice Volume | 0 - 15 |

- ID 390 Output Master Volume

| Value | MAX Level |
| --- | --- |
| 63 - 60 | 0dB |
| 59 - 56 | -2dB |
| 55 - 52 | -4dB |
| 51 - 48 | -6dB |
| 47 - 44 | -8dB |
| 43 - 40 | -10dB |
| 39 - 36 | -12dB |
| 35 - 32 | -14dB |
| 31 - 28 | -16dB |
| 27 - 24 | -18dB |
| 23 - 20 | -20dB |
| 19 - 16 | -22dB |
| 15 - 12 | -24dB |
| 11 - 8 | -26dB |
| 7 - 4 | -28dB |
| 3 - 0 | -30dB |

- ID 391Line VolumeID393Synth Output VolumeID 394CD VolumeID395Mic Output Volume

| Value | MAX Level |
| --- | --- |
| 31 | 12.0dB |
| 30 | 10.5dB |
| 29 | 9.0dB |
| 28 | 7.5dB |
| 27 | 6.0dB |
| 26 | 4.5dB |
| 25 | 3.0dB |
| 24 | 1.5dB |
|  | ¥¥¥ |
| 6 | -25.5dB |
| 5 | -27.0dB |
| 4 | -28.5dB |
| 3 | -30.0dB |
| 2 | -31.5dB |
| 1 | -33.0dB |
| 0 | -34.5dB |

ID 396Mono In Volume

| Value | MAX Level |
| --- | --- |
| 15 | 0dB |
| 14 | -3.0dB |
| 13 | -6.0dB |
| 12 | -9.0dB |
| 11 | -12.0dB |
| 10 | -15.0dB |
| 9 | -18.0dB |
| 8 | -21.0dB |
| 7 | -24.0dB |
| 6 | -27.0dB |
| 5 | -30.0dB |
| 4 | -33.0dB |
| 3 | -36.0dB |
| 2 | -39.0dB |
| 1 | -42.0dB |
| 0 | -45.0dB |

- ID398Line Recording VolumeID 399Mic Recording VolumeID 400(CD Recording Volume)ID401(Loop Recording VolumeID 402Line Voice VolumeID403(Mic Voice Volume)ID404CD Voice VolumeID 405Loop Voice Volume

| Value | MAX Level |
| --- | --- |
| 15 | 22.5dB |
| 14 | 21.0dB |
| 13 | 19.5dB |
| 12 | 18.0dB |
| 11 | 16.5dB |
| 10 | 15.0dB |
| 9 | 13.5dB |
| 8 | 12.0dB |
| 7 | 10.5dB |
| 6 | 9.0dB |
| 5 | 7.5dB |
| 4 | 6.0dB |
| 3 | 4.5dB |
| 2 | 3.0dB |
| 1 | 1.5dB |
| 0 | 0.0dB |

- ID 397Recording Monitor Volume

| Value | MAX Level |
| --- | --- |
| 63 | 0dB |
| 62 | -1.5dB |
| 61 | -3.0dB |
| 60 | -4.5dB |
| 59 | -6.0dB |
| 58 | -7.5dB |
| 57 | -9.0dB |
| 56 | -10.5dB |
| ¥¥¥ | ¥¥¥ |
| 6 | -85.5dB |
| 5 | -87.0dB |
| 4 | -88.5dB |
| 3 | -90.0dB |
| 2 | -91.5dB |
| 1 | -93.0dB |
| 0 | -94.5dB |