Modem Riser 56K

Manual

Motorola

SM56 AC-L Soft Modem

Quick Start User's Guide

Contents

1	Introduction

2	Preparing The Computer For SM56 AC-L Installation

3	Installing The Modem Hardware

4	Installing The Modem Software on Windows 9x and Windows 2000

5	Installing The Modem Software on Windows NT

6	Verifying Correct Modem Installation

7	Troubleshooting

8	Un-installing and Upgrading the SM56 AC-L Modem

9	Changing The Operating System

10 	IDE Device Compatibility

11	Contact Information

Appendix A	SM56 AC-L Main Features

Appendix B	Software License Agreement

1	Introduction

Motorola's SM56 AC-L modem can be used on Intel reference motherboard platforms using Intel's core logic chipsets supporting AC-Link 2.1. Motorola's SM56 AC-L modem is a feature-rich, V.90 modem at an attractive price. It supports all fax and data fall back modes, and while it does include telephone answering machine(TAM) operation, it does not support speakerphone functionality. This document details installation, troubleshooting, and usage of the SM56 AC-L software modem.

The SM56 AC-L modem provides high-speed communications between your personal computer and a remote location, such as an Internet Service Provider (ISP), so you can:

- Receive data at up to 56 Kbps in V.90 or K56Flex modes

- Get automatic fallback to V.34 (33.6 Kbps) rates under bad line conditions and on non-V.90/K56Flex headends

- Send faxes from your personal computer at rates up to 14.4 Kbps

- Use your PC as a telephone answering machine (TAM)

- The SM56 AC-L modem runs on Windows 95/98, in a Windows 95/98 DOS box, Windows NT 4.0, and Windows 2000.

- Note: The modem cannot be used in pure DOS or on any other operating system. Note that Windos 9x, NT 4.0, and 2000 require different modem drivers. The driver for one OS will  not work on the other OS.Make sure you have the correct drivers before installing

- Computer OEMs that bundle the SM56 AC-L modem can benefit from sizable cost reductions, hardware reduction, and lower power consumption. End users benefit from quick, easy and affordable software upgrades, which help them keep current with the latest communications technology.

- Important: There are three different SM56 modem types, and each supports a different feature-set, as follows:

- Product Name					Modem Type			Main Feature-set Supported

- Motorola SM56 AC-L Fax Modem		Data/Fax only			All data and fax modes

- No telephone answering machine (TAM)

- No speakerphone

- No caller ID

- No distinctive ring

- ______________________________________________________________________________________

- Motorola SM56 AC-L Voice Modem	Data/Fax/TAM			All data and fax modes

- Telephone answering machine (TAM)

- Caller ID and distinctive ring

- No speakerphone

- ______________________________________________________________________________________

- Motorola SM56 AC-L Speakerphone	Data/Fax/TAM/Speakerphone	All data and fax modes

- Modem									Telephone answering machine (TAM)

- Full duplex speakerphone

- ______________________________________________________________________________________

- You can check which modem you have by selecting Start->Settings->Control Panel, then double click on the Modems icon. The modem name will appear in the Modem Properties window.

- 2	Preparing the Computer For SM56 AC-L Installation

- To ensure problem-free installation of the SM56 AC-L modem, ensure that a COM port is available for it to install on. The SM56 AC-L install program interrogates lower COM ports (below COM 5) and installs the modem on the first  available and unused port. Although the SM56 AC-L functions perfectly on any available COM port, some communications applications may not be able to use the modem if it gets installed above COM 4. Refer to the Troubleshooting section in this document for information on how to force the modem onto a COM port  below COM 5 after install is complete.

- You can check which COM ports are on your computer as follows:

- 1. Open the Windows 95/98 Control Panel.

- 2. Double click the System icon.

- 3. Choose the Device Manager tab.

- 4. Expand the Ports (COM and LPT) branch to see which COM ports are already installed.

- 3	Installing The Modem Hardware

- 1. Power down the computer.

- 2. Locate the AMR/MDC connector on the computer's motherboard, and carefully insert the SM56 AC-L modem board.

- 3. Connect the Wall jack on the modem board to an analog phone jack using an RJ-11 phone cable.

- 4. Power ON the computer.

- Important: When using the SM56 AC-L modem, or any modem for that matter, you should always connect the modem line input to a direct analog telephone line. This will maximize modem performance by avoiding any problems introduced by internal PBX systems, etc.

- 4	Installing Modem Software on Windows 9x and Windows 2000

- Important:  These SM56 AC-L modem drivers can be used on Windows 95/98 only.

- (Note: The Windows Install Wizard may vary slightly with different versions of the OS and different service packs)

- On starting Windows for the first time after installing the SM56 AC-L modem, the Plug and Play (PnP) sub-system detects the new hardware and begins an install wizard for the modem. The SM56 AC-L modem is detected as a new PCI Device. The wizard fist looks for the modem information (INF) file, which is included on Motorola's distribution disk or CD-ROM. This file contains information about the modem, including device type, device driver information, as well as the AT command/response sets that it supports.

- Select Driver from Disk Provided by Hardware Manufacturer (or Have Disk depending on the OS version) and browse to the SM56 AC-L modem INF file (called SM56ACL.INF) on the Motorola disk/CD-ROM. Windows will find the information on the disk. You may get an error message Cannot Locate File...... If you do, simply repoint the install wizard to the Motorola disk/CD-ROM and it will find the file the second time round.

- Windows copies the necessary SM56 AC-L files to the computer

- After modem installation is complete, Windows reports the SM56 AC-L modem as a Motorola SM56 AC-L Modem in the Control Panel and Device Manager.

- For D/F/TAM and D/F/TAM/Speakerphone modems, Windows finds a second device after the modem has been installed. Windows notifies you that it has found a Serial Wave Device for the modem and prompts for a Wave Device driver. Re-point to the Motorola disk/CD-ROM and click OK. It finds the INF file and installs the appropriate drivers. This serial wave device is used for telephone answering machine (TAM) playback.

- Note:

- The SM56 AC-L also installs a utility on the Windows System Tray (right side of the Task Bar). This application includes real time modem status (dialing, negotiating, connect in data mode, connect rate, etc.), in addition to user selection for COM port, country and language, and enable/disable Windows 9x DOS box support. There is also an audio test included, which allows you to verify if your sound subsystem supports Direct X 3.0 (or later) -- needed to hear call progress (dialing and negotiation).

- 5	Installing SM56 Software on Windows NT 4.0

- Important -- The SM56 software for Windows NT is Data/Fax only, there is no speakerphone or TAM supported.

- Windows NT 4.0 does not fully support plug and play (PnP). Therefore, after installing the modem board in the computer and booting, the operating system will not automatically recognize that new hardware is present. You must manually install the modem as follows:

- Step 1 -- Install a COM port for the modem

- You must install a COM port for the SM56 modem as follows:

- 1. Open the Windows NT Control Panel.

- 2. Double click the Ports icon. The Ports window opens.

- 3. Select Add.

- 4. Select a COM port number (in the range 1-4 preferably) that is not already installed, and allocate to it the standard resources as follows:

- COM Port		IO Address		IRQ

- 1			3F8			4

- 2			2F8			3

- 3			3E8			4

- 4			2E8			3

- 5. When your selections are complete click OK.

- 6. You will be prompted to restart the computer, do not do so yet. Select Don't Restart Now.

- 7. Close the Ports window.

- Step 2 -- Install the SM56 AC-L modem software

- 1. Open the Windows NT Control Panel.

- 2. Double click the Modems icon....the Install New Modem dialog appears.

- 3. Click the "Don't detect my modem; I will select it from a list" check box. Then click Next.

- 4. Click the Have Disk button.

- 5. Browse to the Windows NT SM56 AC-L modem install software. Click misgmodm.inf to highlight it. Then click Open, then click OK.

- 6. Select the Motorola SM56 AC-L Modem from the list. Click Next.

- 7. Click the port number selected in Step 1 above to highlight it as the COM port for the modem. Then click Next. The wizard will now copy all the necessary files and install the SM56 AC-L modem on your PC.

- 8. When complete, restart the PC to activate and start the COM port and SM56 AC-L modem devices. You are now ready to use the modem.

- 6	Verifying Correct SM56 AC-L Modem Installation

- Verifying Software Operation

- 1. Open the Windows Control Panel and double click the Modems icon.

- 2. Select the Diagnostics tab, and single click the Motorola SM56 AC-L Modem to select it.

- 3. Click on the More Info button.

- Within a few seconds a window will appear with the modem responses to various ATI commands issued to it. If this happens the modem software is properly installed and functioning correctly.

- Verifying Proper Hardware Operation

- To verify correct SM56 AC-L modem card hardware operation, you can use the following local analog loopback (LAL) test procedure, as follows:

- 1. Important: Remove the telephone line cable from the modem card.

- 2. Open Hyperterminal.

- 3. Optionally, select a connection name and icon.

- 4. In the Connect Using window, select the Motorola SM56 AC-L...... modem.  Click OK.

- 5. Enter a number in the Phone Number box...any number since we will not be dialing it. Click OK.

- 6.On the next screen select Cancel.

- 7. Enter AT and hit <Enter>.

- The response OK should appear.

- 8. Enter ATS46 = 23 and hit <Enter>.

- 9. Enter AT&T1 and hit <Enter>. Wait a few seconds.

- 10. Type some letters at the keyboard. If the hardware is functioning correctly, the letters you type appear on the HyperTerminal display.

- 11. To exit this test mode, type +++ in quick succession (note no carriage return) to escape to command mode, then type ATH <Enter> to hang up.

- 7	Troubleshooting

- If there is a problem when using the SM56 AC-L modem, and your communications application does not explain the problem, check the following list of symptoms and tips.

- My Communications Software Cannot Use the Modem at COM 5

- Some communications applications, such as older versions of AOL, do not communicate with a COM port higher than COM4. Sometimes, depending on your operating system and hardware configuration, the SM56 AC-L install program will not be able to install the modem on a COM port number below 5. If the modem installs on COM 5 or higher...

- You can manually force the COM number port using the SM56 Ac-L Modem helper application in the Windows System tray (just to the right of the Windows Task Bar).

- Windows 95/98 DOS box applications do not work with the modem

- By default DOS box support is disabled after installing the SM56 AC-L modem. You must enable it via the SM56 AC-L Modem helper application in the Windows System tray (just to the right of the Windows Task Bar). Make sure the DOS box check box is checked. If not, click it to check it, exit the SM56 AC-L Modem helper application and restart the computer. You must restart the computer to complete DOS box enabling.

- If the Enable DOS box check box is already checked (i.e. enabled), and DOS box application still do not work, try:

- 1.   Open the Control Panel.

- 2.   Double click the System icon.

- 3.    Expand the Modem branch.

- 4.   Double click the Motorola SM56 AC-L modem.

- 5.   Select the Modem tab.  Make a note of the COM port number.

- 6.   Select the Resources tab. Make a note of the IRQ number listed.

- 7.   For DOS support to operate correctly, the SM56 modem cannot occupy the standard IRQ for the COM port number.

- 8.   Uncheck the Use Automatic Resources check box.

- Double click on the Interrupt Request label and change the IRQ to a different number that is not in conflict with another device. (If there is no free IRQ:  free one, or change other device IRQ levels to free a non-standard IRQ).

- The standard I/O addresses and IRQs for COM Ports 1-4 are as follows:

- COM Port		IO Address		IRQ

- 1			3F8			4

- 2			2F8			3

- 3			3E8			4

- 4			2E8			3

- There is no dial tone

- 1.   Ensure that the telephone cable is securely connected at both ends.

- 2.   Unplug the telephone line cable from the computer, and connect it directly to a telephone from the wall outlet. Check for a dial tone. If there is none, the problem is in the telephone line or system. Call the service provider.

- When Attempt To Dial You Get An Error Message

- When you try to dial a connection an error message box pops up, indicating that your are not connected to a proper analog phone line. This is the SM56 AC-L digital line guard feature which protects against hardware damage if the modem is plugged into a high current phone outlet -- such as a digital phone jack. The SM56 AC-L automatically detects the over-current and goes back on-hook before hardware damage occur. Check the phone outlet and make sure that it is a real analog phone line. Try another analog phone jack.

- I Cannot Hear Call Progress

- 1.   Make sure your speakers are connected to the speaker output jack on your sound system.

- 2.	Call progress reporting on the SM56 AC-L requires that Direct X 3.0 or later be installed on your PC, and that your sound system supports Direct X (3.0 and later) playback. To test, go into the SM56 AC-L Modem helper Desktop Tray application and select the Call Progress tab. Then click the Test button. If you do not hear anything then your sound system does not support Direct X 3.0. Contact your sound card vendor for driver updates or more information.

- The modem cannot complete a connection to another modem

- 1.   Ensure that your modem is dialing the correct number. Ensure that you've specified the correct area code, if one is required.

- 2.  Determine whether the remote modem is correctly configured to communicate with yours.

- The modem does not answer incoming calls

- 1.   Ensure that the automatic answer parameter is set to one of the enabled options, using the ATS0 command (ATS0=1 to answer after one ring, ATS0=2 to answer after two rings, and so on).

- 2.   Ensure that no other devices, such as fax or answering machines, are answering calls before the modem does.

- .

- The modem disconnects while transmission is in progress

- 1.   Ensure that the telephone cable is securely connected at both ends.

- 2.   Ensure that call-waiting is disabled. In most areas, the command *70 or  #70 disables call-waiting. Check with your telephone company for the correct key sequence.  (With call-waiting, the incoming call's click sound may be disrupting your call.)

- 3.   IDE activity (hard drive and CD-ROM devices) can cause an AC-Link modem to drop connections. Refer to the IDE Device Compatibility section for more details.

- Data is not transmitted or received for unusually long periods of time

- 1.   Re-dial the call.  (The telephone line connection may be poor.)

- 2.   Try another ISP number, the server could be busy.

- The computer runs slower than usual

- 1.   Close any open applications that you are not using.

- You cannot enter tone selections successfully when calling tone-driven applications

- When dialing a remote system that requires you to enter selections using the telephone keys, such as a voice-mail depot or bank-account information provider, you can lengthen the duration of the tones your modem sends, so that the remote system can detect them better.  To adjust DTMF tone length, use the AT+VTDn command, where n specifies the tone duration. Check the SM56 AC-L online Help (via the SM56 AC-L Modem helper application on the Windows Desktop Tray) for a list of supported AT Commands.

- The modem does not respond to AT commands

- 1.   Ensure that your communications software is configured to use the same COM port as the modem's COM port.

- 2.   Reset modem parameters to default options by entering AT&F; then re-enter custom options.

- The modem responds to commands, but they do not appear on the screen

- Make sure that the local display echo option is enabled by entering ATE1.

- You've installed a new peripheral device; now the modem does not work

- 1.   In the Windows desktop tray, select Start.

- 2.   Select Help. The Windows Help Topics window appears.

- 3.   Select the Contents tab.

- 4.   Select If You Have a Hardware Conflict.

- 5.   A series of troubleshooting actions appears.  Follow the appropriate sequence.

- The modem connects; then meaningless characters appear

- 1.   Open the Control Panel. Double click the Modems icon.

- 2.   Click Motorola SM56 AC-L Modem once to select..

- 3.   Click on Properties.

- 4.   Select Connection.

- 5.   Click on Advanced.

- 6.   Check the Use Error Control-Required to Connect box.

- The modem cannot connect; the Error Control option is selected

- The modem may be connecting at a rate higher than appropriate for the line conditions.

- 1.   Use the AT%B command to limit the SM56 AC-L maximum connection rate. (For a list of AT commands, refer to the on-line User's Guide.)

- 2.   Lower the rate, using AT commands, until the problem is corrected. You can add AT commands to do this; refer to the next section .

- How Do I  Add AT Commands Before the Modem Dials My ISP?

- 1.   Open the Control Panel.

- 2.   Double-click the Modems icon.

- 3.   Click on once to select the Motorola SM56 AC-L Modem.

- 4.   Click on Properties.

- 5.   Select Connection.

- 6.   Click on Advanced.

- 7.   In the Extra Settings box, chain AT commands as needed. You do not need to enter the AT prefix, but it will not hurt if you do.

- 8	Un-installing and Upgrading the SM56 AC-L Modem

- SM56 AC-L Modem Un-Install Procedure

- To un-install the modem (all builds prior to Build 67 preview 12) :

- 1. Open the Windows Device Manager.

- 2. Select the SM56 AC-L Modem from under the Modem key.

- 3. Click the Remove button to remove the modem device.

- 4. Shut down the computer and remove the AMR/MDC card from the computer. The SM56 AC-L modem software remains on the hard disk. If you do not remove the modem card, the modem will auto-install the next time you start Windows

- Note: To prevent the modem auto-installing the next time Windows is started (modem board installed), browse to \Windows\Inf\Other and delete the Motorola SM56 AC-L INF files.

- To un-install Build 67 preview 12 and later:

- 1. Open the Windows Control Panel

- 2. Double click the Add/Remove Programs icon.

- 3. Select Motorola SM56 AC-L Modem Un-install and click Add/Remove.

- 4. Shut down the computer and remove the AMR/MDC card from the computer. The SM56 AC-L modem software remains on the hard disk. The next time you start the computer with the SM56 AC-L modem board installed, the modem drivers will auto-install.

- Note: To prevent the modem auto-installing the next time Windows is started (modem board installed), browse to \Windows\Inf\Other and delete the Motorola SM56 AC-L INF files.

- SM56 AC-L Modem Upgrade Procedure

- Upgrading From Builds Earlier Than Build 67 Preview 12

- If you wish to upgrade the SM56 AC-L software from any Build earlier than Build 67 Preview 12 you must perform a complete un-install procedure, restart the PC, then perform a complete new install.

- Upgrade the SM56 AC-L modem to a newer version as follows:

- 1. Obtain the latest driver set from Motorola.

- 2. Completely remove the earlier SM56 AC-L drivers. In Device Manager select the SM56 AC-L and click Remove. Browse to the \Program Files folder (or \Windows\Motorola folder, depending on the build number) and delete Motset95 folder. Browse to \Windows\System and delete sm56ich.vxd and sm56ich1.vxd

- 3. Restart your computer. Perform a brand new install (see install section earlier).

- Upgrading From Build 67 Preview 12 And Later

- Upgrade the SM56 AC-L modem to a newer version as follows:

- 1. Obtain the latest driver set from Motorola.

- 2. Run the upgrade utility -- sm56set.exe

- 9	Changing the Operating System

- The SM56 AC-L modem drivers are the same for Windows 95 and Windows 98. If the modem is installed on Windows 95 and you upgrade to Windows 98, the SM56 AC-L will continue to function as before. No new drivers are required.

- However, if you change from Windows 95/98 to Windows NT 4.0 or Windows 2000 you must get a new set of drivers for that operating system. Contact your SM56 AC-L modem supplier for driver status and updates.

- 10	IDE Device Compatibility

- Higher interrupt priority devices and high volume hard drive or CD-ROM activity on AC-Link PC's can pre-empt modem operation and potentially cause retrains or even disconnects. Examples include playing a game while downloading a file from The Net, or sometimes, even playing a graphic/disk intensive online game such as Quake. To avoid this you must configure your IDE devices to use DMA mode, in the BIOS (if selectable) and in Windows.

- Changing the BIOS:

- 1. Restart your computer

- 2. Enter the BIOS configuration system menus.

- 3. Locate the hard drive configuration screens.

- 4. Make sure that PIO 4 and DMA2 are selected -- note that some BIOS's may not have these selections.

- Changing in Windows 9x:

- 1. Boot into Windows.

- 2. Go into the Device Manager.

- 3. Select Disk Drives, and double click the hard drive tag.

- 4. Select the Settings tab and click to enable DMA.

- 5. repeat for your CD-ROM drive.

- 11	Contact Information

- If you have a problem with the SM56 AC-L modem, ensure that the problem and its solution are not shown in the Troubleshooting section. If you cannot resolve it through this list first contact your direct SM56 AC-L modem or PC supplier.

- Modem related information should include as much detail as possible to allow Intel and Motorola engineers to qualify and reproduce (if necessary) the problems, including:

- Information about  your modem:

- SM56 AC-L version number (find this by recording the modem's response to the ATI3 command)

- Information about  your setup:

- The telephone number you are calling from.

- The telephone number you are calling to.

- If performing a lab test, a detailed description of the equipment used.

- The remote modem information.

- Direct external analog telephone line or through a PBX -- use direct analog lines where possible.

- Information about  the problem:

- The actions and steps that you performed.

- A description of what you saw; be specific.

- A description of what you expected to see.

- If possible, a description of what you saw using a non-AC-Link modem under the same conditions.

- Appendix A:  SM56 AC-L Main Features

- Compatibility with Windows 95/98 and Win 9x DOS Box communication applications.

- Full Windows Plug and Play support.

- Powerful modem installation, un-installation, and upgrade engines.

- Data and fax mode operation.

- Support for various data modulation modes:

- - V.90 connection rates if the headend is a true V.90 location. V.90 downstream rates  to 56Kbps. Upstream rates to 33.6Kbps (V.34).

- - Fallback to K56flex® mode if the headend is K56flex, not V.90. K56flex® downstream rates  to 56Kbps. Upstream rates to 33.6Kbps(V.34).

- - Connection at V.34 rates (33.6 Kbps) if the headend is not V.90 or K56flex®.

- - V.32bis, V.32, V.22bis, V.23, V.22/B212, V.21, Bell 103.

- Error correction - V.42, LAPM, MNP2-4.

- Data compression - V.42bis, MNP5.

- Fax modes supported - V.17, V.27ter, V.29.

- Supports US Caller ID.

- Supports Telephone Answering Machine (TAM).

- Tone and pulse (SiLabs Rev. B chipset) dialing.

- Call progress monitoring through the PC sound system (using Direct X 3.0 and later).

- Adaptive rate re-negotiation (up and down) during a connection to compensate for line dynamics.

- Includes Local Analog Loopback (LAL) for quick hardware troubleshooting.

- Includes a very useful SM56 AC-L Modem helper application to allow country and language selections, report real time modem status, and provides sound system test utility for Direct X 3.0 (and later).

- Online User's Guide in standard Windows HLP format. Includes modem usage and troubleshooting details, as well as a complete listing of supported AT commands.

Appendix B: Software License Agreement

THE FOLLOWING AGREEMENT IS A LEGAL AGREEMENT BETWEEN YOU (EITHER AN INDIVIDUAL OR ENTITY), AND MOTOROLA, INC. (FOR ITSELF AND ITS LICENSORS).  THE RIGHT TO USE THIS SOFTWARE IS BASED ON THE CONDITION THAT YOU AGREE TO THE FOLLOWING LICENSE.  USING THIS PRODUCT CONSTITUTES ACCEPTANCE OF THE TERMS AND CONDITIONS SET FORTH IN THIS AGREEMENT.  IF YOU DO NOT AGREE TO THE TERMS OF THIS AGREEMENT, THEN DO NOT USE THE SOFTWARE

The enclosed computer program(s), SM56 Software Modem ("Software") is licensed, not sold, to you by Motorola, Inc. ("Motorola") for use only under the terms of this License, and Motorola reserves any rights not expressly granted to you. Motorola and its licensors retain ownership of the Software itself and its accompanying written materials, which are protected by the copyright laws of your country and international treaty provisions.

1.	License.  This License allows you to use one copy of the Software with the Beta Kit (Intel PIIX Bus Master IDE Driver for Windows NT) supplied from Intel.  To "use" the Software means that the Software is either loaded in the temporary memory (e.g., RAM) of a computer or installed on the permanent memory of a computer (e.g., hard disk). Licensee hereby acknowledges that the licenses granted under this Agreement may be suspended in some cases if Licensee assert an Essential patent for an ITU or prior CITT analog modem standard against Motorola or a third party.  The use of the Software is restricted to personal, non-commercial testing of the Software.  Any other use is expressly prohibited.

2.	Restrictions.  The Software contains trade secrets in a human or machine perceivable form and, to protect them, you may not REVERSE ENGINEER, DECOMPILE, DISASSEMBLE OR OTHERWISE REDUCE THE SOFTWARE TO ANY HUMAN OR MACHINE PERCEIVABLE FORM.  YOU MAY NOT MODIFY, ADAPT, TRANSLATE, RENT, LEASE, LOAN OR CREATE DERIVATIVE WORKS BASED UPON THE SOFTWARE OR ANY PART THEREOF.

3.	Termination.  This License is effective until terminated.  This License will terminate immediately without notice form Motorola or judicial resolution if you fail to comply with any provision of this License.  Motorola can terminate this license at will upon proper notice to you.  In any event this License will terminate 6 months from the date you received this Software.  Upon such termination you must destroy the Software, all accompanying written materials and all copies thereof, and Sections 5, 6, 7 and 8 will survive any termination.

4.	Export Law Assurances.  You agree that neither the Software nor any direct product thereof is being or will be shipped, transferred or re-exported, directly or indirectly, into any country prohibited by the United States Export Administration Act and the regulations thereunder or will be used for any purpose prohibited by the Act.

5.	Warranty.  The Software and written materials are provided "AS IS" and without warranty of any kind.  Motorola's entire liability and your sole and exclusive remedy for any breach of the foregoing limited warranty will be, at Motorola's option, replacement of the disk(s) or refund the amount paid for this Software License.

NO OTHER WARRANTY IS PROVIDED BY MOTOROLA, AND MOTOROLA AND ITS LICENSORS EXPRESSLY DISCLAIM ALL OTHER WARRANTIES, EITHER EXPRESS OF IMPLIED, INCLUDING BUT NOT LIMITED TO IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT.  MOTOROLA DOES NOT WARRANT THAT THE OPERATION OF THE SOFTWARE WILL BE UNINTERRUPTED OR ERROR-FREE, OR THAT DEFECTS IN THE SOFTWARE WILL BE CORRECTED.  NO ORAL OR WRITTEN REPRESENTATIONS MADE BY MOTOROLA OR AN AGENT THEREOF SHALL CREATE A WARRANTY OR IN ANY WAY INCREASE THE SCOPE OF THIS WARRANTY.  BECAUSE SOME JURISDICTIONS DO NOT ALLOW THE EXCLUSION OR LIMITATION OF IMPLIED WARRANTIES, THE ABOVE LIMITATIONS MAY NOT APPLY TO YOU.

6.	Limitation of Remedies and Damages.  Regardless of whether any remedy set forth herein fails of its essential purpose, in no event shall Motorola or any of the licensors, directors, officers, employees or affiliates of the foregoing be liable to you for any consequential, incidental, indirect, special or similar damages whatsoever (including, without limitation, damages for loss of business profits, business interruption, loss of business information and the like), whether foreseeable or unforeseeable, arising out of the use or inability to use the Software or accompanying written materials, regardless of the basis of the claim and even if Motorola or a Motorola representative has been advised of the possibility of such damage.  Motorola's liability to you for direct damages for any cause whatsoever, and regardless of the basis of the form of the action, will be limited to the price paid for the Software that caused the damages.  THIS LIMITATION WILL NOT APPLY IN CASE OF PERSONAL INJURY ONLY WHERE AND TO THE EXTENT THAT APPLICABLE LAW REQUIRES SUCH LIABILITY WITHOUT THIS LIMITATION.  BECAUSE SOME JURISDICTIONS DO NOT ALLOW THE EXCLUSION OR LIMITATION OF LIABILITY FOR CONSEQUENTIAL OR INCIDENTAL DAMAGES, THE ABOVE LIMITATION MAY NOT APPLY TO YOU.

7.	Complete Agreement.  This License constitutes the entire agreement between the parties with respect to the use of the Software and related documentation, and supersedes all prior or contemporaneous understandings or agreements, written or oral, regarding such matter.  No amendment to or modification of this License will be binding unless in writing and signed by a duly authorized representative of Motorola.

Note:  This equipment may be connected to Type 1 telecommunications carrier facilities with a maximum DC Loop Resistance of 1371 ohms, and interoperate correctly with central office switching facilities.  This equipment may not operate correctly if connected to Type 1 telecommunications carrier facilities with a DC Loop Resistance greater than 1371 ohms.

© 1999 SMI.

Prepared by: Lauren Haven