Chapter 2

SYSTEM COMPONENTS

This section describes the major components of the mother-
board.

Motherboard revision number:	R01

# n  CPU

The CPU is a 64-bit microprocessor chip that forms the basis of the high-performance system with 16KB internal cache memory on chip.

## CPU Installation

NOTE:
Static electricity can destroy electronic devices. Whenever you handle any option outside of its protective packaging, first discharge any static electricity from your body by touching a protective grounding device or unpainted metal on the rear panel of the system unit.

To install CPU in a ZIF socket:

1.	Find the ZIF socket on the motherboard. (See p.2-1.)

2.	Lift the ZIF socket arm up to the vertical position.

3.	Align the CPU so its Pin 1 corner (beveled corner) is at the Pin 1 corner of the ZIF socket. Then insert the CPU's pins into the corresponding holes in the socket.

NOTE:
You are suggested to install a cooling fan to cool the CPU.

4.	Press the arm downwards to the horizontal position. You will feel some resistance while doing so. This is normal as the pressure starts to secure the CPU in place.

5.	Set the jumpers (JP5, JP7, JP8, JP10~JP12) according to the CPU type and speed. (See P.3-3.)

# n  Chipset

The 82430 VX PCIset consists of the VX System Controller (TVX), the two VX Data Paths (TDX), and the PCI ISA IDE Accelerator (PIIX3).

·	PCI 2.1 compliant

·	Integrated DRAM controller

-	64-bit path to memory

-	4MB to 128MB main memory

-	EDO/Fast Page mode DRAM support

-	Support for Symmetrical DRAMs

-	supports SDRAM

-	5 QWord deep buffer for 3-1-1-1 posted writes, and
	DWord and burst merging

-	Supports mixed memory technologies
	(EDO/FP/SDRAM)

-	Supports 3V or 5V DRAMs

·	Integrated second level cache controller

-	Direct mapped organization

-	Supports 256K and 512K pipelined burst SRAM

-	Cache hit read/write cycle timings at 3-1-1-1 with
	burst SRAM

-	Back-to-back read/write cycles at 3-1-1-1-1-1-1-1

-	Supports write-back

·	Fully synchronous 25/30/33MHz PCI bus interface

-	Five PCI bus masters (including PIIX3)
-	Converts back-to-back sequential PCI memory
	writes to PCI burst writes

-	CPU-to-PCI memory writes posting with 5 DWord
	deep buffers

-	PCI-to-DRAM read prefetching (5 QWord)

-	PCI-to-DRAM posting (18 DWord)

-	Multi-transaction timer to support multiple short PCI
	transactions

·	Supports the Universal Serial Bus (PIIX3 USB)

·	208-pin QFP system controller (TVX), two 100-pin QFP data paths (TDX)

# n  ROM BIOS

The BIOS holds the fundamental functions and acts as a communication channel between the motherboard and the rest of the system. You can configure the motherboard through the SETUP program. Details regarding the ROM BIOS are thoroughly discussed in Chapter 5.

# n  System Memory

There are four 72-pin SIMM and two 168-pin DIMM sockets. Each can support 1MB, 2MB, 4MB and 8MB SIMM/DIMM modules. Refer to Chapter 4 for DRAM configuration and installation.

# n  Cache Memory

The effectiveness of the cache is determined predominantly by the size and organization of the cache. The motherboard design allows you to configure the external cache for 256KB or 512KB. It uses the write-back cache algorithm to minimize access time to the main memory, which improves overall system performance. Refer to Chapter 4 for proper configuration and installation.