Chapter 5

Award BIOS SETUP

# n  Introduction

The SETUP program allows you to enter the system configuration information. This information is needed by the system to identify the type of devices installed and to set up special features. Typical configuration information includes the date and time, the type of disk drives, and the amount of memory.

The configuration information is stored in a special kind of memory called CMOS (Complementary Metal Oxide Semiconductor) RAM. CMOS RAM data are backed up by a RTC backup battery.

The configuration information has been set up in the factory for optimal performance. You only need to run SETUP when:

·	you see an error message on the screen requesting you to run SETUP

·	you make certain changes for special features

·	you update the configuration information after adding or removing hardware

·	you correct the system date and time

## Running SETUP and Moving Around

NOTE:
The SETUP program may have been updated after this manual was published. Your actual SETUP screens may vary slightly from those shown in this chapter.

There are two ways to access the SETUP program:

·	Turn on the system and press Delete immediately.

·	Turn on the system and press Delete when you see the message that briefly appears at the bottom of the screen telling you how to enter SETUP.

The figure below shows the first SETUP menu (main menu). The main menu provides nine setup functions and two exit choices.

| STANDARD CMOS SETUP BIOS FEATURES SETUP CHIPSET FEATURES SETUP POWER MANAGEMENT SETUP PNP/PCI CONFIGURATION LOAD BIOS DEFAULTS LOAD SETUP DEFAULTS | INTEGRATED PERIPHERALS PASSWORD SETTING IDE HDD AUTO DETECTION SAVE & EXIT SETUP EXIT WITHOUT SAVING |
| --- | --- |

SETUP Main Menu

In this main menu, use arrow keys to move among the items and press Enter to accept or enter the sub-menu. In a sub-menu, use arrow keys to move among the items and press +/-/PgUp/PgDn to select a value.

A brief description of keyboard usage is listed below:

| Key | Function |
| --- | --- |
| Esc | 1. Quit without saving. 2. Exit to the main menu. |
| á, â, à, ß | Moves the cursor to a desired item. |
| +, -, PgUp PgDn | Cycle through the pre-defined values for the selected item. Pressing the plus key or PgUp brings up the next value; pressing the minus key or PgDn does the contrary. |
| F1 | Displays the help screen for the selected item. |
| F2, Shift-F2 | Changes the colors on the screen. |
| F5 | Restore the previous values from CMOS for the option setup menus*. |
| F6 | Loads the BIOS default values for the option setup menus*. |
| F7 | Loads the SETUP default values for the option setup menus*. |

*	Option setup menus refer to BIOS FEATURES SETUP, CHIPSET FEATURES
	SETUP, POWER MANAGEMENT SETUP, PNP/PCI CONFIGURATION, and
	INTEGRATED PERIPHERALS menus.

# n  STANDARD CMOS SETUP

When you select STANDARD CMOS SETUP from the main menu, the submenu as shown below appears that contains all the items in a standard BIOS (Basic Input/Output System).

Date (mm:dd:yy)		: Tue, May 14 1996
Time (hh:mm:ss)		: 10 : 36 : 12

HARD DISKS	: TYPE	SIZE	CYLS	HEAD	 PRECOMP	LANDZ	 SECTOR	MODE
----------------------------------------------------------------------------------------
Primary Master	: Auto	  0	   0	   0	     0		  0	     0	AUTO
Primary Slave	: Auto	  0	   0	   0	     0		  0	     0	AUTO
Secondary Master	: Auto	  0	   0	   0	     0		  0	     0	AUTO
Secondary Slave	: Auto	  0	   0	   0	     0		  0	     0	AUTO

Drive A		: 1.44 M, 3.5 in.
Drive B		: None

Video		: EGA / VGA
Halt On		: All Errors

STANDARD CMOS SETUP Menu

The followings describe all the items in this menu.

### Date / Time

The date and time might be incorrect when you start up your system for the first time. Enter the correct value for each field. Note that the time is based on a 24-hour format.

### Primary/Secondary Master/Slave

This item sets the hard disk type.

CAUTION:
The hard disk will not work properly if you enter incorrect information. For a hard disk type not matched or listed, you can select "User" to manually define your own type.

The available options are:

Options	Descriptions
None	Select this option if no hard disk is installed.
1 to 45	Select one of the 45 pre-defined types which matches
	your hard disk.
User	Select this option to manually enter the parameters
	such as number of cylinders, number of heads,
	landing zone, write precompensation, number of
	sectors/track, and access mode.
Auto	Select this option if your hard disk supports the "auto-
	detect" function. If you select "Auto", you must also
	use the IDE HDD Auto Detect function. (See "IDE
	HDD AUTO DETECT" in this chapter.)

NOTE:
If you change the controller of the hard disk interface, note that for an ESDI interface, select type "1"; for a SCSI or CD-ROM interface, select "None".

### Drive A/ B

This item sets the floppy disk drive type. The available options are None, 360K 5.25 in, 1.2M 5.25 in, 720K 3.5 in, 1.44M 3.5 in, and 2.88M 3.5 in.

### Video

This item sets the type of video in your system. The available options are EGA/VGA, CGA 40, CGA 80, and MONO.

### Halt On

This item sets if the system will stop when an error is detected during power up.

The available options are:

Options		Descriptions
All Errors		The system will stop and you will be
		prompted whenever the POST detects a
		non-fatal error.
No Errors		The system will not stop for any error that
		may be detected.
All, But Keyboard	The system will not stop for a keyboard
		error; it will stop for all other errors 
All, But Diskette	The system will not stop for a floppy disk
		drive error; it will stop for all other errors.
All, But Disk/key	The system will not stop for a keyboard or
		floppy disk drive error; it will stop for all other
		errors.

# n  BIOS FEATURES SETUP

When you select BIOS FEATURES SETUP from the main menu, the submenu as shown below appears that contains items for special enhanced features.

Virus Warning			: Disabled
CPU Internal Cache		: Enabled
External Cache			: Enabled
Quick Power On Self Test		: Disabled

Boot Sequence			: A, C
Swap Floppy Drive		: Disabled
Boot Up Floppy Seek		: Enabled
Boot Up NumLock Status		: On

Typematic Rate Setting		: Disabled
Typematic Rate (Chars/Sec)	: 6
Typematic Delay (msec)		: 250
Security Option			: Setup

OS Select For DRAM>64MB	: Non-OS2

BIOS FEATURES SETUP Menu

The followings describe all the items in this menu.

### Virus Warning

If this item is set to Enabled, during and after the system boots up, any attempt to write to the hard disk's boot sector or partition table will halt the system and the following error message will appear. In the mean time, you can run an anti-virus program to locate the problem.

!WARNING!
Disk boot sector is to be modified
Type Y to accept write or N to abort write

The available options are Enabled and Disabled.

NOTE:
This function is available only for DOS or other operating systems that do not trap INT13.

### CPU Internal Cache

This item allows you to use the internal cache on the CPU. The available options are Enabled and Disabled.

### External Cache

This item allows you to use the external cache. The available options are Enabled and Disabled.

### Quick Power On Self Test

When set to Enabled, this item speeds up Power On Self Test (POST) by shortening or skipping some check items.

The available options are Enabled and Disabled.

### Boot Sequence

This item sets the sequence of booting.

The available options are:

Options		Descriptions
C, A		The system will try to boot from drive C (hard
		disk) first whenever turned on. If the hard disk is
		not bootable, the system will then try to boot
		from drive A.
A, C		The system will try to boot from drive A first
		whenever turned on. If a diskette with operating
		system does not exist, the system will then try
		to boot from drive C (hard disk).
C, CDROM, A	The system will try to boot from the drive C
		whenever turned on; If a drive C with operating
		system does not exist, the system will then try
		to boot from the CD-ROM. If CD-ROM is not
		bootable, the system will then try to boot from
		drive A.
CDROM, C, A	The system will try to boot from the CD-ROM
		whenever turned on; If a CD-ROM with
		operating system does not exist, the system will
		then try to boot from drive C. If drive C is not
		bootable, the system will then try to boot from
		drive A.

NOTE:
This function is available only for IDE type disks. For SCSI type disks, the sequence is always "A, C".

### Swap Floppy Drive

This feature allows you to exchange the drive names of the two floppy disk drives, if installed. Make sure that Drive A and Drive B item in the STANDARD CMOS SETUP menu are updated accordingly.

The available options are Enabled and Disabled.

### Boot Up Floppy Seek

This item sets if the system will verify the floppy disk drive type during POST.

The available options are Enabled and Disabled. Select Disabled to bypass the floppy disk drive check and thus speed up POST.

### Boot Up NumLock Status

This item sets if the Num Lock key will be automatically activated after system startup. The available options are On and Off.

### Typematic Rate Setting

This item is the master control of the following two items: Typematic Rate and Typematic Delay.

The available options are Enabled and Disabled. When set to Disabled, the values of the following two items will be determined by the keyboard.

### Typematic Rate (Chars/Sec)

This item controls the rate at which a character is repeated (in characters/second) whenever a key is held down.

The available options are 6, 8, 10, 12, 15, 20, 24, and 30.

### Typematic Delay (msec)

This item controls the delay time (in msec) when a key is pressed and held down.

The available options are 250, 500, 750, and 1000.

### Security Option

This feature allows the password to prevent unauthorized use of the system and the SETUP program.

The available options are:

Options	Descriptions
System	The system will not boot and access to SETUP will be
	denied if the correct password is not entered at the
	prompt.
Setup	The system will boot, but access to SETUP will be
	denied if the correct password is not entered at the
	prompt.

NOTE:
This item is effective only when a password is set. (See "PASSWORD SETTING" in this chapter.)

### OS Select For DRAM>64MB

If your system memory size is larger than 64MB, you have to define the communication type between BIOS and your operating system.

The available options are OS/2 and Non-OS2.

# n  CHIPSET FEATURES SETUP

When you select CHIPSET FEATURES SETUP from the main menu, the submenu as shown below appears that contains items for chipset-specific features.

IDE HDD Block Mode		: Enabled
IDE Primary Master PIO		: Auto
IDE Primary Slave PIO		: Auto
IDE Secondary Master PIO	: Auto
IDE Secondary Slave PIO		: Auto
Onboard Primary PCI IDE		: Enabled
Onboard Secondary PCI IDE	: Enabled
Cyrix Branch Target Buffer	: Enabled

8 Bit I/O Recovery Time		: 1
16 Bit I/O Recovery Time		: 1
SDRAM (CAS Lat/RAS-to-CAS)	: 3/3
DRAM Timing			: 70ns

CHIPSET FEATURES SETUP Menu

NOTE:
CHIPSET FEATURES Setup is for technicians or advanced users only. You are not advised to change the default settings.

# n  POWER MANAGEMENT SETUP

When you select POWER MANAGEMENT SETUP from the main menu, the submenu as shown below appears that contains items for Power Management features.

Power Management		: Min Saving
PM Control By APM		: Yes
Video Off Method			: V/H SYNC+Blank
MODEM Use IRQ		: 3

Doze Mode			: 1 Min
Standby Mode			: 1 Min
Suspend Mode			: 1 Min
HDD Power Down		: 15 Min

POWER MANAGEMENT SETUP Menu

The Power Management feature helps conserve power by putting the system into low power modes when the system is inactive for a period of time, called "time-out." The system will wake up whenever an activity is detected (e.g. pressing a key).

Power Management works in a 3-stage manner following this order:

1.	Doze mode

2.	Standby mode

3.	Suspend mode

The time-out period set for these three modes are sequential in effect. That is after the Doze mode time-out is reached, the Power Management starts the time-out for Standby mode, and after Standby mode the time-out for Suspend mode.

The followings describe all the items in this menu.

### Power Management

This item is the master control of the Power Management features.

The available options are:

Options	Descriptions
Disabled	The Power Management feature does not take
	effect.
User Defined	All the settings can be defined for the Power
	Management to work in the way as specified by
	the user.
Min Saving	Some items are pre-defined for the Power
	Management to assumedly save minimum power.
	The pre-defined settings are:
		Doze Mode:		1 Hour
		Standby Mode:		1 Hour
		Suspend mode:		1 Hour
		HDD Power Down:	15 Min
Max Saving	Some items are pre-defined for the Power
	Management to assumedly save maximum
	power. The pre-defined settings are:
		Doze Mode:		1 Min
		Standby Mode:		1 Min
		Suspend Mode:		1 Min
		HDD Power Down:	1 Min

### PM Control By APM

If the APM (Advanced Power Management) driver is installed, this item allows you to enable or disable control from APM.

The available options are:

Options	Descriptions
No	The system BIOS will ignore APM.
Yes	The system BIOS will wait for APM's prompt before it
	enters any power saving mode.

### Video Off Method

This item defines the screen-off status.

The available options are:

Options	Descriptions
Blank Screen	The screen will be blank.
V/H SYNC + Blank	The screen will be blank and the V-SYNC
	and H-SYNC signals will be turned off.
DPMS Supported	This function is enabled for video systems
	supporting DPM.

### MODEM Use IRQ

This item defines the modem IRQ channel. The available options are 3, 4, 5, 7, 9, 10, 11, and NA.

### Doze Mode

Setting this item allows your system to enter Doze mode when no "major device activities" have occurred during the set time-out period. (Major devices refer to the keyboard, mouse, FDD, hard disk and SIO/PIO.) Any major activity will bring the system out of Doze mode.

The available options are Disabled, 1 Min, 2 Min, 4 Min, 6 Min, 8 Min, 10 Min, 20 Min, 30 Min, 40 Min and 1 Hour.

### Standby Mode

Setting this item allows your system to enter Standby mode when no "major device activities" have occurred during the set time-out period. (Major devices refer to the keyboard, mouse, FDD, hard disk and SIO/PIO.) Any major activity will bring the system out of Standby mode.

The available options are Disabled, 1 Min, 2 Min, 4 Min, 6 Min, 8 Min, 10 Min, 20 Min, 30 Min, 40 Min and 1 Hour.

### Suspend Mode

Setting this item allows your system to enter Suspend mode when no "major device activities" have occurred during the set time-out period. (Major devices refer to the keyboard, mouse, FDD, hard disk and SIO/PIO.) Any major activity will bring the system out of Suspend mode.

The available options are Disabled, 1 Min, 2 Min, 4 Min, 6 Min, 8 Min, 10 Min, 20 Min, 30 Min, 40 Min and 1 Hour.

### HDD Power Down

Setting this item causes the hard disk to enter power saving mode (motor off) if it is not in use during the set time-out period.

The available options are Disabled, and 1 Min to 15 Min.

# n  PNP/PCI CONFIGURATION

When you select PNP/PCI CONFIGURATION from the main menu, the submenu as shown below appears that contains items for configuring "Plug & Play" and the PCI devices.

PNP OS Installed			: No
Resources Controlled By		: Manual
Reset Configuration Data		: Disabled

IRQ-3	assigned to	: Legacy ISA
IRQ-4	assigned to	: Legacy ISA
IRQ-5	assigned to	: PCI/ISA PnP
IRQ-7	assigned to	: PCI/ISA PnP
IRQ-9	assigned to	: PCI/ISA PnP
IRQ-10	assigned to	: PCI/ISA PnP
IRQ-11	assigned to	: PCI/ISA PnP
IRQ-12	assigned to	: PCI/ISA PnP
IRQ-14	assigned to	: PCI/ISA PnP
IRQ-15	assigned to	: PCI/ISA PnP
DMA-0	assigned to	: PCI/ISA PnP
DMA-1	assigned to	: PCI/ISA PnP
DMA-3	assigned to	: PCI/ISA PnP
DMA-5	assigned to	: PCI/ISA PnP
DMA-6	assigned to	: PCI/ISA PnP
DMA-7	assigned to	: PCI/ISA PnP

PCI IRQ Actived By		: Level
PCI IDE 2nd Channel		: Enabled
PCI IDE IRQ Map To		: PCI-AUTO
   Primary IDE INT#		: A
   Secondary IDE INT#		: B

PNP/PCI CONFIGURATION Menu

The Plug & Play function automatically sets the IRQ channel, the memory address and the I/O address for your system.

NOTE:
PNP/PCI Setup is for technicians or advanced users only. You are not advised to change the default settings.

# n  INTEGRATED PERIPHERALS

When you select INTEGRATED PERIPHERALS from the main menu, the submenu as shown below appears that contains items for configuring the on-board I/O devices.

Onboard FDD Controller		: Enabled
Onboard Serial Port 1		: COM1/3F8
Onboard Serial Port 2		: COM2/2F8
Infra Red (IR) Function		: Disabled
IR Transfer Mode			: Half-Dup
Onboard Parallel Port		: 378/IRQ7
Onboard Parallel Mode		: SPP

INTEGRATED PERIPHERALS Menu

### Onboard FDD Controller

This item allows you to enable or disable the on-board floppy disk drive controller.

The available options are Enabled and Disabled. If you want to use a separate floppy disk drive controller board, select Disabled.

### Onboard Serial Port 1/2

This item defines the on-board serial port and sets its address.

The available options are COM1/3F8, COM2/2F8, COM3/3E8, COM4/2E8, and Disabled.

### Infra Red (IR) Function

This item sets the IR port standard depending on the device connected. The available options are ASKIR, HPSIR, and Disabled.

### IR Transfer Mode

This item sets the IR transfer mode.

The available options are Half-Dup (receiving or sending only) and Full-Dup (simultaneous receiving and sending.)

### Onboard Parallel Port

This item sets the address and interrupt channel for the on-board parallel port.

The available options are 3BCH/IRQ7, 378H/IRQ7, 278H/IRQ5, and Disabled.

### Onboard Parallel Mode

This item sets the on-board parallel port mode. Your system supports SPP (Standard Parallel Port), EPP (Enhanced Parallel Port) and ECP (Extended Capabilities Port) standards which turn the standard parallel port into a high speed bi-directional peripheral port. You can select the mode if the device connected to your parallel port supports it.

The available options are SPP, EPP, ECP, and ECP+EPP.

### ECP Mode Use DMA

This item appears on the screen If you selected ECP or ECP+EPP for the previous item. The available options are 1 and 3.

# n  LOAD BIOS DEFAULTS

The BIOS default values have been set to provide the minimum performance for the system.

If you select LOAD BIOS DEFAULTS from the main menu, BIOS default values will be loaded to the BIOS FEATURES SETUP, CHIPSET FEATURES SETUP, POWER MANAGEMENT SETUP, PNP/PCI CONFIGURATION and INTEGRATED PERIPHERALS menus.

# n  LOAD SETUP DEFAULTS

The SETUP default values have been set to assumedly provide maximum performance for the system.

If you select LOAD SETUP DEFAULTS from the main menu, BIOS default values will be loaded to the BIOS FEATURES SETUP, CHIPSET FEATURES SETUP, POWER MANAGEMENT SETUP, PNP/PCI CONFIGURATION and INTEGRATED PERIPHERALS menus.

# n  PASSWORD SETTING

The PASSWORD SETTING option works in conjunction with Security Option in the BIOS FEATURES SETUP menu to prevent unauthorized access to the system and SETUP program (See "Security Option" in this chapter.)

To enter or change the password, select the option from the main menu and follow the on-screen prompts. To disable the password, simply press Enter without typing any characters.

# n  IDE HDD AUTO DETECT

If your IDE hard disk supports the "auto-detect" function, the IDE HDD Auto Detect feature can automatically detect the parameters of the hard disk, including size, cylinders, number of heads, sectors per track, write precompensation, and the access mode.

NOTE:
By selecting the Primary/Secondary Master/Slave items from the STANDARD CMOS SETUP option, you can set the hard disk type manually.

The SETUP program will display all possible modes supported by the hard disk including Normal, LBA (Logical Block Addressing), and Large modes. See below for explanations.

1.	Normal mode

This is the generic access mode in which neither the BIOS nor the IDE controller will make any transformations during accessing.

The maximum number of cylinders, heads, and sectors for Normal mode are 1024, 16, and 63. If Normal mode is selected, the maximum accessible hard disk size will be 528MB even though its physical size may be greater. See the formula below:

|  | no. of cylinders | (1024) |
| --- | --- | --- |
| x | no. of head | (16) |
| x | no. of sectors | (63) |
| x | bytes per sector | (512) |
|  |  | 528MB |

2.	LBA mode

This is a new access mode for overcoming the 528MB bottleneck. The number of cylinders, heads, and sectors shown may not be the number physically contained in the hard disk.

During accessing, the IDE controller will transform the logical address described by sector, head, and cylinder number into its own physical address inside the hard disk.

The maximum size supported by LBA mode is 8.4GB (Gigabytes). See the formula below:

|  | no. of cylinders | (1024) |
| --- | --- | --- |
| x | no. of head | (255) |
| x | no. of sectors | (63) |
| x | bytes per sector | (512) |
|  |  | 8.4GB |

3.	Large mode

This is an extended access mode supported by the SETUP program for some IDE hard disks that contain more than 1024 cylinder without LBA support. Also for users who do not want LBA, the Large mode provides an alternative.

Example of Large mode:

| Cylinders | Heads | Sector | Sector |
| --- | --- | --- | --- |
| 1120 | 16 | 59 | Normal |
| 560 | 32 | 59 | Large |

BIOS tricks the operating system that the number of cylinders is less than 1024 by dividing it by 2. At the same time, the number of heads is multiplied by 2. A reverse transformation process will be made inside INT 13h in order to access the right hard disk address.

The maximum size supported by Large mode is 1GB. See the formula below:

|  | no. of cylinders | (1024) |
| --- | --- | --- |
| x | no. of head | (32) |
| x | no. of sectors | (63) |
| x | bytes per sector | (512) |
|  |  | 1GB |

NOTE:
To support LBA or Large mode of hard disks, the hard disk service routine uses INT 13h . If you are running under an operating system which replaces INT 13h, the system may fail to access a hard disk with LBA/Large mode selected.

# n  EXITING SETUP

To exit the SETUP program, you can choose SAVE & EXIT SETUP or EXIT WITHOUT SAVING from the main menu.