Chapter 4

memory configuration

This chapter tells you how to configure the system DRAM and cache SRAM. The figure below shows the DRAM and SRAM locations:

# n  System DRAM

NOTE:
To avoid memory problems, you are suggested to use 60ns EDO DRAM chips throughout the system.

## DRAM Configuration

### EDO/FP

| Bank 0 (SI1, SI2) | Bank 1 (SI3, SI4) | Total |
| --- | --- | --- |
| 1M x 32 | -- | 8MB |
| 1M x 32 | 1M x 32 | 16MB |
| 2M x 32 | -- | 16MB |
| 2M x 32 | 1M x 32 | 24MB |
| 2M x 32 | 2M x 32 | 32MB |
| 4M x 32 | -- | 32MB |
| 4M x 32 | 1M x 32 | 40MB |
| 4M x 32 | 2M x 32 | 48MB |
| 4M x 32 | 4M x 32 | 64MB |
| 8M x 32 | -- | 64MB |
| 8M x 32 | 1M x 32 | 72MB |
| 8M x 32 | 2M x 32 | 80MB |
| 8M x 32 | 4M x 32 | 96MB |
| 8M x 32 | 8M x 32 | 128MB |

### SDRAM/EDO (3.3V)

| SI6 | SI5 | Total |
| --- | --- | --- |
| 1M x 64 | -- | 8MB |
| 1M x 64 | 1M x 64 | 16MB |
| 2M x 64 | -- | 16MB |
| 2M x 64 | 1M x 64 | 24MB |
| 2M x 64 | 2M x 64 | 32MB |
| 4M x 64 | -- | 32MB |
| 4M x 64 | 1M x 64 | 40MB |
| 4M x 64 | 2M x 64 | 48MB |
| 4M x 64 | 4M x 64 | 64MB |
| 8M x 64 | -- | 64MB |
| 8M x 64 | 1M x 64 | 72MB |
| 8M x 64 | 2M x 64 | 80MB |
| 8M x 64 | 4M x 64 | 96MB |
| 8M x 64 | 8M x 64 | 128MB |

## DRAM Installation

NOTE:
Static electricity can destroy electronic devices. Whenever you handle any option outside of its protective packaging, first discharge any static electricity from your body by touching a protective grounding device or unpainted metal on the rear panel of the system unit.

CAUTION:
Do not install SIMM and DIMM at the same time.

### SIMM

1.	Locate the SIMM sockets (SI1~SI4) on the motherboard. (See the figure on p.4-1.)

2.	Align the SIMM's notched end with the socket's corre-
sponding end and firmly insert the SIMM into the socket at an angle. Maintaining this angle, insert the SIMM all the way into the slot. Then, push the SIMM towards the plastic clips to snap it into place.

### DIMM

1.	Locate the DIMM sockets (SI5, SI6) on the motherboard. (See the figure on p.4-1.)

2.	Align the DIMM module with the socket and firmly insert the DIMM into the socket. Then, push the plastic clips to snap it into place.

# n  Cache Memory

NOTE:
Static electricity can destroy electronic devices. Whenever you handle any option outside of its protective packaging, first discharge any static electricity from your body by touching a protective grounding device or unpainted metal on the rear panel of the system unit.

1.	Install 256KB/512KB module to the PB SRAM socket SI7. (See the figure on p.4-1.)

2.	Set JP3 and JP4 according to the cache size. (See P.3-3.)

# n  Memory Mapping

# n  Memory Address

| Hex Address | Device |
| --- | --- |
| 00000000 - 0009FFFF | basic memory |
| 000A0000 - 000BFFFF | video memory |
| 000C0000 - 000C7FFF | VGA BIOS |
| 000C8000 - 000DFFFF | reserved for ROM on I/O adapters |
| 000E0000 - 000FFFFF | system ROM BIOS or shadow RAM |

# n  I/O Mapping

| Hex Address | Device |
| --- | --- |
| 0000 - 001F | DMA controller 1 |
| 0020 - 003F | interrupt controller 1 and chipset programmable register |
| 0040 - 005F | timer |
| 0060 - 006F | keyboard controller |
| 0070 - 007F | real time clock, NMI mask |
| 0080 - 009F | DMA page register |
| 00A0 - 00BF | interrupt controller 2 |
| 00C0 - 00DF | DMA controller 2 |
| 00F0 | clear numeric coprocessor busy |
| 00F1 | reset numeric coprocessor |
| 00F8 - 00FF | numeric coprocessor |
| 0100 - 3FF | adapter I/O |